| # | Clarification Title | Clarification Description(Rev.00) | Conclusion (Rev.00) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description(Rev.01) | Conclusion (Rev.01) (VENDOR to Confirm & Consider in the Final Offers) |
| --- | --- | --- | --- | --- | --- |
| - GENERAL |  |  |  |  |  |
|  | Documents Priority | Vendor to confirm that in case of contradiction between this TCL and offer; vendor confirmation of compliance through this technical clarification is governed and shall be followed by vendor after order. In such case offer won’t be considered as basis of any claim after order. | Confirmed. |  | TJ Confirmed & Closed. |
|  | Deviation List | Bidder shall confirm his full compliance to meet all purchaser documents listed and attached to MR, code / standards listed in MR with ref. No. "BK-GCS-PEDCO-120-ME-MR-0002" as well as MR requirements. All deviations shall be listed in project deviation form, attach to this TCL, signed and stamped. Notes: 1-Any exception and deviation from the requirements of MR and the related specifications including referenced codes & standards shall be clearly declared by vendor in writing for purchaser’s approval at bid stage by using of the format included in “Letter Of Conformity” attached to material requisition. Raising no deviation to any single requirement means full compliance of vendor quotation to that requirement. Purchaser has no responsibility to discover deviations or alternatives scattered over the proposal in addition to what proclaimed explicitly in deviation list; hence such deviation (if any) is disregarded and shall not be considered for vendor claim in future. (Vendor to confirm) 2- Bidder shall confirm that all MR attachments listed in MR are available for her for review at bid stage. 3-Considering items 1 & 2, please note that any deviation at detail stage will not be accepted by purchaser. | 1-Confirmed. 2- Confirmed. 3-Confirmed. |  | 1- TJ Confirmed & Closed. 2- TJ Confirmed & Closed. 3- TJ Confirmed & Closed. |
|  | Documents Priority | Bidder to confirm his compliance with documents priority stated in MR. Any conflict between the documents shall be notified by bidder to purchaser and resolved by his agreement | Confirmed. |  | TJ Confirmed & Closed. |
|  | Requirement For Documents | Bidder to confirm that documents required after purchase order shall be submitted in accordance with “RFD Form” attached to the MR. | Confirmed. |  | TJ Confirmed & Closed. |
|  | Sub-Vendors | Bidder to confirm that Sub-Vendors shall be selected from Project’s approved AVL. | Confirmed. |  | TJ Confirmed & Closed. |
|  | Specification | Bidder to confirm that surface preparation and painting of all equipment material including valves , pipes , fittings and everything within the package are in accordance with the project painting specification attached to MR. | Confirmed. |  | TJ Confirmed & Closed. |
|  | Specification | Bidder to confirm that Packing, Marking, Preservation and Preparation for Shipment will be done in accordance to the project specification attached to the MR. | Confirmed. |  | TJ Confirmed & Closed. |
|  | Material Certificate | Bidder to confirm that for all pressure parts and those required special consideration or stainless steel as well as for supports the material certificate should be in accordance with EN 10204, 3.1 Material certificate for non-pressure parts shall be in accordance with EN 10204, 2.2. | Confirmed. |  | TJ Confirmed & Closed. |
|  | Scope | Bidder shall confirm the scope of work and supply are fully in accordance with MR and its attachment with following items: 1- The consequences of HAZOP and SIL study shall be in vendor scope of supply and work. 2- Participation in technical meeting whenever it is necessary by Purchaser/Client to close engineering document after order is in vendor scope 3- Lifting beams (Spreader beams) for erection with sling shackle and etc. shall be designed and supplied by vendor 4- N2 purging preservation with pressure regulator, valve & flexible hoses during transportation & 18 months of site storage based on requirements in MR shall be in vendor scope of supply / work. 5- All utility conditions shall be as per document "Process basis of Design Document; Doc. No: BK-GNRAL-PEDCO-000-PR-DB-0001". Moreover bidder shall consider utility noted condition as per project design data. If other utility condition is needed, whole required facilities is in vendor scope of work/supply. 6- Anchor bolts | 1- Confirmed. |  | 1- TJ Confirmed & Closed. 2- TJ Confirmed & Closed. 3- TJ Confirmed & Closed. 4- TJ Confirmed & Closed. 5- TJ Confirmed & Closed. 6- TJ Confirmed & Closed. |
|  | Spare Part | Vendor to supply all the specified spare parts in the below attached file (as minimum): | Please resend the attached document. |  | TJ Confirmed to follow Spare part list, Commissioning & Special tools which would be sent separately by Hirgan (as per attachment) & Closed. |
|  | New Item |  |  |  | Vendor confirmed to follow the MOP for sub vendor list. MOP will be sent as per attachment. Any cost impact of deviation from MOP list would be burden by vendor. |
| - PROCESS |  |  |  |  |  |
|  |  | PFD & H&M shall be provided | Confirmed, will be submitted after PO |  | TJ confirmed to provide PFD, H&MB by the next offer. (19 Feb) |
|  |  | Equipment list shall be provided | Confirmed, please refer to the attached file. |  | TJ confirmed to reflect changes in the related document. |
|  | 3. PROCESS DESIGN | , The maximum concentration of water in the dry gas at the outlet of the gas dehydration package calculated based on Dew Point equal to 5 °C (determined according to 10 degrees distance from minimum operating temperature) and the result is that 11.63 Ib/MMSCF in winter case and 11.43 Ib/MMSCF in summer case. | Confirmed |  | TJ confirmed to reflect changes in the related document. |
|  | 3.2. Process Description | Normal flare header back pressure is 0.5 bar gage .Therefore, add ejector if needed. | Confirmed |  | TJ confirmed & closed. |
|  |  | please revise base on last revision of design basis | Confirmed |  | TJ confirmed to reflect changes in the related document. |
|  |  | Please specified make up flow rate. | Confirmed, detail design will be submitted after PO. |  | TJ confirmed to reflect changes in the related document. |
|  | 2.5. Site Conditions & Location | Please revised base on last revision of project design basis. | Confirmed |  | TJ confirmed to reflect changes in the related document. |
|  | 2.4.2. Fuel Gas | Operating pressure on fuel gas header is 4.5 barg | Confirmed |  | TJ confirmed to reflect changes in the related document. |
|  | New item Native files |  |  |  | TJ confirmed to provide native process files like HYSYS,… |
|  | New item Optimization |  |  |  | TJ confirmed to provide technical reasons of any item removal or change in order to optimize package. |
|  | New item Analyzer |  |  |  | TJ confirmed that moisture analyzer is in the scope of work. |
| - MECHANIC |  |  |  |  |  |
|  | Document Requirement | Bidder to submit all required documents with tender as per “RFD Form” in the MR. For example below documents: 1-Marked up and Filled out MR scope of supply and works (It shall be signed and stamped) 2-GA DWG with outline dimensions and weights related to this project 3- A complete Equipment List 4-Noise Data 5-Utility Consumption and load List 6-Consummable material e.g. Glycol, Chemical and … 7-Spare Part List | 1. Confirmed, please refer to attached file 2. Confirmed, sample is added, details will be submitted after PO. 3. Confirmed, please refer to attached file. In detail will be submitted after PO. 4,5,6. Confirmed, will be submitted after PO 7. Confirmed, please refer to attached file. |  | 1-7. TJ confirmed & closed. |
|  | Material | Material shall be in accordance with ASME code not ASTM. ASME number and grade of all supplied material to be specified in next revision of offer. | Confirmed |  | TJ confirmed & closed. |
|  | Applied Standards | Applied Standards for equipment design, fabrication and test shall be listed. ASME Sec. VIII Div. 1, IPS (Iranian Petroleum Standards) & specifically IPS-G-ME-150 and ASCE 7-10 are applicable and Shall be added to the references. | Confirmed |  | TJ confirmed & closed. |
|  | Vendor’s Scope | Design and supplying Heat Tracing and insulation if required to be in the vendor’s scope. | Confirmed |  | TJ confirmed & closed. |
|  | Vendor’s Scope | All vents and drains for skid mounted equipment in the package to be valved, manifolded and to be routed to the edge of skid. | Confirmed. |  | TJ confirmed & closed. |
|  | Vendor’s Scope | Access ladder and platforms, stairway shall be studied, designed and supplied by vendor. Relevant detail drawings shall be submitted for approval in detail design stage. Gratings shall be hot dip galvanized. Please confirm. | Confirmed |  | TJ confirmed & closed. |
|  | Vendor’s Scope | All required facility for utility distribution in Battery Limit to be in the vendor’s scope of supply and work. Note: Each utility will be supplied in one tie-in and will be returned in one tie-out so any required accessories for distribution of these utilities in Battery Limit is in the vendor’s scope. | Confirmed |  | TJ confirmed & closed. |
|  | Vendor’s Scope | All required piping material including valve, fitting, inter connection piping, pipe supports and etc. within battery limit Note: All Piping and Tubing to be prefabricated and match marked and a relevant MTO and isometric DWG for installation to be provided by vendor. | Confirmed |  | TJ confirmed & closed. |
|  | Vendor’s Scope | Fuel skid shall be supplied completely shop prefabricated (piped, wired to local JB) and ready for site installation/operation. Please confirm. | Confirmed |  | TJ confirmed & closed. |
|  | Vendor’s Scope | All temporary items (blind flange, cover/cap, gasket, bolts/nuts, etc.) required for hydro testing at site shall be supplied by vendor. Please confirm. | Confirmed |  | TJ confirmed & closed. |
|  | Vendor’s Scope | All required injection packages which are required for proper operation of this dehydration package to be in the vendor’s scope of supply. Please confirm. Note: Please provide a list for them in next revision offer. | Confirmed |  | TJ confirmed & closed. |
|  | Vendor’s Scope | All required supports for equipment (such as legs, skirts and etc.) shall be provided by the vendor. | Confirmed |  | TJ confirmed & closed. |
|  | Vendor’s Scope | All installation material such as anchor bolts, templates, shims, leveling jack screws and leveling pads, sliding plate, PTFE sheet, roller, etc., as applicable to proposed package design, shall be included in vendor scope. Please confirm. | Confirmed |  | TJ confirmed & closed. |
|  | Vendor’s Scope | All heat treatment, including PWHT, post forming, solution annealing, stabilization heat treatment, etc. as required referenced specifications, referenced codes & standards (including material standards), and/or recommended by vendor for specified applications, shall be included in vendor scope and relevant procedure as well as certified report shall be provided. | Confirmed. |  | TJ confirmed & closed. |
|  | Technical Specification | Please submit technical specifications for the Burner (type, quantity, arrangement, type of ignition) | Confirmed |  | TJ confirmed & closed. |
|  | Technical Specification | MDMT for design of equipment shall be considered as per the project requirement. Please confirm. | Confirmed |  | TJ confirmed & closed. |
|  | Project Requirement | Please confirm this package is fully suitable for outdoor installation/operation without requiring any additional protection. | Confirmed |  | TJ confirmed & closed. |
|  | Project Requirement | The package shall be designed and constructed so that requires minimum site work (preparation and welding) while fulfills transportation limits. Please confirm. | Confirmed |  | TJ confirmed & closed. |
|  | Special Tools(new added) | All special tools required for erection, operation and maintenance of the package (including auxiliaries) shall be supplied by vendor. Please confirm. | Confirmed |  | TJ confirmed & closed. |
|  | Inspection and Test | Test procedures and acceptance criteria will be checked after order with project specifications and related standards. Vendor to confirm to follow project standard and spec. | Confirmed |  | TJ confirmed & closed. |
|  | Project Requirement | - Regard to the injection packages, please confirm to follow the following requirement: - Metering pumps shall be equipped with internal (for diaphragm liquid end) and external relief valves - Motor absorbed power for internal PSV setting with 10% accumulation shall be added in offer and shall be considered in selection of motor nameplate power. - In all metering pumps, suction and discharge shall be of double ball check valve - Bladder material shall be compatible to process fluid. - Damping peak to peak shall be less than 2.5%. - Pressure gauges shall be oil filled type in the package | Confirmed |  | 1-7. TJ confirmed & closed. |
|  | Documentation | Bidder to confirm that all native files due to all calculations and drawings shall be submitted for purchaser approval after order. | Confirmed |  | TJ confirmed & closed. |
|  | Documentation | Regarding technical offer, diameter, dimension and thickness of each equipment will be checked after receiving vendor calculation and data after order and comments in detail shall be followed without cost impact. | Confirmed |  | TJ confirmed & closed. |
|  | Documentation | Welding book (including WPS, PQR, weld map and NDT map) shall be prepared by vendor and submitted for purchaser approval (pressure parts, and structural). No welding shall be commenced before receiving approved welding book. Please confirm. | Confirmed |  | TJ confirmed & closed. |
|  | Documentation | All engineering documents to be referenced for site works (procedures, welding documents, detail drawings, test and inspection requirements, etc.) shall be provided by vendor as part of post order engineering documentation. Please confirm. | Confirmed |  | TJ confirmed & closed. |
|  | Documentation | Below item shall be added - NDT - Hydrostatic test report - PWHT report (if required by code) - VPIS - Manufacturing & assembly - Name plate - Painting procedure and sand blast - As built drawing - Final book Please modify your proposal. | Confirmed |  | TJ confirmed & closed. |
|  | Comments on Vendor’s Proposal and its attachments | Please refer to the comments on the offer. | Confirmed. |  | TJ confirmed & closed. |
|  | Guarantee | The guarantee period shall be eighteen (18) months from the date of delivery or twelve (12) months from the date of mechanical completion & start-up (if applicable), whichever occurs sooner. Under this warranty, all material, parts, and assemblies found to be defect during the guarantee period shall be replaced by vendor without any charge. Please confirm. | Confirmed. |  | TJ confirmed & closed. |
|  | Technical Point | Local stress analysis of shells/heads under external loads from piping system and attachments (platforms, pipe supports, davit, and lifting lug) as per WRC Bulletin 107, 297 or FE shall be included in strength calculation. And also, nozzle load shall be considered according to “Specification for allowable loads on static equipment nozzles” The original file of all analysis including all input data and results should be provided by Vendor. ASME Section VIII Div.2 should be used for local stress analysis. Please note that wherever WRC is not applicable FEA shall be done to evaluate local stresses. The original file (native file) of all analysis including all input data and results should be provided by Vendor. Please confirm. | Confirmed. |  | TJ confirmed & closed. |
|  |  | For wall thickness greater than 50mm, integrally reinforced nozzle shall be used . For below cases, head type should be hemispherical: -High pressure vessels Shell thickness ≥ 70 mm | Confirmed. |  | TJ confirmed & closed. |
|  |  | Based on the Material Requisition, Bidder shall submit “Specifications / Data Sheets for Package, Equipment, Vessels, Tanks, Pumps, Fans, Blowers, Burners, etc.” | Confirmed. |  | TJ confirmed & closed. |
|  |  | The pumps shall have suction strainers. Mesh size and flow area shall be in accordance with the pump manufacturer’s recommendation. Pulsation dampers shall be provided and installed both at suction and discharge. | Confirmed. |  | TJ confirmed & closed. |
|  |  | Package SUPPLIER shall calculate the NPSHa available at pump suction nozzle as per the details of their design. The purchaser shall reconfirm the NPSHA once the piping has been designed. Acceleration losses in the suction shall be minimized through use of the straightest, shortest suction line possible. | Confirmed. |  | TJ confirmed & closed. |
|  |  | A performance and mechanical running test shall be performed at the pump manufacturer’s shop. SUPPLIER shall submit specific test agendas (procedures) to the PURCHASER for review and approval. All pertinent test conditions, procedures and acceptance criteria should be clearly mentioned and should be in conformance with the purchase order and pertinent test codes. It is critical that all aspects of the test be mutually agreed upon in writing in the form of a final test agenda. This final test agenda must be completed prior to the start of any testing program. | Confirmed. |  | TJ confirmed & closed. |
|  |  | Version for all Software to be considered in accordance with code edition. For PV-Elite: PVElite/CodeCalc/ Nozzle Pro 2019 should be considered for preliminary general mechanical calculation. ASME Material Data Base shall be set to 2010. Version of PVElite can be discussed in K.O.M. For AutoCad: Native files of drawings to be compatible with AutoCad 2014. For HTRI :7 For SAP : 20.10 In any case, verification of software is Vendor’s responsibility. | Confirmed. |  | TJ confirmed & closed. |
|  |  | For nozzle necks made by plate full radiography shall be considered. Please confirm. | Confirmed. |  | TJ confirmed & closed. |
|  |  | Trial Assembly of all removable Internals (If supplied by Vendor) is vessel vendor’s scope of work, please confirm. | Confirmed. |  | TJ confirmed & closed. |
|  |  | All documentation shall be in editable electronic format (scanned pages are not acceptable). Vendor to Confirm. | Confirmed. |  | TJ confirmed & closed. |
|  |  | All changes and modifications on VENDOR’s documents shall have the affected areas highlighted by 'clouding' or 'side-lining' and the use of a triangle adjacent to the alterations denoting the revision status. Previous revision highlighting shall be removed at subsequent issues. PURCHASER will take no action on any change without such marking. And VENDOR shall be responsible for any consequence caused by missing such marking. | Confirmed. |  | TJ confirmed & closed. |
|  |  | Vendor to confirm “welding at site” is not permitted. | Confirmed. |  | TJ confirmed & closed. |
|  | Site Tests | Site Acceptance Test (SAT) and 72 hours performance test of whole the package at site to be done by vendor. Negative tolerances for dehydration level (the maximum water concentration at the outlet of the package shall not be more than the specified value in the relevant duty specification), flowrate and pressure of the outlet gas of the package will not be accepted. | Confirmed. |  | TJ confirmed & closed. |
|  | Tolerances on Utility Consumption | Please specify the utility and power consumption for this package and specify the maximum tolerances for them at this stage. These tolerances shall be followed during performance test of the package. | Confirmed. |  | TJ confirmed & closed. |
|  | Vendor’s Scope | All required steel structure for the package to be prefabricated and supplied by vendor. Also a detail DWG and procedure for installation of them to be provided by vendor | Confirmed. | . | TJ confirmed & closed. |
| - PIPING & MATERIAL |  |  |  |  |  |
|  | Package P&ID | Vendor P&ID of Package considering tie-ins in it and sizes shall be submitted. | Confirmed. |  | TJ confirmed to send preliminary tie-ins at revised offer. |
|  | Package 3D Model | PDMS format (version 12.1) of 3D Model of Package shall be submitted by vendor. | Confirmed. |  | TJ confirmed & closed. |
|  | General Material | Specification For Material Requirements in Sour service” Document No:. BK-GNRAL-PEDCO-000-PI-SP-0008 shall be met where required. Hence: - Material shall be Killed, Normalized and fine grain practiced. - All carbon steel welds shall be post weld heat treated even if not required by the applicable design/construction codes. - Plates and welded pips shall be HIC tested as per par. 6.4 of this specification. HIC testing also shall be performed on carbon steel materials clad with corrosion resistance alloys. | Confirmed. |  | TJ confirmed & closed. TJ confirmed to reflect changes in the related document. |
|  | General Material | Removable internals and internal bolt/nuts shall be S.S316 (L). | Confirmed. |  | TJ confirmed & closed. TJ confirmed to reflect changes in the related document. |
|  | Painting/Coating | All C.S equipment shall be painted in accordance with project specification. | Confirmed. |  | TJ confirmed & closed. TJ confirmed to reflect changes in the related document. |
|  | Insulation | Insulation material and thickness shall be in accordance project specification. | Confirmed. |  | TJ confirmed & closed. TJ confirmed to reflect changes in the related document. |
|  | Other Equipment | Material for other items such as still column and filters shall be specified. | Confirmed. |  | TJ confirmed & closed. TJ confirmed to reflect changes in the related document. |
|  | Corrosion Allowances | Typical C.A such as 3 mm for sour services is not accepted. Vendor shall provide corrosion study and material selection report after P.O. Corrosion Allowances shall be specified for material in your offer. | Confirmed. |  | TJ confirmed & closed. TJ confirmed to reflect changes in the related document. |
|  | Lean Glycol Air Cooler | Material shall be S.S316L. | Confirmed. |  | TJ confirmed & closed. TJ confirmed to reflect changes in the related document. |
|  | Piping Stress Analysis (Displacements at the Tie-in points) | Tie-in points shall be designed so that movements and rotation tend to zero. (By VENDOR) | Confirmed. |  | TJ confirmed & closed. TJ confirmed to reflect changes in the related document. |
|  | Piping Stress Analysis (Allowable loads at the Tie-in points) | VENDOR shall be specified maximum allowable forces and moments at the tie-in points for review and purchaser approval. In case of unavoidable over loaded at the tie-in points, then the loads should be submitted to the VENDOR for consideration and approval. | Confirmed. |  | TJ confirmed & closed. TJ confirmed to reflect changes in the related document. |
| - INSTRUMENT & CONTROL |  |  |  |  |  |
|  | Control System Requirements | Packaged control type B is requested as per BK-GNRAL-PEDCO-000-IN-SP-0004. Unit shall be fully remote controlled (monitoring and control functions) by the DCS and ESD systems. There is no VENDOR supplied control cabinet in control Room for these package LCP is required for main monitor/control commands such as burner control command. BMS logic shall be provided by vendor to be implemented in DCS/ESD. The package is equipped with its instruments, wired by means of appropriate cables and cable routing. The cables are connected to the junction boxes located at the skid edge battery limits or at edge of base plate. All controls are incorporated into the DCS/ESD by the IPCS Integrator. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed. |  | TJ shall prepare control block diagram for Hirgan review and comment. Estimated I/O count for ESD/DCS to be added in vendor proposal. LCP location to be specified in GA drawing. BMS logic diagram will be provided after PO . Since package performance guarantee is in vendor scope , required man hour for SAT and Commissioning for system interconnections to be considered. |
|  | Panel Requirements | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0002, 0003 for panels mounted in control building and field panels and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed. |  | TJ confirmed for field panels & closed. |
|  | Instrumentation | Regarding supplying instrument on datasheet), bidder is requested to consider all instruments mentioned on project datasheet. VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0001 for instrumentation devices and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed. |  | TJ confirmed & closed. |
|  | Instrumentation Valves | VENDOR shall follow below documents for valves and other documents as mentioned in these documents. - BK-GNRAL-PEDCO-000-IN-SP-0005 - BK-GNRAL-PEDCO-000-IN-SP-0006 - BK-GNRAL-PEDCO-000-IN-SP-0007 VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed. |  | TJ confirmed & closed. |
|  | Connection with MCC | - if MCC is supplied by VENDOR all interfaces shall be supplied by VENDOR. Below signals are typically defined in this project to be provided for electrical interfaces: - Start command (2 sec high pulse) - Stop command (2 sec high pulse) - Trip command (low signal for trip) - Run/Stop Status (high signal for run status) - Local/Remote status (high signal for local status) - Tripped on Fault (high signal for fault status) - Available for remote control (high signal for available status) | Confirmed. |  | MCC is scope out. |
|  | Cable | - VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0010 for cables and all other details mentioned in this document. (All cable instruments for internal package and for connection to local control panel of package shall be supplied by Vendor.) VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed. |  | TJ confirmed & closed. |
|  | Terminals | All terminals shall be screw type. VENDOR shall note that each core/pair/etc. of cable shall be wired to dedicated terminal | Confirmed. |  | TJ confirmed & closed. |
|  | Panel Dimension | VENDOR shall specify the estimated number and dimension of panels. | Confirmed. |  | TJ confirmed for field panels & closed. |
|  | Required Power | VENDOR shall specify the estimated power required (non-UPS and UPS) | Confirmed. |  | UPS Power consumption for LCP or any other devices to be specified by vendor (if any) |
|  | Special Tools | VENDOR shall supply all special tools for compressor and axillary devices. This is included the special cables (not mentioned in BK-GNRAL-PEDCO-000-IN-SP-0010), MODBUS sockets, software and etc. The list shall be provided separately. | Confirmed. |  | TJ confirmed & closed. |
|  | Software | VENDOR shall supply all the software (including the passwords, licenses and program sources). The programs shall be supplied with all tags, descriptions and etc. The software programs shall be installed on engineering laptops and also supplied on CD/DVD. | Confirmed. |  | There is no software. TJ confirmed & closed. |
|  | Required Doc. | Vendor is requested to prepare P&ID diagram, showing all instrumentation and control loops for package battery limit. | Confirmed. |  | Dedicated symbols for LCP, DCS and ESD with interconnecting signals shall be specified in P&ID. |
|  | Required Doc. | All required sequence logic, control loop safety protection and installation which are necessary for package performance shall be shown on vendor P&ID and prepared by vendor. | Confirmed. |  | Refer to item No.1 |
|  | Required Certificate | Required certificates for instrumentation, as minimum -certificate of origin, Material Certificates, Test/Calibration Certificates, Certificate of Conformance shall be added to technical proposal. (prepared by Vendor after P.O.) | Confirmed. |  | TJ confirmed & closed. |
|  | Scope of Supply | Vendor to confirm to supply below items for submitted package (Type B): -Accessories (impulse lines , fittings ,labels, local routings , cables glands , …) -Utility distribution inside the battery limit, -Documentation. -All computing hardware and software, (if required). -All instrument fully installed, piped & wired on skid, -All other instrument equipment, including but not limited to panels, (Local Panel when required), -All Junction Boxes, (Terminals, plugs, etc.), Cables, inside the skid battery limit and special cables, -Cable trays and their accessories (labels, supports, etc. ), -All piping and tubing. Please confirm. | Confirmed. |  | Refer to item No. |
|  | Required proposal | As minimum vendor document list, equipment list, instrument MTO, Data sheet, to be added to the proposal. | Confirmed. |  | Vendor to complete proposal. |
|  | Spare Part | Commissioning / Spare part list for instrumentation and control panels to be provided. | Confirmed. |  | Vendor to complete proposal as per MR. procedure.. |
|  | Catalog | Local control panel specification or data sheet/catalogue to be added to vendor proposal. | Confirmed. |  | Vendor to complete proposal. |
| - SAFETY |  |  |  |  |  |
|  | Fire & Safety Requirements | Vendor shall confirm: - Max. Sound Pressure level shall be 85 dBA for normal operation. For temporary operation (such as PSV) IPS-E-SF-900 shall be followed. - For HAC, mainly Zone 2, IIB, T3 shall be considered. For any pumps or equipment located inside the pit or sump, and 1.5 m around each vent (if any) Zone 1 shall be considered. - SIL requirement is subject to change and after SIL study will be finalized. - Vendor are responsible to attend the HAZOP & SIL meetings related to mentioned package and follow the HAZOP / SIL study recommendations without cost claim. - In addition to stairway, a ladder shall be located at the opposite side of the platform. | Confirmed. |  | TJ confirmed & closed. |
| - ELECTRICAL |  |  |  |  |  |
| - 1 | Vendor Scope of Work | Electrical system scope of work shall be clarified by vendor & any deviation shall be reported. | Confirmed. |  | TJ confirmed & closed. |
| - 2 | Vendor Scope of Work | All documents related to electrical equipment should be submitted by vendor such as preliminary load list, motor data sheet, motor outline drawing & etc. | Confirmed. |  | TJ confirmed & closed. |
|  | Vendor Scope of Work | Vendor list & sub vendor list of manufacture shall be according to project AVL. | Confirmed. |  | TJ confirmed & closed. |
|  | Specification | All electrical equipment shall comply with electrical references & standard that mentioned on material requisition. (IPS-M-EL-131 (2) ) & IPS-E-EL-100(1) | Confirmed. |  | TJ confirmed & closed. |
|  | Specification | Motor shall be manufactured & tested in accordance with the latest applicable sections of the IEC standards. Project specification shall be applied as well. | Confirmed. |  | TJ confirmed & closed. |
|  | Specification | Environmental criteria for design of electrical systems that shall be considered by vendor (According to Electrical System Design Criteria) | Confirmed. |  | TJ confirmed & closed. |
|  | Specification | Hazard Area classification, Gas group and temperature class of motor shall be at least Exd, IIB, T3, with protection degree of IP54 for motor enclosure and IP55 for terminal box. | Confirmed. |  | TJ confirmed & closed. |
|  | Vendor Scope of Work | Local control station/panel for electrical equipment should be supply by vendor. It shall be according to “BK-GCS-PEDCO-120-EL-DT-0006” | Confirmed. |  | TJ confirmed & closed. |
|  | Vendor Scope of Work | All accessories related to cable entry/termination in electrical equipment (motors, JBs, etc.)in the package such as cable glands & etc. shall be supplied by vendor. | Confirmed. |  | TJ confirmed & closed. |
|  | Load List | Electrical preliminary load list of motor include Mechanical power (BHP), Rated power, efficiency, power factor and rated current shall be specified in the proposal. | Confirmed. |  | TJ confirmed & closed. |
|  | Load List | API factor, effect of ambient condition especially de-rate factor for altitude shall be considered for specifying rated power of motor and procedure of obtaining rated power of motor from BHP shall be cleared in the proposal. | Confirmed. |  | TJ confirmed & closed. |
|  | Data Sheet | Insulation class shall be F and temperature rise shall be B. | Confirmed. |  | TJ confirmed & closed. |
|  | Spare Part | A complete set of spare parts for commissioning shall be supplied for each motor. | Confirmed. |  | TJ confirmed & closed. |
|  | Vendor Scope of Work | Supplier shall design the routing of the power & control cable inside package. | Confirmed. |  | TJ confirmed & closed. |
|  | Vendor Scope of Work | Supply of cables between Junction boxes and Package control panel is in vendor scope of work. In other word, All cabling listed in “Customer cable table” shall be supplied and connected by customer/contractor. Cables shall be according to “BK-GNRAL-PEDCO-000-EL-SP-0014”. | Confirmed. |  | TJ confirmed & closed. |
|  | Vendor Scope of Work | If AC UPS or DC UPS & batteries are required in the field for package, vendor shall provide all necessary documents (such as calculation, list etc.) & supply required equipment according to purchaser standards. | Confirmed.. |  | TJ confirmed & closed. |
|  | Specification | Glands shall be of threaded type or fitted in equally strong way | Confirmed. |  | TJ confirmed & closed. |
| - CIVIL & STRUCTURE |  |  |  |  |  |
|  |  | Based on STD2800 this site is located at High Risk Area which is named Seismic Zone 2. also according to ASCE 7 (2010) Seismic Zone is equal to Zone 3 with Seismic Importance Factor 1.25 . Besides according to INBC-Part 6 amount of Wind Speed shall be considered 120 Km/hr. | Confirmed. |  | TJ confirmed & closed. |
|  |  | The technical specifications of The anchor bolts are outside the scope of the seller’s work. But seller’s must be followed the standard drawing of anchor bolt to the number “BK-GNRAL-PEDCO-000-ST-DW-0002”. | Confirmed. |  | TJ confirmed & closed. |
|  |  | Dimensional specifications of base plates & foundations are inside the scope of the seller’s work. | Confirmed. |  | TJ confirmed & closed. |
|  |  | Determining the center of gravity of the equipment to control the overturning are in the scope of the seller’s work. | Confirmed. |  | TJ confirmed & closed. |
|  |  | The following codes & standards should be considered in the civil & structure section: AWS , ASCE7-10 , INBC6 | Confirmed. |  | TJ confirmed & closed. |
|  |  | Structure design and related setting bolts are inside the scope of the seller’s work. | Confirmed. |  | TJ confirmed & closed. |