| # | Clarification Title | Clarification Description | Conclusion (VENDOR to Confirm & Consider in the Final Offers) |
| --- | --- | --- | --- |
| GENERAL |  |  |  |
|  |  | The Bid shall be in total compliance with Material Requisition for PIG TRAPS and referenced project specifications, codes and standards unless specific deviations are clearly identified by the vendor. |  |
|  |  | Vendor is sole responsible to consider this clarification and modify his offer based on this TCL. All modifications which need to be considered on the next revision of offer based on this clarification should be clearly marked with revision and cloud marks. All consequences due to changes without cloud mark are in vendor’s responsibility. Anyway, this clarification has priority over the offer. |  |
|  |  | Bidder to Confirm that scope of work and supply are in full compliance with Material Requisition (MR) section 4.1 and 4.2 and its attachments and relevant codes and standards mentioned on MR and specifications. Vendor shall mention any deviation / alternative compared with requirements of MR and its attachments on the signed and stamped Deviation Form with sufficient technical reasons. PURCHASER has no responsibility to discover any deviations entered in Vendor’s proposal. Deviations which are not listed in the Deviation Form are not valid and in case of order, no deviation or alternative would be acceptable except the ones specified on the mentioned form and finalized before order at clarification stage. |  |
|  |  | The Vendor shall have overall responsibility for the coordination with his Sub-Vendors, and for the proper execution of any accessory item assembly, testing, delivery and any other service defined herein. |  |
|  |  | Regarding Sub-Vendors/Sub-Suppliers, Vendor shall note that Contractor / Purchaser have the right to reject Sub-Vendors/Sub-Suppliers if his capability is judged poor or not proven. |  |
|  |  | Purchaser’s approval shall in no way be construed as relieving Vendor of any of his obligation, responsibilities or liabilities under the Contract and/or applicable Codes and Standards. |  |
|  |  | Transportation to delivery point (site) shall be considered as optional price in vendor’s proposal. |  |
|  |  | Inhibition materials against internal corrosion for transportation and longtime storage at stockyard is in vendor scope of supply. Requirements of section 12 of Project Specification for Pig Traps (BK-PPL-PEDCO-320-ME-SP-0001) shall be followed by vendor. Bidder to Confirm. |  |
|  |  | All commercial issues which are included in technical offer will be considered for information only. Such points should be specified on commercial offer by Bidder |  |
|  |  | In case of order, Purchaser would not review documents sent by Vendor before receiving Vendor Print Index and Schedule. Vendor Print Index and Schedule (VPIS) shall be prepared as first document after PO. All documents shall be issued after getting approval of this mentioned document. |  |
|  |  | In case of order, vendor should provide his Inspection and Test Plan based on requirements specified on MR and its attachments and also Vendor’s experiences and send for approval. Purchaser’s ITP and relevant specifications attached to MR are minimum requirement and vendor should consider them as a base and provide comprehensive ITP. In case of order, Vendor should provide complete NDE Map based on requirements specified on MR and its attachments and also vendor’s experiences and send it for approval. Approved Vendor I carrying out all required Inspection and Test.TP and approved Vendor Procedures/NDE Map will be the base for |  |
|  |  | Providing required facilities for Inspection and Test shall be in Vendor’s scope of work. |  |
|  |  | Vendor shall unconditionally guarantee the materials and workmanship on all material and/or services for a period of 12 months after successful start-up at the Purchaser’s Production Site or 18 months after the date of delivery whichever comes earlier. Bidder to confirm. |  |
|  |  | It is the responsibility of the vendor to ensure that there is no inconsistency in proposal and related attachments. So, any discrepancy between Vendor proposal attachments shall be removed. |  |
|  |  | Bidder to fill out the attached TBA (Technical Bid Analysis) and submit along with his revised offer. |  |
|  |  | Painting (Internal and external) shall be according to the Project 'Specification for Painting.', Doc. No. : BK-GNRAL-PEDCO-000-PI-SP-0006. |  |
|  |  | Comments on the Technical offers to be implemented for next revision. |  |
|  |  | Vendor shall limit the NO. of sub vendor to 3 and sub vendor List shall be accepted by MOP. |  |
| PROCESS |  |  |  |
|  |  | PL-3201 : Design temperature to be revised 5/85 °C, Design Pressure: 62Barg,Operating temperature to be revised 57.74 /58.3 °C, Operating pressure : 49.40 / 50.90 Barg |  |
|  |  | PR-3201 : Design temperature to be revised 5/85 °C, Design Pressure: 62Barg,Operating temperature to be revised 15 / 31.3°C, Operating pressure : 40 Barg |  |
| MECHANIC |  |  |  |
|  |  | Modifications due to HAZOP are implementing on mechanical datasheets of all items. It is mandatory that Bidder consider all requirements the mechanical datasheets in his revised offer. Bidder to confirm. |  |
|  |  | Design code for Pig Launcher &Pig Receiver should be ASME SECTION VIII , ASME B31.4 |  |
|  |  | Version for all Software to be considered in accordance with code edition. For PV-Elite: PVElite 2019 should be considered for general mechanical calculation. ASME Material Data Base shall be set to 2010. Version of PVElite can be discussed in K.O.M. For Nozzle Pro: Nozzle Pro V15 (PRG2019) For AutoCad: Native files of drawings to be compatible with AutoCad 2014. |  |
|  |  | Table of Material Requisition section 4.1 and 4.2 (including “PIG TRAPS”) shall be followed by vendor and shall be completely specified in vendor proposal. Vendor shall mark (X) for each item when accepting Purchaser’s requirements and shall send together with his proposal for approval. If vendor does not complete this table, it is assumed that all items are accepted by Vendor and will be supplied by him. |  |
|  |  | The final length of the pig trap shall be such that to accommodate an intelligent pig |  |
|  |  | All requirements for using intelligent pig shall be considered by vendor |  |
|  |  | For traps, the barrels are sloped (1:100) down toward the pipeline. |  |
|  |  | Spare parts requirements shall be based on Material Requisition of PIG TRAPS. Also “Specification for Spare Parts Requirements”, E&C-QC-SP-1 shall be followed by vendor. These be considered as minimum requirements and do not relieve Vendor’s responsibility to ensure the adequacy of spares for the Start-Up and Commissioning. |  |
|  |  | Based on Material Requisition, bidder shall submit following document along with bid as minimum: • Sub-Vendor List • Manufacturing schedule • Vendor`s Reference list • Quality Assurance Plan • Signed Deviation List (or No Deviation form) • Inspection and Test Plan • Special tools for erection and maintenance (if any) • Copy of Quality system certificate(s) (ISO 9001/2 or 3, …) • Spare Part list for start-up and commissioning as per Material Requisition and project specification • Two Years Spare parts list • Marked table of MR Sec. 8.1 and 8.2 • List of vendor’s production machines and devices or Bidder’s catalogues. Bidder to Confirm and submit. |  |
|  |  | Vendor’s Reference list shall include below information at least: - Project name - Client name - Year built - Equipment type - Material - Design Condition - Dimensions / Thickness - Fabricated weight - Design Code |  |
|  |  | • Seismic parameters updated as below: Code: ASCE 7-10 Importance Factor: acc. to datasheet Site Class: D Ss = 0.1.25 S1 = 0.58 Fa = 1 Fv =1.5 SDS=0.83 SD1=0.58 R, z/h and ap: acc. to datasheet • Wind Requirements Code: ASCE 7-10 33% wind shall be applied for Hydrotest. Design wind speed: 110km/h @10m Wind Exposure: C Wind Imp. Factor: 1.15 |  |
|  |  | TRAPS shall be supplied with all required accessories such as internal parts, nozzles, supports, lifting devices, name plates, earth lugs and so on. All process internals such as packing, mist eliminator, distributor, weir plate, baffles and so on are included in the Vendor's scope of supply. |  |
|  |  | Any attachments which are directly to be welded internally or externally to the TRAPS shall be made of the same material grade used for the shell or heads as appropriate. |  |
|  |  | In case of order, the minimum documents which are listed in the table of Attachment 1 of MR shall be submitted by the Vendor. In addition, if deemed necessary, more specific documents may be required for completion of job and shall be provided by Vendor. Bidder to Confirm. |  |
|  |  | All documentation shall be fully searchable and unlocked (scanned pages are not acceptable). Bidder to Confirm. |  |
|  |  | The Vendor documents, once have been revised, shall clearly identify all changes respect to latest issue, In particular: Text Documents: • Changed Line: Triangle holding the revision number in the left margin and revised line will be bold and underlined. • All changed pages shall be clearly marked in revision page (the page after cover page of documents). Drawings: • Clouded for Insertions and Deletions plus a Triangle holding the revision number. Purchaser will take no action on any change without such marking. Vendor shall be responsible for any consequence caused by missing such marking. |  |
|  |  | Vendor shall be legally responsible for mechanical design and stability/integrity of the equipment and shall carry out all calculations and check to as certain that the wall thickness of all parts (Pressure and non-pressure) complies with the requirements of codes, MR and specifications attached with MR. The thicknesses specified in the MR and attachments shall be considered as minimum specified. In addition, Vendor shall include in his design suitable additional margin required for machining / thinning based on the type of operation. |  |
|  |  | The thickness after forming of any pressure retaining parts shall not be less than the calculated thickness (Only positive tolerances on plate thickness are acceptable.) |  |
|  |  | Local stress analysis of shells/heads under external loads from piping system and attachments (platforms, pipe supports, davit, and lifting lug) a All nozzle loads shall not be less than specified values in a "appendix n" of specification for pressure vessels Doc No.: BK-GNRAL-PEDCO-000-ME-SP-0001". As per WRC Bulletin 107, 297 or FE shall be included in strength calculation. And also, nozzle load shall be considered according to “The original file of all analysis including all input data and results should be provided by Vendor. ASME Section VIII Div.2 (if any) should be used for local stress analysis. Allowable stresses based on ASME Section VIII Div. 1 shall be considered. |  |
|  |  | Selecting and design of lifting devices shall be performed by vendor based on project standard drawing and all other relevant project specifications. Finalizing the type of lifting device is based on purchaser approval with no additional cost impact. |  |
|  |  | Deliverable documents of vendor and his sub-vendor(s) shall be submitted with native files for all revisions. All native e-files should be submitted in original format (e.g., AutoCAD, PV-Elite, Nozzle Pro., Word, Excel, etc.) along with PDF file. Bidder to Confirm. |  |
|  |  | For raw material of each TRAP in sour service, Certificate shall be supplied by the Vendor assuring that components and manufacturing techniques meet the requirements of NACE MR 0175/ISO 15156. Material shall be HIC and SSC resistant. Specification For Material Requirements in Sour service Document No. BK-GNRAL-PEDCO-000-PI-SP-0008.PAINT All requirement specified thereunder, including but not limited to the followings, shall be met: • Hardness test • PWHT • HIC Test • Impact Test • 100% RT |  |
|  |  | All pressure parts and retaining pressure parts for TRAPS in sour service shall fulfill impact test requirements of ASME Sec.VIII Div.1 UG-84 rules. |  |
|  |  | Guarantee shall include mechanical guarantee as per data sheet and specification. Bidder to Confirm. |  |
|  |  | All requirements including but not limited to the followings shall be met in design of nozzles: 1- All nozzles shall be set-in 2- Thickness Basis shall be selected “minimum” 3- Diameter basis for nozzle calculation shall be “Outside Diameter” for nozzles made from pipe and plate and self-reinforced nozzles. For LWN nozzles, diameter basis shall be “Inside Diameter”. 4- Nozzle design pressure shall be equal to “MAWP + Static Head to Element Bottom. 5- Project standard drawing shall be followed for selecting nozzle neck schedule (thickness), reinforcement pad thickness and outside diameter and etc. |  |
|  |  | Providing Standard cap for butt welded nozzles with size below 16” and elliptical head for such nozzles larger than 16” is in vendor’s scope of supply. Mentioned cap/head shall be welded to the end of nozzle neck and used in hydro-test and shipping. Vendor shall consider 50 mm extra projection for mentioned nozzles for welding of attached cap/head. Extra length of nozzle will be cut off in site. Related calculation in hydrostatic test pressure condition shall be performed by vendor and submitted for purchaser review and approval. |  |
|  |  | All flanges shall be in accordance with ASME B16.5 for 24" or less and ASME B16.47 Series "B" for larger. |  |
| PIPING & MATERIAL |  |  |  |
|  | Equipment G.A Drawing | G.A Drawing (Detail Drawing) of Equipment shall be submitted by vendor after PO. |  |
|  | Piping Stress Analysis | VENDOR shall specify maximum allowable forces and moments at the nozzles for review and purchaser approval. In case of unavoidable over loaded at the nozzles, then the loads should be submitted to the VENDOR for consideration and approval. |  |
|  | Material | NDT shall be as per code. |  |
|  |  | PSL2 shall be mentioned in all API 5L materials. |  |
|  |  | Saddle Wear Plate shall be A516 Gr. 70 and others should be A283 Gr. C. |  |
|  |  | Gasket shall be S.S. 316, Spiral Wound Graphite Filled. |  |
|  |  | Internal & External Coating: Acc. To the Project 'Specification for Painting', Doc. No. : BK-GNRAL-PEDCO-000-PI-SP-0006. |  |
| INSTRUMENT & CONTROL |  |  |  |
|  |  | Suitable door switch /Status indicator flap to be considered by vendor, showing door status by signal or flap. ( 24VDC digital switch for pig launcher and flap indicator switch for pig receiver is required to be provided by vendor |  |
| STRUCTURE |  |  |  |
|  |  | Based on STD2800 this site is located at High Risk Area which is named Seismic Zone 2. also according to ASCE 7 (2010) Seismic Zone is equal to Zone 3 with Seismic Importance Factor 1.25 . Besides according to INBC-Part 6 amount of Wind Speed shall be considered 120 Km/hr. |  |
|  |  | The technical specifications of The anchor bolts are outside the scope of the seller’s work. But seller’s must be followed the standard drawing of anchor bolt to the number “BK-GNRAL-PEDCO-000-ST-DW-0002”. |  |
|  |  | Dimensional specifications of base plates & foundations are inside the scope of the seller’s work. |  |
|  |  | Determining the center of gravity of the equipment to control the overturning are in the scope of the seller’s work. |  |
|  |  | The following codes & standards should be considered in the civil & structure section: AWS , ASCE7-10 , INBC6. |  |