| # | Clarification Title | Clarification Description(Rev.00) | Conclusion (Rev.00) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description(Rev.01) | Conclusion (Rev.01) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description(Rev.02) | Conclusion (Rev.02) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description(Rev.03) | Conclusion (Rev.03) (VENDOR to Confirm & Consider in the Final Offers) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GENERAL |  |  |  |  |  |  |  |  |  |
| 1. | Documents Priority | Vendor to confirm that in case of contradiction between this TCL and offer; vendor confirmation of compliance through this technical clarification is governed and shall be followed by vendor after order. In such case offer won’t be considered as basis of any claim after order. | Confirmed | Noted | Closed |  |  |  |  |
| 2. | Deviation List | Bidder shall confirm his full compliance to meet all purchaser documents listed and attached to MR, code / standards listed in MR with ref. No. "BK-GCS-PEDCO-120-ME-MR-0001" as well as MR requirements. All deviations shall be listed in project deviation form, attach to this TCL, signed and stamped. Notes: 1-Any exception and deviation from the requirements of MR and the related specifications including referenced codes & standards shall be clearly declared by vendor in writing for purchaser’s approval at bid stage by using of the format included in “Letter Of Conformity” attached to material requisition. Raising no deviation to any single requirement means full compliance of vendor quotation to that requirement. Purchaser has no responsibility to discover deviations or alternatives scattered over the proposal in addition to what proclaimed explicitly in deviation list; hence such deviation (if any) is disregarded and shall not be considered for vendor claim in future. (Vendor to confirm) 2- Bidder shall confirm that all MR attachments listed in MR are available for her for review at bid stage. 3-Considering items 1 & 2, please note that any deviation at detail stage will not be accepted by purchaser. 4- Please find attached purchaser comments / clarification due to deviations raised by bidder | Confirmed | Noted | Closed |  |  |  |  |
| 3. | Documents Priority | Bidder to confirm his compliance with documents priority stated in MR. Any conflict between the documents shall be notified by bidder to purchaser and resolved by his agreement | Confirmed | Noted | Closed |  |  |  |  |
| 4. | Requirement For Documents | Bidder to confirm that documents required after purchase order shall be submitted in accordance with “RFD Form” attached to the MR. | Confirmed | Noted | Closed |  |  |  |  |
| 5. | Sub-Vendors | Bidder to confirm that Sub-Vendors shall be selected from Project’s approved AVL. | Confirmed | Noted | Closed |  |  |  |  |
| 6. | Specification | Bidder to confirm that surface preparation and painting of all equipment material including valves , pipes , fittings and everything within the package are in accordance with the project painting specification attached to MR. | Confirmed | Noted | Closed |  |  |  |  |
| 7. | Specification | Bidder to confirm that Packing, Marking, Preservation and Preparation for Shipment will be done in accordance to the project specification attached to the MR. | Confirmed | Noted | Closed |  |  |  |  |
| 8. | Inspection and Test Plan | Inspection and Test Plan / Quality Control Plan and their related inspection level (Witness, Hold, Review and Approval) shall be at least in accordance with MR and all project requirement such as project ITP and “QA inspection and test” section of datasheets. Any deviation is not accepted unless otherwise specified in deviation list and approved by Purchaser. | Confirmed | Noted | Closed |  |  |  |  |
| 9. | Material Certificate | Bidder to confirm that for all pressure parts and those required special consideration or stainless steel as well as for supports the material certificate should be in accordance with EN 10204, 3.1 Material certificate for non-pressure parts shall be in accordance with EN 10204, 2.2. | Confirmed | Noted | Closed |  |  |  |  |
| - 10. 10. | Scope | Bidder shall confirm the scope of work and supply are fully in accordance with MR and its attachment with following items: 1- The consequences of HAZOP and SIL study shall be in vendor scope of supply and work. 2- Participation in technical meeting whenever it is necessary by Purchaser/Client to close engineering document after order is in vendor scope 3- Lifting beams (Spreader beams) for erection with sling shackle and etc. shall be designed and supplied by vendor 4- N2 preservation with pressure regulator, valve & flexible hoses during transportation & 18 months of site storage based on requirements in MR shall be in vendor scope of supply / work. 5- All utility conditions shall be as per document "Process basis of Design Document; Doc. No: BK-GNRAL-PEDCO-000-PR-DB-0001". Moreover bidder shall consider utility noted condition as per project design data. If other utility condition is needed, whole required facilities is in vendor scope of work/supply. 6- Anchor bolts | - Confirmed - Confirmed - Confirmed. It is not preferred to be supplied by TAPCO/LMF. But if the purchaser insists could be supplied. - Confirmed - Confirmed - Confirmed | 3-To be provided by vendor. Noted for other items. | Vendor confirm to provide item 3. Closed. |  |  |  |  |
| 11. | NEW ITEM |  |  | Vendor shall follow and supply the attached spare part list: | Vendor confirms to follow this requirement. Closed |  |  |  |  |
| PROCESS |  |  |  |  |  |  |  |  |  |
|  | COMPRESSOR SKID | There are not cooling water in site | Confirmed, our cooling system is closed cooling water for cooling the compressor and the cooling water will be cooled by air cooler. | Please consider the MR description for cooling circuit but in addition, please provide water consumption if water cooled coolers will be considered for oil and water coolers. |  |  | TAPCO: Please see attached Consumption list. |  |  |
|  | Scope | Process gas separator(s) Process gas cooler(s) Is not in vendor scope of work(please revised) | Will be considered and technical offer will be revised. | Noted | Closed |  |  |  |  |
|  | Datasheet | vendor proposal shall be revised base on last revision of compressor data sheet (d02) | Data sheet rev d02 has not been received. | Will be sent ASAP. |  | vendor proposal shall be revised base on last revision of compressor data sheet (D06) | TAPCO: Revised Technical Offer is attached. |  |  |
|  | Allowable Particle Size | please specified maximum acceptable liquid particle in suction nozzle of compressor | 10 PPM | Noted | Closed |  |  |  |  |
|  | Consumption List | utility consumption for compressor package shall be specified | It is already attached. | We are waiting for consumption list in next revision offer |  |  | TAPCO: Please see attached Consumption list. |  |  |
|  | P and ID | Please submitted P& id of spill back system for each train. | it is already included in process P&ID. | Noted, vendor confirmed that spill back will be provided by him | Closed |  |  |  |  |
|  | New item |  |  | Vendor shall confirm that his selected materials for compressor package are suitable by considerig depreizing scenario, especially for MDMT. | Vendor confirms to follow this requirement. Closed. |  |  |  |  |
|  | Proposal pdf file; 1-Technical specification; 1-1 site/location condition |  |  |  |  | In proposal, site conditions shall be revised base on last revision of Process Basis of Design; BK-GNRAL-PEDCO-000-PR-DB-0001_D04 | TAPCO: Considered and revised. |  |  |
|  | Proposal pdf file; 1.2.4 Compressor data; Compressor design capacity |  |  |  |  | In proposal, compressor design capacity shall be revised base on last revision of Process Data Sheets For Gas Compressors Packages" BK-GCS-PEDCO-120-PR-DT-0030_D02. Process turn down shall be considered 35% of lower Flow rate. | TAPCO: Considered and revised. |  |  |
|  | Attachment 7-16 in proposal package; Utility consumption list; - Nitrogen - Plant cooling water system |  |  |  |  | - NITROGEN purity shall be considered 98 % VOL MAX. - Please specify service water. specification is suitable for compressor cooling water system. | TAPCO: 1-Noted. 2-In detail engineering will be specified. |  |  |
|  | New item |  |  |  |  | Refer to Attachment#2 of this TCL for Process comments on the offer. | TAPCO: Replied and attached. |  |  |
|  | New Item |  |  |  |  |  |  | Refer to Attachment#2 of this TCL for Process comments on the offer. |  |
| MECHANIC |  |  |  |  |  |  |  |  |  |
|  | Document Requirement | Bidder to submit all required documents with tender as per “RFD Form” in the MR. For example below documents: 1-Marked up and Filled out MR scope of supply and works (It shall be signed and stamped) 2-GA DWG with outline dimensions and weights related to this project 3-Noise Data 4-Utility Consumption List | - It is attached - The submitted GAD is sample and completely similar to this project and its dimension can be used for this project. Detailed GAD will be submitted after P.O. during detail engineering. - Noise data is below 85 db in one meter and related detail noise data will be submitted after PO. - It is attached. | 1-We are waiting for it in next revision 2- We are waiting for a plan layout to demonstrate the arrangement of all 3 compressors and auxiliary systems. 3-Noted 4-We are waiting for updated consumption list. | Closed |  |  |  |  |
|  | Material | ASTM number and grade of all supplied material to be specified in next revision offer. Some material to be changed in next revision offer, related ASTM No. to be provided in next revision offer. | Will be considered in next revision. | We are waiting for it in next revision | Closed |  |  |  |  |
|  | Applied Standards | Applied Standards for equipment design, fabrication and test shall be listed | Will be considered in next revision. | We are waiting for it in next revision | Closed |  |  |  |  |
|  | Design Standards | Design standard of coupling shall be API 671 4th edition flexible disk type. Coupling model, material, balance grade and spacer length to be specified at this stge. API 671 data sheet shall be filled out and submitted after order. | It is already considered in technical proposal. API format could not be issued by our sub supplier. They will issue in their special format. | Noted also Vendor shall provide API 671 spacer type flexible metal disk coupling with considering G2.5 for balancing grade and method 3 as per API 671. | Vendor confirms to follow this requirement Closed |  |  |  |  |
|  | Vendor’s Scope | Design and supplying Heat Tracing and insulation if required to be in the vendor’s scope. | Confirmed | Noted | Closed |  |  |  |  |
|  | Vendor’s Scope | All vents and drains of the package to be valved, manifolded and to be routed to the edge of skid. | Confirmed | Noted | Closed |  |  |  |  |
|  | Inspection and Test | Test procedures and acceptance criteria will be checked after order with project specifications and related standards. Vendor to confirm to follow project standard and spec. | Confirmed | Noted | Closed |  |  |  |  |
|  | Comments on Vendor’s Proposal and its attachments | Please refer to the Attachment #1 of this TCL for Mechanical comments on the offer. | It is considered and will be revised | Refer to the attachements |  |  |  |  |  |
|  | New item |  |  |  |  | For revision D06 of compressor mechanical data sheets refer to attachment#1 of this TCL. Vendor to revise offer according to this new data sheet. | TAPCO: Replied and attached. |  |  |
|  | NEW ITEM |  |  |  |  | Refer to Attachment#1 of this TCL for MA comments on the offer. | TAPCO: Replied and attached. |  |  |
|  | New Item |  |  |  |  |  |  | For MA comments refer to Attachment#1 of this TCL. |  |
| PIPING & MATERIAL |  |  |  |  |  |  |  |  |  |
|  | Compressor Tags | Tag of Compressor in all vendor documents (e.g. G.A. Drawing) to be modified according to Project tags (C-2101 & C-2102). | Confirmed. | Noted | Closed |  |  |  |  |
|  | Compressor Stages Arrangement | Due to arrangement of Suction Drums & Air Coolers in scope of Client, Location of 1st Stage of Compressor shall be in Left side of Compressor Package in Compressor G.A. Drawing in Plan View and 2nd Stage of Compressor in Right side. | This GAD is preliminary and will be revised after PO. Could be discussed at the meeting. | Vendor would be requested to revise GAD in this regard after P.O; vendor is to modify the arrangement free of charge. | Vendor confirms to follow this requirement Closed |  |  |  |  |
|  | Compressor Skids Arrangement | Location of Oil Cooler in page 3 of G.A. Drawing is not ok and shall be in back, left or right side of Compressor. | This GAD is preliminary and will be revised after PO. Could be discussed at the meeting. | Vendor would be requested to revise GAD in this regard after P.O; vendor is to modify the arrangement free of charge. | Vendor confirms to follow this requirement Closed |  |  |  |  |
|  | Compressor Package Exclusions | In Technical Proposal document in page 36, Item 2 of “Exclusions” is by vendor. Piping between all Compressors skids are in scope of work of Compressor vendor. | Confirmed. Will be revised. | Noted | Closed |  |  |  |  |
|  | Compressor Data Sheet | In page 21 of vendor Data Sheet, According to Project P&ID, size of Discharge line of 2nd Stage is 6". | Size of interconnecting between the stages of compressors will be selected by the vendor to reach the client’s required discharge capacity and pressure. | Noted | Closed |  |  |  |  |
|  | Compressor Material | Material of CYLINDER shall be C.S and material for PISTON shall be S.S. | Material of cylinder is EN-GJS-400 which is cast iron and LMF guarantee it. Material of piston is X5CrNi18-10 which is SS. | Vendor shall follow previous comment | Vendor confirms to follow this requirement Closed | Shall be reflected in TAPCO / LMF Data sheet. | TAPCO: Since the fluid (Gas) consists of H2s and Co2; Carbon Steel could not be used because CS is not corrosion proof. We only guarantee the submitted material and will not guarantee CS. The submitted Cast Iron is special which is EN-GJSA-XNiMn23-4 (ASTM A571 D2M) | Material selection and guarantee is in vendor scope of work and vendor shall guarantee any material deviation from project requirements. |  |
|  | Compressor Material | CYLINDER LINER shall also be C.S. | It is ASTM A278 Cl40 which is cast iron and will be guaranteed by LMF. | Vendor shall follow previous comment | Vendor confirms to follow this requirement Closed | Shall be reflected in TAPCO / LMF Data sheet. | TAPCO: Since the fluid (Gas) consists of H2s and Co2; Carbon Steel could not be used because CS is not corrosion proof. We only guarantee the submitted material and will not guarantee CS. The submitted Cast Iron is special which is EN-GJSA-XNiMn23-4 (ASTM A571 D2M) | Material selection and guarantee is in vendor scope of working and vendor shall guarantee any material deviation from project requirements. |  |
|  | Compressor Material | Material requirement shall be in compliance with NACE MR 0175/ISO 15156 and Specification for Material Requirements in Sour service BK-GNRAL-PEDCO-000-PI-SP-0008. | Confirmed | Noted | Closed |  |  | Shall be reflected in TAPCO / LMF Data sheet. |  |
|  | Compressor Piping Stress Analysis | Pulsation and vibration study shall be performed by vendor. Vendor shall locate Tie-in points in positions where pulsation and vibration do not affect purchaser side's piping. Tie-in point has zero displacement and rotation, otherwise displacements shall be specified by vendor at this tie-in point. In the event that pulsation study result has impact on purchaser piping routes and/or pipe supports, all vendor requirements shall be informed to purchaser by marking-up pipe route clearly. Since space management and piping static stress analysis are in purchaser scope of work, vendor to obtain purchaser approval on marked-up isometric drawings. | Confirmed | Noted | Closed |  |  |  |  |
|  | Compressor Piping Stress Analysis | Vendor shall provide allowable piping loads at the tie-in points. Allowable nozzle loads shall be in accordance with "Appendix N in Specification For Pressure Vessels". In case of unavoidable over loaded, then the loads should be submitted to the vendor for consideration and approval. |  |  |  |  |  |  |  |
| INSTRUMENT & CONTROL |  |  |  |  |  |  |  |  |  |
|  | Control System Requirement | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0004 and provide the requirement of type C package as mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed | Noted | Closed |  |  |  |  |
|  | Panel Requirement | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0002 and BK-GNRAL-PEDCO-000-IN-SP-0003 for panels mounted in control building and field panels and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed | Noted | Closed |  |  |  |  |
|  | Instrumentation | Regarding supplying instrument on datasheet), bidder is requested to consider all instruments mentioned on project datasheet. VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0001 for instrumentation devices and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed | Noted | Closed |  |  |  |  |
|  | Instrumentation Valves | VENDOR shall follow below documents for valves and other documents as mentioned in these documents. - BK-GNRAL-PEDCO-000-IN-SP-0005 - BK-GNRAL-PEDCO-000-IN-SP-0006 - BK-GNRAL-PEDCO-000-IN-SP-0007 VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed | Noted | Closed |  |  |  |  |
|  | Connection with MCC | if MCC is supplied by VENDOR all interfaces shall be supplied by VENDOR. Below signals are typically defined in this project to be provided for electrical interfaces: - Start command (2 sec high pulse) - Stop command (2 sec high pulse) - Trip command (low signal for trip) - Run/Stop Status (high signal for run status) - Local/Remote status (high signal for local status) - Tripped on Fault (high signal for fault status) Available for remote control (high signal for available status) | MCC is not in our scope. | To be considered in optional MCC. However required signals as per purchaser’s comment to be considered in UCP design. |  | To be implemented in proposal. |  |  |  |
|  | Cable | - VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0010 for cables and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed | Noted | Closed |  |  |  |  |
|  | Terminals | All terminals shall be screw type. VENDOR shall note that each core/pair/etc. of cable shall be wired to dedicated terminal | Confirmed | Noted | Closed |  |  |  |  |
|  | Panel Dimension | VENDOR shall specify the estimated number and dimension of panels. | It would be around 2100x1600x800 mm. | Please specify the quantity of panels |  | To be Specified by vendor to meet control room free space. | TAPCO: 4 panels for each compressor. |  |  |
|  | Required Power | VENDOR shall specify the estimated power required (non-UPS and UPS) | 230 V or 110 V | 110 VAC (UPS), 230 VAC (non-UPS) will be supplied by contractor. Vendor to specify power consumption accordingly. |  | To be Specified by vendor to meet main UPS calculation. | TAPCO: Utility consumption is attached. |  |  |
|  | Special Tools | - VENDOR shall supply all special tools for compressor and axillary devices. This is included the special cables (not mentioned in BK-GNRAL-PEDCO-000-IN-SP-0010), MODBUS sockets, software and etc. The list shall be provided separately. | Special tools list is attached. | Vendor shall provide special tools as per the previous comment. | Vendor confirms to follow this requirement Closed |  |  |  |  |
|  | Protection Instruments | The protective instruments shall be according to API-670. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed. API 670 Annex H-6. | Noted | Closed |  |  |  |  |
|  | Software | VENDOR shall supply all the software (including the passwords, licenses and program sources). The programs shall be supplied with all tags, descriptions and etc. The software programs shall be installed on engineering laptops and also supplied on CD/DVD. | Confirmed | Noted | Closed |  |  |  |  |
|  | Load Sharing | VENDOR shall clearly specify the scenario for load sharing. The interface signals, required hardware (e.g. panels, if any) and additional control or on/off valves. | It will be submitted after PO. | One common load Sharing panel for all train to be supplied by vendor as minimum. | Vendor confirms to follow this requirement. Closed | Load sharing necessity to be rechecked by vendor. |  |  |  |
|  | Start-up | VENDOR shall clearly state the start-up sequence. It shall be mentioned if any additional instrument or valve is required. Also, the interface signals shall be mentioned (if any) | Confirmed. | LCP and LGBs for start up to be supplied by vendor | Vendor confirms to follow this requirement. Closed | One LCP/Gauge board per train is requested. Vendor to confirm | TAPCO: Confirmed |  |  |
|  | Comments on Vendor’s Proposal and its attachments | Please refer to the Attachment #2 of this TCL for Instrument comments on the offer. | It is applied on the attached revised technical offer. | We are waiting for next revision offer |  |  | TAPCO: it is attached. |  |  |
|  | NEW ITEM |  |  | 1-UCP with redundant CPU dedicated for each train to be provided by vendor. 2-SIL rated loops for protection and trip to be merged inside UCP panel for each train. 3- Monitoring of machine protection signals could be also merged inside UCP panel due to vendor design (required transmitters/transducer for temperature/vibration and etc. sensors to be considered for UCP connection otherwise separate panel, at least VMS to be provided. ) 4-Loose instruments as per purchaser’s P&ID and also spill back valve is in vendor’s scope of supply |  | Common UCP for all train is requested. It shall be implemented technical proposal and system config. Block diagram. | TAPCO: Confirmed. |  |  |
|  | NEW ITEM |  |  | Special cables (if any) shall be supplied by vendor. | Vendor confirms to follow this requirement. Closed |  |  |  |  |
|  | NEW ITEM |  |  | All original software and licenses, workstation and laptops as per project documents to be considered by vendor. | Vendor confirms to follow this requirement. Closed |  |  |  |  |
|  | New Item |  |  |  |  |  |  | vendor confirm all requested item but Related proposal attachment such as revised control block diagram and Revised P&ID are not submitted. all responsibility of Design/ Fabrication fully in compliance with project requirements is by vendor. |  |
| SAFETY |  |  |  |  |  |  |  |  |  |
|  | Noise level | Vendor to confirm that Noise level @1m should be 85 dbA. | Confirmed | Noted | Closed |  |  |  |  |
|  | Area Classification | Electrical devices should be certified for Zone 2 Gas group IIB, T3. | Confirmed | Noted | Closed |  |  |  |  |
| ELECTRICAL |  |  |  |  |  |  |  |  |  |
|  | Vendor Scope of Work | Electrical system scope of work shall be clarified by vendor & any deviation shall be reported. | It is specified in our technical offer and is based on the client’s scope of supply and it would be inside the Skid. | Indoor LV MCC shall be quoted as option by vendor. |  |  | TAPCO: It is not in our scope | Closed |  |
|  | Vendor Scope of Work | All documents related to electrical equipment should be submitted by vendor such as motor data sheet, motor outline drawing & etc. | This kind of documents will be submitted after PO. But you may find a sample motor data sheet as per attached for your information. | Noted but vendor shall confirm that all purchaser’s comment at that stage will be followed by him. In addition, Electrical load list and electrical scope of work and interconnection diagram to be provided at this stage | Vendor confirms to follow this requirement. Closed |  |  |  |  |
|  | Vendor Scope of Work | Vendor list & sub vendor list of manufacture shall be according to project AVL. | Confirmed | Noted | Closed |  |  |  |  |
|  | Specification | All electrical equipment shall comply with electrical references & standard that mentioned on material requisition. (IPS-M-EL-131 (2) & IPS-M-EL-132 (2)) | Confirmed | Noted | Closed |  |  |  |  |
|  | Specification | Motor shall be manufactured & tested in accordance with the latest applicable sections of the IEC standards. Project specification shall be applied as well. | Confirmed | Noted | Closed |  |  |  |  |
|  | Specification | Environmental criteria for design of electrical systems that shall be considered by vendor (According to Electrical System Design Criteria) | Confirmed | Noted | Closed |  |  |  |  |
|  | Specification | Hazard Area classification, Gas group and temperature class of motor shall be at least Exd, IIB, T3, with protection degree of IP54 for motor enclosure and IP55 for terminal box. | Confirmed | Noted | Closed |  |  |  |  |
|  | Data Sheet | Anti-condensation heaters shall be provided for all medium voltage & high voltage motors. The voltage of anti-condensation heaters shall be 230 v single phase for heaters up to 3 Kw. For heaters above 3 kw, 400v three phase & neutral shall be used. The anti-condensation heaters shall be wired to a separate terminal box mounted on the motor frame. | Confirmed | Noted | Closed |  |  |  |  |
|  | Vendor Scope of Work | Local control station/panel for electrical equipment should be supply by vendor. It shall be according to “BK-GCS-PEDCO-120-EL-DT-0006” | Confirmed | Noted | Closed |  |  |  |  |
|  | Vendor Scope of Work | All accessories related to cable entry/termination in electrical equipment (motors, JBs, etc.) such as cable glands & etc. shall be supplied by vendor. | Confirmed | Noted | Closed |  |  |  |  |
|  | Specification | 2 temperature detector (RTD) for each winding & 1 one RTD for each bearing should be provided for MV motor | Confirmed | Noted | Closed |  |  |  |  |
|  | Specification | Differential protection current transformer shall be provided for motors above 1000 Kw. | Confirmed. | Noted | Closed |  |  |  |  |
|  | Load List | Electrical load list of motor include Mechanical power (BHP), Rated power, efficiency, power factor and rated current shall be specified in the proposal. | It is already considered in the technical offer. | Noted | Closed |  |  |  |  |
|  | Load List | API factor, effect of ambient condition especially de-rate factor for altitude shall be considered for specifying rated power of motor and procedure of obtaining rated power of motor from BHP shall be cleared in the proposal. | Already considered in offer. | Noted | Closed |  |  |  |  |
|  | Specification | Compressor motors, shall be 11 Kv & 50 Hz. (Please check page 14 of TCL) LV motors rated up to 150 Kw, shall be 0.4 Kv & 50 Hz. All motors shall be capable of operating continuously at rated torque at any frequency between -2% & +2% with voltage variation of ±5%. | Confirmed | Noted | Closed |  |  |  |  |
|  | Data Sheet | Insulation class shall be F and temperature rise shall be B. | Confirmed | Noted | Closed |  |  |  |  |
|  | Spare Part | A complete set of spare parts for commissioning shall be supplied for each motor. | Please let us have your required spare parts list. | Refer to item A.11 in above | Closed |  |  |  |  |
|  | Test & Inspection | The equipment shall be factory tested. Certified copies of test reports shall be submitted to the purchaser. | Confirmed | Noted | Closed |  |  |  |  |
|  | Data Sheet (Pg 13 of technical Offer) | In electrical service item, electrical data for auxiliary pumps of compressor shall be specified by vendor in data sheet. | It is specified in Utility Consumption list. | Noted | Closed |  |  |  |  |
|  | Data Sheet | Electrical data sheet “BK-GCS-PEDCO-120-EL-DT-0009” shall be completed by vendor. | We have not received this document, although, it will be filled in after PO. | Noted | Closed |  |  |  |  |
|  | Vendor Scope of Work | Supplier shall design the routing of the power & control cable inside package. | Confirmed. Power cable is out of TAPCO/LMF scope. | Noted | Closed |  |  |  |  |
|  | Vendor Scope of Work | Supply of cables between Junction boxes and Package control panel is in vendor scope of work. In other word, All cabling listed in “Customer cable table” shall be supplied and connected by customer/contractor. Cables shall be according to “BK-GNRAL-PEDCO-000-EL-SP-0014”. | Cabling inside the skid is in vendor scope. | Noted | Closed |  |  |  |  |
|  | Vendor Scope of Work | If UPS & batteries are required, vendor shall provide all necessary documents (such as calculation, list etc.) & supply required equipment according to purchaser standards. | Confirmed | Noted | Closed |  |  |  |  |
|  | Specification | Glands shall be of threaded type or fitted in equally strong way | Confirmed | Noted | Closed |  |  |  |  |
|  | TCL File | Attachment deviation cannot be studied. The file is damaged. | Please see attached. | We are waiting for it in next revision |  |  |  |  |  |
|  | TCL File | The attached files named from 7-1 till 7-14 are not according to pg 37 of TCL. In addition, there is not attachment 7-8 (Electrical Design Document). | Total pages of TCL is 16, what do you mean page 37?! | We are waiting for it in next revision |  |  |  |  |  |
|  | New item |  |  |  |  | Power demand for Auxiliary consumption of each compressor will be supplied by client. But SLD of this auxiliary panel & interconnection with UCP shall be issued by vendor. | TAPCO: Confirmed |  |  |
| CIVIL & STRUCTURE |  |  |  |  |  |  |  |  |  |