| # | Clarification Title | Clarification Description(Rev.00) | Conclusion (Rev.00) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description(Rev.01) | Conclusion (Rev.01) (VENDOR to Confirm & Consider in the Final Offers) |
| --- | --- | --- | --- | --- | --- |
| - GENERAL |  |  |  |  |  |
|  | Documents Priority | Vendor to confirm that in case of contradiction between this TCL and offer; vendor confirmation of compliance through this technical clarification is governed and shall be followed by vendor after order. In such case offer won’t be considered as basis of any claim after order. | Confirmed |  | Vendor confirmed and closed. |
|  | Deviation List | Bidder shall confirm his full compliance to meet all purchaser documents listed and attached to MR, code / standards listed in MR with ref. No. "BK-GCS-PEDCO-120-ME-MR-0002" as well as MR requirements. All deviations shall be listed in project deviation form, attach to this TCL, signed and stamped. Notes: 1-Any exception and deviation from the requirements of MR and the related specifications including referenced codes & standards shall be clearly declared by vendor in writing for purchaser’s approval at bid stage by using of the format included in “Letter Of Conformity” attached to material requisition. Raising no deviation to any single requirement means full compliance of vendor quotation to that requirement. Purchaser has no responsibility to discover deviations or alternatives scattered over the proposal in addition to what proclaimed explicitly in deviation list; hence such deviation (if any) is disregarded and shall not be considered for vendor claim in future. (Vendor to confirm) 2- Bidder shall confirm that all MR attachments listed in MR are available for her for review at bid stage. 3-Considering items 1 & 2, please note that any deviation at detail stage will not be accepted by purchaser. | Confirmed |  | Vendor confirmed and closed. |
|  | Documents Priority | Bidder to confirm his compliance with documents priority stated in MR. Any conflict between the documents shall be notified by bidder to purchaser and resolved by his agreement | Confirmed |  | Vendor confirmed and closed. |
|  | Requirement For Documents | Bidder to confirm that documents required after purchase order shall be submitted in accordance with “RFD Form” attached to the MR. | Confirmed |  | Vendor confirmed and closed. |
|  | Sub-Vendors | Bidder to confirm that Sub-Vendors shall be selected from Project’s approved AVL. | Confirmed |  | Vendor confirmed and closed. |
|  | Specification | Bidder to confirm that surface preparation and painting of all equipment material including valves , pipes , fittings and everything within the package are in accordance with the project painting specification attached to MR. | Confirmed |  | Vendor confirmed and closed. |
|  | Specification | Bidder to confirm that Packing, Marking, Preservation and Preparation for Shipment will be done in accordance to the project specification attached to the MR. | Confirmed |  | Vendor confirmed and closed. |
|  | Material Certificate | Bidder to confirm that for all pressure parts and those required special consideration or stainless steel as well as for supports the material certificate should be in accordance with EN 10204, 3.1 Material certificate for non-pressure parts shall be in accordance with EN 10204, 2.2. | Confirmed |  | Vendor confirmed and closed. |
|  | Scope | Bidder shall confirm the scope of work and supply are fully in accordance with MR and its attachment with following items: 1- The consequences of HAZOP and SIL study shall be in vendor scope of supply and work. 2- Participation in technical meeting whenever it is necessary by Purchaser/Client to close engineering document after order is in vendor scope 3- Lifting beams (Spreader beams) for erection with sling shackle and etc. shall be designed and supplied by vendor 4- N2 preservation with pressure regulator, valve & flexible hoses during transportation & 18 months of site storage based on requirements in MR shall be in vendor scope of supply / work. 5- All utility conditions shall be as per document "Process basis of Design Document; Doc. No: BK-GNRAL-PEDCO-000-PR-DB-0001". Moreover bidder shall consider utility noted condition as per project design data. If other utility condition is needed, whole required facilities is in vendor scope of work/supply. 6- Anchor bolts | Confirmed |  | Vendor confirmed and closed. |
|  | NEW ITEM Inspection and Test Plan |  |  | Inspection and Test Plan / Quality Control Plan and their related inspection level (Witness, Hold, Review and Approval) shall be at least in accordance with MR and all project requirement such as project ITP and “QA inspection and test” section of datasheets. Any deviation is not accepted unless otherwise specified in deviation list and approved by Purchaser. | Vendor confirmed and closed. |
|  | NEW ITEM Spare Part |  |  | Vendor to supply all the specified spare parts in the below attached file (as minimum): | Vendor confirmed and closed. |
|  | NEW ITEM Supervision |  |  | Vendor to provide man day of installation supervision. | Vendor confirmed to consider by the next offer. |
| - PROCESS |  |  |  |  |  |
|  | Dew Point | The dew point of the product gas shall not exceed 5°C. | Confirmed | closed | Vendor confirmed and closed. |
|  | Lean glycol | Lean glycol consumption and purity for Dehydration Package shall be specified | Confirmed, after PO | This data shall be specified for review and comparison with other manufactures. | Vendor confirmed and closed. (With the next offer) |
|  | P&ID | P&ID shall be provided | Confirmed, after PO | Comments after receiving vendor data. In this step, instead of p & ID, the schematic of the equipment and process communication should be specified | Delham accepted project P&ID will be followed. cost impact due to any change will be compensated by their side |
|  | PFD | PFD & H&M shall be provided | Confirmed, after PO | Comments after receiving vendor data | Delham accepted project PFD & H&MB will be followed. cost impact due to any change will be compensated by their side |
|  | Flare network pressure | Flare network pressure shall be considered 0.5 barg | Confirmed | closed | Vendor confirmed and closed. (any required provisions should be considered in the next offer) |
| - MECHANIC |  |  |  |  |  |
|  | Document Requirement | Bidder to submit all required documents with tender as per “RFD Form” in the MR. For example below documents: 1-Marked up and Filled out MR scope of supply and works (It shall be signed and stamped) 2-GA DWG with outline dimensions and weights related to this project 3- A complete Equipment List 4-Noise Data 5-Utility Consumption and load List 6-Consummable material e.g. Glycol, Chemical and … 7-Spare Part List | Confirmed, please see attachment 9. | Vendor shall follow and supply this requirement. | Vendor confirmed and closed. |
|  | Material | Material shall be in accordance with ASME code not ASTM. ASME number and grade of all supplied material to be specified in next revision of offer. | Confirmed |  | Vendor confirmed and closed. |
|  | Applied Standards | Applied Standards for equipment design, fabrication and test shall be listed. ASME Sec. VIII Div. 1, IPS (Iranian Petroleum Standards) & specifically IPS-G-ME-150 and ASCE 7-10 (instead of UBC) are applicable and Shall be added to the references. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor’s Scope | Design and supplying Heat Tracing and insulation if required to be in the vendor’s scope. | Confirmed, but not recommended | Vendor shall follow and supply this requirement. | Vendor confirmed and closed. |
|  | Vendor’s Scope | All vents and drains for skid mounted equipment in the package to be valved, manifolded and to be routed to the edge of skid. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor’s Scope | Access ladder and platforms, stairway shall be studied, designed and supplied by vendor. Relevant detail drawings shall be submitted for approval in detail design stage. Gratings shall be hot dip galvanized. Please confirm. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor’s Scope | All required facility for utility distribution in Battery Limit to be in the vendor’s scope of supply and work. Note: Each utility will be supplied in one tie-in and will be returned in one tie-out so any required accessories for distribution of these utilities in Battery Limit is in the vendor’s scope. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor’s Scope | All required piping material including valve, fitting, inter connection piping, pipe supports and etc. within battery limit Note: All Piping and Tubing to be prefabricated and match marked and a relevant MTO and isometric DWG for installation to be provided by vendor. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor’s Scope | Fuel skid shall be supplied completely shop prefabricated (piped, wired to local JB) and ready for site installation/operation. Please confirm. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor’s Scope | All temporary items (blind flange, cover/cap, gasket, bolts/nuts, etc.) required for hydro testing at site shall be supplied by vendor. Please confirm. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor’s Scope | All required injection packages which are required for proper operation of this dehydration package to be in the vendor’s scope of supply. Please confirm. Note: Please provide a list for them in next revision offer. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor’s Scope | All required supports for equipment (such as legs, skirts and etc.) shall be provided by the vendor. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor’s Scope | All installation material such as anchor bolts, templates, shims, leveling jack screws and leveling pads, sliding plate, PTFE sheet, roller, etc., as applicable to proposed package design, shall be included in vendor scope. Please confirm. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor’s Scope | All heat treatment, including PWHT, post forming, solution annealing, stabilization heat treatment, etc. as required referenced specifications, referenced codes & standards (including material standards), and/or recommended by vendor for specified applications, shall be included in vendor scope and relevant procedure as well as certified report shall be provided. | Confirmed |  | Vendor confirmed and closed. |
|  | Technical Specification | Please submit technical specifications for the Burner (type, quantity, arrangement, type of ignition) | Confirmed |  | Vendor confirmed and closed. |
|  | Technical Specification | MDMT for design of equipment shall be considered as per the project requirement. Please confirm. | Confirmed |  | Vendor confirmed and closed. |
|  | Project Requirement | Please confirm this package is fully suitable for outdoor installation/operation without requiring any additional protection. | Confirmed |  | Vendor confirmed and closed. |
|  | Project Requirement | The package shall be designed and constructed so that requires minimum site work (preparation and welding) while fulfills transportation limits. Please confirm. | Confirmed |  | Vendor confirmed and closed. |
|  | Special Tools | All special tools required for erection, operation and maintenance of the package (including auxiliaries) shall be supplied by vendor. Please confirm. | Confirmed |  | Vendor confirmed and closed. |
|  | Inspection and Test | Test procedures and acceptance criteria will be checked after order with project specifications and related standards. Vendor to confirm to follow project standard and spec. | Confirmed |  | Vendor confirmed and closed. |
|  | Project Requirement | Regard to the injection packages, please confirm to follow the following requirement: - Metering pumps shall be equipped with internal (for diaphragm liquid end) and external relief valves - Motor absorbed power for internal PSV setting with 10% accumulation shall be added in offer and shall be considered in selection of motor nameplate power. - In all metering pumps, suction and discharge shall be of double ball check valve - Bladder material shall be compatible to process fluid. - Damping peak to peak shall be less than 2.5%. - Pressure gauges shall be oil filled type in the package | Confirmed |  | Vendor confirmed and closed. |
|  | Documentation | Bidder to confirm that all native files due to all calculations and drawings shall be submitted for purchaser approval after order. | Confirmed |  | Vendor confirmed and closed. |
|  | Documentation | Regarding technical offer, diameter, dimension and thickness of each equipment will be checked after receiving vendor calculation and data after order and comments in detail shall be followed without cost impact. | Confirmed |  | Vendor confirmed and closed. |
|  | Documentation | Welding book (including WPS, PQR, weld map and NDT map) shall be prepared by vendor and submitted for purchaser approval (pressure parts, and structural). No welding shall be commenced before receiving approved welding book. Please confirm. | Confirmed |  | Vendor confirmed and closed. |
|  | Documentation | All engineering documents to be referenced for site works (procedures, welding documents, detail drawings, test and inspection requirements, etc.) shall be provided by vendor as part of post order engineering documentation. Please confirm. | Confirmed |  | Vendor confirmed and closed. |
|  | Documentation | Below item shall be added to section “Prior to fabrication”: - VPIS Below items shall be added to section “After fabrication”: - marking, packing, shipping procedure - manufacturing & assembly - name plate - ITP - Painting procedure and sand blast Please modify your proposal. | Confirmed, please see revised offer | Vendor shall follow this requirement. | Vendor confirmed and closed. |
|  | Documentation | VPIS (00-183-TEG-VPIS-101) Review code for data sheets and thermal calculations shall be IFA not IFI. Please modify. | Confirmed |  | Vendor confirmed and closed. |
|  | Documentation | Reference list (00-183-TEG-RFL-101) Project description is not correct. Please modify. | Confirmed, modified |  | Vendor confirmed and closed. |
|  | Comments on Vendor’s Proposal and its attachments | Please refer to the Attachment #1 of this TCL for Mechanical comments on the offer. | Confirmed |  | Vendor confirmed and closed. |
|  | Guarantee | The guarantee period shall be eighteen (18) months from the date of delivery or twelve (12) months from the date of mechanical completion & start-up (if applicable), whichever occurs sooner. Under this warranty, all material, parts, and assemblies found to be defect during the guarantee period shall be replaced by vendor without any charge. Please confirm. | Confirmed |  | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | Local stress analysis of shells/heads under external loads from piping system and attachments (platforms, pipe supports, davit, and lifting lug) as per WRC Bulletin 107, 297 or FE shall be included in strength calculation. And also, nozzle load shall be considered according to “Specification for allowable loads on static equipment nozzles” The original file of all analysis including all input data and results should be provided by Vendor. ASME Section VIII Div.2 should be used for local stress analysis. Please note that wherever WRC is not applicable FEA shall be done to evaluate local stresses. The original file (native file) of all analysis including all input data and results should be provided by Vendor. Please confirm. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | For wall thickness greater than 50mm, integrally reinforced nozzle shall be used For below cases, head type should be hemispherical: -High pressure vessels Shell thickness ≥ 70 mm | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | Based on the Material Requisition, Bidder shall submit “Specifications / Data Sheets for Package, Equipment, Vessels, Tanks, Pumps, Fans, Blowers, Burners, etc.” | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | The pumps shall have suction strainers. Mesh size and flow area shall be in accordance with the pump manufacturer’s recommendation. Pulsation dampers shall be provided and installed both at suction and discharge. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | Package SUPPLIER shall calculate the NPSHa available at pump suction nozzle as per the details of their design. The purchaser shall reconfirm the NPSHA once the piping has been designed. Acceleration losses in the suction shall be minimized through use of the straightest, shortest suction line possible. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | A performance and mechanical running test shall be performed at the pump manufacturer’s shop. SUPPLIER shall submit specific test agendas (procedures) to the PURCHASER for review and approval. All pertinent test conditions, procedures and acceptance criteria should be clearly mentioned and should be in conformance with the purchase order and pertinent test codes. It is critical that all aspects of the test be mutually agreed upon in writing in the form of a final test agenda. This final test agenda must be completed prior to the start of any testing program. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | Version for all Software to be considered in accordance with code edition. For PV-Elite: PVElite/CodeCalc/ Nozzle Pro 2019 should be considered for preliminary general mechanical calculation. ASME Material Data Base shall be set to 2010. Version of PVElite can be discussed in K.O.M. For AutoCad: Native files of drawings to be compatible with AutoCad 2014. For HTRI :7 For SAP : 20.10 In any case, verification of software is Vendor’s responsibility. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | For nozzle necks made by plate full radiography shall be considered. Please confirm. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | Trial Assembly of all removable Internals (If supplied by Vendor) is vessel vendor’s scope of work, please confirm. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | All documentation shall be in editable electronic format (scanned pages are not acceptable). Vendor to Confirm. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | VENDOR’s documents shall have the affected areas highlighted by 'clouding' or 'side-lining' and the use of a triangle adjacent to the alterations denoting the revision status. Previous revision highlighting shall be removed at subsequent issues. PURCHASER will take no action on any change without such marking. And VENDOR shall be responsible for any consequence caused by missing such marking. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | Vendor to confirm “welding at site” is not permitted. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | All required steel structure for the package to be prefabricated and supplied by vendor. Also a detail DWG and procedure for installation of them to be provided by vendor. | Vendor confirmed and closed. |
| - PIPING & MATERIAL |  |  |  |  |  |
|  | Package P&ID | P&ID of Package considering tie-ins in it shall be submitted. | Confirmed, after PO | P&ID of Package according to comment shall be submitted before PO. | Delham accepted project P&ID will be followed. cost impact due to any change will be compensated by their side. |
|  | Technical Proposal | G.A Drawing of Package shall be added in Item “Documentation-Prior to Fabrication” in page 23 of Technical Proposal. | Confirmed, please see revised offer rev.01 | Last received offer is not revised according to this comment. | Updated drawing will be provided by vendor. (with next offer by 06.FEB. 2022) |
|  | Equipment Drawing | All Coordinates in Equipment Drawing shall be revised as per Binak Project or to be deleted. | Confirmed | Updated Equipment Drawing is not received. | Updated drawing will be provided by vendor. (with next offer) |
|  | Equipment Drawing | Notes in Equipment Drawing shall be revised as per Binak Project. | Confirmed | Updated Equipment Drawing is not received. | Updated drawing will be provided by vendor. (with next offer) |
|  | Equipment Drawing | Dimensions of Skids and overall dimension of Package shall be shown in Equipment Drawing. | Confirmed, after PO | Updated Equipment Drawing according to comment shall be submitted before PO. | Updated drawing will be provided by vendor. (with next offer) |
|  | Piping Stress Analysis | Tie-in points shall be designed so that movements and rotation tend to zero. (By VENDOR) | Confirmed, after PO | Noted. | Vendor confirmed and closed. |
|  | Piping Stress Analysis | VENDOR shall be specified maximum allowable forces and moments at the tie-in points for review and purchaser approval. In case of unavoidable over loaded at the tie-in points, then the loads should be submitted to the VENDOR for consideration and approval. | Confirmed | Noted. | Vendor confirmed and closed. |
|  | Material For Gas Glycol Contactor | - A 3 mm thick AISI 316L stainless steel cladding shall be applied below the bottom tray when bubble caps are used or below the chimney tray when structured packing or swirl tubes are used. - All internals shall be AISI 316L stainless steel. | Confirmed | Shall be reflected in your offer. | Vendor confirmed and closed. |
|  | Material For Still Column | - Material of the Still Column shall be C.S. + 3mm S.S. 316L cladding. - Overhead vapour line piping shall be AISI 316L or duplex stainless steel. | S.S. is suggested | Shall be reflected in your offer. | Vendor confirmed and closed. |
|  | Material For Reflux Exchanger | Shell and tube material shall be S.S.316L. | Confirmed | Shall be reflected in your offer. | Vendor confirmed and closed. |
|  | Material For Rich/Lean Glycol heat exchanger | Shell and tube material shall be S.S.316L. | Confirmed | Shall be reflected in your offer. | Vendor confirmed and closed. |
|  | Material For Glycol Regenerator Reboiler | - Shell material shall be C.S. + 316L Cladding. - Tube material shall be S.S. 316L. | Not suggested | Vendor is responsible for material selection. Please confirm. | Vendor confirmed and closed. |
|  | Material For Overhead Air Cooler | The material shall be S.S. 316L. | Confirmed | Shall be reflected in your offer. | Vendor confirmed and closed. |
|  | Material For wetted parts | ASTM A516 and ASTM A105 shall be normalized for wetted parts. | Confirmed | Shall be reflected in your offer. | Vendor confirmed and closed. |
|  | Material For internal Bolts/Nuts | Internal Bolts/Nuts shall be SA-193 Gr.B8M/SA-194 Gr.8M. | Confirmed | Shall be reflected in your offer. | Vendor confirmed and closed. |
|  | Material (General) | HIC and SSC shall be considered in accordance with “Technical Specification For Material Requirements In Sour Service” (BK-GNRAL-PEDCO-000-PI-SP-0008). | Confirmed, if required | Closed. | Vendor confirmed and closed. |
|  | Material (General) | Project PMS shall be followed for piping work. | Confirmed | Closed. | Vendor confirmed and closed. |
|  | NEW ITEM Package 3D Model |  |  | PDMS format (version 12.1) of 3D Model of Package shall be submitted by vendor | Vendor confirmed and closed. (After P.O) |
| - INSTRUMENT & CONTROL |  |  |  |  |  |
|  | Control System Requirements | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0004 and provide the requirement of type C package as mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0004 and provide the requirement of type B package as mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Vendor confirmed Type B and closed. (will be provided with next offer) |
|  | Panel Requirements | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0002,0003 for panels mounted in control building and field panels and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed |  | Vendor confirmed and closed. |
|  | Instrumentation | Regarding supplying instrument on datasheet), bidder is requested to consider all instruments mentioned on project datasheet. VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0001 for instrumentation devices and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed |  | Vendor confirmed and closed. |
|  | Instrumentation Valves | VENDOR shall follow below documents for valves and other documents as mentioned in these documents. - BK-GNRAL-PEDCO-000-IN-SP-0005 - BK-GNRAL-PEDCO-000-IN-SP-0006 - BK-GNRAL-PEDCO-000-IN-SP-0007 VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed |  | Vendor confirmed and closed. |
|  | Connection with MCC | if MCC is supplied by VENDOR all interfaces shall be supplied by VENDOR. Below signals are typically defined in this project to be provided for electrical interfaces: - Start command (2 sec high pulse) - Stop command (2 sec high pulse) - Trip command (low signal for trip) - Run/Stop Status (high signal for run status) - Local/Remote status (high signal for local status) - Tripped on Fault (high signal for fault status) Available for remote control (high signal for available status) | Confirmed |  | Vendor confirmed and closed. |
|  | Cable | - VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0010 for cables and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed |  | Vendor confirmed and closed. |
|  | Terminals | All terminals shall be screw type. VENDOR shall note that each core/pair/etc. of cable shall be wired to dedicated terminal | Confirmed |  | Vendor confirmed and closed. |
|  | Panel Dimension | VENDOR shall specify the estimated number and dimension of panels. | Confirmed |  | Vendor confirmed and closed. |
|  | Required Power | VENDOR shall specify the estimated power required (non-UPS and UPS) | Confirmed |  | Vendor confirmed and closed. |
|  | Special Tools | VENDOR shall supply all special tools for compressor and axillary devices. This is included the special cables (not mentioned in BK-GNRAL-PEDCO-000-IN-SP-0010), MODBUS sockets, software and etc. The list shall be provided separately. | Confirmed |  | Vendor confirmed and closed. |
|  | Software | VENDOR shall supply all the software (including the passwords, licenses and program sources). The programs shall be supplied with all tags, descriptions and etc. The software programs shall be installed on engineering laptops and also supplied on CD/DVD. | Confirmed |  | Vendor confirmed and closed. |
|  | Marked up comments | Marked up comment as per attachments shall be clarified and replied by vendor through TCL | Confirmed |  | Vendor confirmed and closed. |
| - SAFETY |  |  |  |  |  |
|  | Fire & Safety requirements | Vendor shall confirm: - Max. Sound Pressure level shall be 85dBA for normal operation. For temporary operation (such as PSV) IPS-E-SF-900 shall be followed. - For HAC, mainly Zone 2, IIB, T3 shall be considered. For “YYY-P-103”, and 1.5 m around each vent (if any) Zone 1 shall be considered. - SIL requirement is subject to change and after SIL study will be finalized. - Vendor are responsible to attend the HAZOP & SIL meeting related to mentioned package and follow the HAZOP / SIL study recommendations without cost claim. Please be informed that F&G, fire proofing and firefighting are excluded by vendor, which is OK by Safety. They will be considered in Safety documents. | Confirmed | Any discrepancy between confirmed items and vendor proposal is in vendor responsibility. CLOSED. | Vendor confirmed and closed. |
| - ELECTRICAL |  |  |  |  |  |
|  | Vendor Scope of Work | Electrical system scope of work shall be clarified by vendor & any deviation shall be reported. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor Scope of Work | All documents related to electrical equipment should be submitted by vendor such as preliminary load list, motor data sheet, motor outline drawing & etc. | Confirmed, After PO |  | Vendor confirmed and closed. |
|  | Vendor Scope of Work | Vendor list & sub vendor list of manufacture shall be according to project AVL. | Confirmed |  | Vendor confirmed and closed. |
|  | Specification | All electrical equipment shall comply with electrical references & standard that mentioned on material requisition. (IPS-M-EL-131 (2) ). | Confirmed |  | Vendor confirmed and closed. |
|  | Specification | Motor shall be manufactured & tested in accordance with the latest applicable sections of the IEC standards. Project specification shall be applied as well. | Confirmed |  | Vendor confirmed and closed. |
|  | Specification | Environmental criteria for design of electrical systems that shall be considered by vendor (According to Electrical System Design Criteria) | Confirmed |  | Vendor confirmed and closed. |
|  | Specification | Hazard Area classification, Gas group and temperature class of motor shall be at least Exd, IIB, T3, with protection degree of IP54 for motor enclosure and IP55 for terminal box. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor Scope of Work | Local control station/panel for electrical equipment should be supply by vendor. It shall be according to “BK-GCS-PEDCO-120-EL-DT-0006” | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor Scope of Work | All accessories related to cable entry/termination in electrical equipment (motors, JBs, etc.) such as cable glands & etc. shall be supplied by vendor. | Confirmed |  | Vendor confirmed and closed. |
|  | Load List | Electrical preliminary load list of motor include Mechanical power (BHP), Rated power, efficiency, power factor and rated current shall be specified in the proposal. | Confirmed |  | Vendor confirmed and closed. |
|  | Load List | API factor, effect of ambient condition especially de-rate factor for altitude shall be considered for specifying rated power of motor and procedure of obtaining rated power of motor from BHP shall be cleared in the proposal. | Confirmed |  | Vendor confirmed and closed. |
|  | Data Sheet | Insulation class shall be F and temperature rise shall be B. | Confirmed |  | Vendor confirmed and closed. |
|  | Spare Part | A complete set of spare parts for commissioning shall be supplied for each motor. | Confirmed |  | Vendor confirmed and closed. |
|  | Test & Inspection | The equipment shall be factory tested. Certified copies of test reports shall be submitted to the purchaser. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor Scope of Work | Supplier shall design the routing of the power & control cable inside package. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor Scope of Work | - Supply of cables between Junction boxes and Package control panel is in vendor scope of work. In other word, All cabling listed in “Customer cable table” shall be supplied and connected by customer/contractor. - Cables shall be according to “BK-GNRAL-PEDCO-000-EL-SP-0014”. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor Scope of Work | If AC UPS or DC UPS & batteries are required, vendor shall provide all necessary documents (such as calculation, list etc.) & supply required equipment according to purchaser standards. | Confirmed |  | Vendor confirmed and closed. |
|  | Specification | Glands shall be of threaded type or fitted in equally strong way | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor Scope of Work | - All skid electrical equipment shall be provide by vendor | Confirmed |  | Vendor confirmed and closed. |
| - CIVIL & STRUCTURE |  |  |  |  |  |
|  | NEW ITEM |  |  | Based on STD2800 this site is located at High Risk Area which is named Seismic Zone 2. also according to ASCE 7 (2010) Seismic Zone is equal to Zone 3 with Seismic Importance Factor 1.25 . Besides according to INBC-Part 6 amount of Wind Speed shall be considered 120 Km/hr. It is necessary to consider the above in the data sheets. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | The technical specifications of The anchor bolts are outside the scope of the seller’s work. But seller’s must be followed the standard drawing of anchor bolt to the number “BK-GNRAL-PEDCO-000-ST-DW-0002”. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | Dimensional specifications of base plates & foundations are inside the scope of the seller’s work. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | Determining the center of gravity of the equipment to control the overturning are in the scope of the seller’s work. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | The following codes & standards should be considered in the civil & structure section: AWS , ASCE7-10 , INBC6 | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | Structure design and related setting bolts are inside the scope of the seller’s work. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | According to the zero edition of the PLOT PLAN document, the piping section: H.P.P. = 100, which is declared 100.20 in the document under review | Issued document was sample. Revised one will be submitted by vendor after P.O. It is accepted by Hirgan but cost of any changes should be compensated by vendor. |
|  | NEW ITEM |  |  | Altitude (MSL) in this document is declared 16.25, which is not correct and The declared MATCH LINE coordinates do not match our PLOT PLAN | Issued document was sample. Revised one will be submitted by vendor after P.O. It is accepted by Hirgan but cost of any changes should be compensated by vendor. |
|  | NEW ITEM |  |  | The area considered for BATTERY LIMIT in this document is more than the space allocated on PLOT PLAN to this package | Issued document was sample. Revised one will be submitted by vendor after P.O. It is accepted by Hirgan but cost of any changes should be compensated by vendor. |
|  | NEW ITEM |  |  | According to the structure design by vendor, it should be provide 3D model of the mentioned structure by vendor. | Vendor confirmed and closed. |