| # | Clarification Title | Clarification Description(Rev.00) | Conclusion (Rev.00) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description(Rev.01) | Conclusion (Rev.01) (VENDOR to Confirm & Consider in the Final Offers) |
| --- | --- | --- | --- | --- | --- |
| - GENERAL |  |  |  |  |  |
|  | Documents Priority | Vendor to confirm that in case of contradiction between this TCL and offer; vendor confirmation of compliance through this technical clarification is governed and shall be followed by vendor after order. In such case offer won’t be considered as basis of any claim after order. | Confirmed. | Noted | Closed |
|  | Deviation List | Bidder shall confirm his full compliance to meet all purchaser documents listed and attached to MR, code / standards listed in MR with ref. No. "BK-GCS-PEDCO-120-ME-MR-0002" as well as MR requirements. All deviations shall be listed in project deviation form, attach to this TCL, signed and stamped. Notes: 1-Any exception and deviation from the requirements of MR and the related specifications including referenced codes & standards shall be clearly declared by vendor in writing for purchaser’s approval at bid stage by using of the format included in “Letter Of Conformity” attached to material requisition. Raising no deviation to any single requirement means full compliance of vendor quotation to that requirement. Purchaser has no responsibility to discover deviations or alternatives scattered over the proposal in addition to what proclaimed explicitly in deviation list; hence such deviation (if any) is disregarded and shall not be considered for vendor claim in future. (Vendor to confirm) 2- Bidder shall confirm that all MR attachments listed in MR are available for her for review at bid stage. 3-Considering items 1 & 2, please note that any deviation at detail stage will not be accepted by purchaser. | Noted and confirmed. GasTech’s Deviation, alternative design and optional items has been presented in section 1.1 of GasTech Technical Proposal (Q20-1483) Deviation will be submitted along with revised Technical offer. | Noted but if there will be any deviation, it shall be clearly specified in deviation list and purchaser’s approval on it shall be taken from him, otherwise it will not be accepted. | Deviation form will be sent along with revised offer. |
|  | Documents Priority | Bidder to confirm his compliance with documents priority stated in MR. Any conflict between the documents shall be notified by bidder to purchaser and resolved by his agreement | Noted and confirmed. | Noted | Closed |
|  | Requirement For Documents | Bidder to confirm that documents required after purchase order shall be submitted in accordance with “RFD Form” attached to the MR. | Noted and confirmed. | Noted | Closed |
|  | Sub-Vendors | Bidder to confirm that Sub-Vendors shall be selected from Project’s approved AVL. | Noted and confirmed. MOP will be followed for sub-vendor selection. Please let us know if there is any other project specification. | Noted | Closed |
|  | Specification | Bidder to confirm that surface preparation and painting of all equipment material including valves , pipes , fittings and everything within the package are in accordance with the project painting specification attached to MR. | Noted and confirmed. | Noted | Closed |
|  | Specification | Bidder to confirm that Packing, Marking, Preservation and Preparation for Shipment will be done in accordance to the project specification attached to the MR. | Noted and confirmed. | Noted | Closed |
|  | Material Certificate | Bidder to confirm that for all pressure parts and those required special consideration or stainless steel as well as for supports the material certificate should be in accordance with EN 10204, 3.1 Material certificate for non-pressure parts shall be in accordance with EN 10204, 2.2. | Noted and confirmed. | Noted | Closed |
|  | Scope | Bidder shall confirm the scope of work and supply are fully in accordance with MR and its attachment with following items: 1- The consequences of HAZOP and SIL study shall be in vendor scope of supply and work. 2- Participation in technical meeting whenever it is necessary by Purchaser/Client to close engineering document after order is in vendor scope 3- Lifting beams (Spreader beams) for erection with sling shackle and etc. shall be designed and supplied by vendor 4- preservation with pressure regulator, valve & flexible hoses during transportation & 18 months of site storage based on requirements in MR shall be in vendor scope of supply / work. 5- All utility conditions shall be as per document "Process basis of Design Document; Doc. No: BK-GNRAL-PEDCO-000-PR-DB-0001". Moreover bidder shall consider utility noted condition as per project design data. If other utility condition is needed, whole required facilities is in vendor scope of work/supply. 6- Anchor bolts | - GasTech will participate in HAZOP and SIL study meeting, and any consequences of these meetings will be affected in document preparation, package design and fabrication, but its cost/time impacts should be borne by client. - Noted and confirmed. - Noted and confirmed. - Noted and confirmed. - Noted and confirmed. - Noted and confirmed. | - All cost impact related to bad and not safe design of the package, shall be tolerated by vendor. - Noted - NotedINSPECTION - Noted - Noted - Noted | Gastech accepted to participate in the meetings and any consequences made by these meetings (as per project specifications and standards) would be tolerated by Gastech. Any other items (operational & maintenance facilities) not included in project specifications (IPS & applicable standards) would be provided by Gastech and the cost would be provided by Client. |
|  | NEW ITEM Spare Part |  |  | Vendor to supply all the specified spare parts in the below attached file (as minimum): | Gastech confirmed and closed. |
|  | NEW ITEM |  |  |  | Tag numbering procedure to be sent to Gastech in order to be tagged of equipment based on that. |
| - PROCESS |  |  |  |  |  |
|  | Dew Point | The dew point of the product gas shall not exceed 5°C. | Noted. As it is described in section 1.1.1 of GasTech Submitted Technical Proposal “According to Duty Specification for Gas Dehydration Package (Doc.#BK-GCS-PEDCO-120-PRSP-0001_D00), the maximum concentration of water in the dry gas at the outlet of the gas dehydration package shall be 1 Ib/MMSCFD, which is very low for these applications, client is requested to check the necessity of this dehydration level. It is worth mentioning that according to represented in Section 6, product specification table in the Design Basis document (BK-GNRAL-PEDCO-000-PR-DB-0001_D01), the product water content is about 11.26 ~ 11.43 lb/MMSCDF. Client is requested to re-check and advise. Client is requested to clarify. | Duty Specification for Gas Dehydration Package (Doc.#BK-GCS-PEDCO-120-PRSP-0001), have been revised, as per section 5.3 new revision of Duty Specification for Gas Dehydration Package revision D02 The maximum concentration of water in the dry gas at the outlet of the gas dehydration package calculated based on Dew Point equal to 5 °C (determined according to 10 degrees distance from minimum operating temperature) and the result is that 11.63 Ib/MMSCF in winter case and 11.43 Ib/MMSCF in summer case. | Gastech confirmed performance of package: Dew point should be 5°C @ outlet pressure. Any technical changes (removing any equipment) should be proposed as a technical inquiry and be confirmed by Contractor. |
|  | Lean glycol | Lean glycol consumption and purity for Dehydration Package shall be specified | It has been specified in section 4.2 and also attachment#1 PFD and heat and material balance. | Closed. Process simulation and heat and material balance shall be revised base on rev D03 process flow diagram. | Gastech to specify total glycol loss. (mechanical + thermodynamic) |
|  | Dehydration Package | vendor proposal shall be revised base on last revision of Process Flow Diagram (PFD) (D02) | Noted and confirmed. It will be submitted along with revised Technical offer. Please be informed that according to above PFD and H&MB, gas stream temperature has been decreased dehydration package (please see stream No 13 and 14) while it is not possible as lean glycol temperature should be higher than inlet gas stream to prevent HC condensation and foaming on contactor also there is a Gas/Lean Exchanger where Lean TEG temperature decreases using gas stream. So dry gas temperature will be higher. Please see GasTech submitted H&MB for more details. |  | Outlet conditions of dry Gas stream as per PFD & H&MB. (Outlet temp. < 65 ℃) |
| - 6 | Flare network pressure | Flare network pressure shall be considered 0.5 barg | Noted. |  | Gastech confirmed to provide Ejector along with technical details. |
|  | Dehydration Package | P&ID document (BK-GCS-PEDCO-120-PR-PI-0013-D01) is for guidance only. It should be noted that, requiring for all the equipment in the package finalized by vendor. | Noted and confirmed |  | Closed |
|  | NEW ITEM |  |  |  | Gastech confirmed below items: 1. Inlet gas stream filter will be removed. 2. AC-705, V-706 will be removed after providing technical supports. 3. Carbon & after filter will be reduced to one number, while outlet stream air cooler will be removed. 4. Removed items breakdown price will be provided. 5. Native simulation file (Aspen, Hysys R.11) will be provided. 6. Technical reason of TEG type instead of mole sieve. |
| - MECHANIC |  |  |  |  |  |
|  | Document Requirement | Bidder to submit all required documents with tender as per “RFD Form” in the MR. For example below documents: 1-Marked up and Filled out MR scope of supply and works (It shall be signed and stamped) 2-GA DWG with outline dimensions and weights related to this project 3- A complete Equipment List 4-Noise Data 5-Utility Consumption and load List 6-Consummable material e.g. Glycol, Chemical and … 7-Spare Part List | - Please see the enclosed Stamped scope of work and supply. - Preliminary GA and weight will be submitted along with revised Technical Proposal. - It has been submitted in attachment#3 of technophile proposal. - Noise will be less than 85db @1m. - Utility consumption has been submitted in section 3.3 of submitted Technical Proposal. - It will be submitted in revised offer. - It has been submitted in attachment#4 of Technical proposal. | Noted | Closed |
|  | Material | Material shall be in accordance with ASME code not ASTM. ASME number and grade of all supplied material to be specified in next revision of offer. | Noted and confirmed. | Noted | Closed |
|  | Applied Standards | Applied Standards for equipment design, fabrication and test shall be listed. ASME Sec. VIII Div. 1, IPS (Iranian Petroleum Standards) & specifically IPS-G-ME-150 and ASCE 7-10 are applicable and Shall be added to the references. | Noted. Please see section 2.7 of submitted technical proposal where major applicable standards have been listed. | Noted | Closed |
|  | Vendor’s Scope | Design and supplying Heat Tracing and insulation if required to be in the vendor’s scope. | Noted and confirmed | Noted | Closed |
|  | Vendor’s Scope | All vents and drains for skid mounted equipment in the package to be valved, manifolded and to be routed to the edge of skid. | Noted and confirmed | Noted | Closed |
|  | Vendor’s Scope | Access ladder and platforms, stairway shall be studied, designed and supplied by vendor. Relevant detail drawings shall be submitted for approval in detail design stage. Gratings shall be hot dip galvanized. Please confirm. | Noted and confirmed | Noted | Closed |
|  | Vendor’s Scope | All required facility for utility distribution in Battery Limit to be in the vendor’s scope of supply and work. Note: Each utility will be supplied in one tie-in and will be returned in one tie-out so any required accessories for distribution of these utilities in Battery Limit is in the vendor’s scope. | Noted and confirmed | Noted | Closed |
|  | Vendor’s Scope | All required piping material including valve, fitting, inter connection piping, pipe supports and etc. within battery limit Note: All Piping and Tubing to be prefabricated and match marked and a relevant MTO and isometric DWG for installation to be provided by vendor. | Noted and confirmed | Noted | Closed |
|  | Vendor’s Scope | Fuel skid shall be supplied completely shop prefabricated (piped, wired to local JB) and ready for site installation/operation. Please confirm. | Noted and confirmed. Whole regeneration skid will be supplied as skid mounted and prefabricated. | Noted | Closed |
|  | Vendor’s Scope | All temporary items (blind flange, cover/cap, gasket, bolts/nuts, etc.) required for hydro testing at site shall be supplied by vendor. Please confirm. | Noted and confirmed | Noted | Closed |
|  | Vendor’s Scope | All required injection packages which are required for proper operation of this dehydration package to be in the vendor’s scope of supply. Please confirm. Note: Please provide a list for them in next revision offer. | Noted and confirmed | Noted | Closed |
|  | Vendor’s Scope | All required supports for equipment (such as legs, skirts and etc.) shall be provided by the vendor. | Noted and confirmed | Noted | Closed |
|  | Vendor’s Scope | All installation material such as anchor bolts, templates, shims, leveling jack screws and leveling pads, sliding plate, PTFE sheet, roller, etc., as applicable to proposed package design, shall be included in vendor scope. Please confirm. | Noted and confirmed | Noted | Closed |
|  | Vendor’s Scope | All heat treatment, including PWHT, post forming, solution annealing, stabilization heat treatment, etc. as required referenced specifications, referenced codes & standards (including material standards), and/or recommended by vendor for specified applications, shall be included in vendor scope and relevant procedure as well as certified report shall be provided. | Noted and confirmed | Noted | Closed |
|  | Technical Specification | Please submit technical specifications for the Burner (type, quantity, arrangement, type of ignition) | One Atmospheric natural draft burner will be supplied. Details will be provided after PO. | Noted | Closed |
|  | Technical Specification | MDMT for design of equipment shall be considered as per the project requirement. Please confirm. | Noted and confirmed. | Noted | Closed |
|  | Project Requirement | Please confirm this package is fully suitable for outdoor installation/operation without requiring any additional protection. | Noted and confirmed. | Noted | Closed |
|  | Project Requirement | The package shall be designed and constructed so that requires minimum site work (preparation and welding) while fulfills transportation limits. Please confirm. | Noted and confirmed. Whole regeneration skid will be supplied as skid mounted and prefabricated. | Noted | Closed |
|  | Special Tools(new added) | All special tools required for erection, operation and maintenance of the package (including auxiliaries) shall be supplied by vendor. Please confirm. | Noted and confirmed. | All special tools((i.e. spread bar and skirt bracing if required based vendor’s lifting calculation & relevant bolts/nuts/gasket, Hydraulic bolt tensioning device for bolts greater than 1-1/2” and etc ) required for erection, operation and maintenance of the package (including auxiliaries) shall be supplied by vendor. Please confirm. | Closed |
|  | Inspection and Test | Test procedures and acceptance criteria will be checked after order with project specifications and related standards. Vendor to confirm to follow project standard and spec. | Noted and confirmed. | Noted | Closed |
|  | Project Requirement | - Regard to the injection packages, please confirm to follow the following requirement: - Metering pumps shall be equipped with internal (for diaphragm liquid end) and external relief valves - Motor absorbed power for internal PSV setting with 10% accumulation shall be added in offer and shall be considered in selection of motor nameplate power. - In all metering pumps, suction and discharge shall be of double ball check valve - Bladder material shall be compatible to process fluid. - Damping peak to peak shall be less than 2.5%. - Pressure gauges shall be oil filled type in the package | - Noted and confirmed. - Noted and confirmed. - Noted and confirmed. - Noted and confirmed. - Noted and confirmed. - Noted and confirmed. | Noted | Closed |
|  | Documentation | Bidder to confirm that all native files due to all calculations and drawings shall be submitted for purchaser approval after order. | Native file of documents will be submitted. Native Report file for calculation will be submitted. | Noted | Closed |
|  | Documentation | Regarding technical offer, diameter, dimension and thickness of each equipment will be checked after receiving vendor calculation and data after order and comments in detail shall be followed without cost impact. | Noted and confirmed. | Noted | Closed |
|  | Documentation | Welding book (including WPS, PQR, weld map and NDT map) shall be prepared by vendor and submitted for purchaser approval (pressure parts, and structural). No welding shall be commenced before receiving approved welding book. Please confirm. | Noted and confirmed. | Noted | Closed |
|  | Documentation | All engineering documents to be referenced for site works (procedures, welding documents, detail drawings, test and windseism requirements, etc.) shall be provided by vendor as part of post order engineering documentation. Please confirm. | Noted and confirmed. | Noted | Closed |
|  | Documentation | Below item shall be added - NDT - Hydrostatic test report - PWHT report (if required by code) - VPIS - Manufacturing & assembly - Name plate - Painting procedure and sand blast - As built drawing - Final book Please modify your proposal. | Comment is not clear. | Please modify your proposal by adding below items: • NDT • Hydrostatic test report • PWHT report (if required by code) • VPIS • Manufacturing & assembly • Name plate • Painting procedure and sand blast • As built drawing • Final book | Gastech confirmed to provide mentioned documentations after P.O. |
|  | Comments on Vendor’s Proposal and its attachments | Please refer to the comments on the offer. | Please see GasTech Clarification in attached file. | Please refer to Attachment#1 of this TCL for mechanical comments on the offer | Refer to the replies on the proposal. |
|  | Guarantee | The guarantee period shall be eighteen (18) months from the date of delivery or twelve (12) months from the date of mechanical completion & start-up (if applicable), whichever occurs sooner. Under this warranty, all material, parts, and assemblies found to be defect during the guarantee period shall be replaced by vendor without any charge. Please confirm. | Noted and confirmed. If the defect is related to Design, Supply or fabrication of GasTech. | Noted | Closed |
|  | NEW ITEM |  |  | Local stress analysis of shells/heads under external loads from piping system and attachments (platforms, pipe supports, davit, and lifting lug) as per WRC Bulletin 107, 297 or FE shall be included in strength calculation. And also, nozzle load shall be considered according to “Specification for allowable loads on static equipment nozzles” The original file of all analysis including all input data and results should be provided by Vendor. ASME Section VIII Div.2 should be used for local stress analysis. Please note that wherever WRC is not applicable FEA shall be done to evaluate local stresses. The original file (native file) of all analysis including all input data and results should be provided by Vendor. Please confirm. | Gastech confirmed & closed. |
|  | NEW ITEM |  |  | For wall thickness greater than 50mm, integrally reinforced nozzle shall be used For below cases, head type should be hemispherical: -High pressure vessels Shell thickness ≥ 70 mm | Gastech confirmed & closed. |
|  | NEW ITEM |  |  | Based on the Material Requisition, Bidder shall submit “Specifications / Data Sheets for Package, Equipment, Vessels, Tanks, Pumps, Fans, Blowers, Burners, etc.” | Gastech confirmed & closed. |
|  | NEW ITEM |  |  | The pumps shall have suction strainers. Mesh size and flow area shall be in accordance with the pump manufacturer’s recommendation. Pulsation dampers shall be provided and installed both at suction and discharge. | Gastech confirmed & closed. |
|  | NEW ITEM |  |  | Package SUPPLIER shall calculate the NPSHa available at pump suction nozzle as per the details of their design. The purchaser shall reconfirm the NPSHA once the piping has been designed. Acceleration losses in the suction shall be minimized through use of the straightest, shortest suction line possible. | Gastech confirmed & closed. |
|  | NEW ITEM |  |  | A performance and mechanical running test shall be performed at the pump manufacturer’s shop. SUPPLIER shall submit specific test agendas (procedures) to the PURCHASER for review and approval. All pertinent test conditions, procedures and acceptance criteria should be clearly mentioned and should be in conformance with the purchase order and pertinent test codes. It is critical that all aspects of the test be mutually agreed upon in writing in the form of a final test agenda. This final test agenda must be completed prior to the start of any testing program. | Gastech confirmed & closed. |
|  | NEW ITEM |  |  | Version for all Software to be considered in accordance with code edition. For PV-Elite: PVElite/CodeCalc/ Nozzle Pro 2019 should be considered for preliminary general mechanical calculation. ASME Material Data Base shall be set to 2010. Version of PVElite can be discussed in K.O.M. For AutoCad: Native files of drawings to be compatible with AutoCad 2014. For HTRI :7 For SAP : 20.10 In any case, verification of software is Vendor’s responsibility. | Gastech confirmed & closed. |
|  | NEW ITEM |  |  | For nozzle necks made by plate full radiography shall be considered. Please confirm. | Gastech confirmed & closed. |
|  | NEW ITEM |  |  | Preassembly of all removable Internals (If supplied by Vendor) is in vessel vendor’s scope of work, please confirm. | Gastech confirmed to be fully responsible of conformity of installing internal equipment. |
|  | NEW ITEM |  |  | All documentation shall be in editable electronic format (scanned pages are not acceptable). Vendor to Confirm. | Gastech confirmed & closed. |
|  | NEW ITEM |  |  | All changes and modifications on VENDOR’s documents shall have the affected areas highlighted by 'clouding' or 'side-lining' and the use of a triangle adjacent to the alterations denoting the revision status. Previous revision highlighting shall be removed at subsequent issues. PURCHASER will take no action on any change without such marking. And VENDOR shall be responsible for any consequence caused by missing such marking. | Gastech confirmed & closed. |
|  | NEW ITEM Site Tests |  |  | Site Acceptance Test (SAT) and 72 hours performance test of whole the package at site to be done by vendor. Negative tolerances for dehydration level (the maximum water concentration at the outlet of the package shall not be more than the specified value in the relevant duty specification), flow rate and pressure of the outlet gas of the package will not be accepted. | Gastech confirmed & closed. |
|  | NEW ITEM Tolerances on Utility Consumption |  |  | Please specify the utility and power consumption for this package and specify the maximum tolerances for them at this stage. This tolerances shall be followed during performance test of the package. | Gastech confirmed & closed. |
|  | NEW ITEM Vendor’s Scope |  |  | All required steel structure for the package to be prefabricated and supplied by vendor. Also a detail DWG and procedure for installation of them to be provided by vendor. | Gastech confirmed & closed. |
| - PIPING & MATERIAL |  |  |  |  |  |
|  | Package P&ID | In P&ID of Package tagging of Equipment & Pipes shall be modified according to project Plot Plan & project tagging. | As there was no tag and numbering in Project P&ID and also duty specification, equipment tag number has been selected based on GasTech reference. Also, We haven’t received project Plot Plan. | Tags in vendor offer shall comply with Project documents. Pls apply previous comment. | -Tag numbering should be based on project tag numbering procedure, will be sent by Hirgan. |
|  | Package P&ID | All Tie-ins with Client shall be shown with tag and also size of tie-ins in P&ID shall be specified. | It will be submitted after PO. | Requested Data is General and basic Data which is necessary before PO. Pls apply previous comment. | Gastech confirmed to send preliminary Tie-ins at revised offer. |
|  | Technical Proposal | G.A Drawing of Package with its dimensions shall be added in Technical Proposal. | Preliminary GA will be submitted along with revised Technical Proposal. | Revised Technical Proposal is not received. | Gastech confirmed and closed. |
|  | General Material | Specification For Material Requirements in Sour service” Document No:. BK-GNRAL-PEDCO-000-PI-SP-0008 shall be met where required. Hence: - Material shall be Killed, Normalized and fine grain practiced. - All carbon steel welds shall be post weld heat treated even if not required by the applicable design/construction codes. - Plates and welded pips shall be HIC tested as per par. 6.4 of this specification. HIC testing also shall be performed on carbon steel materials clad with corrosion resistance alloys. | Noted and confirmed. | Shall be reflected in your offer. Closed. | Gastech confirmed and closed. |
|  | General Material | Removable internals and internal bolt/nuts shall be S.S316 (L). | Noted and confirmed. | Shall be reflected in your offer. Closed. | Gastech confirmed and closed. |
|  | Painting/Coating | All C.S equipment shall be painted in accordance with project specification. | Noted and confirmed. | Shall be reflected in your offer. Closed. | Gastech confirmed and closed. |
|  | Insulation | Insulation material and thickness shall be in accordance project specification. | Noted and confirmed. | Shall be reflected in your offer. Closed. | Gastech confirmed and closed. Any deviation regarding to project specification should be compensated by Gastech. |
|  | Still Column Material | - Material could be C.S+3mm S.S316L cladding. - Overhead vapor line piping shall be AISI 316L | - Considering the equipment Dimension and Thickness, its material will be SS316L. - Noted and confirmed. | Closed. - Shall be reflected in your offer. Closed | Gastech confirmed and closed. |
|  | Reboiler Material | - Shell Material shall be C.S. + 316L cladding - Tube shall be 316L | - There is no requirement for cladding. In case client want to specify its material, Considering the equipment Dimension and Thickness, its material will be SS316L. -There is no tube on Reboiler. Fire Box will be C.S. | Vendor is responsible and shall guarantee the material. Please confirm. | Gastech confirmed to guarantee material performance (C.S) for reboiler. |
|  | Dehydration Column Material | A 3 mm thick AISI 316L stainless steel cladding shall be applied below the bottom tray when bubble caps are used or below the chimney tray when structured packing or swirl tubes are used. | - Noted and confirmed. | Shall be reflected in your offer. Closed. | Gastech confirmed and closed. |
|  | Still Column Condenser | S.S type shall be 316L. | - Noted and confirmed. | Shall be reflected in your offer. Closed. | Gastech confirmed and closed. |
|  | Attachment 7# | This document is not existed in the offer package. Vendor shall provide material selection report for whole dehydration package. (Piping and equipment). | It will be prepared and submitted after PO. | Closed. | Gastech confirmed and closed. |
|  | Corrosion Allowances | Corrosion Allowances shall be specified for material. | Noted. It will be added in revised Technical Offer | Shall be reflected in your offer. Closed. | Gastech confirmed and closed. |
|  | Piping Stress Analysis (Displacements at the Tie-in points) | Tie-in points shall be designed so that movements and rotation tend to zero. (By VENDOR) | Noted and confirmed. | Noted. | Gastech confirmed and closed. |
|  | Piping Stress Analysis (Allowable loads at the Tie-in points) | VENDOR shall be specified maximum allowable forces and moments at the tie-in points for review and purchaser approval. In case of unavoidable over loaded at the tie-in points, then the loads should be submitted to the VENDOR for consideration and approval. | It will be submitted after PO. | Noted. | Gastech confirmed and closed. |
|  | NEW ITEM Package 3D Model |  |  | New Comment: PDMS format (version 12.1) of 3D Model of Package shall be submitted by vendor. | Gastech confirmed and closed. |
|  | NEW ITEM |  |  | Piping material selection and calculation should be submitted by Gastech. | Gastech confirmed to send the document ASAP. (before sending proposal) |
| - INSTRUMENT & CONTROL |  |  |  |  |  |
|  | Comments on Vendor’s Proposal and its attachments | Please refer to the comments on the offer. | Please see the reply in attached file. |  | P&ID shall be revised by specific symbol and legend. Control/safety block diagram to be provided by vendor as per final clarifications. |
|  | New Item Control System Requirements |  |  | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0004 and provide the requirement of type C package as mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | PLC type indoor control panel is requested. Redundant CPU and power supply shall be considered. SIL requirements for Safety /Protection loops (such as redundant I/O and safety relays.,,,) to be considered. BMS logic shall be included. F&G is scope out. One field mounted LCP for Burners and local control/monitor to be considered. Confirmed by Gastech. Closed. |
|  | New Item Panel Requirements |  |  | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0002,0003 for panels mounted in control building and field panels and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed by Gastech. Closed. |
|  | New Item Instrumentation |  |  | Regarding supplying instrument on datasheet), bidder is requested to consider all instruments mentioned on project datasheet. VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0001 for instrumentation devices and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed by Gastech. Closed. |
|  | New Item Instrumentation Valves |  |  | VENDOR shall follow below documents for valves and other documents as mentioned in these documents. - BK-GNRAL-PEDCO-000-IN-SP-0005 - BK-GNRAL-PEDCO-000-IN-SP-0006 - BK-GNRAL-PEDCO-000-IN-SP-0007 VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed by Gastech. Closed. |
|  | New Item Connection with MCC |  |  | if MCC is supplied by VENDOR all interfaces shall be supplied by VENDOR. Below signals are typically defined in this project to be provided for electrical interfaces: - Start command (2 sec high pulse) - Stop command (2 sec high pulse) - Trip command (low signal for trip) - Run/Stop Status (high signal for run status) - Local/Remote status (high signal for local status) - Tripped on Fault (high signal for fault status) Available for remote control (high signal for available status) | MCC will be considered by contractor. All motors inside package skid shall be equipped with LCS. |
|  | New Item Cable |  |  | - VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0010 for cables and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed by Gastech. Closed. Special cables between package skid and UCP ( if any) will be provided by vendor |
|  | New Item Terminals |  |  | All terminals shall be screw type. VENDOR shall note that each core/pair/etc. of cable shall be wired to dedicated terminal | Confirmed by Gastech. Closed. |
|  | New Item Panel Dimension |  |  | VENDOR shall specify the estimated number and dimension of panels. | Panel’s numbers and power consumption to be added in revised proposal. |
|  | New Item Required Power |  |  | VENDOR shall specify the estimated power required (non-UPS and UPS) | Power consumption to be added in revised proposal. |
|  | New Item Special Tools |  |  | VENDOR shall supply all special tools for compressor and axillary devices. This is included the special cables (not mentioned in BK-GNRAL-PEDCO-000-IN-SP-0010), MODBUS sockets, software and etc. The list shall be provided separately. | Confirmed by Gastech. Closed. |
|  | New Item Software |  |  | VENDOR shall supply all the software (including the passwords, licenses and program sources). The programs shall be supplied with all tags, descriptions and etc. The software programs shall be installed on engineering laptops and also supplied on CD/DVD. | Confirmed by Gastech. Closed. |
| - SAFETY |  |  |  |  |  |
|  | Fire & Safety Requirements | Vendor shall confirm: - Max. Sound Pressure level shall be 85 dBA for normal operation. For temporary operation (such as PSV) IPS-E-SF-900 shall be followed. - For HAC, mainly Zone 2, IIB, T3 shall be considered. For any pumps or equipment located inside the pit or sump, and 1.5 m around each vent (if any) Zone 1 shall be considered. - SIL requirement is subject to change and after SIL study will be finalized. - Vendor are responsible to attend the HAZOP & SIL meetings related to mentioned package and follow the HAZOP / SIL study recommendations without cost claim. - In addition to stairway, a ladder shall be located at the opposite side of the platform. | - Noted and confirmed. - Noted and confirmed. - Noted and confirmed. -GasTech will participate in HAZOP and SIL study meeting, and any consequences of these meetings will be affected in document preparation, package design and fabrication, but its cost/time impacts should be borne by client. - Noted and confirmed. | Any discrepancy between confirmed items in TCL and vendor’s proposal is fully in vendor responsibility. Closed. | -Closed -Closed -Closed -Refer to item 9. General part. -Closed |
| - ELECTRICAL |  |  |  |  |  |
|  | Vendor Scope of Work | All documents related to electrical equipment should be submitted by vendor such as preliminary load list, motor data sheet, motor outline drawing & etc. | Preliminary load list will be submitted along with revised offer. Motor datasheet and outline drawing will be submitted after PO as they have no use in this stage. | Noted. | Closed. |
|  | Vendor Scope of Work | Vendor list & sub vendor list of manufacture shall be according to project AVL. | Noted and confirmed. MOP will be followed for sub-vendor selection. Please let us know if there is any other project specification. | Noted. | Closed. |
|  | Specification | All electrical equipment shall comply with electrical references & standard that mentioned on material requisition. (IPS-M-EL-131 (2) ). | Noted and confirmed. | Noted. | Closed. |
|  | Specification | Motor shall be manufactured & tested in accordance with the latest applicable sections of the IEC standards. Project specification shall be applied as well. | Noted and confirmed. | Noted. | Closed. |
|  | Specification | Environmental criteria for design of electrical systems that shall be considered by vendor (According to Electrical System Design Criteria) | Noted and confirmed. | Noted. | Closed. |
|  | Specification | Hazard Area classification, Gas group and temperature class of motor shall be at least Exd, IIB, T3, with protection degree of IP54 for motor enclosure and IP55 for terminal box. | Noted and confirmed. | Noted. | Closed. |
|  | Vendor Scope of Work | Local control station/panel for electrical equipment should be supply by vendor. It shall be according to “BK-GCS-PEDCO-120-EL-DT-0006” | Noted and confirmed. | Noted. | Closed. |
|  | Vendor Scope of Work | All accessories related to cable entry/termination in electrical equipment (motors, JBs, etc.) such as cable glands & etc. shall be supplied by vendor. | Noted and confirmed. | Noted. | Closed. |
|  | Load List | Electrical preliminary load list of motor include Mechanical power (BHP), Rated power, efficiency, power factor and rated current shall be specified in the proposal. | Preliminary load list will be submitted along with revised technical offer. | Noted. | Closed. |
|  | Load List | API factor, effect of ambient condition especially de-rate factor for altitude shall be considered for specifying rated power of motor and procedure of obtaining rated power of motor from BHP shall be cleared in the proposal. | Preliminary load list will be submitted along with revised technical offer. | Noted. | Closed. Will be added in revised tech. proposal. |
|  | Data Sheet | Electrical Motor Insulation class shall be F and temperature rise shall be B. | Noted and confirmed. | Noted. | Closed. |
|  | Spare Part | A complete set of spare parts for commissioning shall be supplied for each motor. | Noted and confirmed. | Noted. | Closed. |
|  | Test & Inspection | The equipment shall be factory tested. Certified copies of test reports shall be submitted to the purchaser. | Noted and confirmed. | Noted. | Closed. |
|  | Vendor Scope of Work | Supplier shall design the routing of the power & control cable inside package. | Noted and confirmed. | Noted. | Closed. |
|  | Vendor Scope of Work | - Supply of cables between Junction boxes and Package control panel is in vendor scope of work. In other word, All cabling listed in “Customer cable table” shall be supplied and connected by customer/contractor. - Cables shall be according to “BK-GNRAL-PEDCO-000-EL-SP-0014”. | -Power Cable supply up to motors JB and Control Cable supply up to motor LCS will be by others. Power Cable supply for Lighting up to Lighting main JB will be by others and from lighting main JB up to lighting fixtures TB will be supplied by GasTech. -Noted and confirmed. | Noted. | Closed. |
|  | Vendor Scope of Work | If AC UPS or DC UPS & batteries are required, vendor shall provide all necessary documents (such as calculation, list etc.) & supply required equipment according to purchaser standards. | Noted and confirmed. | Noted. | Closed. |
|  | Specification | Glands shall be of threaded type or fitted in equally strong way | Noted and confirmed. | Noted. | Closed. |
| - CIVIL & STRUCTURE |  |  |  |  |  |
|  | NEW ITEM |  |  | -Based on STD2800 this site is located at High Risk Area which is named Seismic Zone 2. Also according to ASCE 7 (2010) Seismic Zone is equal to Zone 3 with Seismic Importance Factor 1.25 . -Besides according to INBC-Part 6 amount of Wind Speed shall be considered 120 Km/hr. It is necessary to consider the above in the data sheets. | Gastech confirmed and closed. |
|  | NEW ITEM |  |  | The technical specifications of The anchor bolts are outside the scope of the seller’s work. But seller’s must be followed the standard drawing of anchor bolt to the number “BK-GNRAL-PEDCO-000-ST-DW-0002”. | Gastech confirmed to calculate load at base and sizing of anchor bolts. Anchor bolt supplement is out of scope of Gastech. |
|  | NEW ITEM |  |  | Dimensional specifications of base plates & foundations are inside the scope of the seller’s work. | Gastech confirmed and closed. (After P.O) |
|  | NEW ITEM |  |  | Determining the center of gravity of the equipment to control the overturning are in the scope of the seller’s work. | Gastech confirmed and closed. (After P.O) |
|  | NEW ITEM |  |  | The following codes & standards should be considered in the civil & structure section: AWS , ASCE7-10 , INBC6 | Gastech confirmed and closed. |
|  | NEW ITEM |  |  | According to the structure design by vendor, it should be provide 3D model of the mentioned structure by vendor. | Gastech confirmed and closed. |