| # | Clarification Title | Clarification Description(Rev.00) | Conclusion (Rev.00) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description(Rev.01) | Conclusion (Rev.01) (VENDOR to Confirm & Consider in the Final Offers) |
| --- | --- | --- | --- | --- | --- |
| - GENERAL |  |  |  |  |  |
|  | Documents Priority | Vendor to confirm that in case of contradiction between this TCL and offer; vendor confirmation of compliance through this technical clarification is governed and shall be followed by vendor after order. In such case offer won’t be considered as basis of any claim after order. | Confirmed |  | Vendor confirmed & closed. |
|  | Deviation List | Bidder shall confirm his full compliance to meet all purchaser documents listed and attached to MR, code / standards listed in MR with ref. No. "BK-GCS-PEDCO-120-ME-MR-0002" as well as MR requirements. All deviations shall be listed in project deviation form, attach to this TCL, signed and stamped. Notes: 1-Any exception and deviation from the requirements of MR and the related specifications including referenced codes & standards shall be clearly declared by vendor in writing for purchaser’s approval at bid stage by using of the format included in “Letter Of Conformity” attached to material requisition. Raising no deviation to any single requirement means full compliance of vendor quotation to that requirement. Purchaser has no responsibility to discover deviations or alternatives scattered over the proposal in addition to what proclaimed explicitly in deviation list; hence such deviation (if any) is disregarded and shall not be considered for vendor claim in future. (Vendor to confirm) 2- Bidder shall confirm that all MR attachments listed in MR are available for her for review at bid stage. 3-Considering items 1 & 2, please note that any deviation at detail stage will not be accepted by purchaser. | 1-please provide the format of Letter of Conformity (it has not been attached to the Material Requisition). regarding deviation, please see the attached Deviation List (Attach No. 9) 2-Confirmed. 3-Noted. | Please find the Attachment 3 of M.R. NO. BK-GCS-PEDCO-120-ME-MR-0002-D00 and fill out and sign. | Vendor confirmed & closed. |
|  | Documents Priority | Bidder to confirm his compliance with documents priority stated in MR. Any conflict between the documents shall be notified by bidder to purchaser and resolved by his agreement | Confirmed |  | Vendor confirmed & closed. |
|  | Requirement For Documents | Bidder to confirm that documents required after purchase order shall be submitted in accordance with “RFD Form” attached to the MR. | Confirmed |  | Vendor confirmed & closed. |
|  | Sub-Vendors | Bidder to confirm that Sub-Vendors shall be selected from Project’s approved AVL. | Confirmed |  | Vendor confirmed & closed. |
|  | Specification | Bidder to confirm that surface preparation and painting of all equipment material including valves, pipes, fittings and everything within the package are in accordance with the project painting specification attached to MR. | Confirmed |  | Vendor confirmed & closed. |
|  | Specification | Bidder to confirm that Packing, Marking, Preservation and Preparation for Shipment will be done in accordance to the project specification attached to the MR. | Confirmed |  | Vendor confirmed & closed. |
|  | Material Certificate | Bidder to confirm that for all pressure parts and those required special consideration or stainless steel as well as for supports the material certificate should be in accordance with EN 10204, 3.1 Material certificate for non-pressure parts shall be in accordance with EN 10204, 2.2. | Confirmed |  | Vendor confirmed & closed. |
|  | Scope | Bidder shall confirm the scope of work and supply are fully in accordance with MR and its attachment with following items: 1- The consequences of HAZOP and SIL study shall be in vendor scope of supply and work. 2- Participation in technical meeting whenever it is necessary by Purchaser/Client to close engineering document after order is in vendor scope 3- Lifting beams (Spreader beams) for erection with sling shackle and etc. shall be designed and supplied by vendor 4- N2 purging preservation with pressure regulator, valve & flexible hoses during transportation & 18 months of site storage based on requirements in MR shall be in vendor scope of supply / work. 5- All utility conditions shall be as per document "Process basis of Design Document; Doc. No: BK-GNRAL-PEDCO-000-PR-DB-0001". Moreover bidder shall consider utility noted condition as per project design data. If other utility condition is needed, whole required facilities is in vendor scope of work/supply. 6- Anchor bolts | - MFS representative will be attending in HAZOP & SIL meeting. Related cost that might occur of conclusion of this meeting should be handled by others. - Confirmed - Confirmed - Confirmed - Confirmed - Only The design will be by MFS. | - Not accepted, any cost impact related to the vendor’s not safe design shall be tolerated by vendor. - Noted - Noted - Noted - Noted Not accepted, vendor to supply anchor bolts required for the package installation. | - Vendor confirmed & closed. - Vendor confirmed & closed. - Vendor confirmed & closed. - Vendor confirmed & closed. - Vendor confirmed & closed. - Vendor shall only design the anchor bolt. Hirgan will supply it. |
|  | NEW ITEM Spare Part |  |  | Vendor to supply all the specified spare parts in the below attached file (as minimum): | Vendor confirmed & closed. |
|  | NEW ITEM Spare Part |  |  |  | Vendor confirmed to follow the MOP for sub vendor list. MOP will be sent as per attachment. Any cost impact of deviation from MOP list would be burden by vendor. Vendor confirmed & closed. |
| - PROCESS |  |  |  |  |  |
|  | P&ID | Vendor should update last submitted P&ID | Noted. The revised P&ID will be submitted within 22th of January. | The provided pid is schematic and should be reviewed and modified by the manufacturer. | Vendor confirmed & closed. |
|  | Flare network pressure | Flare network pressure shall be considered 0.5 barg | We considered 1.2 bara at battery limit, that is a normal value for flare network. However, if 0.5 barg must be matched, an increase of stripping gas consumption should be considered, from 15 kg/h to 30 kg/h. No other impacts are foreseen. | Please consider ejector if need. | Vendor proposed that there is no need for an ejector. Vendor confirmed & closed. |
|  | Dehydration Package | Vendor proposal shall be revised base on last revision of Process Flow Diagram (PFD) (D02) | Please, note that the proposed PFD (D02 revision) is not completely in line with bid P&ID and other bid documents. In PFD, lean glycol filters, overhead air cooler and water pot (shown in P&ID and required in the Duty Specification for Gas Dehydration Package doc n. BK-GCS-PEDCO-120-PR-SP-0001-D00, para 5.1) are not shown. We have included them in our proposal. In addition, we proposed some changes respect bid PFD: - Surge drum installed downstream glycol/glycol exchanger, to decrease operating temperature and increase the available NPSH to glycol pumps - Rich Glycol After Filter has not been foreseen because the carbon filter is cartridge type (no bed type) with no risk of carbon carryover in the outlet stream. The after filter is not necessary. | PFD and P&ID is schematic and vendor should be study and refer best offer. If need deleted and add equipment. Overhead air cooler and water pot can be deleted in next proposal. | Vendor confirmed to provide technical reasons for any item removal. (aircooler, filter, water pot). (by the next offer, 12 Feb 2022) Flare network condition: 100 ℃ Vendor confirmed & closed. |
|  | Dehydration Package | P&ID document (BK-GCS-PEDCO-120-PR-PI-0013-D01) is for guidance only. It should be noted that, requiring for all the equipment in the package finalized by vendor. | Confirmed |  | Vendor confirmed & closed. |
| - MECHANIC |  |  |  |  |  |
|  | Document Requirement | Bidder to submit all required documents with tender as per “RFD Form” in the MR. For example below documents: 1-Marked up and Filled out MR scope of supply and works (It shall be signed and stamped) 2-GA DWG with outline dimensions and weights related to this project 3-A complete Equipment List 4-Noise Data 5-Utility Consumption and load List 6-Consummable material e.g. Glycol, Chemical and … 7-Spare Part List 8-NDT 9-Hydrostatic test report 10-PWHT report (if required by code) 11-VPIS 12-Marking, packing, shipping procedure 13-Manufacturing & assembly 14-Name plate 15-ITP 16-Painting procedure and sand blast 17-Final book | Please refer to the signed and stamped marked up and filled out MR scope of supply and works (Attach No. 10), GA DWG (Attach No. 2), Organization chart (Attach No. 16) which have been attached to the recent revision of our proposal Also, the spare part lists have been indicated in our technical proposal. Other documents have already been submitted or will be submitted after P.O. |  | Vendor confirmed & closed. |
|  | Applied Standards | Applied Standards for equipment design, fabrication and test shall be listed. IPS (Iranian Petroleum Standards) & specifically IPS-G-ME-150 and ASCE 7-10 shall be added to the references. | Please see the revised technical proposal. |  | Vendor confirmed & closed. |
|  | Vendor’s Scope | Design and supplying Heat Tracing and insulation if required to be in the vendor’s scope. | Confirmed |  | Vendor confirmed & closed. |
|  | Vendor’s Scope | All vents and drains for skid mounted equipment in the package to be valved, manifolded and to be routed to the edge of skid. | Confirmed |  | Vendor confirmed & closed. |
|  | Vendor’s Scope | Access ladder and platforms, stairway shall be studied, designed and supplied by vendor. Relevant detail drawings shall be submitted for approval in detail design stage. Gratings shall be hot dip galvanized. Please confirm. | Confirmed |  | Vendor confirmed & closed. |
|  | Vendor’s Scope | All required facility for utility distribution in Battery Limit to be in the vendor’s scope of supply and work. Note: Each utility will be supplied in one tie-in and will be returned in one tie-out so any required accessories for distribution of these utilities in Battery Limit is in the vendor’s scope. | Confirmed |  | Vendor confirmed & closed. |
|  | Vendor’s Scope | All required piping material including valve, fitting, inter connection piping, pipe supports and etc. within battery limit Note: All Piping and Tubing to be prefabricated and match marked and a relevant MTO and isometric DWG for installation to be provided by vendor. | Confirmed |  | Vendor confirmed & closed. |
|  | Vendor’s Scope | Fuel skid shall be supplied completely shop prefabricated (piped, wired to local JB) and ready for site installation/operation. Please confirm. | Confirmed |  | Vendor confirmed & closed. |
|  | Vendor’s Scope | All temporary items (blind flange, cover/cap, gasket, bolts/nuts, etc.) required for hydro testing at site shall be supplied by vendor. Please confirm. | All equipment will be hydro tested separately at shop. the hydrotesting at site and required temporary items will not be in our scope. | Noted but If there will be any requirement for hydro test at site, mentioned items shall be provided by vendor. | Hydro test shall be done at shop, if under vendor fault it would be done by vendor cost on site. Vendor confirmed & closed. |
|  | Vendor’s Scope | All required injection packages which are required for proper operation of this dehydration package to be in the vendor’s scope of supply. Please confirm. Note: Please provide a list for them in next revision offer. | Confirmed Following chemical injection packages to be considered: - pH control injection package - antifoam injection package Each package will consist of a storage tank (about 1 m3 capacity) and a metering pump (10 liters/h capacity). |  | Noted and closed. |
|  | Vendor’s Scope | All required supports for equipment (such as legs, skirts and etc.) shall be provided by the vendor. | Confirmed |  | Vendor confirmed and closed. |
|  | Vendor’s Scope | All installation material such as anchor bolts, templates, shims, leveling jack screws and leveling pads, sliding plate, PTFE sheet, roller, etc., as applicable to proposed package design, shall be included in vendor scope. Please confirm. | We will only design the anchor bolts and PTFE sheet. Roller is not applicable in this project. Except above mentioned, confirmed. | Not accepted, vendor to follow this items | Noted and closed. |
|  | Vendor’s Scope | All heat treatment, including PWHT, post forming, solution annealing, stabilization heat treatment, etc. as required referenced specifications, referenced codes & standards (including material standards), and/or recommended by vendor for specified applications, shall be included in vendor scope and relevant procedure as well as certified report shall be provided. | Confirmed |  | Vendor confirmed and closed. |
|  | Technical Specification | Please submit technical specifications for the Burner (type, quantity, arrangement, type of ignition) | Burner will be natural draft type (due to the low duty), low NOx, complete with a ignition transformer to. More details shall be provided after contract award and vendor selection. |  | Vendor claimed as burner size is small, pilot is not necessary. Noted and closed. |
|  | Technical Specification | MDMT for design of equipment shall be considered as per the project requirement. Please confirm. | Confirmed |  | Vendor confirmed and closed. |
|  | Project Requirement | Please confirm this package is fully suitable for outdoor installation/operation without requiring any additional protection. | Confirmed |  | Vendor confirmed and closed. |
|  | Project Requirement | The package shall be designed and constructed so that requires minimum site work (preparation and welding) while fulfills transportation limits. Please confirm. | Confirmed |  | Vendor confirmed and closed. |
|  | Special Tools | All special tools required for erection, operation and maintenance of the package (including auxiliaries) shall be supplied by vendor. Please confirm. | Not applicable |  | Spreader beam, bolt tensioner, shackle, steel rope shall be supplied by vendor and closed. Otherwise reported as deviation. |
|  | Inspection and Test | Test procedures and acceptance criteria will be checked after order with project specifications and related standards. Vendor to confirm to follow project standard and spec. | Confirmed |  | Vendor confirmed and closed. |
|  | Project Requirement | Regard to the injection packages, please confirm to follow the following requirement: - Metering pumps shall be equipped with internal (for diaphragm liquid end) and external relief valves - Motor absorbed power for internal PSV setting with 10% accumulation shall be added in offer and shall be considered in selection of motor nameplate power. - In all metering pumps, suction and discharge shall be of double ball check valve - Bladder material shall be compatible to process fluid. - Damping peak to peak shall be less than 2.5%. - Pressure gauges shall be oil filled type in the package | Noted and confirmed. |  | Vendor confirmed and closed. |
|  | Documentation | Bidder to confirm that all native files due to all calculations and drawings shall be submitted for purchaser approval after order. | Confirmed, Except the ones considered as know-how. |  | Vendor excluded HYSY native file and any other process native files. It would be considered as deviation. |
|  | Documentation | Regarding technical offer, diameter, dimension and thickness of each equipment will be checked after receiving vendor calculation and data after order and comments in detail shall be followed without cost impact. | Confirmed |  | Vendor confirmed and closed. |
|  | Documentation | Welding book (including WPS, PQR, weld map and NDT map) shall be prepared by vendor and submitted for purchaser approval (pressure parts, and structural). No welding shall be commenced before receiving approved welding book. Please confirm. | Confirmed |  | Vendor confirmed and closed. |
|  | Documentation | All engineering documents to be referenced for site works (procedures, welding documents, detail drawings, test and inspection requirements, etc.) shall be provided by vendor as part of post order engineering documentation. Please confirm. | Confirmed |  | Vendor confirmed and closed. |
|  | Comments on Vendor’s Proposal and its attachments | Please refer to the proposal for Mechanical comments on the offer. | Please refer to the revised proposal. | Please refer to the Attachment#1 of this TCL for comments on proposal |  |
|  | Guarantee | The guarantee period shall be eighteen (18) months from the date of delivery or twelve (12) months from the date of mechanical completion & start-up (if applicable), whichever occurs sooner. Under this warranty, all material, parts, and assemblies found to be defect during the guarantee period shall be replaced by vendor without any charge. Please confirm. | Confirmed, however, the mentioned guarantee will be given provided that the goods receive proper treatment in the field, are dried out, started-up and operated under the conditions not exceeding in the design condition and in accordance with the instructions included in MFS’s scope. |  | Vendor confirmed and closed. |
|  | NEW ITEM Site Tests |  |  | Site Acceptance Test (SAT) and 72 hours performance test of whole the package at site to be done by vendor. Negative tolerances for dehydration level (the maximum water concentration at the outlet of the package shall not be more than the specified value in the relevant duty specification), flowrate and pressure of the outlet gas of the package will not be accepted. | Vendor Confirmed to provide supervision based on MR & closed. |
|  | NEW ITEM Tolerances on Utility Consumption |  |  | Please specify the utility and power consumption for this package and specify the maximum tolerances for them at this stage. This tolerances shall be followed during performance test of the package. | Vendor confirmed and closed. |
|  | NEW ITEM Vendor’s Scope |  |  | All required steel structure for the package to be prefabricated and supplied by vendor. Also a detail DWG and procedure for installation of them to be provided by vendor. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | Local stress analysis of shells/heads under external loads from piping system and attachments (platforms, pipe supports, davit, and lifting lug) as per WRC Bulletin 107, 297 or FE shall be included in strength calculation. And also, nozzle load shall be considered according to “Specification for allowable loads on static equipment nozzles” The original file of all analysis including all input data and results should be provided by Vendor. ASME Section VIII Div.2 should be used for local stress analysis. Please note that wherever WRC is not applicable FEA shall be done to evaluate local stresses. The original file (native file) of all analysis including all input data and results should be provided by Vendor. Please confirm. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | For wall thickness greater than 50mm, integrally reinforced nozzle shall be used . For below cases, head type should be hemispherical: -High pressure vessels Shell thickness ≥ 70 mm | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | Based on the Material Requisition, Bidder shall submit “Specifications / Data Sheets for Package, Equipment, Vessels, Tanks, Pumps, Fans, Blowers, Burners, etc.” | Vendor confirmed and closed (after P.O.). |
|  | NEW ITEM |  |  | The pumps shall have suction strainers. Mesh size and flow area shall be in accordance with the pump manufacturer’s recommendation. Pulsation dampers shall be provided and installed both at suction and discharge. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | Package SUPPLIER shall calculate the NPSHa available at pump suction nozzle as per the details of their design. The purchaser shall reconfirm the NPSHA once the piping has been designed. Acceleration losses in the suction shall be minimized through use of the straightest, shortest suction line possible. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | A performance and mechanical running test shall be performed at the pump manufacturer’s shop. SUPPLIER shall submit specific test agendas (procedures) to the PURCHASER for review and approval. All pertinent test conditions, procedures and acceptance criteria should be clearly mentioned and should be in conformance with the purchase order and pertinent test codes. It is critical that all aspects of the test be mutually agreed upon in writing in the form of a final test agenda. This final test agenda must be completed prior to the start of any testing program. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | Version for all Software to be considered in accordance with code edition. For PV-Elite: PVElite/CodeCalc/ Nozzle Pro 2019 should be considered for preliminary general mechanical calculation. ASME Material Data Base shall be set to 2010. Version of PVElite can be discussed in K.O.M. For AutoCad: Native files of drawings to be compatible with AutoCad 2014. For HTRI :7 For SAP : 20.10 In any case, verification of software is Vendor’s responsibility. | Vendor confirmed and closed. |
|  | NEW ITEM |  |  | For nozzle necks made by plate full radiography shall be considered. Please confirm. | Vendor confirmed & closed. |
|  | NEW ITEM |  |  | Trial Assembly of all removable Internals (If supplied by Vendor) is vessel vendor’s scope of work, please confirm. | Vendor confirmed to be responsible of conformity of installing internal equipment. |
|  | NEW ITEM |  |  | All documentation shall be in editable electronic format (scanned pages are not acceptable). Vendor to Confirm. | Vendor confirmed & closed. |
|  | NEW ITEM |  |  | All changes and modifications on VENDOR’s documents shall have the affected areas highlighted by 'clouding' or 'side-lining' and the use of a triangle adjacent to the alterations denoting the revision status. Previous revision highlighting shall be removed at subsequent issues. PURCHASER will take no action on any change without such marking. And VENDOR shall be responsible for any consequence caused by missing such marking. | Vendor confirmed & closed. |
|  | NEW ITEM |  |  | Vendor to confirm “welding at site” is not permitted. | Vendor confirmed & closed. |
| - PIPING & MATERIAL |  |  |  |  |  |
|  | Package P&ID | In P&ID of Package tagging of Equipments & Pipes shall be modified according to project Plot Plan & project tagging. | Noted. Revised P&ID will be submitted within next 22th of January. | Waiting for receiving revised P&ID. | Vendor Confirmed & Closed. |
|  | Package P&ID | All Tie-ins with Client shall be shown with tag and also size of tie-ins in P&ID shall be specified. | Noted and confirmed. Revised P&ID will be submitted within next 22th of January. | Waiting for receiving revised P&ID. | Vendor confirmed to send preliminary Tie-ins at revised offer. |
|  | Technical Proposal | G.A Drawing of Package with its dimensions shall be added in Technical Proposal. | Please see the Attached GA drawing (Attach No. 2) | Distance between two skids in G.A Drawing shall be specified too. | Vendor shall check the possibility of supply the package on a single skid. |
|  | General Material | Vendor shall provide material selection report for whole dehydration package. (Piping and equipment). | Please, note that material selection is already defined Data Sheet. A detailed report, if required, will be provided after contract award. | Comments that confirmed in TCL shall be reflected in your offer. Closed. | Vendor confirmed & closed. |
|  | General Material | Specification For Material Requirements in Sour service” Document No:. BK-GNRAL-PEDCO-000-PI-SP-0008 shall be met where required. Hence: - Material shall be Killed, Normalized and fine grain practiced. - All carbon steel welds shall be post weld heat treated even if not required by the applicable design/construction codes. - Plates and welded pips shall be HIC tested as per par. 6.4 of this specification. HIC testing also shall be performed on carbon steel materials clad with corrosion resistance alloys. | Confirmed. | Comments that confirmed in TCL shall be reflected in your offer. Closed. | Revised offer shall be sent accordingly. Vendor Confirmed. |
|  | General Material | Removable internals and internal bolt/nuts shall be S.S316 (L). | Confirmed | Shall be reflected in your offer. Closed. | Vendor confirmed & closed. |
|  | Painting/Coating | All C.S equipment shall be painted in accordance with project specification. | Confirmed | Shall be reflected in your offer. Closed. | Revised offer shall be sent accordingly. Vendor Confirmed. |
|  | Insulation | Insulation material and thickness shall be in accordance project specification. | Confirmed | Shall be reflected in your offer. Closed. | Vendor confirmed & closed. |
|  | Glycol Contactor | - A 3 mm thick AISI 316L stainless steel cladding shall be applied below the bottom tray when bubble caps are used or below the chimney tray when structured packing or swirl tubes are used. - All internals shall be AISI 316L stainless steel. -External Bolts shall be A193 Gr. B7M (Hot Dip Galvanized) -External Nuts shall be A194 Gr. 2HM (Hot Dip Galvanized) - Corrosion Allowance should be 3mm. | Confirmed. However, for bottom scrubber, we recommend 6 mm c.a. | Confirmed items Shall be reflected in your offer. Closed. | Vendor confirmed & closed. |
|  | Still Column | -Shell material could be C.S+3mm S.S316L. - Overhead vapour line piping shall be AISI 316L. -External Bolts shall be A193 Gr. B7M (Hot Dip Galvanized) -External Nuts shall be A194 Gr. 2HM (Hot Dip Galvanized) | Confirmed. Due to the small diameter, a solid SS 316L still column may be considered as alternative. | Confirmed items Shall be reflected in your offer. Closed. | Vendor shall check Solid S.S or cladding for this equipment and revised the data sheet accordingly. |
|  | Stripping Column | - External Bolts shall be A193 Gr. B7M (Hot Dip Galvanized) -External Nuts shall be A194 Gr. 2HM (Hot Dip Galvanized) | Confirmed | Confirmed items Shall be reflected in your offer. Closed. | HDG shall be applied on stud bolts in revised data sheet. |
|  | Glycol Flash Drum | -External Bolts shall be A193 Gr. B7M (Hot Dip Galvanized) -External Nuts shall be A194 Gr. 2HM (Hot Dip Galvanized) - Corrosion Allowance should be 3mm. | It’s advisable to consider 6 mm c.a. due to acid gases presence in the fluid. | Closed | SA-516 and SA-105 shall be normalized in revised data sheet. |
|  | Water Separation Pot | -External Bolts shall be A193 Gr. B7M (Hot Dip Galvanized) -External Nuts shall be A194 Gr. 2HM (Hot Dip Galvanized) -Material could be C.S+ 3mm. | Confirmed | Closed | Vendor confirmed & closed. |
|  | Particle Glycol Filter | - Corrosion Allowance should be 3mm. | Confirmed | Closed | Vendor confirmed & closed. |
|  | Carbon Glycol Filter | - Corrosion Allowance should be 3mm. | Confirmed | Closed | Vendor confirmed & closed. |
|  | Lean Glycol Filter | - Corrosion Allowance should be 3mm. - Removable Internal shall be S.S 316L. | Confirmed | Confirmed items Shall be reflected in your offer. Closed. | Vendor confirmed & closed. |
|  | Rich/ Lean TEG Exchanger | Material for Shell/Plate shall be S.S. 316L. | Confirmed | Confirmed items Shall be reflected in your offer. Closed. | Flange and Nozzle material shall also be S.S 316L. |
|  | TEG REBOILER | - Shell Material shall be C.S+316L Cladding - Tube Material shall be S.S 316L. | Noted and confirmed. | Requirements in comment Number 5 shall be reflected in data sheet. Closed. | SA-516 and SA-105 shall be normalized in revised data sheet. |
|  | Dry Gas / Lean TEG Exchanger | - Shell and tube material is incorrect and shall be revised. | Noted. DS has been revised. | DS is incorrect again. | Shell and tube material is incorrect and shall be revised. |
|  | Lean TEG Circulation Pumps | Pump Material could be C.S+3mm. | Confirmed. | Closed | Closed. |
|  | Piping Stress Analysis (Displacements at the Tie-in points) | Tie-in points shall be designed so that movements and rotation tend to zero. (By VENDOR) | Confirmed, after P.O. | Noted | Vendor confirmed & closed. |
|  | Piping Stress Analysis (Allowable loads at the Tie-in points) | VENDOR shall be specified maximum allowable forces and moments at the tie-in points for review and purchaser approval. In case of unavoidable over loaded at the tie-in points, then the loads should be submitted to the VENDOR for consideration and approval. | Confirmed, after P.O. | Noted | Vendor confirmed & closed. |
|  | NEW ITEM Package 3D Model | PDMS format (version 12.1) of 3D Model of Package shall be submitted by vendor. |  |  | Vendor Confirmed & Closed. |
| - INSTRUMENT & CONTROL |  |  |  |  |  |
|  | Comments on Vendor’s Proposal and its attachments | Please refer to the proposal and PID for comments on the offer. | Noted. Revised P&ID will be submitted within next 22th of January. | Open | P&ID to be revised. Showing dedicated signal required for client DCS, ESD and package LCP and BMS logic module. Project Tag numbering to be followed. Analyzer to be added as per P&ID or shall be listed in vendor deviation list. |
|  | New item Panel Requirements |  |  | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0002,0003 for panels mounted in control building and field panels and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | There is no room mounted control panel. Project spec. For local panels will be followed. Closed. |
|  | New item Control System Requirements |  |  | Packaged control type B is requested as per BK-GNRAL-PEDCO-000-IN-SP-0004. Unit shall be fully remote controlled (monitoring and control functions) by the DCS and ESD systems. There is no VENDOR supplied control cabinet in control Room for these packageLCP is required for main monitor/control commands such as burner control command. BMS logic shall be provided by vendor to be implemented in DCS/ESD. The package is equipped with its instruments, wired by means of appropriate cables and cable routing. The cables are connected to the junction boxes located at the skid edge battery limits or at edge of base plate. All controls are incorporated into the DCS/ESD by the IPCS Integrator. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Vendor confirmed & I/O count For ESD and DCS to be provided at bid stage by vendor. IS signal for analogue to be respected. |
|  | New item Instrumentation |  |  | Regarding supplying instrument on datasheet), bidder is requested to consider all instruments mentioned on project datasheet. VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0001 for instrumentation devices and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Vendor confirmed & closed. |
|  | New item Instrumentation Valves |  |  | VENDOR shall follow below documents for valves and other documents as mentioned in these documents. - BK-GNRAL-PEDCO-000-IN-SP-0005 - BK-GNRAL-PEDCO-000-IN-SP-0006 - BK-GNRAL-PEDCO-000-IN-SP-0007 VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Vendor confirmed & closed. |
|  | New item Connection with MCC |  |  | if MCC is supplied by VENDOR all interfaces shall be supplied by VENDOR. Below signals are typically defined in this project to be provided for electrical interfaces: - Start command (2 sec high pulse) - Stop command (2 sec high pulse) - Trip command (low signal for trip) - Run/Stop Status (high signal for run status) - Local/Remote status (high signal for local status) - Tripped on Fault (high signal for fault status) Available for remote control (high signal for available status) | MCC is scope out. |
|  | New item Cable |  |  | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0010 for cables and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Vendor confirmed & All cables in field package (between instrument and JB/LCP is by vendor (will be provided as loose item). multi cable between package field and control room is by contractor. Special cables will be provided by vendor ( if any) Vendor confirmed & closed. |
|  | New item Terminals |  |  | All terminals shall be screw type. VENDOR shall note that each core/pair/etc. of cable shall be wired to dedicated terminal | Vendor confirmed & closed. |
|  | New item Panel Dimension |  |  | VENDOR shall specify the estimated number and dimension of panels. | Will be specified for JB and LCP after PO. Vendor confirmed & closed. |
|  | New item Required Power |  |  | VENDOR shall specify the estimated power required (non-UPS and UPS) | Power Consumption for UPS required LCP and Analyzer( if any) to be specified |
|  | New item Special Tools |  |  | VENDOR shall supply all special tools for equipment and axillary devices. This is included the special cables (not mentioned in BK-GNRAL-PEDCO-000-IN-SP-0010), MODBUS sockets, software and etc. The list shall be provided separately. | Confirmed ( if any) Vendor confirmed & closed. |
|  | New item Software |  |  | VENDOR shall supply all the software (including the passwords, licenses and program sources). The programs shall be supplied with all tags, descriptions and etc. The software programs shall be installed on engineering laptops and also supplied on CD/DVD. | BMS logic diagram for DCS implementation to be provided by vendor. DCS and ESD logic during preparation by system vendor shall be review and completed by package vendor to guarantee BMS Operation .vendor assistance in SAT is required to considered ( daily rate) to guarantee complete startup- commissioning of the package. Vendor confirmed & closed. |
|  | New item Required Doc. |  |  | Vendor is requested to prepare P&ID diagram, showing all instrumentation and control loops for package battery limit. | Refer to item no. 1 |
|  | New item Required Doc. |  |  | All required sequence logic, control loop safety protection and installation which are necessary for package performance shall be shown on vendor P&ID and prepared by vendor. | Refer to item 1 and 12. |
|  | New item Required Certificate |  |  | Required certificates for instrumentation, as minimum -certificate of origin, Material Certificates, Test/Calibration Certificates, Certificate of Conformance shall be added to technical proposal. (Prepared by Vendor after P.O.) | Vendor confirmed & closed. |
|  | New item Scope of Supply |  |  | Vendor to confirm to supply below items for submitted package (Type B): -Accessories (impulse lines , fittings ,labels, local routings , cables glands , …) -Utility distribution inside the battery limit, -Documentation. -All computing hardware and software, (if required). -All instrument fully installed, piped & wired on skid, -All other instrument equipment, including but not limited to panels, (Local Panel when required), -All Junction Boxes, (Terminals, plugs, etc.), Cables, inside the skid battery limit and special cables, -Cable trays and their accessories (labels, supports, etc. ), -All piping and tubing. Please confirm. | Vendor confirmed & closed. |
|  | New item Required proposal |  |  | As minimum vendor document list, equipment list, instrument MTO, Data sheet,... to be added to the proposal. | Vendor confirmed & closed. |
|  | New item Spare Part |  |  | Commissioning / Spare part list for instrumentation and control panels to be provided. | Vendor confirmed & closed. |
|  | New item Catalog |  |  | Local control panel specification or data sheet/catalogue to be added to vendor proposal. | Vendor confirmed & closed. |
|  | New item Air Consumption |  |  | Vendor shall specify air consumption regard for all valves in utility consumption list. | Will be added in the next offer (12 Feb 2022). |
| - SAFETY |  |  |  |  |  |
|  | Fire & Safety Requirements | Vendor shall confirm: - Max. Sound Pressure level shall be 85 dBA for normal operation. For temporary operation (such as PSV) IPS-E-SF-900 shall be followed. - For HAC, mainly Zone 2, IIB, T3 shall be considered. For any pumps or equipment located inside the pit or sump, and 1.5 m around each vent (if any) Zone 1 shall be considered. - SIL requirement is subject to change and after SIL study will be finalized. - Vendor are responsible to attend the HAZOP & SIL meetings related to mentioned package and follow the HAZOP / SIL study recommendations without cost claim. - In addition to stairway, a ladder shall be located at the opposite side of the platform. | - Confirmed - Confirmed - Noted, if the changes effect our design we will notify. - MFS representative will attend in HAZOP/SIL study meeting. However, he consequences cost, which might be occur, should be handled by others. - Confirmed | Any discrepancy between confirmed items in TCL and vendor’s proposal is fully in vendor responsibility. - Closed. | - Vendor confirmed and closed. - Vendor confirmed and closed. - Vendor confirmed and closed. - Vendor confirmed and closed. - Vendor confirmed and closed. |
| - ELECTRICAL |  |  |  |  |  |
|  | Vendor Scope of Work | Electrical system scope of work shall be clarified by vendor & any deviation shall be reported. | Noted, Please refer to the Deviation list (Attach No. 9) and Signed & Stamped Marked up and Filled out MR scope of supply and works (Attach No. 10) | Note 3 of attach9 is acceptable but note 6 is not acceptable. Any lighting fixtures, cables, glands, junction boxes, sockets, tray, ladder in the package is in vendor scope of work. | Vendor confirmed to supply all lighting accessories for package & Closed. |
|  | Vendor Scope of Work | All documents related to electrical equipment should be submitted by vendor such as preliminary load list, motor data sheet, motor outline drawing & etc. | Confirmed. |  | Vendor confirmed & closed. |
|  | Vendor Scope of Work | Vendor list & sub vendor list of manufacture shall be according to project AVL. | Confirmed. |  | Vendor confirmed & closed. |
|  | Specification | All electrical equipment shall comply with electrical references & standard that mentioned on material requisition. (IPS-M-EL-131 (2) ). | Confirmed. |  | Vendor confirmed & closed. |
|  | Specification | Motor shall be manufactured & tested in accordance with the latest applicable sections of the IEC standards. Project specification shall be applied as well. | Confirmed. |  | Vendor confirmed & closed. |
|  | Specification | Environmental criteria for design of electrical systems that shall be considered by vendor (According to Electrical System Design Criteria) | Confirmed. |  | Vendor confirmed & closed. |
|  | Specification | Hazard Area classification, Gas group and temperature class of motor shall be at least Exd, IIB, T3, with protection degree of IP54 for motor enclosure and IP55 for terminal box. | Confirmed. |  | Vendor confirmed & closed. |
|  | Vendor Scope of Work | Local control station/panel for electrical equipment should be supply by vendor. It shall be according to “BK-GCS-PEDCO-120-EL-DT-0006” | Confirmed. |  | Vendor confirmed & closed. |
|  | Vendor Scope of Work | All accessories related to cable entry/termination in electrical equipment (motors, JBs, etc.) such as cable glands & etc. shall be supplied by vendor. | Confirmed, except the ones indicated in the Deviation list (Attach No. 9) and Signed & Stamped Marked up and Filled out MR scope of supply and works (Attach No. 10). | Any lighting fixtures, cables, glands, junction boxes, sockets, tray, ladder in the package is in vendor scope of work. | Vendor confirmed to supply all lighting accessories for package. |
|  | Load List | Electrical preliminary load list of motor include Mechanical power (BHP), Rated power, efficiency, power factor and rated current shall be specified in the proposal. | Noted & confirmed. Please refer to Utility Consumption List (Attach No.6) |  | Vendor confirmed & closed. |
|  | Load List | API factor, effect of ambient condition especially de-rate factor for altitude shall be considered for specifying rated power of motor and procedure of obtaining rated power of motor from BHP shall be cleared in the proposal. | Confirmed, after PO -P=39KW, API Factor=1.15, Effect of Ambient =0.87 Pnew=39*1.15/0.87~55 P=1.36KW, API Factor=1.25, Effect of Ambient =0.87 Pnew=1.36*1.25/0.87~2.2 |  | Vendor confirmed & closed. |
|  | Data Sheet | Insulation class shall be F and temperature rise shall be B. | Confirmed |  | Vendor confirmed & closed. |
|  | Spare Part | A complete set of spare parts for commissioning shall be supplied for each motor. | Confirmed |  | Vendor confirmed & closed. |
|  | Test & Inspection | The equipment shall be factory tested. Certified copies of test reports shall be submitted to the purchaser. | Confirmed |  | Vendor confirmed & closed. |
|  | Vendor Scope of Work | Supplier shall design the routing of the power & control cable inside package. | Confirmed |  | Vendor confirmed & closed. |
|  | Vendor Scope of Work | - Supply of cables between Junction boxes and Package control panel is in vendor scope of work. In other word, All cabling listed in “Customer cable table” shall be supplied and connected by customer/contractor. - Cables shall be according to “BK-GNRAL-PEDCO-000-EL-SP-0014”. | Confirmed |  | Vendor confirmed & closed. |
|  | Vendor Scope of Work | If AC UPS or DC UPS & batteries are required, vendor shall provide all necessary documents (such as calculation, list etc.) & supply required equipment according to purchaser standards. | UPS feeder by Client. Please refer to Utility Consumption List (Attach No.6) | Accepted but any AC UPS or DC UPS & batteries which are required in the field for package is in vendor scope of work. | UPS for signaling will be provided by client. |
|  | Specification | Glands shall be of threaded type or fitted in equally strong way | Confirmed |  | Vendor confirmed & closed. |
|  | Vendor Scope of Work | - All skid electrical equipment shall be provide by vendor | Confirmed, except the ones indicated in the Deviation list (Attach No. 9) and Signed & Stamped Marked up and Filled out MR scope of supply and works (Attach No. 10). | Note 3 of attach9 is acceptable but note 6 is not acceptable. Any lighting fixtures, cables, glands, junction boxes, sockets, tray, ladder in the package is in vendor scope of work. | Vendor confirmed to supply all lighting accessories for package. |
| - CIVIL & STRUCTURE |  |  |  |  |  |
|  | New Item |  |  | Based on STD2800 this site is located at High Risk Area which is named Seismic Zone 2. also according to ASCE 7 (2010) Seismic Zone is equal to Zone 3 with Seismic Importance Factor 1.25 . Besides according to INBC-Part 6 amount of Wind Speed shall be considered 120 Km/hr. It is necessary to consider the above in the data sheets. | Vendor confirmed and closed. |
|  | New Item |  |  | The technical specifications of The anchor bolt are outside the scope of the seller’s work. | Vendor confirmed and closed. |
|  | New Item |  |  | Dimensional specifications of base plates & foundations are in the seller’s work. | Vendor confirmed and closed. |
|  | New Item |  |  | Determining the center of gravity of the equipment to control the overturning are in the scope of the seller’s work. | Vendor confirmed and closed. |
|  | New Item |  |  | The following codes & standards should be considered in the civil & structure section: AWS , ASCE7-10 , INBC6 | Vendor confirmed and closed. |