| طرح نگهداشت و افزایش تولید 27 مخزن |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DUTY SPECIFICATION FOR GAS DEHYDRATION PACKAGE (PK-2101) نگهداشت و افزایش تولید میدان نفتی بینک |  |  |  |  |  |  |  |
| D04 | JUL.2022 | IFA | M.Aryafar | M.Fakharian | M.Mehrshad |  |
| D03 | APR.2022 | IFA | M.Aryafar | M.Fakharian | M.Mehrshad |  |
| D02 | JAN.2022 | IFA | M.Aryafar | M.Fakharian | M.Mehrshad |  |
| D01 | DEC.2021 | IFC | M.Aryafar | M.Fakharian | M.Mehrshad |  |
| D00 | AUG.2021 | IFC | M.Asgharnejad | M.Fakharian | Sh.Ghalikar |  |
| Rev. | Date | Purpose of Issue/Status | Prepared by: | Checked by: | Approved by: | Client Approval |
| Class: 2 |  | CLIENT Doc. Number: F0Z-708735 |  |  |  |  |
| Status: | IDC: Inter-Discipline Check IFC: Issued For Comment IFA: Issued For Approval AFD: Approved For Design AFC: Approved For Construction AFP: Approved For Purchase AFQ: Approved For Quotation IFI: Issued For Information AB-R: As-Built for CLIENT Review AB-A: As-Built –Approved |  |  |  |  |  |

REVISION RECORD SHEET

| PAGE | D00 | D01 | D02 | D03 | D04 |  | PAGE | D00 | D01 | D02 | D03 | D04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | X |  |  |  |  | 66 |  |  |  |  |  |
| 2 | X | X |  |  |  |  | 67 |  |  |  |  |  |
| 3 | X | X |  |  |  |  | 68 |  |  |  |  |  |
| 4 | X | X |  |  |  |  | 69 |  |  |  |  |  |
| 5 | X |  |  |  |  |  | 70 |  |  |  |  |  |
| 6 | X | X | X | X |  |  | 71 |  |  |  |  |  |
| 7 | X | X | X |  |  |  | 72 |  |  |  |  |  |
| 8 | X | X |  | X | X |  | 73 |  |  |  |  |  |
| 9 | X | X | X | X |  |  | 74 |  |  |  |  |  |
| 10 |  |  |  |  |  |  | 75 |  |  |  |  |  |
| 11 |  |  |  |  |  |  | 76 |  |  |  |  |  |
| 12 |  |  |  |  |  |  | 77 |  |  |  |  |  |
| 13 |  |  |  |  |  |  | 78 |  |  |  |  |  |
| 14 |  |  |  |  |  |  | 79 |  |  |  |  |  |
| 15 |  |  |  |  |  |  | 80 |  |  |  |  |  |
| 16 |  |  |  |  |  |  | 81 |  |  |  |  |  |
| 17 |  |  |  |  |  |  | 82 |  |  |  |  |  |
| 18 |  |  |  |  |  |  | 83 |  |  |  |  |  |
| 19 |  |  |  |  |  |  | 84 |  |  |  |  |  |
| 20 |  |  |  |  |  |  | 85 |  |  |  |  |  |
| 21 |  |  |  |  |  |  | 86 |  |  |  |  |  |
| 22 |  |  |  |  |  |  | 87 |  |  |  |  |  |
| 23 |  |  |  |  |  |  | 88 |  |  |  |  |  |
| 24 |  |  |  |  |  |  | 89 |  |  |  |  |  |
| 25 |  |  |  |  |  |  | 90 |  |  |  |  |  |
| 26 |  |  |  |  |  |  | 91 |  |  |  |  |  |
| 27 |  |  |  |  |  |  | 92 |  |  |  |  |  |
| 28 |  |  |  |  |  |  | 93 |  |  |  |  |  |
| 29 |  |  |  |  |  |  | 94 |  |  |  |  |  |
| 30 |  |  |  |  |  |  | 95 |  |  |  |  |  |
| 31 |  |  |  |  |  |  | 96 |  |  |  |  |  |
| 32 |  |  |  |  |  |  | 97 |  |  |  |  |  |
| 33 |  |  |  |  |  |  | 98 |  |  |  |  |  |
| 34 |  |  |  |  |  |  | 99 |  |  |  |  |  |
| 35 |  |  |  |  |  |  | 100 |  |  |  |  |  |
| 36 |  |  |  |  |  |  | 101 |  |  |  |  |  |
| 37 |  |  |  |  |  |  | 102 |  |  |  |  |  |
| 38 |  |  |  |  |  |  | 103 |  |  |  |  |  |
| 39 |  |  |  |  |  |  | 104 |  |  |  |  |  |
| 40 |  |  |  |  |  |  | 105 |  |  |  |  |  |
| 41 |  |  |  |  |  |  | 106 |  |  |  |  |  |
| 42 |  |  |  |  |  |  | 107 |  |  |  |  |  |
| 43 |  |  |  |  |  |  | 108 |  |  |  |  |  |
| 44 |  |  |  |  |  |  | 109 |  |  |  |  |  |
| 45 |  |  |  |  |  |  | 110 |  |  |  |  |  |
| 46 |  |  |  |  |  |  | 111 |  |  |  |  |  |
| 47 |  |  |  |  |  |  | 112 |  |  |  |  |  |
| 48 |  |  |  |  |  |  | 113 |  |  |  |  |  |
| 49 |  |  |  |  |  |  | 114 |  |  |  |  |  |
| 50 |  |  |  |  |  |  | 115 |  |  |  |  |  |
| 51 |  |  |  |  |  |  | 116 |  |  |  |  |  |
| 52 |  |  |  |  |  |  | 117 |  |  |  |  |  |
| 53 |  |  |  |  |  |  | 118 |  |  |  |  |  |
| 54 |  |  |  |  |  |  | 119 |  |  |  |  |  |
| 55 |  |  |  |  |  |  | 120 |  |  |  |  |  |
| 56 |  |  |  |  |  |  | 121 |  |  |  |  |  |
| 57 |  |  |  |  |  |  | 122 |  |  |  |  |  |
| 58 |  |  |  |  |  |  | 123 |  |  |  |  |  |
| 59 |  |  |  |  |  |  | 124 |  |  |  |  |  |
| 60 |  |  |  |  |  |  | 125 |  |  |  |  |  |
| 61 |  |  |  |  |  |  | 126 |  |  |  |  |  |
| 62 |  |  |  |  |  |  | 127 |  |  |  |  |  |
| 63 |  |  |  |  |  |  | 128 |  |  |  |  |  |
| 64 |  |  |  |  |  |  | 129 |  |  |  |  |  |
| 65 |  |  |  |  |  |  | 130 |  |  |  |  |  |

CONTENTS

1.0	INTRODUCTION	4

2.0	Scope	4

3.0	NORMATIVE REFERENCES	5

3.1	Local Codes and Standard	5

3.2	The Project Documents	6

3.3	ENVIRONMENTAL DATA	6

3.4	Order of Precedence	6

4.0	Abbreviations	6

5.0	BASIS OF DESIGN	7

5.1	General	7

5.2	Feed specification	8

5.3	Product Specification	9

5.4	Control System	9

5.5	Turn Down Ratio	9

- INTRODUCTION

Binak oilfield in Bushehr province is a part of the southern oilfields of Iran, is located 20 km northwest of Genaveh city.

With the aim of increasing production of oil from Binak oilfield, an EPC/EPD Project has been defined by NIOC/NISOC and awarded to Petro Iran Development Company (PEDCO). Also PEDCO (as General Contractor) has assigned the EPC-packages of the Project to "Hirgan Energy - Design and Inspection" JV.

As a part of the Project, a New Gas Compressor Station (adjacent to existing Binak GCS) shall be constructed to gather of 15 MMSCFD (approx.) associated gases and compress & transfer them to Siahmakan GIS.

GENERAL DEFINITION

The following terms shall be used in this document.

| CLIENT: | National Iranian South Oilfields Company (NISOC) |
| --- | --- |
| PROJECT: | Binak Oilfield Development – Surface Facilities; New Gas Compressor Station |
| EPD/EPC CONTRACTOR (GC): | Petro Iran Development Company (PEDCO) |
| EPC CONTRACTOR: | Joint Venture of : Hirgan Energy – Design & Inspection (D&I) Companies |
| VENDOR: | The firm or person who will fabricate the equipment or material. |
| EXECUTOR: | Executor is the party which carries out all or part of construction and/or commissioning for the project. |
| THIRD PARTY INSPECTOR (TPI): | The firm appointed by EPD/EPC CONTRACTOR (GC) and approved by CLIENT (in writing) for the inspection of goods. |
| SHALL: | Is used where a provision is mandatory. |
| SHOULD: | Is used where a provision is advisory only. |
| WILL: | Is normally used in connection with the action by CLIENT rather than by an EPC/EPD CONTRACTOR, supplier or VENDOR. |
| MAY: | Is used where a provision is completely discretionary. |

- Scope

This  specification  outlines  the  minimum  process  requirements  for  design  of  Associated  gas dehydration package in Binak Plant.

- NORMATIVE REFERENCES

## Local Codes and Standard

IPS-C-PI-240 		Construction Standard for Plant Piping System

IPS-C-PI-350 		Plant Piping System Pressure Testing.

IPS-E-PI-200 		Engineering Standard for Flexibility Analysis

IPS-E-PI-221.1		Engineering Standard for Piping Material Selection.

IPS-E-PI-240 		Engineering Standard for Plant Piping System

IPS-E-PR-308		Engineering Standard for Numbering system

IPS-G-ME-150	Towers, Reactors, Pressure Vessels and Internals

IPS-E-GN-100	 Engineering Standard for Units

IPS-E-PR-170   	Engineering Standard for Process Flow Diagram

IPS-E-PR-200   	Engineering Standard for Basic Engineering Design Data

IPS-E-PR-230   	Engineering Standard for Piping& Instrumentation Diagrams (P & IDs)

IPS-E-PR-308   	Engineering Standard for Numbering System

IPS-E-PR-330   	Engineering Standard for Process Design of Production and

Distribution Compressed Air Systems

IPS-E-PR-340    	Engineering Standard for Process Design of Fuel Systems

IPS-E-PR-360    	Engineering Standard for Process Design of Liquid & Gas

Transfer & Storage

IPS-E-PR-440    	Engineering Standard for Process Design of Piping Systems

(Process Piping & Pipeline Sizing)

IPS-E-PR-450    	Engineering Standard for Process Design of Pressure Relieving

Systems Inclusive Safety Relief Valves

IPS-E-PR-460    	Engineering Standard for Process Design of Flare & Blowdown Systems

IPS-E-PR-470   	Engineering Standard for Process Design of Emergency Measures

IPS-E-PR-771   	Engineering Standard for Process Requirements of Heat Exchanging

Equipment

IPS-E-PR-785   	Engineering Standard for Process Design of Air Cooled Heat Exchangers

(Air Coolers)

IPS-E-PR-810    	Engineering Standard for Process Design of Furnaces

IPS-E-PR-830   	Engineering Standard for Process Design of Valves & Control Valves

IPS-E-PR-850   	Engineering Standard for Process Requirements of Vessels, Reactors &

Separators

IPS-E-PR-880		Engineering Standard for Process Design of Gas (Vapor) – Liquid

Separators

IPS-E-PR-190		Layout and spacing

## The Project Documents

BK-GCS-PEDCO-120-PR-PF-0001	Process Flow Diagram (PFD)

BK-GNRAL-PEDCO-000-PR-DB-0001	Process Basis of Design

BK-GCS-PEDCO-120-PR-PI-0013	P&ID - Gas Compression Dehydration Package

BK-GNRAL-PEDCO-000-PR-DC-0001	Process Design Criteria

## ENVIRONMENTAL DATA

Refer to "Process Basis of Design”; Doc. No. :  BK-GNRAL-PEDCO-000-PR-DB-0001

## Order of Precedence

In case of any conflict between the contents of this document or any discrepancy between this document and other project documents or reference standards, this issue must be reported to the CLIENT. The final decision in this situation will be made by CLIENT.

- Abbreviations

NISOC:	National Iranian South Oil Company

PFD:	Process Flow Diagram

P&ID:	Piping and Instrumentation Diagram

BFPD:	Barrel Fluid per Day

- BASIS OF DESIGN

## General

The selected process for removing of water from sour gas is TEG solution. Package supplier shall review and verify the best applicable method for this unit.This configuration is expected to represent the likely selected flow scheme but is subject to change by the package supplier and client’s preference. Water saturated sour gas from the last stage of compression will be sent to the sour gas dehydration package. In sour gas dehydration package the inlet stream will be brought in counter current contact with TEG in sour gas glycol contactor, in which water vapor is absorbed in the TEG. Water‐rich TEG from the bottom of the contactor column is sent under level control to the regeneration system, where at first it passes through a coil in the top of glycol regen still column. This coil provides cooling effect for reflux in still column and pre‐ heats the rich TEG. Rich TEG after removal of dissolved and entrained gases in glycol flash drum and heating in rich/lean glycol exchanger will enter still column, in which the excess water is removed from TEG solution by heating and boiling. Vapor phase out from top of still column for send to flare header. Hot, regenerated lean TEG accumulates in the glycol surge drum, from where it is initially cooled and then filtered as necessary. Finally, the lean glycol is pumped by lean glycol circulation pumps back to the sour gas glycol contactor. In additionally due to the pressure of the LP flare (0.2 barg at least) It’s necessary that considered the manufacture by vendor such as the ejector for venting gas in to the flare if required. Also permissible amount of pressure drop of dehydration package is 1 barg.

Dehydration package (PK-2101) shall include the following major equipment as shown on the ) BK-GCS-PEDCO-120-PR-PI-0013)  within boundary limit, but not be limited to:

- Gas Glycol Contactor

- Dehydrated Gas/Lean Glycol Heat Exchanger

- Glycol Regenerator Still Column

- Glycol Surge Drum

- Glycol Flash Drum

- Lean Glycol Circulation Pumps

- Rich Glycol Filter

- Glycol Charcoal Filter

- Glycol After Filters

- Lean Glycol Filters (IF REQUIRED)

- Lean/Rich Glycol Exchanger

- Glycol Regenerator Re-boiler

D044444

Feed Specification

## Product Specification

Vendor is required to guarantees below water content in outgoing dry gas.

The  maximum  concentration  of  water  in  the  dry  gas  at  the  outlet  of  the  gas  dehydration  package calculated based on Dew Point equal to  5 °C (determined  according to 10 degrees distance from minimum operating  temperature) and the result is that 11.63 Ib/MMSCF in winter case and 11.43 Ib/MMSCF in summer case.

## Control System

Packaged control type B is requested as per BK-GNRAL-PEDCO-000-IN-SP-0004.  Unit shall be fully remote controlled (monitoring and control functions) by the DCS and ESD systems. There is no VENDOR supplied control cabinet in control Room for these packages. LCP is required for main monitor/control commands such as burner control command. BMS logic shall be provided by vendor to be implemented in DCS/ESD.

The package is equipped with its instruments, wired by means of appropriate cables and cable routing. The cables are connected to the junction boxes located at the skid edge battery limits or at edge of base plate. All controls are incorporated into the DCS/ESD by the system Vendors.

## Turn Down Ratio

Sour gas dehydration package shall be designed in order to treat 35% of its normal capacity.