| Item No. | Clarification Description(Rev.00) | Conclusion (Rev.00) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description (Rev.01) | Conclusion (Rev.01) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description(Rev.02) | Conclusion (Rev.02) (VENDOR to Confirm & Consider in the Final Offers) |
| --- | --- | --- | --- | --- | --- | --- |
| - GENERAL |  |  |  |  |  |  |
|  | VENDOR shall confirm his full compliance to meet purchaser’s specifications, code and standard listed in specifications and mechanical data sheets, PIDs, Plot Plans and etc. Any deviations which not mentioned in deviation list will not be accepted by purchaser at detail stage. If there is not any Deviation or Exception from purchaser’s technical documents, VENDOR shall sign with "NO DEVIATION" and submitted with his proposal. | Confirm. | Noted. Closed. |  |  |  |
|  | VENDOR is sole responsible to consider this clarification and modify his offer based on this TCL. All modifications which need to be considered on the next revision of offer based on this clarification should be clearly marked with revision and cloud marks. All consequences due to changes without cloud marks are in VENDOR’s responsibility. Anyway, this clarification has priority over the offer. | Confirm. | Noted. Closed. |  |  |  |
|  | The VENDOR shall have overall responsibility for the coordination with his Sub-Vendors, and for the proper execution of any accessory item assembly, testing, delivery and any other service defined herein. | Confirm. | Noted. Closed. |  |  |  |
|  | All equipment shall be designed to meet the requirements shown in the Project Specification and data sheets. | Confirm. | Noted. Closed. |  |  |  |
|  | All documents required after purchase order shall be submitted in accordance with VENDOR documents requirements of the PMR. | Please kindly note that based on the scope of supply which would be air cooler, the marked item providing the document is not applicable. (Please refer to attached file). | There is no attached file on this clarification nor in vendor’s offer, please clearly let us know about the deviations. However, any deviation from the PMR. Will not be acceptable. | Please kindly note that based on the scope of supply which would be air cooler, the marked item providing the document is not applicable. (Not AAC scope of work) (Also attached file was refer to Technical proposal) | Scope of supply shall be fully supplied by vendor, otherwise, deviation form to be filled out. |  |
|  | All required special tools shall be considered in VENDOR main offer (they shall be supplied by bidder without any cost impact). A qualified and complete list has to be included in the bid documentation | Confirm. | Noted. Closed. |  |  |  |
|  | VENDOR shall confirm that Packing, Marking, Preservation and Preparation for Shipment will be done in accordance to project specification mentioned in PMR. | Confirm. | Noted. Closed. |  |  |  |
|  | VENDOR shall confirm project performance and mechanical guarantee based on data in PMR. | Air cooler Thermal, mechanical design and guarantee and performance guarantee is vendor scope of work in compliance with client requirement and specifications. | Noted. Closed. |  |  |  |
|  | VENDOR should confirm that all sub-vendors shall be selected from project Approved Bidder List (A.V.L.). In case of any deviation, bidder shall submit his deviation list for purchaser. Also VENDOR is fully responsible for his Sub-vendors. | Confirm. | Noted. Closed. |  |  |  |
|  | Material specification consists of class and relative grade shall be submitted by detail based on ASME/ ASTM for the whole package. VENDOR shall prepare material and material grade as per data sheets of equipment. Any substitution shall be expressed in deviation form. Meanwhile substitution after Purchase order is not approved. | Confirm. | Noted. Closed. |  |  |  |
|  | VENDOR to confirm his compliance with documents priority stated in PMR. Any conflict between the documents shall be notified by bidder to purchaser and resolved by his agreement. | Confirm. | Noted. Closed. |  |  |  |
|  | VENDOR shall confirm that surface preparation and painting of all equipment material including valves, pipes, fittings and everything within the package are in accordance with project painting specification. | Please kindly note that the air cooler does not include the pipes and valves. | Interconnecting pipe & valves (between bundles & bays) to be supply by vendor. | Please kindly note that the air cooler does not include (have) the pipes and valves, between bundles & bays. | Noted. Closed. |  |
|  | PURCHASER’s approval shall in no way be construed as relieving VENDOR of any of his obligation, responsibilities or liabilities under the CONTRACT and/or applicable Codes and Standards. | Confirm. | Noted. Closed. |  |  |  |
|  | In case of order, PURCHASER would not review documents sent by VENDOR before receiving VENDOR Print Index and Schedule. VENDOR Print Index and Schedule (VPIS) shall be prepared as first document after PO. All documents shall be issued after getting approval of this mentioned document. | Confirm. | Noted. Closed. |  |  |  |
|  | In case of order, VENDOR should provide her quality control plan (VQCP) based on requirements specified on MR and its attachments and also VENDOR’s experiences and send for approval. PURCHASER’s ITP and relevant specifications attached to MR are minimum requirement and VENDOR should consider them as a base and provide comprehensive Quality Control Plan. | Confirm. | Noted. Closed. |  |  |  |
|  | Providing Required Facilities for Inspection and Test shall be in VENDOR’s Scope of work. | Confirm. | Noted. Closed. |  |  |  |
|  | Utility consumption summary shall be submitted by VENDOR along with his proposal. | Confirm. | Noted. Closed. |  |  |  |
|  | All static and dynamic loads on foundations/Steel Structures shall be specified by VENDOR. | Confirm. | Noted. Closed. |  |  |  |
|  | All Native files of drawings & calculations (like CodeCalc; FEA; ANSYS; HTRI file; Auto-CAD DWG,..) shall be provided by bidder at detail stage. | Confirm. | Noted. Closed. |  |  |  |
|  | S.S. Name plates shall be fitted to all machines and equipment, according to project standard DWG. | Confirm. | Noted. Closed. |  |  |  |
|  | VENDOR shall submit the documents which are specified at “BID stage”. Fail to submit the necessary documents will be deemed as offer is disqualified. | Confirm. | Noted. Closed. |  |  |  |
|  | VENDOR shall confirm that project PMS will be followed for all pipes, valves, gaskets, etc. and also for Tie-in points in battery limit. | Confirmed for clause 5 in PMS. For materials, your data sheets will be followed. Also Please kindly note that the air cooler does not include the pipes and valves .The tie-in point as per PID would be as follow : Mechanical: flange to flange. Instrument: Instrument JB Motor: LV Motor Terminal box. | All the inter-connecting pipes and valves, as well as flanges and fittings, and in general all the inter-connecting components to be supplied by vendor, please confirm. | Please kindly note that the air cooler does not include (have) the pipes and valves, between bundles & bays. The tie-in point as per PID would be as follow : Mechanical: flange to flange. Instrument: Instrument JB Motor: LV Motor Terminal box. | Noted. Closed. |  |
|  | VENDOR shall consider all required consumable materials for site welding, inspection, tests and etc. of equipment, piping, structures and.... | Confirm. | Noted. Closed. |  |  |  |
|  | No fabrication shall be commenced until complete calculations. Shop drawings have been received and approved by purchaser. | Confirm. | Noted. Closed. |  |  |  |
|  | External supports & attachments, clips for piping (wherever required) shall be supplied by VENDOR. | Confirm. | Noted. Closed. |  |  |  |
|  | Please confirm that all spare parts (commissioning and two years) shall be sent with the equipment to site at the same time. | Confirm. | The spare parts should be packed and marked properly before the shipment at vendor’s shop. Packing should be done in accordance with packing and shipping procedure and should be done in a way that the keeping at the ware house will be possible properly. Please confirm. | Confirm. | Noted. Closed. |  |
|  | Vendor shall provide in his quotation, recommended spare parts list compatible with job specification for commissioning, two years of normal operation and capital spares with unit price and their validity period. Recommended spare parts shall have full description and enough data in accordance with related job specification (PMR & NIOEC-SP-40-03 & Doc. No. E&D-QC-SP-1). | Please be noted, spare part list has been changed based on this TCL, mechanical section, Items 38, 39, 40. Please confirm or clarify. | Noted. Closed. |  |  |  |
|  | Please confirm, any loose items include of electrical & instrument local panel, cables and pipes on battery limit of each equipment will be supplied by vendor. | Please kindly note that based on the MR and PID, the scope would be as per below and also, the piping is not in air cooler supply scope. - LV Motor - Instrument JB - Instruemnt Cable - I/P Converter, regulator and tubing to AV fan. | Inter-connecting piping (If applicable) is in vendor’s scope of work and supply. Please confirm. | Confirm. | Noted. Closed. |  |
|  | For all pressure parts and those required special consideration (such compliance with NACE) as well as for supports the material certificate should be in accordance with EN 10204, 3.1. Material certificate for non-pressure parts shall be in accordance with EN 10204, 2.2. Please confirm. | Confirm. | Noted. Closed. |  |  |  |
|  | Please confirm that all the attached documents listed in attachment No.1 of the material requisition with Ref. BK-GCS-PEDCO-120-ME-MR-0004_D00 are already in your possession. Some updated documents (latest revision) were handed over with this TCL. | Confirm. | Noted. Closed. |  |  |  |
| - PROCESS |  |  |  |  |  |  |
|  | Refer to PDS it shall be considered 10%extra on duty so please edit duty multiplier in thermal file to 1.1and redesign. | Confirmed. | Noted& Closed |  |  |  |
|  | Input maximum ambient temperature in thermal file too. | Confirmed. | Noted& Closed |  |  |  |
|  | Input data of ground clearance in your thermal file is not sufficient please recheck based on mechanical request and concept. | Confirmed. | Noted |  |  |  |
|  | Total tube sheet thickness which is input in your thermal file is not enough please revised and add tubes inserted from tube sheet too. | Total tube sheet thickness will be specified by mechanical calculations, and please be noted, it has no effect on thermal calculations. | Total tube sheet thickness and tubes extended from tube sheet shall be input in your thermal design file and it is essential in thermal design and its extra surface. | Confirm. | Noted& Closed |  |
|  | Please input plenum height in your thermal file. | Plenum height will be calculated by software, and it’s depend on fan & bay size, so shall not be specified by users and it has no effect on thermal calculations. | Bidder is responsible to select real plenum chamber type and input its height in the thermal file. | Confirm. Please be noted, due to small size of cooler, BOX plenum will be considered. | Noted& Closed |  |
|  | Please recheck and edit un-finned tubes length based on API and HTRI. | Its min 6mm based on API, AAC will be fabricated 6mm as well. But in thermal file, 20mm input for coverage worse cases. | It is uncorrected it shall be filled based on API 661 (7.1.11.6) and reduces tube sheet thickness and tubes extended from tube sheet (based on HTRI consideration for this tap) | This Item API 661 (7.1.11.6), is a fabrication tip and has no relevant to HTRI & thermal design. All data input in HTRI is based on considering worse case design. Also please be noted, Un-finned tube length is different from Un-heated tube length, Which tube sheet thickness is effected on Un-heated tube length. | Noted. Unheated tube length and un finned tube length both to be specified. |  |
|  | However, fins tip thickness are in vendor scope of work but AAC data input is deviated from our previous experience by this company it was about 0.2 mm. | 0.2 is for embedded fin type, in this project fins are extruded type with 0.48 base & 0.41 tip. | Noted & Closed |  |  |  |
|  | Barometric design pressure is 0.9142 bar. | Confirm. | Noted & Closed |  |  |  |
|  | Please redesign based on new attached heat curve. | Confirm. | Noted |  |  |  |
|  | Please design air-cooled with min humidity and fan selection based on maximum humidity. | Confirmed. | Noted it will be checked in your offer. |  |  |  |
|  | Please add and consider all PDS notes in your design too. | Confirmed. | But it is not implemented. | Please clarify which note has not been considered in design??? | Bundle to be self draining. The last row should 1% sloped. Please confirm. |  |
|  | Total static pressure drop for fan selection shall be total static pressure drop plus 20percent of drop of air across the bundle. | Confirmed. | But it is not implemented.it is different between your thermal file and AAC data sheet. |  |  |  |
|  | Please select correct type of plenum chamber type instead of software default. | Confirmed. | It is not revised | Due to small size of air cooler, suitable plenum is box and has been considered box. | Noted& Closed |  |
|  | New Items AE-2101 The other process cases of the exchanger shall be simulated &reported in AAC design too. | Confirmed. |  |  |  |  |
|  | New Items AE-2101 - HC inlet temperature as per PDS is 124.8°C and will be finalized by compressor vendor. Please edit in AAC data sheet. - Inlet operating pressure as per PDS is 19 bar g please edit in your data sheet. - All thermal data in AAC data sheet has been deviated from AAC thermal file. They shall be uniformed. | Confirmed. |  |  |  |  |
|  | New items AE-2102 Minimum ambient temperature is different with Hirgan’s data sheet in winter case. |  |  |  |  |  |
| - MECHANIC |  |  |  |  |  |  |
|  | Following documents shall be submitted; 1.Sub-vendor list 2.Deviation list (project deviation format shall be signed and attached) 3.Quality plan 4.All revised general arrangements based on new thermal design 5.Air cooler estimated weight 6.Delivery plan 7.Reference list | Confirm. 5- will be submitted after finalizing thermal design. | This clause will be closed after finalizing the thermal design. |  |  |  |
|  | Breakdown price of following items shall be included in the proposal, as well as total price. 1) Material and Fabrication for each Section Separately 2) Pre-commissioning & commissioning spare parts (END-QC-SP-1) 3) 2 years operational spare parts (END-QC-SP-1) 4) Packing & transportation 5) Other fee (if any) | Confirm. | Noted. Closed. |  |  |  |
|  | Vendor to confirm the project specifications with Ref. BK-GCS-PEDCO-120-ME-SP-0001_D00 and BK-GNRAL-PEDCO-000-ME-SP-0001_D00 are also considered for design of air coolers in addition of ASME Sec. VIII Div.1 and API 661. Also IPS standards IPS-E-PR-785 and IPS-G-ME-245 are to be considered. | Confirm. | Noted. Closed. |  |  |  |
|  | Following works to be considered as exclusion of vendor scope; 1.Fire proofing material 2.Permanent gasket and bolt of process nozzle | Confirm. | Inter connecting bolt and nuts, as well as gaskets and pipes (If required) to be supplied by vendor. Please confirm. | Confirm. | Noted. Closed. |  |
|  | Following items shall be quoted as option in vendor proposal: 1- Shop & site run in test (including vibration verification) Noted that trial assembly to be included in shop run in test (for one bay per item). 2- Site supervision for installation Please confirm and revise your offer. | Confirm. | Noted. Closed. |  |  |  |
|  | Mechanical guarantee and Guarantee period to be included in BID. | Confirm. | Noted. Closed. |  |  |  |
|  | A summary table of air coolers including items No, unit No, Bay No., bundle No., weights and power consumption is needed in vendor proposal. | Confirm. Weights will be submitted after finalizing thermal design. | The summary table for both winter and summer cases to be reported, moreover, all reported data to be in strict accordance with of thermal calculations and Hirgan Process data sheet. | Confirmed. Please refer to Technical proposal page 3. | Noted. Closed. |  |
|  | Vendor should consider all Mechanical data sheet notes and implement them on their designs and data sheets. | Confirm. | Noted. Closed. |  |  |  |
|  | Bidder shall confirm to verify nozzle external loads calculations according to API 661 PART 7.1.10 by Finite Element Analysis (nozzle loads shall be used two (3) times of API 661). Please add it as a note in relevant data sheets. | Confirm. | 3 X API nozzles loads to be taken into consideration, please confirm. | Confirmed. | Noted. Closed. |  |
|  | Vendor shall consider bundle expansion on structure. | Confirm. | Consequently, sliding pad on the sliding header to be provided by vendor, its material and characteristics to be the same as project specifications / reference standards, sliding header to be considered oppose to the front header (Where the nozzles located), please confirm. | Confirmed. | Noted. Closed. |  |
|  | Possibility of lateral header movement to be considered by vendor. | Noted. | Mechanical calculations for lateral force to be also done. | Confirmed. | Noted. Closed. |  |
|  | Requirements of Expanded tube to tube sheet joint, No. of grooves should be followed as per clauses 9.3.2, 9.3.3 of API 661 and TEMA Class R requirements. As regards special services like H2S, lethal and hydrogen services strength welding is required. | Requirements of API 661 will be considered. TEMA is for tubular heat exchangers. | Please confirm that TEMA will be followed for tube to tube sheet junctions along with API 661. | Confirmed. API 661 has some minor discrepancies with TEMA with this regard. So API 661 will be considered where discrepancy existed. | The one which is more stringent one to be followed, the matter will be finalized during detail design, at this stage, please consider the most stringent one please. |  |
|  | Tube hole diameters, tolerances, grooving and finish shall be in accordance with TEMA Class R. Tubes shall be expanded into the tube sheet up to within 3mm (1/8") of the airside face of the tube sheet. Minimum tube sheet thickness shall be 25 mm. | Requirements of API 661 will be considered. TEMA is for tubular heat exchangers. | Please confirm that TEMA will be followed for the matter which is mentioned in this clause along with API 660, moreover, in no case the expansion length shall not be less than the tube O.D. please confirm. | API 661 has some minor discrepancies with TEMA with this regard. So API 661 will be considered where discrepancy existed. API 660 is for shell and tube heat exchangers. | The one which is more stringent one to be followed, the matter will be finalized during detail design, at this stage, please consider the most stringent one please. |  |
|  | The minimum thickness of tube sheets and plug sheets in case of plug type headers and removable cover plate headers shall be in accordance with TEMA standard but shall not be less than 25mm including corrosion allowance. | Confirmed. | Noted. Closed. |  |  |  |
|  | On the side frames there shall be lugs for lifting the bundles stiffly welded to the side members and with a hole of 25 mm minimum. | Confirm. | Noted. Closed. |  |  |  |
|  | Fan guards for air coolers shall be furnished to provide personnel protection during operation. | Noted. | Air seal to be provided on bundle in order not to let the air flows from the hollow spaces. | Confirmed. | Noted. Closed. |  |
|  | No carbon steel parts to be welded shall have a carbon content higher than 0.25%. please confirm. | Confirm. | Noted. Closed. |  |  |  |
|  | Material requirement should be in compliance with NACE MR 0175/ISO 15156 and Technical Specification for Material Requirements in Sour Service Doc. No. BK-GNRAL-PEDCO-000-PI-SP-0008. Please confirm. | Confirmed. | Noted. Closed. |  |  |  |
|  | Corrosion allowance shall be multiplied by two for pass partition plates or stiffeners. Please confirm. | Confirmed. | Noted. Closed. |  |  |  |
|  | All header flanges shall be welding neck type. Vent and drain nozzle shall be 2” LWN type with ASME rating flange and blinded. All nozzles smaller than 2” to be LWN. Please confirm and revise your technical offer accordingly. | Confirmed. | Noted. Closed. |  |  |  |
|  | Hydrostatic test shall be in accordance with the requirements of ASME code Sec. VIII Div.1. Please confirm. | Confirmed. | Noted. Closed. |  |  |  |
|  | All butt welds in headers shall be examined by full RT. All corners welds shall be full UT. Please confirm. | Confirmed. | Noted. Closed. |  |  |  |
|  | Fan rings, Fan deck, Plenum, walkway grating including boltings shall be hot dip galvanized | Confirmed. | Noted. Closed. |  |  |  |
|  | Material of louver blades (if required) shall be high quality galvanized carbon steel. Please confirm. | Confirmed. | Noted. Closed. |  |  |  |
|  | TI/PI and vent and drain nozzle sizes shall be specified in relevant data sheet. | Confirmed. | Noted. Closed. |  |  |  |
|  | Fan performance curves for all fans shall be included to relevant data sheet in quotation. | Confirmed. | Noted. Closed. |  |  |  |
|  | Bidder to clarify mechanical efficiency of separate power transmissions Em value and related reference. | Confirm. | Noted. Closed. |  |  |  |
|  | Access Platform, Maintenance Platform (if required) and Ladders for access to platforms and platform’s gratings are in vendor scope of supply. please confirm Access Platform: Shall be provided on all the four sides. Maintenance Platform : Full steel platform duly interconnected along full length and width of air coolers for fan/motors etc. | Confirm. | Noted. Closed. |  |  |  |
|  | Platforms shall be a minimum of 750 mm wide and 2 m headroom. Design load of platforms shall be 500 kg/m^2. | Confirm. | Noted. Closed. |  |  |  |
|  | The effect of the pipe supports loads on your structure shall be borne by your side without cost impact according to your experience. just the additional price for pipe support steel structure weight can be paid. | Confirm. | Noted. Closed. |  |  |  |
|  | For gasket material and relevant flange specification of blinded flanges, vendor shall follow Piping material specification. | Noted. Note 22 of your data sheets will be considered for flange gasket material. | Gaskets for nozzles shall be spiral wound type, graphite filled with inner ring S.S.316 and outer ring S.S 316. Please confirm. | Confirmed. | Noted. Closed. |  |
|  | A vibration switch shall be provided one per motor for each bay within the equipment. The mounting location shall be carefully selected to represent high vibration / trip in case of bearing / fan damage and ease of maintenance. | Confirmed. | Vibration switch type and locations to be clearly shown on the GA. Drawing. Please confirm. | Confirmed. Would be provided In GA after P.O. |  |  |
|  | Motors electrical area classification and service factor shall be specified. | EExde IIB T3 motor is considered. The SF of motor is 1. | Based on mechanical data sheet, EExd, IIB, T3, (IP 55) shall be considered by vendor. | Confirmed. | Noted. Closed. |  |
|  | Minimum stock thickness for extruded fins shall be 0.40 mm. This thickness shall apply at the root of the fin. Fins Stock thickness shall be specified in related data sheet. | Confirmed. | Noted. Closed. |  |  |  |
|  | The maximum allowable number of fins is 400 per meter of the tube length. In order to prevent fouling, fin surfaces shall be smooth. | Confirmed. | Noted. Closed. |  |  |  |
|  | The width of the tube bundle shall be chosen with due regard to transport and handling aspects, and shall not, in general, exceed 3 meters. At least one inlet nozzle is required per meter of bundle width. | Confirmed. | Noted. Closed. |  |  |  |
|  | Spare part for commission and 2 years are included at scope of supply and not as optional. | Confirmed. | Noted. Closed. |  |  |  |
|  | Please confirm to consider the following items in spare parts for pre-commissioning, commissioning and start-up in the scope of supply. Plug: 10% Plug gaskets: 10% (min. 2 sets). Standard bearing: minimum 20% per size and type and material Gaskets for blind nozzles: 100% (min. one set of gasket per each type) Stud bolts & nuts & washer : 5% (min. 2 sets). | Confirmed. | Noted. Closed. |  |  |  |
|  | Please confirm to consider the following items in spare parts for 2 years operation in the scope of supply. Plugs Steel 1% & Nonferrous 2% (minimum one number) Plug gasket 5% (minimum one number) Tube support 10% (minimum one number) Gaskets for blind nozzles: 200% (min. two set of gasket per each type) Stud bolts & nuts & washer : 10% (min. 2 sets). The following items as per project requirements attached to MR. Blade Bearings Belts Bolting for Structure Fan Blades Vibration Detectors | Confirmed. | Noted. Closed. |  |  |  |
|  | Please confirm to consider the following items in Special tools in the scope of supply as minimum. 1-Level Gauge for Pitch Angle Setting of Fan Blades 2-Torque Wrench for Fan Blade Setting Bolts 3-Torque Wrench for Header Plugs | Confirmed. | Noted. Closed. |  |  |  |
|  | Preservation (N2 purging) shall be considered in vendor scope of supply / work. Please reconfirm. | Confirmed. | Noted. Closed. |  |  |  |
|  | All plates used in sour service shall be examined by ultrasonic test for lamination, please confirm compliance. | Confirmed. | Noted. Closed. |  |  |  |
|  | Belt drivers shall be anti-static, Please confirm. | Confirmed. | Noted. Closed. |  |  |  |
|  | All driver, fans, bearings shall be waterproof for outdoor installation. Please confirm. | IP55 for motor is considered. | Noted. Closed. |  |  |  |
|  | Earth lugs is in vendor scope of supply. Shall be considered, Please revise your offer accordingly. | Confirmed. | Noted. Closed. |  |  |  |
|  | Rotor shaft power at API point shall be specified at all fan characteristics and fan performance curves. Please revise your offer accordingly. | Confirmed. | Noted. Closed. |  |  |  |
|  | Structure surface preparation and coating shall be hot deep galvanized (ASTM A123). Please confirm and revise offer accordingly. | Confirm. | Noted. Closed. |  |  |  |
|  | Fan hub shall be hot deep galvanized. Please confirm. | Confirmed. | Noted. Closed. |  |  |  |
|  | Air humidity to be corrected in fan data sheet. Worse case humidity shall be considered in fan calculation.(100%) | Confirmed. | Noted. Closed. |  |  |  |
|  | Half of fans (50%) shall be equipped with variable pitch automatic system to control fluid temperature, please revise offer accordingly. Related data sheet for fan. This price shall be included SS 316 Tube fitting, actuators, positioner, IP convertor and all necessary parts for pneumatic circuit. Please confirm. | Confirmed, however, based on PID for both fan HY is considered. Please clarify. | The latest revision of PID is attached. It should be reviewed by vendor and confirmed. | Please kindly note that still in PID for all fans the I/P is mentioned. Please clarify. |  |  |
|  | Draft type of fans is induced, Please revise offer accordingly. | Confirmed. | Noted. Closed. |  |  |  |
|  | Plug gasket to be solid metal. Please confirm. | Confirmed. | Noted. Closed. |  |  |  |
|  | The last pass of tubes shall be 1% slope. Please revise your offer accordingly. | Confirmed. | Noted. Closed. |  |  |  |
|  | Flange rating shall be revise as per MDS, Please revise your offer accordingly. | Confirmed. | Noted. Closed. |  |  |  |
|  | Louver to be considered, please revise offer accordingly. | Manual Louver to open and close by lever would be considered. | Noted. Closed. |  |  |  |
|  | NEW ITEM: Please specify and report the winter case and summer case individually. | Confirmed. | Noted. Closed. |  |  |  |
|  | NEW ITEM: Heat exchanger's duty to be derived from HTRI file, conflicts to be removed. | Confirmed. | Noted. Closed. |  |  |  |
|  | NEW ITEM: Flow rate is different with Hirgan's data sheet as well as Vendor's HTRI file. Please remove the discrepancy. | Confirmed. | Noted. Closed. |  |  |  |
|  | NEW ITEM: Inlet Temp is dissimilar to Hirgan's data sheet and vendor's HTRI file. Please enhance the difference. | Confirmed. | Noted. Closed. |  |  |  |
|  | NEW ITEM: Face velocity Input value in HTRI file is not similar with this value. please revise. | Confirmed. | Noted. Closed. |  |  |  |
|  | NEW ITEM: Tube Length is 3.048 m in HTRI output. Please check and remove the discrepancy with data sheet. | Confirmed. | Noted. Closed. |  |  |  |
|  | NEW ITEM: Nozzle sizes are specified in Hirgan's data sheet and HTRI file and vendor’s data sheet are not the same, also, nozzle schedules should be finalized after mechanical design. please correct. | Confirmed. | Noted. Closed. |  |  |  |
|  | NEW ITEM: Above mentioned comments similarly to be seen in Item AE-2102. | Confirmed. | Noted. Closed. |  |  |  |
| - PIPING & MATERIAL |  |  |  |  |  |  |
|  | In Vendor Technical Offer in page 21 of 38, for AE-2101 rating of nozzles shall be 300#. | Confirmed. | Closed. |  |  |  |
|  | In Vendor Technical Offer in page 28 of 38, for AE-2102 rating of nozzles shall be 600#. | Confirmed. | Closed. |  |  |  |
|  | In Vendor Technical Offer in pages 25 & 32 of 38, Elevation of TOS (Top of Structure) will be at least 5.5 meter. | Confirmed. | Closed. |  |  |  |
|  | VENDOR shall specify maximum allowable forces and moments at the nozzles for review and purchaser approval. In case of unavoidable over loaded at the nozzles, then the loads should be submitted to the VENDOR for consideration and approval. | According to doc: Specification For Air Cooled Heat Exchangers, page 15 of 27, clause 7.1.10.1: max allowable nozzle loads=3× API will be considered in design stage. Any cost impact caused by over loads shall be considered. | Closed. |  |  |  |
|  | Material requirement should be in compliance with NACE MR 0175/ISO 15156 and Technical Specification for Material Requirements in Sour service Doc. No. BK-GNRAL-PEDCO-000-PI-SP-0008. | Confirmed. | Closed. |  |  |  |
|  | Surface preparation & coating shall be acc. to "Specification for Painting" with Doc. No. BK-GNRAL-PEDCO-000-PI-SP-0006. | Confirmed. | Closed. |  |  |  |
|  | Plug Material shall be SA-182 F316L. | Confirmed. | Closed. |  |  |  |
|  | Bundle frame should be hot dip galvanized. | Confirmed. | Closed. |  |  |  |
|  | Louvers material should be SA-283 Gr. C. | SA-283 is not applicable. Please be noted, based on API, louver material will be considered as ST-37. | St-37 does not mention in API. Data sheet shall be followed. | Confirmed. |  |  |
| - INSTRUMENT & CONTROL |  |  |  |  |  |  |
|  | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0004 for package control type. (for this package Type B only Electrical LCP is required). Signals from Vibration switch and signal to I/p Converter shall be transferred to pland ESD/DCS). VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | -Please kindly note that this document is not attached. However, as per PID for motors LCS is mentioned which its scope would be clarified by your electrical Dep. -Different cable type and separated JB would be considered for VS and I/P converter. | Closed |  | As per Electrical Dep. Command, LCS is out of vendor scope. VS control signal cable and JB is scope out if VS signal is 24 VDC. I/P control command cable and JB is scope out if signal is 4-20 mA, intrinsically safe. Otherwise if cables are special type vendor shall prepare them. |  |
|  | Regarding supplying instrument on datasheet), bidder is requested to consider all instruments mentioned on project datasheet. VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0001 for instrumentation devices and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list | Please kindly note that there is no specific item for vibration switch and I/P converter, as a result, we have attached the sample DS. | As minimum, general criteria regarding switch rating, instrument material, voltage range,… shall be followed by vendor as per project specification | Confirmed. | Specification for switch: •The switches shall be EExd, IIB T4 and the weatherproof requirements of IP-65 as per IEC-60529. •Switch element shall be of the micro switch snap action type and hermetically sealed. Micro-switches shall be two double pole double throw (DPDT) type •The switch contact rating shall be 110VAC @10Amp. •Repeatability shall be ±1% or better of full scale •The accuracy of the pressure switch assembly shall be at least 1% of the span |  |
|  | - VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0010 for cables and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed. | Closed. |  |  |  |
|  | Instrument selection, connection & cabling (including vibration switches, I/P converters and 50% fans controlling devices) shall be acc. to "Specification For Instrumentation" Doc. No. BK-GNRAL-PEDCO-000-IN-SP-0001 | Please kindly note that there is no specific item for vibration switch and I/P converter , as a result , we have attached the sample DS | As minimum, general criteria regarding switch rating, instrument material, voltage range,… shall be followed by vendor as per project specification | Confirmed. |  |  |
|  | All terminals shall be screw type. VENDOR shall note that each core/pair/etc. of cable shall be wired to dedicated terminal | Confirmed. | Closed. |  |  |  |
|  | VENDOR shall specify the estimated power required (non-UPS and UPS) | Please refer to attached load list. | Closed. |  |  |  |
|  | Cable from LCP/Instruments to Control/Electrical building is by contractor except for special cables (if it is not as per our specifications) which shall be supplied by vendor. | Confirmed .Our instrument cable from VS and I/P converter to JB would be provided. | Closed. |  | VS control signal cable and JB is scope out if VS signal is 24 VDC. I/P control command cable and JB is scope out if signal is 4-20 mA, intrinsically safe. Otherwise if cables are special type vendor shall prepare them. |  |
|  | 50% of fans shall be AV type. All required accessories to be considered. | I/P Converter, tubing, regulators would be considered. | Closed. |  |  |  |
|  | Vibration switch to be (EExd,IIB,T4, IP65, suitable for 110V@10A rating with 24VDC signal ), please confirm. | Please kindly note that based on the BK-GNRAL-PEDCO-000-IN-SP-0001-D02 “The switch contact rating shall be 110VAC @10Amp.Also, based on your comment; contact should be suitable for 24VDC as well? If it is so, It is confirmed. | Closed. |  |  |  |
| - SAFETY |  |  |  |  |  |  |
|  | All electrical equipment shall be certified for Zone IIB, T3. | Confirmed. | Closed. |  |  |  |
|  | The sound pressure level anywhere in the work area shall not exceed 85 dBA. | Confirmed. | Closed. |  |  |  |
| - ELECTRICAL |  |  |  |  |  |  |
|  | Electrical system scope of work shall be clarified by vendor & any deviation shall be reported. | Please kindly note that the based on client MR , the scope would be : Electric motors, also clarify whether the LCS for stop and start the motor locally, is in vendor scope of supply or not? | Confirmed but LCS is in client scope of work. | Confirmed. |  |  |
|  | Vendor list & sub vendor list of manufacture shall be according to project AVL. | Confirmed. | Closed. |  |  |  |
|  | All electrical equipment shall comply with electrical references & standard that mentioned on material requisition. (IPS-E-EL-100) | BK-GNRAL-PEDCO-000-EL-SP-0010_D02, LV motor specification would follow. | BK-GNRAL-PEDCO-000-EL-SP-0010-D03 Closed | Kindly furnish us with the latest revision of the document. |  |  |
|  | Motor shall be manufactured & tested in accordance with the latest applicable sections of the IEC standards. Project specification shall be applied as well. | Confirmed. | Closed. |  |  |  |
|  | Environmental criteria for design of electrical systems that shall be considered by vendor (According to Electrical System Design Criteria) | De-rating factor would be considered for motor sizing. Based on fan data sheet AE-2101 A-B-C, The rotor shaft power would be 2.2*1.1=2.42 Kw =Required absorbed power at +51 and 12.5m elevation. By considering the de-rating factor as : 0.89 , we have : 0.89 * 5.5=4.89 Kw which is higher than 2.42Kw. Also, the required absorbed power based on API661 at min.Temp would be 3.2Kw . The same calculation goes for AE-2102 A-B-C , so we have : 3 *1.1=3.3 Kw which is less than 4.89Kw and also the absorbed power based on API661 at min.Temp would be 4.3Kw So for both motor 5.5 Kw is fine. | Calculation will be checked after PO, but motor sizing shall satisfy all requirements. | Confirmed. API factor and de-rating factor would be considered for motor sizing. |  |  |
|  | Hazard Area classification, Gas group and temperature class of motor shall be at least Exd, IIB, T3, with protection degree of IP54 for motor enclosure and IP55 for terminal box. | Please kindly note that based on process data sheet , the air cooler would be installed on zone 2, so AAC is proposed “ EExde” type | Exd, IIB, T3, shall be considered by vendor. | Confirmed. |  |  |
|  | All accessories related to cable entry/termination in electrical equipment (motors, JBs, etc.) such as cable glands & etc. shall be supplied by vendor. | Confirmed. | Closed. |  |  |  |
|  | Load list shall include all motor & non-motor (if there is). For motors, Mechanical power (BHP), Rated power, efficiency, power factor and rated current shall be specified in the proposal. | Please kindly note that since motor is not finalized , the value of the efficiency , power factor and so on would be provided after P.O | Closed. |  |  |  |
|  | API factor, effect of ambient condition especially de-rate factor for altitude shall be considered for specifying rated power of motor and procedure of obtaining rated power of motor from BHP shall be cleared in the proposal. | Confirmed. | Closed. |  |  |  |
|  | A complete set of spare parts for commissioning shall be supplied for each motor. | Confirmed. | Closed. |  |  |  |
|  | The equipment shall be factory tested. Certified copies of test reports shall be submitted to the purchaser. | Confirmed. | Closed. |  |  |  |
|  | Supplier shall design the routing of the power & control cable inside package. | Confirmed. | Closed. |  |  |  |
|  | Supply of cables between Junction boxes and Package control panel is in vendor scope of work. In other word, All cabling listed in “Customer cable table” shall be supplied and connected by customer/contractor. Cables shall be according to “BK-GNRAL-PEDCO-000-EL-SP-0014”. | Please kindly note that UCP is not in supplier scope of work and vibration would be signaled to ESD and I/P converter to DCS .The tie –point would be instrument JB. | Closed. |  |  |  |
|  | If UPS & batteries are required, vendor shall provide all necessary documents (such as calculation, list etc.) & supply required equipment according to purchaser standards. | There is no UCP and need of battery in scope of supply of air cooler | Closed. |  |  |  |
|  | According to Project Electrical Load List, there are 12 number of Air cooler fans since in page 36 of Offer, there are only 6 numbers of motors. Vendor load list shall be revised. Project Load List has been attached to document file. | Noted. Please kindly note that the project format is not attached, however, it would be provided after P.O when the motor vendor is finalized. | Closed. |  |  |  |
| - CIVIL & STRUCTURE |  |  |  |  |  |  |
|  | Based on STD2800 this site is located at High Risk Area which is named Seismic Zone 2. Also according to ASCE 7 (2010) Seismic Zone is equal to Zone 3 with Seismic Importance Factor 1.25. Besides according to INBC-Part 6 amount of Wind Speed shall be considered 120 Km/hr. It is necessary to consider the above in the data sheets. | Please send to us civil design criteria and specify soil characteristics to determine seismic force. | Based on STD2800 this site is located at High Risk Area which is named Seismic Zone 2 with specific gravity 0.3. Besides, based on ASCE 7-2010 soil is considered in class D. Please consider below parameters in your calculations: Ss = 1.25 S1 = 0.58 Site Coefficient Fa =1.0 Site Coefficient Fv =1.5 SMS=Fa SS SM1=FV S1 PGA = 0.3 Site Coefficient FPGA = 1.2 SDS =0.83 SD1 =0.58 | Confirmed. ASCE 7-2010 will be considered for calculation of seismic load. |  |  |