| طرح نگهداشت و افزایش تولید 27 مخزن |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Standard Detail Drawing for Storage Tanks نگهداشت و افزایش تولید میدان نفتی بینک |  |  |  |  |  |  |  |
| D02 |  | OCT.2022 | FI | H.Adineh | M.Fakharian | M.Mehrshad |  |
| D01 |  | AUG.2022 | IFI | H.Adineh | M.Fakharian | M.Mehrshad |  |
| D00 |  | JUL. 2021 | IFI | M.Asgharnejad | M.Fakharian | Sh.Ghalikar | CLIENT Approval |
| Class: 2 |  |  | CLIENT Doc. Number: F2D-707124 |  |  |  |  |
| Status: |  | IDC: Inter-Discipline Check IFC: Issued For Comment IFA: Issued For Approval AFD: Approved For Design AFC: Approved For Construction AFP: Approved For Purchase AFQ: Approved For Quotation IFI: Issued For Information AB-R: As-Built for CLIENT Review AB-A: As-Built –Approved |  |  |  |  |  |

REVISION RECORD SHEET

| PAGE | D00 | D01 | D02 | D03 | D04 |  | PAGE | D00 | D01 | D02 | D03 | D04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | X |  |  |  |  | 66 |  |  |  |  |  |
| 2 | X | X |  |  |  |  | 67 |  |  |  |  |  |
| 3 | X | X |  |  |  |  | 68 |  |  |  |  |  |
| 4 | X | X |  |  |  |  | 69 |  |  |  |  |  |
| 5 | X |  |  |  |  |  | 70 |  |  |  |  |  |
| 6 | X | X |  |  |  |  | 71 |  |  |  |  |  |
| 7 | X |  |  |  |  |  | 72 |  |  |  |  |  |
| 8 | X | X |  |  |  |  | 73 |  |  |  |  |  |
| 9 | X | X |  |  |  |  | 74 |  |  |  |  |  |
| 10 | X | X |  |  |  |  | 75 |  |  |  |  |  |
| 11 | X | X |  |  |  |  | 76 |  |  |  |  |  |
| 12 | X | X |  |  |  |  | 77 |  |  |  |  |  |
| 13 | X | X |  |  |  |  | 78 |  |  |  |  |  |
| 14 | X | X |  |  |  |  | 79 |  |  |  |  |  |
| 15 | X | X |  |  |  |  | 80 |  |  |  |  |  |
| 16 | X | X |  |  |  |  | 81 |  |  |  |  |  |
| 17 | X | X |  |  |  |  | 82 |  |  |  |  |  |
| 18 | X | X |  |  |  |  | 83 |  |  |  |  |  |
| 19 | X | X |  |  |  |  | 84 |  |  |  |  |  |
| 20 | X | X |  |  |  |  | 85 |  |  |  |  |  |
| 21 | X | X |  |  |  |  | 86 |  |  |  |  |  |
| 22 | X | X |  |  |  |  | 87 |  |  |  |  |  |
| 23 | X | X |  |  |  |  | 88 |  |  |  |  |  |
| 24 | X | X |  |  |  |  | 89 |  |  |  |  |  |
| 25 | X | X |  |  |  |  | 90 |  |  |  |  |  |
| 26 | X | X |  |  |  |  | 91 |  |  |  |  |  |
| 27 | X |  |  |  |  |  | 92 |  |  |  |  |  |
| 28 | X |  |  |  |  |  | 93 |  |  |  |  |  |
| 29 | X |  |  |  |  |  | 94 |  |  |  |  |  |
| 30 | X |  |  |  |  |  | 95 |  |  |  |  |  |
| 31 | X |  |  |  |  |  | 96 |  |  |  |  |  |
| 32 | X |  |  |  |  |  | 97 |  |  |  |  |  |
| 33 | X |  |  |  |  |  | 98 |  |  |  |  |  |
| 34 | X |  |  |  |  |  | 99 |  |  |  |  |  |
| 35 | X |  |  |  |  |  | 100 |  |  |  |  |  |
| 36 | X |  |  |  |  |  | 101 |  |  |  |  |  |
| 37 | X | X |  |  |  |  | 102 |  |  |  |  |  |
| 38 | X |  |  |  |  |  | 103 |  |  |  |  |  |
| 39 | X |  |  |  |  |  | 104 |  |  |  |  |  |
| 40 | X |  |  |  |  |  | 105 |  |  |  |  |  |
| 41 | X |  |  |  |  |  | 106 |  |  |  |  |  |
| 42 | X |  |  |  |  |  | 107 |  |  |  |  |  |
| 43 | X |  |  |  |  |  | 108 |  |  |  |  |  |
| 44 | X |  |  |  |  |  | 109 |  |  |  |  |  |
| 45 | X |  |  |  |  |  | 110 |  |  |  |  |  |
| 46 | X |  |  |  |  |  | 111 |  |  |  |  |  |
| 47 | X |  |  |  |  |  | 112 |  |  |  |  |  |
| 48 | X |  |  |  |  |  | 113 |  |  |  |  |  |
| 49 | X |  |  |  |  |  | 114 |  |  |  |  |  |
| 50 | X |  |  |  |  |  | 115 |  |  |  |  |  |
| 51 | X |  |  |  |  |  | 116 |  |  |  |  |  |
| 52 | X |  |  |  |  |  | 117 |  |  |  |  |  |
| 53 |  |  |  |  |  |  | 118 |  |  |  |  |  |
| 54 |  |  |  |  |  |  | 119 |  |  |  |  |  |
| 55 |  |  |  |  |  |  | 120 |  |  |  |  |  |
| 56 |  |  |  |  |  |  | 121 |  |  |  |  |  |
| 57 |  |  |  |  |  |  | 122 |  |  |  |  |  |
| 58 |  |  |  |  |  |  | 123 |  |  |  |  |  |
| 59 |  |  |  |  |  |  | 124 |  |  |  |  |  |
| 60 |  |  |  |  |  |  | 125 |  |  |  |  |  |
| 61 |  |  |  |  |  |  | 126 |  |  |  |  |  |
| 62 |  |  |  |  |  |  | 127 |  |  |  |  |  |
| 63 |  |  |  |  |  |  | 128 |  |  |  |  |  |
| 64 |  |  |  |  |  |  | 129 |  |  |  |  |  |
| 65 |  |  |  |  |  |  | 130 |  |  |  |  |  |

CONTENTS

1.0	INTRODUCTION	4

2.0	Scope	4

3.0	NORMATIVE REFERENCES	5

3.1	Local Codes and Standards	5

3.2	International Codes and Standards	5

3.3	The Project Documents	6

4.0	Standard drawings	7

4.1	ANCHOR BOLT BOX	8

4.2	FOAM DISTRIBUTOR FOR FIXED ROOF	9

4.3	HINGE AND DAVIT DETAILS FOR MANHOLES	10

4.4	VORTEX BREAKER DETAILS	12

4.5	ANCHOR BOLTS	14

4.6	OPEN VENT	22

4.7	GROUNDING	23

4.8	DIMENSIONAL TOLERANCES FOR STORAGE TANKS	24

4.9	NAME PLATE	25

4.10	DETAILS OF FLANGED & STUD REINFORCED NOZZLES FOR TANKS	287

4.11	STAIRWAYS	28

4.12	PLATFORMS	29

4.13	FITTING FOR VERTIVAL TANK ARRANGMENT AT BLENDING & TOOL	30

4.14	FITTING FOR VERTIVAL TANK 24” DIA ROOF MANHOLE	31

4.15	FITTING FOR VERTIVAL TANK DIPPING PLATFORM (1/2)	32

4.16	FITTING FOR VERTIVAL TANK DIPPING PLATFORM (2/2)	33

4.17	WELDED VERTICAL STORAGE TANK DRAW OFF SUPPORT (1/2)	34

4.18	WELDED VERTICAL STORAGE TANK DRAW OFF SUPPORT (2/2)	35

4.19	TANKAGE SHELL NOZZLES	36

4.20	STANDPIPES FOR 12 m HIGH TANKS	37

4.21	STANDPIPES FOR 8, 12.5 AND 20m DIA. TANKS	38

4.22	PLATFORM STRUCTURE AT STANDPIPES FOR 10 m HIGH TANKS	39

4.23	PLATFORM STRUCTURE AT STANDPIPES FOR 12 m HIGH TANKS	40

4.24	PLATFORM STRUCTURE AT STANDPIPES PLAN AND DETAILS	41

4.25	STRUCTURE WATER AND FUEL TANK	42

4.26	DETAILS OF SHELL PLATE	43

4.27	DETAILS OF OBSERVATION WINDOW	44

4.29	DETAILS OF PIPE SUPPORT	45

4.29	TANK ASSEMBLY	46

4.30	LOCATION OF APPURTENANCES	47

4.31	DETAILS OF BOTTOM AND ROOF PLATE	48

4.32	DETAILS OF ROOF STAGE	49

4.33	DETAILS OF INLET NOZZLE	50

4.34	DETAILS OF OUTLET NOZZLE	51

- INTRODUCTION

Binak oilfield in Bushehr province, a part of the southern oilfields of Iran, is located 20 km northwest of Genaveh city.

With the aim of increasing production of oil from Binak oilfield, an EPC/EPD Project has been defined by NIOC/NISOC and awarded to Petro Iran Development Company (PEDCO). Also PEDCO (as General Contractor) has assigned the EPC-packages of the Project to "Hirgan Energy - Design and Inspection" JV.

GENERAL DEFINITION

The following terms shall be used in this document.

| CLIENT: | National Iranian South Oilfields Company (NISOC) |
| --- | --- |
| PROJECT: | Binak Oilfield Development – General Facilities |
| EPD/EPC CONTRACTOR: | Petro Iran Development Company (PEDCO) |
| EPC CONTRACTOR: | Joint Venture of : Hirgan Energy – Design & Inspection(D&I) Companies |
| VENDOR: | The firm or person who will fabricate the equipment or material. |
| EXECUTOR: | Executor is the party which carries out all or part of construction and/or commissioning for the project. |
| THIRD PARTY INSPECTOR (TPI): | The firm appointed by EPC CONTRACTOR and approved by CLIENT (in writing) for the inspection of goods. |
| SHALL: | Is used where a provision is mandatory. |
| SHOULD: | Is used where a provision is advisory only. |
| WILL: | Is normally used in connection with the action by CLIENT rather than by an EPC/EPD CONTRACTOR, supplier or VENDOR. |
| MAY: | Is used where a provision is completely discretionary. |

- Scope

The purpose of this document is to provide the standard drawing for the accessories of storage    tanks used in this project.

It shall be used in conjunction with data/requisition sheets for present document subject.

- NORMATIVE REFERENCES

## Local Codes and Standards

- IPS-G-ME-100 	Iranian Petroleum Standard – General Standard for Atmospheric Above Ground Welded Steel Tanks

- IPS-E-PI-240	Iranian Petroleum Standard – Engineering Standard for Plant Piping Systems

- IPS-G-GN-210	Iranian Petroleum Standard – General Standard for Packing & Packages

- IPS-E-CE-210	Iranian Petroleum Standard – Engineering Standard for Steel Structures

- IPS Standard Drawings

- NIOEC Standard Drawings

- NISOC Standard Drawings

## International Codes and Standards

- ASME	American Society of Mechanical Engineers

  - B 31.3	Chemical Plant and Petroleum Refinery Piping

  - B 31.8	Gas Transportation & Distribution Piping Systems

  - B16.47	Large Diameter Steel Flanges NPS 26 Through NPS 60

  - B 16.5	Pipe Flanges and Flanged Fittings NPS 1/2 Through NPS 24

  - B 16.9	Steel Butt Welding Fitting

  - B 16.11	Forged Steel Fittings

  - B 16.20	Metallic Gaskets for Pipe Flanges – Ring Joint Gaskets, Spiral Wound and Jacketed

  - B 16.21	Non- Metallic Gasket for Pipe Flanges

  - B 16.47	Large Diameter Steel Flanges NPS 26 Through NPS 60

  - B 1.20.1	Pipe Threads, General Purpose

- API 650	Welded Storage Tanks for Oil Storage

- AISC	Manual of Steel Construction

## The Project Documents

| - BK-GNRAL-PEDCO-000-ME-SP-0002 | Specification for Atmospheric Above Ground Welded Steel Tanks |
| --- | --- |
| - BK-GNRAL-PEDCO-000-ME-SP-0001 | Specification for Pressure Vessels |
| - BK-GNRAL-PEDCO-000-PI-SP-0019 | Specification for Insulation |
| - BK-GNRAL-PEDCO-000-PI-SP-0006 | Specification for Painting |

- Standard drawings

## DETAILS OF FLANGED & STUD REINFORCED NOZZLES FOR TANKS

| DETAIL OF FLANGED & STUD REINFORCED NOZZLES FOR TANKS | S1D06601_001A | NISOC STANDARDS |
| --- | --- | --- |

## STAIRWAYS

| STAIRWAYS | S1M00806_0010 | NISOC STANDARDS |
| --- | --- | --- |

## PLATFORMS

| PLATFORMS | S1M00807_0020 | NISOC STANDARDS |
| --- | --- | --- |

## FITTING FOR VERTIVAL TANK ARRANGMENT AT BLENDING & TOOL

| FITTING FOR VERTIVAL TANK ARRANGMENT AT BLENDING & TOOL |  |  |  |
| --- | --- | --- | --- |

## FITTING FOR VERTIVAL TANK 24” DIA ROOF MANHOLE

| FITTING FOR VERTIVAL TANK 24” DIA ROOF MANHOLE | S1X01135_0010 | NISOC STANDARDS |
| --- | --- | --- |

## FITTING FOR VERTIVAL TANK DIPPING PLATFORM (1/2)

| FITTING FOR VERTIVAL TANK DIPPING PLATFORM | S1X01138_0010 | NISOC STANDARDS |
| --- | --- | --- |

## FITTING FOR VERTIVAL TANK DIPPING PLATFORM (2/2)

| FITTING FOR VERTIVAL TANK DIPPING PLATFORM | S1X01138_0020 | NISOC STANDARDS |
| --- | --- | --- |

## WELDED VERTICAL STORAGE TANK DRAW OFF SUPPORT (1/2)

| WELDED VERTICAL STORAGE TANK DRAW OFF SUPPORT | S2D01108_0010 | NISOC STANDARDS |
| --- | --- | --- |

## WELDED VERTICAL STORAGE TANK DRAW OFF SUPPORT (2/2)

| WELDED VERTICAL STORAGE TANK DRAW OFF SUPPORT | S2D01108_0020 | NISOC STANDARDS |
| --- | --- | --- |

## TANKAGE SHELL NOZZLES

| TANKAGE SHELL NOZZLES | S2L01115_0010 | NISOC STANDARDS |
| --- | --- | --- |

## STANDPIPES FOR 12 m HIGH TANKS

| STANDPIPES FOR 12 m HIGH TANKS | S4D08062_0010 | NISOC STANDARDS |
| --- | --- | --- |

## STANDPIPES FOR 8, 12.5 AND 20m DIA. TANKS

| STANDPIPES FOR 8, 12.5 AND 20m DIA. TANKS | S4L08063_0010 | NISOC STANDARDS |
| --- | --- | --- |

## PLATFORM STRUCTURE AT STANDPIPES FOR 10 m HIGH TANKS

| PLATFORM STRUCTURE AT STANDPIPES FOR 10 m HIGH TANKS | S4M08214_0010 | NISOC STANDARDS |
| --- | --- | --- |

## PLATFORM STRUCTURE AT STANDPIPES FOR 12 m HIGH TANKS

| PLATFORM STRUCTURE AT STANDPIPES FOR 12 m HIGH TANKS | S4M08215_0010 | NISOC STANDARDS |
| --- | --- | --- |

## PLATFORM STRUCTURE AT STANDPIPES PLAN AND DETAILS

| PLATFORM STRUCTURE AT STANDPIPES PLAN AND DETAILS | S4M08224_0010 | NISOC STANDARDS |
| --- | --- | --- |

## STRUCTURE WATER AND FUEL TANK

| STRUCTURE WATER AND FUEL TANK | S4X04231_001A | NISOC STANDARDS |
| --- | --- | --- |

## DETAILS OF SHELL PLATE

| DETAIL OF SHELL PLATE |  |  |
| --- | --- | --- |

## DETAILS OF OBSERVATION WINDOW

| DETAIL OF OBSERVATION WINDOW |  |  |
| --- | --- | --- |

## DETAILS OF PIPE SUPPORT

| DETAIL OF PIPE SUPPORT |  |  |
| --- | --- | --- |

## TANK ASSEMBLY

| TANK ASSEMBLY |  |  |
| --- | --- | --- |

## LOCATION OF APPURTENANCES

| LOCATION OF APPURTENANCES |  |  |
| --- | --- | --- |

## DETAILS OF BOTTOM AND ROOF PLATE

| DETAIL OF BOTTOM AND ROOF PLATE |  |  |
| --- | --- | --- |

## DETAILS OF ROOF STAGE

| DETAIL OF ROOF STAGE |  |  |
| --- | --- | --- |

## DETAILS OF INLET NOZZLE

| DETAIL OF INLET NOZZLE |  |  |
| --- | --- | --- |

## DETAILS OF OUTLET NOZZLE

| DETAIL OF OUTLET NOZZLE |  |  |
| --- | --- | --- |