| طرح نگهداشت و افزایش تولید 27 مخزن |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CALCULATION NOTE FOR HELI PAD, WELL PAD DIESEL STORAGE PAD - W018s نگهداشت و افزایش تولید میدان نفتی بینک |  |  |  |  |  |  |  |
| D01 |  | APR. 2022 | AFC | R.Berlouie | M.Fakharian | M.Mehrshad |  |
| D00 |  | FEB. 2022 | IFC | R.Berlouie | M.Fakharian | M.Mehrshad |  |
| Rev. |  | Date | Purpose of Issue/Status | Prepared by: | Checked by: | Approved by: | CLIENT Approval |
| Class:1 |  |  | CLIENT Doc. Number: |  |  |  |  |
| Status: |  | IDC: Inter-Discipline Check IFC: Issued For Comment IFA: Issued For Approval AFD: Approved For Design AFC: Approved For Construction AFP: Approved For Purchase AFQ: Approved For Quotation IFI: Issued For Information AB-R: As-Built for CLIENT Review AB-A: As-Built –Approved |  |  |  |  |  |

REVISION RECORD SHEET

| PAGE | D00 | D01 | D02 | D03 | D04 |  | PAGE | D00 | D01 | D02 | D03 | D04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X |  |  |  |  |  | 66 |  |  |  |  |  |
| 2 | X |  |  |  |  |  | 67 |  |  |  |  |  |
| 3 | X |  |  |  |  |  | 68 |  |  |  |  |  |
| 4 | X |  |  |  |  |  | 69 |  |  |  |  |  |
| 5 | X |  |  |  |  |  | 70 |  |  |  |  |  |
| 6 | X |  |  |  |  |  | 71 |  |  |  |  |  |
| 7 | X |  |  |  |  |  | 72 |  |  |  |  |  |
| 8 | X |  |  |  |  |  | 73 |  |  |  |  |  |
| 9 | X |  |  |  |  |  | 74 |  |  |  |  |  |
| 10 | X |  |  |  |  |  | 75 |  |  |  |  |  |
| 11 | X |  |  |  |  |  | 76 |  |  |  |  |  |
| 12 | X |  |  |  |  |  | 77 |  |  |  |  |  |
| 13 | X |  |  |  |  |  | 78 |  |  |  |  |  |
| 14 | X |  |  |  |  |  | 79 |  |  |  |  |  |
| 15 | X |  |  |  |  |  | 80 |  |  |  |  |  |
| 16 | X |  |  |  |  |  | 81 |  |  |  |  |  |
| 17 | X |  |  |  |  |  | 82 |  |  |  |  |  |
| 18 | X |  |  |  |  |  | 83 |  |  |  |  |  |
| 19 | X |  |  |  |  |  | 84 |  |  |  |  |  |
| 20 | X |  |  |  |  |  | 85 |  |  |  |  |  |
| 21 | X |  |  |  |  |  | 86 |  |  |  |  |  |
| 22 | X |  |  |  |  |  | 87 |  |  |  |  |  |
| 23 | X |  |  |  |  |  | 88 |  |  |  |  |  |
| 24 | X |  |  |  |  |  | 89 |  |  |  |  |  |
| 25 |  |  |  |  |  |  | 90 |  |  |  |  |  |
| 26 |  |  |  |  |  |  | 91 |  |  |  |  |  |
| 27 |  |  |  |  |  |  | 92 |  |  |  |  |  |
| 28 |  |  |  |  |  |  | 93 |  |  |  |  |  |
| 29 |  |  |  |  |  |  | 94 |  |  |  |  |  |
| 30 |  |  |  |  |  |  | 95 |  |  |  |  |  |
| 31 |  |  |  |  |  |  | 96 |  |  |  |  |  |
| 32 |  |  |  |  |  |  | 97 |  |  |  |  |  |
| 33 |  |  |  |  |  |  | 98 |  |  |  |  |  |
| 34 |  |  |  |  |  |  | 99 |  |  |  |  |  |
| 35 |  |  |  |  |  |  | 100 |  |  |  |  |  |
| 36 |  |  |  |  |  |  | 101 |  |  |  |  |  |
| 37 |  |  |  |  |  |  | 102 |  |  |  |  |  |
| 38 |  |  |  |  |  |  | 103 |  |  |  |  |  |
| 39 |  |  |  |  |  |  | 104 |  |  |  |  |  |
| 40 |  |  |  |  |  |  | 105 |  |  |  |  |  |
| 41 |  |  |  |  |  |  | 106 |  |  |  |  |  |
| 42 |  |  |  |  |  |  | 107 |  |  |  |  |  |
| 43 |  |  |  |  |  |  | 108 |  |  |  |  |  |
| 44 |  |  |  |  |  |  | 109 |  |  |  |  |  |
| 45 |  |  |  |  |  |  | 110 |  |  |  |  |  |
| 46 |  |  |  |  |  |  | 111 |  |  |  |  |  |
| 47 |  |  |  |  |  |  | 112 |  |  |  |  |  |
| 48 |  |  |  |  |  |  | 113 |  |  |  |  |  |
| 49 |  |  |  |  |  |  | 114 |  |  |  |  |  |
| 50 |  |  |  |  |  |  | 115 |  |  |  |  |  |
| 51 |  |  |  |  |  |  | 116 |  |  |  |  |  |
| 52 |  |  |  |  |  |  | 117 |  |  |  |  |  |
| 53 |  |  |  |  |  |  | 118 |  |  |  |  |  |
| 54 |  |  |  |  |  |  | 119 |  |  |  |  |  |
| 55 |  |  |  |  |  |  | 120 |  |  |  |  |  |
| 56 |  |  |  |  |  |  | 121 |  |  |  |  |  |
| 57 |  |  |  |  |  |  | 122 |  |  |  |  |  |
| 58 |  |  |  |  |  |  | 123 |  |  |  |  |  |
| 59 |  |  |  |  |  |  | 124 |  |  |  |  |  |
| 60 |  |  |  |  |  |  | 125 |  |  |  |  |  |
| 61 |  |  |  |  |  |  | 126 |  |  |  |  |  |
| 62 |  |  |  |  |  |  | 127 |  |  |  |  |  |
| 63 |  |  |  |  |  |  | 128 |  |  |  |  |  |
| 64 |  |  |  |  |  |  | 129 |  |  |  |  |  |
| 65 |  |  |  |  |  |  | 130 |  |  |  |  |  |

CONTENTS

1	INTRODUCTION	4

2	scope	4

3	NORMATIVE REFERENCES	4

4	MATERIAL PROPERTIES	5

5	computer software	5

6	Soil	5

7	load combination	6

8	DESIGN OF CELLAR AREA	7

9	Wellpad slabs	13

10	GAS OIL STORAGE FOUNDATION	17

11	HELIPAD FOUNDATION	21

- INTRODUCTION

Binak oilfield in Bushehr province is a part of the southern oilfields of Iran, is located 20 km northwest of Genaveh city.

With the aim of increasing production of oil from Binak oilfield, an EPC/EPD Project has been defined by NIOC/NISOC and awarded to Petro Iran Development Company (PEDCO). Also PEDCO (as General Contractor) has assigned the EPC-packages of the Project to "Hirgan Energy - Design and Inspection" JV.

- scope

This document covers minimum necessary requirements for the check and design of helipad, well pad & diesel Storage Pad relating to well W-018.

- NORMATIVE REFERENCES

  - Local Codes and Standards

- IPS-C-CE-200	Construction Standard for concrete structures

- IPS-E-CE-500	Engineering standard for loads”.

- Iranian Seismic design code for Petroleum facilities .pub.No.038 (3rd edition)

- Iranian National Building Code .INBC No. 9

- Iranian Code of Practice for seismic Resistant Design of Building Standard No.2800 (4th edition)

  - International Codes and Standards

ACI 318-14 “Building Code Requirements for Reinforced concrete“, American Concrete institute.

  - The Project Documents

- BK-GNRAL-PEDCO-000-ST-SP-0001	Specification for Concrete Work

- BK-W028-PEDCO-110-CV-PY-0002          Civil & structural Drawings-W018

- MATERIAL PROPERTIES

Material properties are delivered in the following table:

Table 1.Material Properties

- computer software

Computer software which is used in analysis & design of slab foundation of this area is defined in the following table. The used parameters for concrete are shown in the below figure.

Table 2.Computer software

Figure 1-Applied Concrete Material

According to code No.9 below equation for modulus of elasticity of concrete has been used:

Ec=4700fc=18203.02 N/mm2

- Soil

Assumption for soil parameters such as allowable soil bearing capacity, allowable settlement and subgrade Modulus are extracted from Geotechnical report which has been done for well W-018.

According to Geotechnical report:

δa = 50 mm (Allowable Settlement)

At the Center of Foundation:             Kskgcm3=qaS=0.62         q=3.1 kg/cm2=0.31N/mm2

At the edge of Foundation:                 Kskgcm3=qaS=1.24       q=6.2 kg/cm2 =0.62N/mm2

At the corner of foundation :       Kskgcm3=qaS=2.48        q=12.4 kg/cm2=1.24N/mm2

- load combination

The following combinations of loads shall be used in the design of foundations, and shall be used for displacement and soil reaction forces (according to ASCE07-10).

  - -Ultimate Load Combinations

For the design of the structural elements the following load combinations are considered.

- 1.4(D±T)

- 1.2(D±T) + 1.6L + 0.5S

- 1.2(D±T) + 1.6S + L

- 1.2(D±T) + L+0.5S

- 1.2(D±T) + 1.0E + L + 0.2S

- 0.9(D±T) + 1.0E

- 0.9(D±T) + W

  - Serviceability Load Combinations

To check the deflection and displacements, structural stability and soil pressure, the following load combinations are considered.

- (D±T)

- (D±T) + L

- (D±T) + S

- (D±T) + 0.75L + 0.75S

- (D±T) + 0.7E

- 0.6(D±T)

- 0.6(D±T) + E

- DESIGN OF CELLAR AREA

Figure 2-Cellar pit Area

  - Loads

    - DEAD load (D)

      - Self-weight (dead)

Part 1 of Dead loads Include the self-weight of the concrete Pad.

      - Machinery load (MLoad)

Part 2 of dead load: Assumed 1000 ton load of machine on concrete pad that distributed along it on point that has been shown at bellow. Due to the dynamic nature of machine the impact coefficient 1.25 applied at m load as related load case.

MLoad on each point

      - Dead Load due to cobble stone

Dead load due to concrete pad with thickness of 20cm & cobble stone with the height of 2.2m with cement mortar applied on concrete pad as below.

qs=2.2×2.1=4.62  t/m2

qc=0.2×2.5=0.5  t/m2

qs+qc=5.12  t/m2

Pad Thickness=300mm

MLoad applied  on each point

Figure 3-Applied Point Load On Soil

    - Thermal Load

The ambient thermal load has been assumed based on the thermal variation for structure of 28 ºC.

    - Snow loads

According to Iranian National Building Code No.6 table 6-7-1 this site location is in Zone 1:   so    Ps=0.25 KN/m2

  - CELLAR AREA DESIGN AND RESULTS

    - Settlement Control

Settlement in different service load combinations should be checked by allowable settlement

Figure 4 –Settlement Of foundation under critical service Load Combination (mm)

Maximum settlement of foundation (δ max) under critical load combination is equals to 22.8 mm, which is less than 50mm and that’s acceptable.

Figure 5 –apply soil on each part (center,corner& edge of mat foundation)

Figure 6 –Soil Pressure under service Load Combination (0.1391 N/mm2)

Maximum soil pressure of foundation (qmax) under critical load combination is equals to 0.1391 N/mm2, which is less qall=0.31 N/mm2 (center) and that’s acceptable.

Figure 7 –Punching Shear Control

With assumed loading and with above output from safe 2016 software punching shear checked and result is less than 1 and is acceptable.

Figure8 –Additional to Ø10@200 (top & bott) reinforcement

According to above output from safe 2016 software there is no need to additional reinforcement.

- Wellpad slabs

One of the panels (15.9x20m) which the wellhead machine are located on, has been modeled in safe 2016 software and the software output have been shown in the following:

Figure 9-plan of all panels

  - Loads:

    - DEAD load (D)

Overhead caused by equipment located during drilling assumed 500kg/m2.

    - Thermal Load(t)

The ambient thermal load has been assumed based on the thermal variation for structure of 28 ºC.

    - Live load (L):

For bottom distributed load about 200kg/m2, has been considered for design of structure

    - Snow loads(S):

According to Iranian National Building Code No.6 table 6-7-1 this site location is in Zone 1 so

Pg = 0.25 KN/m2 = 25 kg/m2

    - machinery load(ML)

Apply 45 ton of truck load with 10.1 m x 2.99 m dimension on concrete pad.

Ml=4500010.1*2.9=1536kg/m2=1.536e-2 N/mm2

Figure10 –loading on Pads (THK=200 mm)

  - Soil Pressure Control

Soil pressures in different service load combinations should be checked by allowable value. The following figure is extracted from “SAFE” model.

Figure11 –Soil Pressure diagram

Maximum soil pressure of foundation under critical load combination is equals to 0.0309 N/mm2 (at corner), which is less than allowable bearing capacity of soil(qall=1.24 N/mm2 at the corner).

  - Settlement Control

Settlement in different service load combinations should be checked by allowable value. The following figure is extracted from “SAFE” model.

Figure12–Displacement under critical service load combination (max=5.32 mm)

Figure13–additional reinforcement check

According to above output from safe 2016 software there is no need for additional reinforcement to existing reinforcement (∅10@200 at bottom)

- GAS OIL STORAGE FOUNDATION

Gas Oil Storage foundation has been modeled as a concrete pad with thickness 0.3 m in safe 2016 software.

Figure14–Gas Oil Storage Foundation (18mx10mx0.3m)

A concrete pad with thickness 0.3m has been modelled in safe software 2016.

  - Thermal Load (T)

The ambient thermal load has been assumed based on the thermal variation for structure of 28 ºC.

  - Live load (L)

For bottom distributed load about 600kg/m2, has been considered for design of structure

Figure16–Load applied on Pad

  - Settlement Control

Settlement in different service load combinations should be checked by allowable value. The following figure is extracted from “SAFE” model.

Figure17 –Displacement under service load combination (max=1.19 mm)

  - Soil Pressure Control

Soil pressures in different service load combinations should be checked by allowable value. The following figure is extracted from “SAFE” model.

Figure18–soil Pressure under service load combination (max=0.0166 N/mm2)

Figure19–additional reinforcement check

According to above output from safe 2016 software there is no need for additional reinforcement to existing reinforcement (∅10@200 top & bottom)

- HELIPAD FOUNDATION

Helipad foundation has been modeled as a concrete pad with thickness 0.25 m in safe 2016 software.

Figure20–Helipad Foundation (8mx8mx0.25m)

A concrete pad with thickness 0.25m has been modelled in safe software 2016 as bellow:

Figure21–Slab Property Data (THK=0.25m)

  - Loads on foundation

    - Thermal Load(T)

The ambient thermal load has been assumed based on the thermal variation for structure of 28 ºC.

    - Live load (l)

For Helipad Live load due to table 6-5-1 code No.6 assumed 3KN/m2 (300kg/m2)

Figure22 –Load applied on Helipad

  - Settlement Control

Settlement in different service load combinations should be checked by allowable value. The following figure is extracted from “SAFE” model.

Figure23 –Displacement under service load combination (max=1.05 mm)

  - Soil Pressure Control

Soil pressures in different service load combinations should be checked by allowable value. The following figure is extracted from “SAFE” model

Figure24 –soil Pressure under service load combination (max=0.015N/mm2)

  - Check Additional Reinforcement

Figure25 –Additional reinforcement

According to above output from safe 2016 software there is no need for additional reinforcement to existing reinforcement (∅10@200 top & bottom)