| طرح نگهداشت و افزایش تولید 27 مخزن |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Relief Load Summery نگهداشت و افزایش تولید میدان نفتی بینک |  |  |  |  |  |  |
| D06 | JUL.2023 | AFC | M.Aryafar | M.Fakharian | A.M.Mohseni |  |
| D05 | May.2023 | IFA | M.Aryafar | M.Fakharian | A.M.Mohseni |  |
| D04 | Apr.2023 | IFA | M.Aryafar | M.Fakharian | M.Mehrshad |  |
| D03 | Dec.2022 | IFA | M.Aryafar | M.Fakharian | M.Mehrshad |  |
| D02 | Oct.2022 | IFA | M.Aryafar | M.Fakharian | M.Mehrshad |  |
| D01 | Apr.2022 | IFA | M.Aryafar | M.Fakharian | M.Mehrshad |  |
| D00 | Dec.2021 | IFC | M.Aryafar | M.Fakharian | M.Mehrshad |  |
| Rev. | Date | Purpose of Issue/Status | Prepared by: | Checked by: | Approved by: | CLIENT Approval |
| Class:2 |  | CLIENT Doc. Number: F0Z-708814 |  |  |  |  |
| Status: | IDC: Inter-Discipline Check IFC: Issued For Comment IFA: Issued For Approval AFD: Approved For Design AFC: Approved For Construction AFP: Approved For Purchase AFQ: Approved For Quotation IFI: Issued For Information AB-R: As-Built for CLIENT Review AB-A: As-Built –Approved |  |  |  |  |  |

REVISION RECORD SHEET

| PAGE | D00 | D01 | D02 | D03 | D04 | D05 | D06 |  | PAGE | D00 | D01 | D02 | D03 | D04 |  | D05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | X | X | X | X | X | X |  | 66 |  |  |  |  |  |  |  |
| 2 | X | X | X | X | X | X | X |  | 67 |  |  |  |  |  |  |  |
| 3 | X |  |  |  |  |  |  |  | 68 |  |  |  |  |  |  |  |
| 4 | X |  |  |  |  |  |  |  | 69 |  |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  |  |  | 70 |  |  |  |  |  |  |  |
| 6 | X | X | X | X | X |  |  |  | 71 |  |  |  |  |  |  |  |
| 7 | X | X | X | X | X |  |  |  | 72 |  |  |  |  |  |  |  |
| 8 |  |  | X | X | X | X |  |  | 73 |  |  |  |  |  |  |  |
| 9 |  |  | X |  | X |  |  |  | 74 |  |  |  |  |  |  |  |
| 10 |  |  | X | X |  |  |  |  | 75 |  |  |  |  |  |  |  |
| 11 |  |  | X |  | X |  |  |  | 76 |  |  |  |  |  |  |  |
| 12 |  |  | X | X | X |  |  |  | 77 |  |  |  |  |  |  |  |
| 13 |  |  | X |  |  |  |  |  | 78 |  |  |  |  |  |  |  |
| 14 |  |  | X |  |  |  |  |  | 79 |  |  |  |  |  |  |  |
| 15 |  |  | X | X |  |  |  |  | 80 |  |  |  |  |  |  |  |
| 16 |  |  | X | X |  |  |  |  | 81 |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  |  |  |  | 82 |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  | 83 |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  | 84 |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  | 85 |  |  |  |  |  |  |  |
| 21 |  |  |  |  |  |  |  |  | 86 |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  |  |  | 87 |  |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  |  |  | 88 |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |  | 89 |  |  |  |  |  |  |  |
| 25 |  |  |  |  |  |  |  |  | 90 |  |  |  |  |  |  |  |
| 26 |  |  |  |  |  |  |  |  | 91 |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |  | 92 |  |  |  |  |  |  |  |
| 28 |  |  |  |  |  |  |  |  | 93 |  |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  |  |  | 94 |  |  |  |  |  |  |  |
| 30 |  |  |  |  |  |  |  |  | 95 |  |  |  |  |  |  |  |
| 31 |  |  |  |  |  |  |  |  | 96 |  |  |  |  |  |  |  |
| 32 |  |  |  |  |  |  |  |  | 97 |  |  |  |  |  |  |  |
| 33 |  |  |  |  |  |  |  |  | 98 |  |  |  |  |  |  |  |
| 34 |  |  |  |  |  |  |  |  | 99 |  |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  |  |  | 100 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |  | 101 |  |  |  |  |  |  |  |
| 37 |  |  |  |  |  |  |  |  | 102 |  |  |  |  |  |  |  |
| 38 |  |  |  |  |  |  |  |  | 103 |  |  |  |  |  |  |  |
| 39 |  |  |  |  |  |  |  |  | 104 |  |  |  |  |  |  |  |
| 40 |  |  |  |  |  |  |  |  | 105 |  |  |  |  |  |  |  |
| 41 |  |  |  |  |  |  |  |  | 106 |  |  |  |  |  |  |  |
| 42 |  |  |  |  |  |  |  |  | 107 |  |  |  |  |  |  |  |
| 43 |  |  |  |  |  |  |  |  | 108 |  |  |  |  |  |  |  |
| 44 |  |  |  |  |  |  |  |  | 109 |  |  |  |  |  |  |  |
| 45 |  |  |  |  |  |  |  |  | 110 |  |  |  |  |  |  |  |
| 46 |  |  |  |  |  |  |  |  | 111 |  |  |  |  |  |  |  |
| 47 |  |  |  |  |  |  |  |  | 112 |  |  |  |  |  |  |  |
| 48 |  |  |  |  |  |  |  |  | 113 |  |  |  |  |  |  |  |
| 49 |  |  |  |  |  |  |  |  | 114 |  |  |  |  |  |  |  |
| 50 |  |  |  |  |  |  |  |  | 115 |  |  |  |  |  |  |  |
| 51 |  |  |  |  |  |  |  |  | 116 |  |  |  |  |  |  |  |
| 52 |  |  |  |  |  |  |  |  | 117 |  |  |  |  |  |  |  |
| 53 |  |  |  |  |  |  |  |  | 118 |  |  |  |  |  |  |  |
| 54 |  |  |  |  |  |  |  |  | 119 |  |  |  |  |  |  |  |
| 55 |  |  |  |  |  |  |  |  | 120 |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  | 121 |  |  |  |  |  |  |  |
| 57 |  |  |  |  |  |  |  |  | 122 |  |  |  |  |  |  |  |
| 58 |  |  |  |  |  |  |  |  | 123 |  |  |  |  |  |  |  |
| 59 |  |  |  |  |  |  |  |  | 124 |  |  |  |  |  |  |  |
| 60 |  |  |  |  |  |  |  |  | 125 |  |  |  |  |  |  |  |
| 61 |  |  |  |  |  |  |  |  | 126 |  |  |  |  |  |  |  |
| 62 |  |  |  |  |  |  |  |  | 127 |  |  |  |  |  |  |  |
| 63 |  |  |  |  |  |  |  |  | 128 |  |  |  |  |  |  |  |
| 64 |  |  |  |  |  |  |  |  | 129 |  |  |  |  |  |  |  |
| 65 |  |  |  |  |  |  |  |  | 130 |  |  |  |  |  |  |  |

CONTENTS

1.0	INTRODUCTION	4

2.0	Scope	5

3.0	NORMATIVE REFERENCES	5

3.1	Local Codes and Standards	5

3.2	International Codes and Standards	5

3.3	The Project Documents	5

3.4	ENVIRONMENTAL DATA	5

4.0	RELIEF LOAD SUMMARY TABLE	6

4.1	PSV-2111/2112	6

4.2	PSV-2113/2114	7

4.3	psv-2121-a/b/c	8

4.4	psv-2122/2123-A/B/C	9

4.5	psv-2131-a/b/c	10

4.6	psv-2132/2133-a/b/c	11

4.7	psv-2141/2142	12

4.8	psv-2201	13

4.9	psv-2211-a/b	14

4.10	psv-2271-a/b	15

4.11	psv-2293	16

- INTRODUCTION

Binak oilfield in Bushehr province is a part of the southern oilfields of Iran, is located 25 km northwest of Genaveh city.

With the aim of increasing production of oil from Binak oilfield, an EPC/EPD Project has been defined by NIOC/NISOC and awarded to Petro Iran Development Company (PEDCO). Also PEDCO (as General Contractor) has assigned the EPC-packages of the Project to "Hirgan Energy - Design and Inspection" JV.

As a part of the Project, a New Gas Compressor Station (adjacent to existing Binak GCS) shall be constructed to gather of 15 MMSCFD (approx.) associated gases and compress & transfer them to Siahmakan GIS.

GENERAL DEFINITION

The following terms shall be used in this document.

| CLIENT: | National Iranian South Oilfields Company (NISOC) |
| --- | --- |
| PROJECT: | Binak Oilfield Development – Surface Fcilities; New Gas Compressor Station |
| EPD/EPC CONTRACTOR (GC): | Petro Iran Development Company (PEDCO) |
| EPC CONTRACTOR: | Joint Venture of : Hirgan Energy – Design & Inspection (D&I) Companies |
| VENDOR: | The firm or person who will fabricate the equipment or material. |
| EXECUTOR: | Executor is the party which carries out all or part of construction and/or commissioning for the project. |
| THIRD PARTY INSPECTOR (TPI): | The firm appointed by EPD/EPC CONTRACTOR (GC) and approved by CLIENT (in writing) for the inspection of goods. |
| SHALL: | Is used where a provision is mandatory. |
| SHOULD: | Is used where a provision is advisory only. |
| WILL: | Is normally used in connection with the action by CLIENT rather than by an EPC/EPD CONTRACTOR, supplier or VENDOR. |
| MAY: | Is used where a provision is completely discretionary. |

- Scope

The scope of this document is to present summary of flare loads in BINAK new gas compressor station unit.

- NORMATIVE REFERENCES

## Local Codes and Standards

| - IPS-E-PR-450 | Process design of pressure relieving Systems inclusive safety relief valves |
| --- | --- |

## International Codes and Standards

| - API-STD-520 | Sizing, Selection and Installation of Pressure Relieving Devices in Refineries, Part 1-Sizing and Selection |
| --- | --- |
| - API-STD-521 | Pressure Relieving and Depressuring Systems |

## The Project Documents

| - BK-GNRAL-PEDCO-000-PR-DC-0001 | Process Design Criteria |
| --- | --- |
| - BK-GCS-PEDCO-120-PR-PF-0001 | Process Flow Diagram (PFD) |
| - BK-GCS-PEDCO-120-PR-PI-0002 ~ 0025 | Piping & Instrumentation Diagram |

## ENVIRONMENTAL DATA

Refer to "Process Basis of Design; Doc. No. BK-GNRAL-PEDCO-000-PR-DB-0001 ".

- RELIEF LOAD SUMMARY TABLE

## PSV-2111/2112

| Location | V-2104 |  |  |
| --- | --- | --- | --- |
| Equipment Size (mm) | (ID X TL-TL) 4200 X 12600 |  |  |
| Case | Wetted Fire | Gas Expansion | Blocked Outlet |
| Fluid & State | HC |  |  |
| Operating Pressure (barg) | 5.3 |  |  |
| Normal Temperature (°C) | 32.0 |  |  |
| Set Pressure (barg) | 9 |  |  |
| LHV (kJ/kg) | 431.6 | - | - |
| Relief Temperature (°C) | 284 | 284 | 32 |
| Required Massflow (kg/hr) | 37247.2 | 7227.0 | 11270.0 |
| Specific Heat Ratio (Cp/Cv) | 1.223 | 1.260 | 1.260 |
| Molecular Weight (kg/kmol) | 48.1 | 23.57 | 23.57 |
| Compressibility Factor | 0.954 | 0.974 | 0.956 |
| Total Back Pressure (barg) | 3.4 | 3.4 | 0.7 |
| Allowable Over Pressure (%) | 21 | 21 | 10 |
| Calculated Discharge Area (mm2) | 4134 | 1146 | 1428 |
| Required Discharge Area (mm2) | 8742 | 1452 | 2248 |
| Orifice Designation | Q | k | L |
| Body Size | 6 x 8 | 3 x 4 | 3 x 4 |
| P&ID Number | BK-GCS-PEDCO-120-PR-PI-0004 (1/2) |  |  |

## PSV-2113/2114

| Location | V-2105 |  |  |
| --- | --- | --- | --- |
| Equipment Size (mm) | (ID X TL-TL) 1500 X 4500 |  |  |
| Case | Wetted Fire | Gas Expansion | Blocked Outlet |
| Fluid & State | HC |  |  |
| Operating Pressure (barg) | 5.1 |  |  |
| Normal Temperature (°C) | 36.92 |  |  |
| Set Pressure (barg) | 9 |  |  |
| LHV (kJ/kg) | 1994 | - | - |
| Relief Temperature (°C) | 182.5 | 330.6 | 36.92 |
| Required Massflow (kg/hr) | 1045.9 | 779.9 | 17833.1 |
| Specific Heat Ratio (Cp/Cv) | 1.154 | 1.246 | 1.246 |
| Molecular Weight (kg/kmol) | 18.02 | 24.52 | 24.52 |
| Compressibility Factor | 0.974 | 0.974 | 0.954 |
| Total Back Pressure (barg) | 3.3 | 3.3 | 1.2 |
| Allowable Over Pressure (%) | 21 | 21 | 10 |
| Calculated Discharge Area (mm2) | 176.97 | 126.67 | 2240.6 |
| Required Discharge Area (mm2) | 254.46 | 254.4 | 3421 |
| Orifice Designation | F | F | N |
| Body Size | 1 1/2 x 2 | 1 1/2 x 2 | 4 x 6 |
| P&ID Number | BK-GCS-PEDCO-120-PR-PI-0005 (1/1) |  |  |

## psv-2121-a/b/c

| Location | V-2101-A/B/C |  |  |
| --- | --- | --- | --- |
| Equipment Size (mm) | (ID X TL-TL) 900 X 3000 |  |  |
| Case | Wetted Fire | Gas Expansion | Control Valve Failiure |
| Fluid & State | HC |  |  |
| Operating Pressure (barg) | 4.9 |  |  |
| Normal Temperature (°C) | 36.9 |  |  |
| Set Pressure (barg) | 22 |  |  |
| LHV (kJ/kg) | 1827 | - | - |
| Relief Temperature (°C) | 227.4 | 999 (NOTE) | 60 |
| Required Massflow (kg/hr) | 374.2 | 717.74 | 2166 |
| Specific Heat Ratio (Cp/Cv) | 1.245 | 1.245 | 1.457 |
| Molecular Weight (kg/kmol) | 18.8 | 24.52 | 24.55 |
| Compressibility Factor | 0.975 | 0.975 | 0.822 |
| Total Back Pressure (barg) | 3.1 | 3.1 | 0.5 |
| Allowable Over Pressure (%) | 21 | 21 | 10 |
| Calculated Discharge Area (mm2) | 27.25 | 72.95 | 107.25 |
| Required Discharge Area (mm2) | 153.93 | 153.938 | 153.938 |
| Orifice Designation | D | E | E |
| Body Size | 1 x 2 | 1 x 2 | 1 x 2 |
| P&ID Number | BK-GCS-PEDCO-120-PR-PI-0006 (1,2,3/3) |  |  |

(NOTE): Based on API-521 It is common practice to exclude the fire-relief scenario when specifying the maximum design temperature of the flare headers.

## psv-2122/2123-A/B/C

| Location | C-2101-A/B/C |  |  |
| --- | --- | --- | --- |
| Equipment Size (mm) | - |  |  |
| Case | Wetted Fire | Gas Expansion | Blocked Outlet |
| Fluid & State | HC |  |  |
| Operating Pressure (barg) | 19 |  |  |
| Normal Temperature (°C) | 124.8 |  |  |
| Set Pressure (barg) | 22 |  |  |
| LHV (kJ/kg) | - | - | - |
| Relief Temperature (°C) | - | - | 124.8 |
| Required Massflow (kg/hr) | - | - | 8664 |
| Specific Heat Ratio (Cp/Cv) | - | - | 1.225 |
| Molecular Weight (kg/kmol) | - | - | 24.523 |
| Compressibility Factor | - | - | 0.964 |
| Total Back Pressure (barg) | - | - | 0.9 |
| Allowable Over Pressure (%) | - | - | 10 |
| Calculated Discharge Area (mm2) | - | - | 539.85 |
| Required Discharge Area (mm2) |  |  | 1017 |
| Orifice Designation | - | - | J |
| Body Size | - | - | 2 x 3 |
| P&ID Number | BK-GCS-PEDCO-120-PR-PI-0007 (1,2,3/3) |  |  |

## psv-2131-a/b/c

| Location | V-2102-A/B/C |  |  |
| --- | --- | --- | --- |
| Equipment Size (mm) | (ID X TL-TL) 900 X 3000 |  |  |
| Case | Wetted Fire | Gas Expansion | Blocked Outlet |
| Fluid & State | HC |  |  |
| Operating Pressure (barg) | 18.1 |  |  |
| Normal Temperature (°C) | 60 |  |  |
| Set Pressure (barg) | 22 |  |  |
| LHV (kJ/kg) | 1807 | - | - |
| Relief Temperature (°C) | 224 | 208 | - |
| Required Massflow (kg/hr) | 378.37 | 982.27 | - |
| Specific Heat Ratio (Cp/Cv) | 1.278 | 1.278 | - |
| Molecular Weight (kg/kmol) | 18.02 | 24.523 | - |
| Compressibility Factor | 0.936 | 0.936 | - |
| Total Back Pressure (barg) | 3.4 | 3.4 | - |
| Allowable Over Pressure (%) | 21 | 21 | - |
| Calculated Discharge Area (mm2) | 27.22 | 59.62 | - |
| Required Discharge Area (mm2) | 153.93 | 153.93 |  |
| Orifice Designation | D | D | - |
| Body Size | 1 x 2 | 1 x 2 | - |
| P&ID Number | BK-GCS-PEDCO-120-PR-PI-0009 (1,2,3/3) |  |  |

## psv-2132/2133-a/b/c

| Location | C-2102-A/B/C |  |  |
| --- | --- | --- | --- |
| Equipment Size (mm) | - |  |  |
| Case | Wetted Fire | Gas Expansion | Blocked Outlet |
| Fluid & State | HC |  |  |
| Operating Pressure (barg) | 54.8 |  |  |
| Normal Temperature (°C) | 142.3 |  |  |
| Set Pressure (barg) | 62 |  |  |
| LHV (kJ/kg) | - | - | - |
| Relief Temperature (°C) | - | - | 142.3 |
| Required Massflow (kg/hr) | - | - | 8664 |
| Specific Heat Ratio (Cp/Cv) | - | - | 1.278 |
| Molecular Weight (kg/kmol) | - | - | 24.52 |
| Compressibility Factor | - | - | 0.923 |
| Total Back Pressure (barg) | - | - | 1.0 |
| Allowable Over Pressure (%) | - | - | 10 |
| Calculated Discharge Area (mm2) | - | - | 193.67 |
| Required Discharge Area (mm2) | - | - | 397.60 |
| Orifice Designation | - | - | G |
| Body Size | - | - | 1 1/2 x 3 |
| P&ID Number | BK-GCS-PEDCO-120-PR-PI-0010 (1,2,3/3) |  |  |

## psv-2141/2142

| Location | V-2103 |  |  |
| --- | --- | --- | --- |
| Equipment Size (mm) | (ID X TL-TL) 900 X 3000 |  |  |
| Case | Wetted Fire | Gas Expansion | Blocked Outlet |
| Fluid & State | HC |  |  |
| Operating Pressure (barg) | 53.9 |  |  |
| Normal Temperature (°C) | 60 |  |  |
| Set Pressure (barg) | 62 |  |  |
| LHV (kJ/kg) | 1456 | - | - |
| Relief Temperature (°C) | 286 | 188 | 60 |
| Required Massflow (kg/hr) | 469.593 | 1826.19 | 17252.5 |
| Specific Heat Ratio (Cp/Cv) | 1.461 | 1.461 | 1.461 |
| Molecular Weight (kg/kmol) | 18.96 | 24.562 | 24.562 |
| Compressibility Factor | 0.823 | 0.823 | 0.823 |
| Total Back Pressure (barg) | 0.6 | 0.6 | 1.0 |
| Allowable Over Pressure (%) | 21 | 21 | 10 |
| Calculated Discharge Area (mm2) | 11.36 | 35.26 | 311 |
| Required Discharge Area (mm2) | 153.93 | 153.93 | 629.01 |
| Orifice Designation | D | D | H |
| Body Size | 1 x 2 | 1 x 2 | 2 x 3 |
| P&ID Number | BK-GCS-PEDCO-120-PR-PI-0012 (1/1) |  |  |

## psv-2201(NOTE)

| Location | V-2203 |  |  |
| --- | --- | --- | --- |
| Equipment Size (mm) | (ID X TL-TL) 1500 X 5000 |  |  |
| Case | Wetted Fire | Gas Expansion | Blocked Outlet |
| Fluid & State | Air |  |  |
| Operating Pressure (barg) | 8 |  |  |
| Normal Temperature (°C) | 65 |  |  |
| Set Pressure (barg) | 12.5 |  |  |
| LHV (kJ/kg) | 1949 | - | - |
| Relief Temperature (°C) | 201.7 | 323.4 | - |
| Required Massflow (kg/hr) | 338.521 | 1071.801 | - |
| Specific Heat Ratio (Cp/Cv) | 1.416 | 1.416 | - |
| Molecular Weight (kg/kmol) | 18.02 | 28.95 | - |
| Compressibility Factor | 0.990 | 0.990 | - |
| Total Back Pressure (barg) | 0.0 | 0.0 | - |
| Allowable Over Pressure (%) | 21 | 21 | - |
| Calculated Discharge Area (mm2) | 40.44 | 113.24 | - |
| Required Discharge Area (mm2) | 153.93 | 153.95 |  |
| Orifice Designation | D | E | - |
| Body Size | 1 x 2 | 1 x 2 | - |
| P&ID Number | BK-GCS-PEDCO-120-PR-PI-0015 (2/2) |  |  |

NOTE: 2 X 100 PSVs, ONE Duty ONE Standby

## psv-2211

| Location | V-2204 |  |  |
| --- | --- | --- | --- |
| Equipment Size (mm) | (ID X TL-TL) 1050 X 3150 |  |  |
| Case | Wetted Fire | Gas Expansion | Blocked Outlet |
| Fluid & State | Nitrogen |  |  |
| Operating Pressure (barg) | 8 |  |  |
| Normal Temperature (°C) | 60 |  |  |
| Set Pressure (barg) | 12.5 |  |  |
| LHV (kJ/kg) | 1950 | - | - |
| Relief Temperature (°C) | 195.9 | 323.4 | - |
| Required Massflow (kg/hr) | 408.2 | 500.391 | - |
| Specific Heat Ratio (Cp/Cv) | 1.4 | 1.4 | - |
| Molecular Weight (kg/kmol) | 19.22 | 28 | - |
| Compressibility Factor | 1.0 | 1.0 | - |
| Total Back Pressure (barg) | 0.5 | 0.5 | - |
| Allowable Over Pressure (%) | 21 | 21 | - |
| Calculated Discharge Area (mm2) | 47.363 | 54.241 | - |
| Required Discharge Area (mm2) | 153.9 | 153.9 | - |
| Orifice Designation | D | D | - |
| Body Size | 1 x 2 | 1 x 2 | - |
| P&ID Number | BK-GCS-PEDCO-120-PR-PI-0016 (1/1) |  |  |

## psv-2271

| Location | V-2205 |  |  |
| --- | --- | --- | --- |
| Equipment Size (mm) | (ID X TL-TL) 438 X 2950 |  |  |
| Case | Wetted Fire | Gas Expansion | Blocked Outlet |
| Fluid & State | HC |  |  |
| Operating Pressure (barg) | 4.9 |  |  |
| Normal Temperature (°C) | 36.78 |  |  |
| Set Pressure (barg) | 9 |  |  |
| LHV (kJ/kg) | 1994 | - | - |
| Relief Temperature (°C) | 182.6 | 350.7 | - |
| Required Massflow (kg/hr) | 174.5 | 114.131 | - |
| Specific Heat Ratio (Cp/Cv) | 1.245 | 1.245 | - |
| Molecular Weight (kg/kmol) | 18.02 | 24.523 | - |
| Compressibility Factor | 0.975 | 0.975 | - |
| Total Back Pressure (barg) | 3.2 | 3.2 | - |
| Allowable Over Pressure (%) | 21 | 21 | - |
| Calculated Discharge Area (mm2) | 28.75 | 18.858 | - |
| Required Discharge Area (mm2) | 153.9 | 153.9 | - |
| Orifice Designation | D | D | - |
| Body Size | 1 x 2 | 1 x 2 | - |
| P&ID Number | BK-GCS-PEDCO-120-PR-PI-0022 (1/1) |  |  |

## psv-2293

| Location | V-2107 |  |  |
| --- | --- | --- | --- |
| Equipment Size (mm) | (ID X TL-TL) 1100 X 3600 |  |  |
| Case | Wetted Fire | Gas Expansion | Blocked Outlet |
| Fluid & State | TEG |  |  |
| Operating Pressure (barg) | 0.1 |  |  |
| Normal Temperature (°C) | Amb. |  |  |
| Set Pressure (barg) | 3.5 |  |  |
| LHV (kJ/kg) | 314.3 | - | - |
| Relief Temperature (°C) | 335.9 | 335.9 | - |
| Required Massflow (kg/hr) | 2098.01 | 76.53 | - |
| Specific Heat Ratio (Cp/Cv) | 1.083 | 1.083 | - |
| Molecular Weight (kg/kmol) | 143.6 | 143.6 | - |
| Compressibility Factor | 0.646 | 0.646 | - |
| Total Back Pressure (barg) | 0.1 | 0.1 | - |
| Allowable Over Pressure (%) | 21 | 21 | - |
| Calculated Discharge Area (mm2) | 275 | 10.03 | - |
| Required Discharge Area (mm2) | 397.6 | 153.9 |  |
| Orifice Designation | G | D | - |
| Body Size | 1 1/2 x 3 | 1 x 2 | - |
| P&ID Number | BK-GCS-PEDCO-120-PR-PI-0025 (1/1) |  |  |