|  | Clarification Title | Clarification Description (Rev.00) | Conclusion (Rev.00) ((VENDOR to Confirm & Consider in the Final Offers) | Clarification Description (Rev.01) | Conclusion (Rev.02-Final) (VENDOR to Confirm & Consider in the Final Offers) |
| --- | --- | --- | --- | --- | --- |
| GENERAL |  |  |  |  |  |
|  | Documents Priority | Vendor to confirm that in case of contradiction between this TCL and offer; vendor confirmation of compliance through this technical clarification is governed and shall be followed by vendor after order. In such case offer won’t be considered as basis of any claim after order. | Confirm With considering our attached Deviation list | Closed |  |
|  | Deviation List | Bidder shall confirm his full compliance to meet all purchaser documents listed and attached to MR, code / standards listed in MR with ref. No. "BK-GCS-PEDCO-120-ME-MR-0006" as well as MR requirements. All deviations shall be listed in project deviation form, attach to this TCL, signed and stamped. Notes: 1-Any exception and deviation from the requirements of MR and the related specifications including referenced codes & standards shall be clearly declared by vendor in writing for purchaser’s approval at bid stage by using of the format included in “Letter Of Conformity” attached to material requisition. Raising no deviation to any single requirement means full compliance of vendor quotation to that requirement. Purchaser has no responsibility to discover deviations or alternatives scattered over the proposal in addition to what proclaimed explicitly in deviation list; hence such deviation (if any) is disregarded and shall not be considered for vendor claim in future. (Vendor to confirm) 2- Bidder shall confirm that all MR attachments listed in MR are available for her for review at bid stage. 3-Considering items 1 & 2, please note that any deviation at detail stage will not be accepted by purchaser. | - Confirm with considering our attached Deviation list - Confirm - noted | 1- Deviation list will be finalized in clarification meeting. 2- Closed 3-closed | Meeting conclusion-14.08.2023: Please refer to agreed deviation list. Please refer to last Deviation list |
|  | Documents Priority | Bidder to confirm his compliance with documents priority stated in MR. Any conflict between the documents shall be notified by bidder to purchaser and resolved by his agreement | Our document list attached. | Document list will be finalized in finalized in the KOM after P.O, Attachment #2 of relevant MR will be checked, singed and stamped by the vendor and if there is any deviation will be specified. | Meeting conclusion-14.08.2023: All agreements on this TCL and deviation list priority are higher than MR. Vendor Confirmed. Closed. Attachment #2 of relevant MR will be checked, singed and stamped by the vendor and if there is any deviation will be specified. Closed. Please refer to last Deviation list |
|  | Requirement For Documents | Bidder to confirm that documents required after purchase order shall be submitted in accordance with “Vendor documents min. requirement list” attached to the MR. | Our document list attached. | VPIS Will be finalized in the KOM after P.O ,Attachment #2 of relevant MR will be checked, singed and stamped by the vendor and if there is any deviation will be specified. | Meeting conclusion-14.08.2023: Attachment #2 of relevant MR will be checked, singed and stamped by the vendor and if there is any deviation will be specified. Closed. Please refer to last Deviation list |
|  | Documents | Bidder shall provide following documents while bidding: 1- Copy of valid quality system certificate (ISO 9001/2 or 3) 2- Sub- vendors / sub-suppliers list 3- Deviation list with letter of conformity (Refer to item A2 above) 4- Reference and Experience List 5- Joint venture agreement (If any) with any company to design / fabrication of equipment. In case of joint venture ; qualified documents of the company who is part of joint venture shall be also submitted | quality system certificate (ISO 9001 version 2015), Sub- vendor, Deviation list with letter of conformity, Reference List Attached |  | Meeting conclusion-14.08.2023: Sub- Vendor list shall be according to Project’s AVL (MOP AVL) and shall be approved by the client’s after P.O. Closed. Please refer to last Deviation list |
|  | Sub-vendors | OMEGA is not in project AVL. Bidder to confirm that Sub-Vendors shall be selected from Project’s AVL. | Main part are imported from omega air and assembled in HAVASAN that IS in AVL | Project AVL shall be followed by vendor as attachment#1 | Meeting conclusion-14.08.2023: Sub- Vendor list shall be according to Project’s AVL (MOP AVL) and shall be approved by the client’s after P.O. Closed. Please refer to last Deviation list |
|  | Painting specification | Bidder to confirm that surface preparation and painting of all equipment material including valves, pipes, fittings and everything within the package are in accordance with project painting specification with ref. "BK-GNRAL-PEDCO-000-PI-SP-0006" attached to MR. | Only piping inside of equipment in our scope that accordance with manufacturer standard. | As per project requirement all piping and instruments on skids and between skids within the package shall be in vendor's scope of supply and work. | Meeting conclusion-14.08.2023: All connections and piping between skid inside the packages are in vendor’s scope of supply. The vendor confirmed to following project specifications for between skids. Closed. Please refer to last Deviation list |
|  | Specification | Bidder to confirm that Packing, Marking, Preservation and Preparation for Shipment will be done in accordance to the project specification with Doc. No; BK-GNRAL-PEDCO-000-QC-PR-0045 attached to the MR. | Confirm. | closed |  |
|  | Inspection and Test Plan | Inspection and Test Plan / Quality Control Plan and their related inspection level (Witness, Hold, Review and Approval) shall be at least in accordance with MR and all project requirement such as NDT PROCEDURE. Any deviation is not accepted unless otherwise specified in deviation list and approved by Purchaser. | Our ITP attached that will be finalized during PIM | ITP will be finalized during PIM | Meeting conclusion-14.08.2023: Vendor confirmed. Closed. Please refer to last Deviation list |
|  | Material Certificate | Bidder to confirm that for all pressure parts and those required special consideration or stainless steel as well as for supports the material certificate should be in accordance with EN 10204, 3.1 Material certificate for non-pressure parts shall be in accordance with EN 10204, 2.2 | For all parts that fabricated in our factory it is possible but for other imported parts it is not possible. | Material certificated will be submitted by the vendor according to approved ITP. | Meeting conclusion-14.08.2023: Vendor confirmed. Closed Please refer to last Deviation list |
|  | Scope | Bidder shall confirm the scope of work and supply are fully in accordance with MR and its attachment with considering following items: 1- The consequences of HAZOP and SIL study shall be in vendor scope of supply and work. 2- Participation in technical meeting whenever it is necessary by Purchaser/Client to close engineering document after order is in vendor scope 3- Lifting beams (Spreader beams) for erection with sling shackle and etc shall be designed and supplied by vendor 4- N2 preservation with pressure regulator, valve & flexible hoses during transportation & 18 months of site storage based on requirements in MR shall be in vendor scope of supply / work. 5- All utility conditions shall be as per document "Basic engineering design data (10‐BD‐00‐PR‐SPC‐0001)". Moreover, bidder shall consider utility noted condition as per project design data. If other utility condition is needed, whole required facilities is in vendor scope of work/supply. 6- Anchor bolts | - Our share this project is only preparation on instrument and nitrogen generator package so HAZOP is not applicable - Confirm - Not applicable - We only prepare nitrogen generator, air compressor, 10000 vessel and refrigerant dryer - Confirm - Not confirm | - Project specification will be followed by the Vendor - Closed - Closed - Approved “Preservation procedures” that will be submitted by the Vendor on detail stage. - Closed - 6- not accepted, Project specification will be followed by the Vendor | Meeting conclusion-14.08.2023: - Project MR and specification will be followed by the Vendor. - Closed. - Closed. - N2 preservation will be done according to approved “Preservation procedures” that will be submitted by the Vendor on detail stage. - Closed. - Anchor bolts size and location will be specified by the vendor. Please refer to last Deviation list |
|  | Applied standard | Applied Standards for equipment design, fabrication and test shall be listed | Attached | All project specification shall be followed by the Vendor if there is any deviation shall be specified clearly. | Meeting conclusion-14.08.2023: All project specification shall be followed by the Vendor if there is any deviation shall be specified clearly. Manufacture STD for compressor is acceptable. Please refer to last Deviation list |
|  | outdoor installation/ operation under project site conditions | Vendor shall confirm Air Compressor Packages, Dryer Packages and N2 generation package are fully suitable for outdoor installation/operation under project site conditions “Process Basis of Design; BK-GNRAL-PEDCO-000-PR-DB-0001 “without requiring any additional protection. | Acc. To min. temp. 5 °C is confirmed. | closed |  |
|  | Suitability of utility characteristics available at site | Vendor shall confirm suitability of utility characteristics available at site as per project site conditions “Process Basis of Design; BK-GNRAL-PEDCO-000-PR-DB-0001 “. Supplying all facility for utility distribution and regulation shall be part of the vendor scope. | We only prepare nitrogen generator, air compressor, 10000 vessel and refrigerant dryer | Vendor shall be follow “Process Basis of Design; BK-GNRAL-PEDCO-000-PR-DB-0001” | Meeting conclusion-14.08.2023: Vendor confirmed. Closed. Please refer to last Deviation list |
|  | Warranty | Vendor shall warranty the engineering, materials and workmanship of the equipment/goods. Under this warranty, all materials, parts, and assemblies found to be defective during the guarantee period shall be replaced free of charge by VENDOR. Also, the guarantee period shall be eighteen (18) months from the date of delivery or twelve (12) months from the installation date of each equipment/packages at site. | Confirm | closed |  |
|  | Document | Bidder to confirm that all native files due to all calculations and drawings shall be submitted for purchaser approval after order. | Regarding the imported equipment (N-Gen, Refrigerant Dryer), only the certificate and document that sent from the foreign supplier could submitted. Native file of calculation only for Air receivers and not for Dryer could submitted. | Vendor shall confirm this requirement and if there is any deviation will be specified. | Meeting conclusion-14.08.2023: Attachment #2 of relevant MR will be checked, by the vendor and if there is any documents that native file cannot be submitted will be specified and will be added to the deviation list. Closed. Please refer to last Deviation list |
| PROCESS |  |  |  |  |  |
|  | Process Data | Air compressor package, air dryer packages, nitrogen package ... shall be revised as per project specification/data sheet. Latest revision of project documents has been attached as Attachment#2 of this TCL. Please follow them. | The model of nitrogen generator and nitrogen receiver acc. To new requested capacity (34 Nm3/hr.) and purity of N2 (99.9%) is revised |  | Meeting conclusion-14.08.2023: Mechanical design pressure for pressure part is 12.5 bar g. Vendor confirmed. Closed. Please refer to last Deviation list |
|  | General comment | Vendor shall be capable to generate nitrogen with purity of 99.9% | confirm | Closed |  |
|  | New Comment |  |  |  | Meeting conclusion-14.08.2023: Utility consumption list and equipment list will be submitted on next rev. of technical offer. confirm |
| MECHANIC |  |  |  |  |  |
|  | Document Requirement | Bidder to submit all required documents with tender as per “VDR Form” in the MR. For example, below documents: 1-Marked up and Filled out MR scope of supply and works (It shall be signed and stamped) 2-GAD for air compressors, air dryers and nitrogen generators with dimensions, all auxiliaries and heaviest part weight table (by considering air cooler in the package) 3-Cross sectional drawing for compressors with BOM 4-Performance Curves 5-Equipment data sheet in each standard format 6-Utility consumption and load list 7-Spare part list 8-Special tool list 9-P&ID’s dedicated for air compressor, dryer and nitrogen generation packages with all equipment, instruments and lube oil, cooling and control systems. 10-Noise Data 11-Equipment List on detail for mechanical and other items | - Only the supplement of equipment is in our scope. - Confirm after P.O. - Confirm after P.O. - Confirm - Confirm - We only have electricity consumption that is mentioned in the load list document - Attached - No need to any special list - PID attached - Noise data only for compressor after P.O. - confirm | - Vendor Shall Be Followed Project MR And If There Is Any Deviation Will Be Specify - Closed - Closed - Close - Close - Noted - Noted - Noted - Noted - Noted - Closed | Meeting conclusion-14.08.2023: - Project's MR will be signed and stamped by vendor and will be submitted. - Closed. - Closed. - Client. - Closed. - Closed. - Closed. - Closed. - Closed. - Closed. - Closed Please refer to last Deviation list |
|  | Interconnecting piping | Design and supply of all interconnecting piping between equipment and all instruments, valves and … on them is in the vendor's scope. Also all between skid piping (including supporting) c/w all instrument and valve on them shall be designed, prefabricated and supplied by vendor. Detailed isometric DWGs shall be submitted for them. Note1: On skid piping to be assembled at vendor's shop. Note2; Outside skid piping to be prefabricated and match marked for site assembling. | supplement of equipment and on skid piping is in our scope and Outside skid piping is not in our scope | Not accepted, as per project requirement all piping and instrument on skid and between skids within the package shall be in vendor’s scope of supply and work. | Meeting conclusion-14.08.2023: Vendor confirmed all connection between skids are in vendor’s scope of supply. Also Vendor Confirmed to preparation isometric drawing and loose item. Closed. Please refer to last Deviation list |
|  | Data sheet | Vendor shall submit equipment data sheets for all parts within the package including compressors, dryers, nitrogen generators and chiller/heat exchanger in addition to corresponding catalogues in his base offer. | confirm | Closed |  |
|  | Heat tracing c/w supports, clips and etc. | Design and supply of heat tracing c/w supports, clips, and other provisions (including panels, heating cables and relevant connectors, electrical cables, junction boxes and thermostats) shall be part of the vendor scope. | Total heat dissipation for package will be deliver after P.O. | Pending to vendor submission | Meeting conclusion-14.08.2023: Closed. Please refer to last Deviation list |
|  | Insulation c/w supports, clips and etc. | Design and supply of insulation c/w supports, clips and other provisions shall be part of the vendor scope. | Insulation in our proposed package is not applicable. | Closed |  |
|  | Commissioning spare parts | Pre-commissioning, commissioning and start up spare part shall be supplied by vendor. Vendor shall confirm to provide all required spare parts for pre-commissioning, commissioning, start up as per project requirement. | Refer to our last spare part list | Vendor shall follow project spare part list as Attachment#1. | Meeting conclusion-14.08.2023: Spare part list will be submitted according reference document on next rev. of technical offer. Closed. Please refer to last Deviation list |
|  | Two years spare parts | Two years spare part shall be supplied by vendor as per project spare part list. | Attached. | Vendor shall follow project spare part list as Attachment#1. | Meeting conclusion-14.08.2023: Spare part list will be submitted according reference document on next rev. of technical offer. Closed. Please refer to last Deviation list |
|  | Capital spare part | Capital spare part to be considered by vendor as optional offer. | Will be consider in our commercial proposal | Closed |  |
|  | Material | ASTM material shall be used for Air, Dryer and Nitrogen generation packages. | confirm | Closed |  |
|  | Vent and Drain of Packages | All vents and drains of the packages to be valved, manifolded and to be routed to the edge of skid. | confirm | Closed |  |
|  | Tie-in | All Tie-in for equipment in the packages shall be equipped will flange and valves | confirm | Closed |  |
|  | Coupling | The coupling shall be of the API671 forged-steel, non-lubricated, flexible-element spacer type. The flexible elements shall be stainless steel metallic disk. | Our proposed compressor coupling are flexible coupling with spider (Polyurethane) | Coupling will be dry flexible Spacer with Non-sparking guard and shall be stainless steel metallic disk. Otherwise this item shall be added at deviation list | Meeting conclusion-14.08.2023: This item will be added to the deviation list. (API 671) Coupling will be dry flexible Spacer with Non sparking guard. Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Coupling balancing | Vendor shall specify the coupling balancing grade and balancing method as per API671. | balancing method AS ISO 1940 | Not accepted, vendor shall specify the coupling balancing grade and balancing method as per API671. Otherwise this item shall be added at deviation list | Meeting conclusion-14.08.2023: This item will be added to the deviation list. Coupling Balanced will be according to ISO 1940-1 G2.5. (will be checked by the vendor and if is not according to G2.5 will be added to the deviation list) Closed. Please refer to last Deviation list |
|  | PMI, pickling and passivation | Positive material identification (PMI), pickling and passivation for non-carbon steel material to be included in vendor’s scope of work. | If need to that, will be done. | Noted | Meeting conclusion-14.08.2023: Closed. Please refer to last Deviation list |
|  | Skid of the packages | 1-Vendor shall specify number of skid inside package with dimension and weight and preliminary general arrangement. 2-In case of multi skid, please clarify if there is any special procedure for installing and assembling of loose parts. 3-Each skid shall be furnished with 4-point lifting facility and no. two earth lugs located at opposite sides and nameplate made of SS material. 4-Vendor shall specify space required around the skid for maintenance. | - Specify in GA document that attached after P.O. - Every equipment has own skid - In proposed compressor, we consider only lift truck transportation point. - space required around the skid for maintenance is specified in GA document | - Not accepted, vendor shall specify this requirement before P.O. - -In case of multi skid, please clarify if there is any special procedure for installing and assembling of loose parts - Each skid shall be furnished with 4-point lifting facility and no. two earth lugs located at opposite sides and nameplate made of SS material - Noted | Meeting conclusion-14.08.2023: - Preliminary GAD will be submitted on next Rev. - Vendor Confirmed. - Vendor Confirmed. - Closed. Please refer to last Deviation list |
|  | Inlet condition of compressor | Inlet condition of compressor shall be considered as ambient condition of site as per relevant project specification for Site Condition “Process Basis of Design; BK-GNRAL-PEDCO-000-PR-DB-0001 ". In this regard, air compressor shall be sized for worse case condition of AMB site condition and in any condition the outlet condition as per project duty spec. shall be met. | In the choose of compressor model we consider the summer condition (worse) | Vendor shall be followed Process Basis of Design; BK-GNRAL-PEDCO-000-PR-DB-0001 Otherwise this item shall be added at deviation list | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Lube oil system for air compressor | Lube oil system for bearings of air compressors shall be in vendor’s scope of supply and work and its dedicated P&ID with all auxiliaries such as pumps, oil cooler, reservoir and instruments shall be submitted by vendor in next revision of technical offer. | Our Internal PID of compressor attached. | Noted | Meeting conclusion-14.08.2023: Closed. Please refer to last Deviation list |
|  | Noise level | If noise enclosure will be required for meeting the project noise limit, it shall be supplied by vendor. | confirm | Closed |  |
|  | Material | Below requirements shall be followed by vendor: -Compressor casing shall be made of cast iron. -Rotors and shafts shall be forged and heat‐treated steel. -Timing gear shall be made of forged steel. -Gear box housing material shall be C.S. -The instrument air lines and the other air piping shall be galvanized CS based on project requirement “Piping Corrosion study and material selection report; BK-GCS-PEDCO-120-PI-RT-0001”. -Oil pumps casing shall be C.S. -Oil pumps impellers shall be S.S. -Oil cooler, shell shall be C.S and tube shall be SS 316 L -Oil piping shall be SS 316L. | - Oil pump is a part of GHH rand Airend - Oil cooler are AL - piping on skid of equipment is in our scope | Vendor shall be followed this requirement, otherwise All relevant items shall be added to the deviation/clarification. | Meeting conclusion-14.08.2023: Vendor will be checked the relevant material and if it is necessary will be added to the deviation list. Please refer to last Deviation list |
|  | Motor Power | For motor driven units, the motor nameplate rating (exclusive of the service factor) shall be a minimum of 110 % of the maximum power (including gear and coupling losses) required for any of the rated conditions. Vendor shall confirm that motors nominal power has been considered with derating factor of site ambient condition. | Confirm | Closed |  |
|  | Applied Standards | Applied Standards for equipment design, fabrication and test shall be listed. ISO10440-2 standard shall be followed by vendor for air compressors. | confirm | Closed |  |
|  | Vendor’s Scope | All vents and drains of the package to be valved, manifolded and to be routed to the edge of skid. | CONFIRM | Closed |  |
|  | Inspection and Test | Test procedures and acceptance criteria will be checked after order with project specifications and related standards. Vendor to confirm to follow project standard and spec. | For domestic product confirm | ITP will be finalized during PIM. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Guarantee | Performance, Process and Mechanical Guarantee shall be in the vendor's scope. | confirm | Closed |  |

|  | Lifting/Shipping | Shipping calculation, hydro test stability calculation and relevant drawings & temporary shipping support for all equipment shall be considered in vendor’s scope and submitted for purchaser’s review at detail stage. Please confirm. | For Domestic product it is possible but for imported product Only packing procedure and hydro test procedure submitted. | Not accepted, this item shall be added at deviation list. | Meeting conclusion-14.08.2023: It will be done according to approved procedures. Vendor Confirmed. Closed. Please refer to last Deviation list |
| --- | --- | --- | --- | --- | --- |
|  | preservation | N2 purging preservation with following conditions shall be considering in vendor’s scope of work & supply (it shall be considered in vendor’s base price not as optional): 1-All opening shall be sealed with a suitable metal cover and gasket. Vessels shall be filled with nitrogen gas at a pressure of 0.5 kg/cm2(g). 2-The equipment shall have a temporary gauge to determine purge pressure. 3-Vendor shall submit N2 filling procedure for purchaser/contractor approval at detail stage. 4-All the required ancillaries such as metal covers, bolts & nuts, gaskets and pressure gauge shall be provided by vendor. Please confirm and modify your offer.( using silica gel instead of N2 purge is not acceptable and shall be omit from your offer.) | Only supplement of nitrogen generator and air compressor and refrigerant dryer and vessel in in our scope | Vendor Shall Be Followed Project MR And If There Is Any Deviation Will Be Specify | Meeting conclusion-14.08.2023: All project requirments will be followed by the vendor, otherwised will be added to the deviation List. Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | New item : Air filter |  |  | Inlet air filter for compressors shall be considered and indicated on P&ID | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | New item : Compressor stages, Inter coolers and Separators |  |  | Vendor shall specify and indicate below items on PFD & P&ID for Air and Nitrogen production packages: -The number of screw compressor stages -Inter/After coolers -Separators between and after stages | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Specified in PID |
|  | New item : Calculation report |  |  | Calculation report for each of compressor stage including temperature, pressure, capacity at suction and discharge of compressor and shaft power required at each stage to be specified by vendor. | Meeting conclusion-14.08.2023: Vendor Confirmed, will be submitted before P.O. Closed. |
|  | New item : MAWP & PSV setting |  |  | Compressor's MAWP and PSV setting pressure on discharge line shall be specified by vendor. | Meeting conclusion-14.08.2023: Mechanical design pressure for pressure part is 12.5 bar g. Vendor confirmed. Closed. |
|  | New item : Ladder and platforms |  |  | Access ladder and platforms, stairway shall be studied, designed and supplied by vendor. Relevant detail drawings shall be submitted for approval. They shall comply with project standards attached to MR, as minimum. | Meeting conclusion-14.08.2023: Vendor confirmed. Closed. Please refer to last Deviation list |
|  | New item : Interconnecting piping |  |  | Design and supply of all interconnecting piping between equipment and all instruments, valves and … on them is in the vendor's scope. Also, all between skid piping (including supporting) c/w all instrument and valve on them shall be designed, prefabricated and supplied by vendor. Detailed isometric DWGs shall be submitted for them. Note1: On skid piping to be assembled at vendor’s shop. Note2; Outside skid piping to be prefabricated and match marked for site assembling. | Meeting conclusion-14.08.2023: Vendor confirmed. Closed. Please refer to last Deviation list |
|  | New Item |  |  | For other mechanical comments refer to Attachment#1 of this TCL. | Meeting conclusion-14.08.2023: Vendor confirmed. Closed. |
| MECHANIC FIX |  |  |  |  |  |
|  | Fixed equipment mechanical data sheet such as drum | Vendor shall submit relevant mechanical data sheet to define shell/head/nozzle/support type & materials & dimension and designing assumption (J.E. / C.A. & etc.…) | confirm | Noted and Closed. |  |
|  | Special tools | Special tools for lifting of vessel (i.e. spread bar and skirt bracing if required based vendor’s lifting calculation & relevant bolts/nuts/gasket, Hydraulic bolt tensioning device for bolts greater than 1-1/2” and etc ), shall be considered in vendor’s scope. | No need | Noted and Closed. |  |
|  | Lifting/Shipping | Shipping calculation, hydrotest stability calculation and relevant drawings & temporary shipping support for all equipment shall be considered in vendor’s scope and submitted for purchaser’s review at detail stage. Please confirm. | For Domestic product it is possible but for imported product Only packing procedure and hydro test procedure submitted. | Noted and Closed. |  |
|  | preservation | N2 purging preservation with following conditions shall be considering in vendor’s scope of work & supply (it shall be considered in vendor’s base price not as optional): 1-All opening shall be sealed with a suitable metal cover and gasket. Vessels shall be filled with nitrogen gas at a pressure of 0.5 kg/cm2(g). 2-The equipment shall have a temporary gauge to determine purge pressure. 3-Vendor shall submit N2 filling procedure for purchaser/contractor approval at detail stage. 4-All the required ancillaries such as metal covers, bolts & nuts, gaskets and pressure gauge shall be provided by vendor. Please confirm and modify your offer.( using silica gel instead of N2 purge is not acceptable and shall be omit from your offer.) | Our proposed n-gen regenerated by N2 purge. All connection cover but deliver at empty position | Noted and Closed. Vendor shall be replied or conformed another clause. | Meeting conclusion-14.08.2023: Closed. Please refer to last Deviation list |
|  | Access for internal loading/unloading | Manhole / hand hole Access for internal loading/unloading of drums | Nitrogen generator and desiccant dryer has no manhole or hand hole. | Project’s Process design criteria shall be followed. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Nozzle WRC | Local stress analysis of shells under external loads from piping system and attachments (platforms, pipe supports, davit, lifting lug) shall be included in strength calculation. And also, nozzle load shall be considered according to Allowable Nozzle loads on static Equipment (as per project specification) should be done. Please note that wherever WRC is not applicable FEA shall be done to evaluate local stresses. The original file (native file) of all analysis including all input data and results should be provided by Vendor. Please confirm. | Only nozzle load could submit. | Local stress analysis of shells under external loads from piping system and attachments (platforms, pipe supports, davit, lifting lug) shall be included in strength calculation. And also, nozzle load shall be considered according to Allowable Nozzle loads on static Equipment (as per project specification) should be done. Please note that wherever WRC is not applicable FEA shall be done to evaluate local stresses. The original file (native file) of all analysis including all input data and results should be provided by Vendor. Please confirm. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Loading Data & Anchor Bolt Data | Anchor bolt data (Size, number, B.C.D., material ,…) and final loading data including erection, operating and hydro tests shear and moment due to wind and earthquake shall be provided by vendor 1 month after purchase order. Please confirm. | Anchor bolt in n-gen is not applicable. | Anchor bolt data (Size, number, B.C.D., material ,…) and final loading data including erection, operating and hydro tests shear and moment due to wind and earthquake shall be provided by vendor 1 month after purchase order. Please confirm. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | PWHT | When stress relieving is required, whole equipment in one piece without removable internals shall be located in the Furnace. All surfaces of flanges shall be protected by vendor due to prevent of damage. Use of direct flame is not approved. | It is not applicable Only Native file of calculation of air receiver could submitted. | Noted and closed. |  |
|  | Corrosion allowance | For all parts made of carbon steel, corrosion allowance of 3.2 mm to be considered by vendor. In addition, corrosion allowance shall be in accordance with the project "Corrosion Study & Material Selection Report" attached to MR. This requirement shall be followed by vendor. | Anchor bolt data (Size, number, B.C.D., material,) and final loading data will be mentioned in air receiver detail drawing document. | For all parts made of carbon steel, corrosion allowance of 3.2 mm to be considered by vendor. In addition, corrosion allowance shall be in accordance with the project "Corrosion Study & Material Selection Report" attached to MR. This requirement shall be followed by vendor. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Referenced documents | Referenced documents, specifications and STDs specified in project MR shall be fully followed by vendor. In addition, for major components within the package, following codes and standards (Latest Editions) shall be considered by vendor: 1- Pressure Vessel Design: ASME BPVC VIII Div. 1 2- Materials: ASME BPVC Sec. II, ASTM 4- Nozzle and External Load Design: WRC 537 5- Wind: ASCE 7 6- Seismic: ASCE 7 | since the manufacture and design of Nitrogen Generator are according to EN 13445 and approval of the European standard Pressure Equipment Directive PED 97/23/CE and PED 2014/68/EU, the documents that can be provided for this equipment are based on the above standards and received from Omega Air side. | Referenced documents, specifications and STDs specified in project MR shall be fully followed by vendor. In addition, for major components within the package, following codes and standards (Latest Editions) shall be considered by vendor: 1- Pressure Vessel Design: ASME BPVC VIII Div. 1 2- Materials: ASME BPVC Sec. II, ASTM 4- Nozzle and External Load Design: WRC 537 5- Wind: ASCE 7 6- Seismic: ASCE 7 | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Material | Material ASTM No. of all package parts including grade and designation shall be specified in equipment datasheets. Furthermore, MDMT, design temp. and pressures shall be specified as well as requirement of impact test and heat treatment. | since the manufacture and design of Nitrogen Generator are according to EN 13445 and approval of the European standard Pressure Equipment Directive PED 97/23/CE and PED 2014/68/EU, the documents that can be provided for this equipment are based on the above standards and received from Omega Air side. | Material ASME No. of all package parts including grade and designation shall be specified in equipment datasheets. Furthermore, MDMT, design temp. and pressures shall be specified as well as requirement of impact test and heat treatment. | Meeting conclusion-14.08.2023: This item will be added to the deviation list. Closed. Please refer to last Deviation list |
|  | Plates | Sliding plates, templates, liners, insert plate & shims (whichever is applicable) are in vendor’s scope of supply. | Only supply of equipment is in our scope | Noted and closed. |  |
|  | Piping Support | All internal piping, valves and fittings with their supports for cooling, sealing, purging, vent, drain and lubrication shall be provided and installed by Vendor. Any auxiliary connection inside B.L shall be flanged type and extended up to the skid's boundary. | Only on skid piping is in our scope | All internal piping, valves and fittings with their supports for cooling, sealing, purging, vent, drain and lubrication shall be provided and installed by Vendor. Any auxiliary connection inside B.L shall be flanged type and extended up to the skid's boundary. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Walk way | In order to provide access to all parts of package, Walkways, platforms, ladders & handrails shall be considered by Vendor | Only supply of equipment is in our scope | In order to provide access to all parts of package, Walkways, platforms, ladders & handrails shall be considered by Vendor | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Seismic/Wind parameters | Vendor shall follow project specifications for Seismic/Wind parameters. Below data shall be considered as minimum requirements: - For seismic: - Code: ASCE 7-10 - Importance Factor: 1. 25 - Site Class: D - Ss = 0.781 - S1 = 1.25 - Fa = 1 - Fv =1.5 - Design wind speed: 120 km/h @10m - Wind Exposure: C | since the manufacture and design of Nitrogen Generator are according to EN 13445 and approval of the European standard Pressure Equipment Directive PED 97/23/CE and PED 2014/68/EU, the documents that can be provided for this equipment are based on the above standards and received from Omega Air side. | Vendor shall follow project specifications for Seismic/Wind parameters. Below data shall be considered as minimum requirements: - For seismic: - Code: ASCE 7-10 - Importance Factor: 1. 25 - Site Class: D - Ss = 0.781 - S1 = 1.25 - Fa = 1 - Fv =1.5 - Design wind speed: 120 km/h @10m Wind Exposure: C | Meeting conclusion-14.08.2023: Will be checked by the vendor, and will be added to deviation list if it cannot be followed. Closed. Please refer to last Deviation list |
|  | Grid plate and Grating | The floor shall be covered with grid plate which is suitably supported to prevent excessive deflection. Checkered plate for walkway or other places that end user may use, is not acceptable. Hot dip galvanized grating with four clips per each piece of grating shall be considered for bottom of all walkways. | Only supply of equipment is in our scope | The floor shall be covered with grid plate which is suitably supported to prevent excessive deflection. Checkered plate for walkway or other places that end user may use, is not acceptable. Hot dip galvanized grating with four clips per each piece of grating shall be considered for bottom of all walkways. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Drain nozzle height | Drain valve height shall have enough space for operation and maintenance. | confirm | Noted and closed. |  |
|  | Passages and all open surfaces | Passages and all open surfaces around the equipment shall be covered with gratings. All the grating penetrations (piping, cables, etc.) shall be bordered with a flat bar which will act as a foot guard. | Only supply of equipment is in our scope | Passages and all open surfaces around the equipment shall be covered with gratings. All the grating penetrations (piping, cables, etc.) shall be bordered with a flat bar which will act as a foot guard. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Commissioning & Start-up | All items required for commissioning & start-up inside the package shall be supplied by vendor. Necessary quantities of catalysts, cartridges, chemicals, desiccant, molecular sieve, etc., whichever is applicable and required for start-up and initial operation of the package shall be supplied by vendor. | Refer to our attached Commissioning & Start-up and spare part list | All items required for commissioning & start-up inside the package shall be supplied by vendor. Necessary quantities of catalysts, cartridges, chemicals, desiccant, molecular sieve, etc., whichever is applicable and required for start-up and initial operation of the package shall be supplied by vendor. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | P&IDs | Purchaser P&IDs to be fully reviewed by vendor and if any extra provisions for control and reliable operation is needed, to be announced by vendor ay bid stage. | Only supply of equipment is in our scope | Purchaser P&IDs to be fully reviewed by vendor and if any extra provisions for control and reliable operation is needed, to be announced by vendor ay bid stage. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Shop-assembled | The unit shall be shop-assembled to the maximum extent practical to minimize on-site erection work. All skids and equipment that mate directly (without interconnecting piping by the purchaser) shall be trial fit in the vendor’s shop. The vendor shall identify all loose-shipped components requiring field installation in the proposal. | confirm | Noted and closed. |  |
|  | pressure vessels Specification | All pressure vessels inside the package shall comply with requirements explained in Specification for Welded Pressure Vessel BK-GNRAL-PEDCO-000-ME-SP-0001 attached to material requisition. | Design is Acc. To ASME SEC. VIII, DIV.I | Noted and closed. |  |
|  | Nozzle Flange | All nozzles shall be flanged with Welding neck flanges. For nozzle sizes smaller than 2”, long-welding-neck flanges are required. | CONFIRM | Noted and closed. |  |
|  | Standard Drawings | All details of pressure vessels designed according to ASME BPVC VIII shall follow Standard Drawings for Fixed Equipment attached to this file. | Confirm | Noted and closed. |  |
|  | Vessel Support type | Vessels with diameters (OD) and weights greater than 1000 mm and 1200 kg, respectively; are not allowed to be supported by legs. | ok | Vendor shall be confirmed and closed. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. |
|  | Lifting Lug | The skid shall be provided with lifting lugs for a four-point lift. Lifting and transport of the skid, complete with all equipment mounted, shall not permanently distort or otherwise damage the skid or the machinery mounting surfaces. | Air receiver has 2 lifting lug in the top of vessel and 2 tail lug in the bottom. | Noted and closed. |  |
|  | Overall plot plan | A preliminary overall plot plan shall be added to the next revision by bidder to show approximate arrangement and dimensions of the package. | Dimension added to data sheet of equipment, Detail mentioned in GA that submitted after P.O. | Noted and closed. |  |
|  | Access for internal loading/unloading | Manhole / hand hole Access for internal loading/unloading of drums | Air receiver has manhole | Noted and closed. |  |
|  | Nozzle WRC | Local stress analysis of shells under external loads from piping system and attachments (platforms, pipe supports, davit, lifting lug) shall be included in strength calculation. And also, nozzle load shall be considered according to Allowable Nozzle loads on static Equipment (as per project specification) should be done. Please note that wherever WRC is not applicable FEA shall be done to evaluate local stresses. The original file (native file) of all analysis including all input data and results should be provided by Vendor. Please confirm. | Local stress analysis for nozzles and /or attachments as per WRC 107/297 with PV-ELLITE software Only Native file of calculation of air receiver could submitted. |  | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Loading Data & Anchor Bolt Data | Anchor bolt data (Size, number, B.C.D., material,) and final loading data including erection, operating and hydro tests shear and moment due to wind and earthquake shall be provided by vendor 1 month after purchase order. Please confirm. | Anchor bolt data (Size, number, B.C.D., material,) and final loading data will be mentioned in air receiver detail drawing document. | Noted and closed. |  |
|  | PWHT | When stress relieving is required, whole equipment in one piece without removable internals shall be located in the Furnace. All surfaces of flanges shall be protected by vendor due to prevent of damage. Use of direct flame is not approved. | PWHT: Not applicable | Noted and closed. |  |
|  | Comments on Vendor’s Proposal and its attachments | Please refer to the Attachment #1 of this TCL for Mechanical comments on the offer. | noted | Noted and closed. |  |
| PIPING & MATERIAL |  |  |  |  |  |
|  | PMS | Project PMS shall be followed by vendor. | Because piping between equipment in compressor room are not in our scope and only piping system inside of compressor and dryer and nitrogen generator are in our scope and will be done according of our specification. | The Vendor shall get approval of packages piping material from NISOC. All tie in point material shall be considered according to project PMS. Due to project P&ID (BK-CGS-PEDCO-120-PR-PI-0016 & BK-CGS-PEDCO-120-PR-PI-0015) vendor scope is specified. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | General arrangement drawing | Air & N2 Package General arrangement drawing shall be issued by vendor. | After P.O. will be delivered. | NOTED. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. |
|  | 3D model | PDMS 3D model shall be issued by vendor. | Because piping between equipment in compressor room are not in our scope so PDMS model is not in our scope and only GA and ICW are deliverable. | Due to project P&ID ( BK-CGS-PEDCO-120-PR-PI-0016 & BK-CGS-PEDCO-120-PR-PI-0015) vendor scope is specified PDMS 3d model shall be issued by vendor. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | VPIS | VPIS will be finalized after P.O. (Piping Doc. shall be added to document which listed to document list) | Ok. | NOTED. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. |
| INSTRUMENT & CONTROL |  |  |  |  |  |
| - 1. | Control System Requirements | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0004 and provide the requirement of type C package for nitrogen package, air compressor and sufficient controller for air dryer in field as mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. Also Control and safety system block diagram shall be provided by vendor at bid stage. As per project specification, PLC based UCP (for control and safety) dedicated to each compressor is requested for both package (N2 and air): -One PLC based UCP for each compressor (totally 2.Nos. located on control room) for Air compressor package (type C). -One PLC based UCP for nitrogen package (located on control room) (type C). -Redundancy for CPU/power supply /critical and safety I/Os and communication link to be considered in each UCP. Separate SIL rated system for ESD signals (could be relay based ESD part inside UCP) shall be considered in parallel as safety back up logic. - LCP is required for each compressors for on/off and commissioning/startup and gauge boarding. -Project Spec to be respected for packages type C. -Microprocessor based LCP is required for each dryer. -Regarding Machine monitoring at list minimum requirements as per API to be respected (vibration for trip…) | As mentioned in our technical letter, all compressors and dryers and nitrogen generator have its own exclusive local control panel. These LCPs are micro controller base (ARM series) and has a displayer so they cannot meet the project specification such as redundancy for CPU and power supply and I/O cards and also these LCPs can connect with DCS via hard wire (VFC) or Modbus communication protocol. | OPEN. Vendor shall follow Project specification. Control Block diagram shall be prepared as per TCL requirements. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Instrumentation | Regarding supplying instrument on datasheet), bidder is requested to consider all instruments mentioned on project datasheet. VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0001 for instrumentation devices and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | As mentioned in our technical letter and deviation list all instruments that installed inside of compressors and dryers and nitrogen generator skid are special type that prepare by LCP Europe manufacturer, so in this matter we cannot follow project specification. | OPEN. Vendor shall follow Project specification and AVL. Analyzer sensor and accessories shall be detailed for Air and nitrogen package. | Meeting conclusion-14.08.2023: Vendor Confirmed, technical offer will be corrected. Closed Oxygen analyzer mentioned in tech. letter previously |
|  | Instrumentation Valves | VENDOR shall follow below documents for valves and other documents as mentioned in these documents. - BK-GNRAL-PEDCO-000-IN-SP-0005 - BK-GNRAL-PEDCO-000-IN-SP-0006 - BK-GNRAL-PEDCO-000-IN-SP-0007 VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Please refer to row 2. | OPEN. Vendor shall follow Project specification | Meeting conclusion-14.08.2023: Vendor Confirmed, technical offer will be corrected. Closed. Please refer to last Deviation list |
|  | Connection with MCC | If MCC is supplied by VENDOR : Interface signals between MCC and UCP (VIA IRP) and LCP shall be considered in vendor scope. Below signals are typically defined in this project: - Start command - Stop command - Trip command - Run/Stop Status - Local/Remote status - Fault status | Main MCC is not in our scope but all of mentioned signals will be considered on compressors LCP. | OPEN. MCC is in vendor scope of supply. Vendor shall follow Project specification | Meeting conclusion-14.08.2023: Vendor Confirmed, technical offer will be corrected. Closed. Acc. To our navigation with foreign manufacturer we cannot supply Main MCC |
|  | Cable | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0010 for cables and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | As mentioned in our technical letter and deviation list all instrument cables that used inside of compressors and dryers and nitrogen generator skid are suitable for instruments connection terminals and LCP terminals, so with considering size of terminals in this matter we cannot follow project specification. | OPEN. Vendor shall follow Project specification | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Terminals | All terminals shall be screw type. VENDOR shall note that each core/pair/etc. of cable shall be wired to dedicated terminal | Confirm. | Closed. |  |
|  | Panel Dimension | VENDOR shall specify the estimated number and dimension of panels. | Panels are part on compressors and dryers canopy and are not separate parts. | OPEN. Vendor design shall modified as per item no. 1. | Meeting conclusion-14.08.2023: Vendor Confirmed, technical offer will be corrected. Closed. Please refer to last Deviation list |
|  | Required Power | VENDOR shall specify the estimated power required (non-UPS and UPS) | Will be mentioned in load list. | OPEN. Vendor design shall modified as per item no. 1. | Meeting conclusion-14.08.2023: Vendor Confirmed, technical offer will be corrected. Closed. Load list attached |
|  | Special Tools | VENDOR shall supply all special tools for compressor and axillary devices. This is included the special cables (not mentioned in BK-GNRAL-PEDCO-000-IN-SP-0010), MODBUS sockets, software and etc. The list shall be provided separately. | It is not needed any special tools for our proposed equipment but special parts like sockets and etc. considered on equipment. | Closed. |  |
|  | Protection Instruments | The protective instruments shall be according to API-670. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Please refer to row 2. | OPEN. Project spec regarding machine monitoring sensors shall follow | Meeting conclusion-14.08.2023: Vendor Confirmed, technical offer will be corrected. Closed. Please refer to last Deviation list |
|  | Software | VENDOR shall supply all the software (including the passwords, licenses and program sources). The programs shall be supplied with all tags, descriptions and etc. The software programs shall be installed on engineering laptops and also supplied on CD/DVD. | Our LCPs are micro controller type and their software are not deliverable. | OPEN. Vendor design shall modified as per item no. 1. | Meeting conclusion-14.08.2023: Vendor Confirmed, Closed. Please refer to last Deviation list |
|  | Sequence controller | VENDOR shall clearly specify the scenario for sequence controlling of compressor and dryer also nitrogen package, he interface signals, required hardware (e.g. panels, if any) and additional control or on/off valves. | Please refer to our technical letter. | OPEN. Vendor design shall modified as per item no. 1 | Meeting conclusion-14.08.2023: Vendor Confirmed, Closed. Please refer to last Deviation list |
|  | Start-up | VENDOR shall clearly state the start-up sequence. It shall be mentioned if any additional instrument or valve is required. Also, the interface signals shall be mentioned (if any) | The logic of sequence panel will be delivered after P.O. and any changes can be applied on this logic and all related instruments will be supplied by HAVASAN. | OPEN. Vendor design shall modified as per item no. 1. | Meeting conclusion-14.08.2023: Vendor Confirmed, Closed. Please refer to last Deviation list |
|  | Spare part | VENDOR shall supply all commissioning/ 2 years spare part as per NISOC procedure, attached to PMR. | Please refer to our new spare part list. | OPEN. Vendor design shall modified as per item no. 1 Control panel, valves and analyzer spare part shall be added. | Meeting conclusion-14.08.2023: Vendor Confirmed, Closed. Please refer to last Deviation list |
|  | Hook up | Project Instrument Hook UP drawing (BK-GCS-PEDCO-120-IN-DG-0002), Shall be Followed by vendor | Because lack of space inside of equipment skid and limitation size of connections, it is not possible to follow project specification in this field for inside of equipment skid. | Open. Generally Project drawing shall be followed. Otherwise deviated items shall be added to deviation list. | Meeting conclusion-14.08.2023: Vendor Confirmed, Closed. Please refer to last Deviation list |
| SAFETY |  |  |  |  |  |
|  | Sound Level | Noise level of Machinery / valves inside the package should not exceed 85 dbA @1m. | Confirm | Closed |  |
| ELECTRICAL |  |  |  |  |  |
|  | Specification | All electrical equipment shall comply with electrical references & standard that mentioned on material requisition. IPS-E-EL-100 (1), IPS-M-EL-131 (2). (have been attached) | This specification is for major electrical parts that are used in site but our compressors and dryers local electrical panels have their own special parts and specification with considering equipment protection and safe operation. | - Motors in the packages shall be according to IPS-M-EL-131(2) - All electrical engineering shall be according to IPS-E-EL-100. No deviation is acceptable. | Meeting conclusion-14.08.2023: It will be checked and if there is any deviation will be specified. Closed. Please refer to last Deviation list |
|  | Vendor Scope of Work | All documents related to electrical equipment shall be submitted by vendor such as preliminary load list, electrical equipment list, motor data sheet, motor outline drawing & etc. | All wanted documents as mentioned in document list will be delivered after P.O. |  | Meeting conclusion-14.08.2023: Project's MR will be signed and stamped by vendor and will be submitted. Please refer to last Deviation list |
|  | Vendor Scope of Work | Vendor list & sub vendor list of manufacture shall be according to project AVL. | Confirmed for major parts. |  | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Vendor Scope of Work | The cable & power from switchgear (400V, 3Phase & 50Hz), to main package panel (in field) is by contractor, but the cable in the package from main panel to each equipment (in skid) is by vendor. In other word, from contractor point of view, there are 3 below loads: - Instrument & Plant Air Package (PK-C-2203A) - Instrument & Plant Air Package (PK-C-2203B) - Nitrogen Package (PK-2204) (Tie in between contractor & vendor is main panel which will be provided by vendor). Therefore the total power demand shall be specified by vendor. | Confirm. | Vendor shall consider all electrical protection for its own power panels according to IPS standard. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Vendor Scope of Work | Since the power & control cables in the skid is by vendor, therefore the cable list in the package shall be according to “BK-GNRAL-PEDCO-000-EL-SP-0014”. Interconnection in the skid is by vendor. | Confirm. |  | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. |
|  | Vendor Scope of Work | The engineering & procurement of package shall be according to “BK-GNRAL-PEDCO-000-EL-SP-0011”. | Please refer to our document list. | Not Accepted. Project standard & specification is in priority list. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Vendor Scope of Work | Supplier shall design the routing of the power & control cable inside package & consider according to IPS standard. | Confirm. | Supplier means vendor. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. |
|  | Vendor Scope of Work | If AC UPS or DC UPS & batteries are required, vendor shall provide all necessary documents (such as calculation, list etc.) & supply required equipment according to purchaser standards. | UPS is not required. | AC & DC UPS in the field is by vendor, but in control room is by contractor. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Spare Part | A complete set of spare parts for commissioning shall be supplied for each motor. | Please refer to new spare part list. |  | Meeting conclusion-14.08.2023: Spare part list will be submitted according to project’s requirements Closed. Please refer to last Deviation list |
|  | Test & Inspection | The equipment shall be factory tested. Certified copies of test reports shall be submitted to the purchaser. | FAT will be run for electrical panels. |  | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. |
|  | Vendor Scope of Work | Data sheet & Specification of LV induction motor have been attached. Please follow them. | Confirm, Also please review our deviation list. | In Special cases item of technical offer, no deviation from project specification has been considered by vendor. | Meeting conclusion-14.08.2023: It will be checked and if there is any deviation will be specified. Closed. Please refer to last Deviation list |
|  | Vendor Scope of Work | The method of start & stop of air compressor & interlock with other equipment shall be specified. | Please refer to our technical letter. | After PO schematic diagram shall be issued for both packages by vendor. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. |
|  | Vendor Scope of Work | The interlock between main motor & auxiliary loads of N2 package shall be specified. | Load and unload of compressor will be done according of line pressure and this pressure is depend on N2 generator function and N2 consumption. | After PO schematic diagram shall be issued for both packages by vendor. | Meeting conclusion-14.08.2023: Vendor Confirmed. Closed. Please refer to last Deviation list |
|  | Vendor Scope of Work | I/O List shall be issued by vendor. | According to our document list the I/O document will be delivered after P.O. |  | Meeting conclusion-14.08.2023: Closed. |
| STRUCTURE |  |  |  |  |  |
|  | Earthquake load | Based on STD2800 this site is located at High Risk Area which is named Seismic Zone 2. also according to ASCE 7 (2010) Seismic Zone is equal to Zone 3 with Seismic Importance Factor 1.25 . | All data will be considered in calculation of equipment. | Closed |  |
|  | Wind | According to project spec. mean hourly with 50Yr return period speed shall be considered 120 Km/hr. | All data will be considered in calculation of equipment. | Closed |  |
|  | Anchor bolt | The size and type and location of the anchor bolts should be provided in a guide map. | Confirm. | Closed |  |
|  | Skid | The exact dimensions & size of the skid and the proper alignment under the skid should be determined. | Confirm. | Closed |  |
|  | Loads and center mass | The static & dynamic loads of the equipment and the center of gravity of it shall be determined. | Confirm. | Closed |  |
|  | Room layout | If this equipment must be kept in the room, all the necessary information of the room layout, such as the oil drain system and the required network, or where the air ducts pass, and any required requirements must be clearly mentioned and presented in a layout. | According to project specifications all of the equipment are considered outdoor and under shelter. | Closed |  |
|  | References | The following codes & standards should be considered in the civil & structure section: AWS , ASCE7-10 , INBC6 | We only will issue ICW and foundation load documents after P.O. | Closed |  |