Requisition No.: AZAS14-02-MK-120-IN-MR-0008-D03

Description: MATERIAL REQUISITION FOR ONOFF VALVES (PN6)

Equipment No.: 5

| Item No. | Job Specification No. & Paragraph No. | Requirements of Job Specification | Description of Proposed Alternative | Reason for Proposed Alternative |
| --- | --- | --- | --- | --- |
| 1 | Row No. 24, 36~ 41, 44~47 | TRUNN MOUNT | Floating | For these sizes, it is not common practice to change the standard design (floating) to trunnion. Despite the trunnion type making the valve more complicated, the sealing due to low pressure and the small size of the ball can lead to leaking problems. More importantly, changing the model from floating to trunnion for such size and class will be costly and time-consuming when repair and maintenance are needed. |
| 2 | Row No. 117~154, 157~173 | RENEW SEAT | WELDED RENEWABLE SEAT | Please find below our reply regarding the seat ring specification. Reference is made to API 600 clause 5.3.3.4: "Where separate seat rings are provided, they shall be shoulder or bottom seated, and either threaded or seal welded in place, except that for DN ≤ 50 (NPS ≤ 2), rolled or pressed-in seat rings may be used. Threaded seat rings shall be seal welded; tack welding or stitch welding is not permitted." Therefore, it is clear that our gate valves have welded renewable seats, where the contact between the body and seat ring is seal welded. |
| 3 |  |  |  |  |