| # | Clarification Title | Clarification Description (Rev.00) | Conclusion (Rev.00) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description (Rev.01) | Conclusion (Rev.01) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description (Rev.02) |
| --- | --- | --- | --- | --- | --- | --- |
| GENERAL |  |  |  |  |  |  |
|  | Documents Priority | Vendor to confirm that in case of contradiction between this TCL and offer; vendor confirmation of compliance through this technical clarification is governed and shall be followed by vendor after order. In such case offer won’t be considered as basis of any claim after order. | Confirmed | Closed |  |  |
|  | Deviation List | Bidder shall confirm his full compliance to meet all purchaser documents listed and attached to MR, code / standards listed in MR with ref. No. "BK-GCS-PEDCO-120-ME-MR-0008" as well as MR requirements. All deviations shall be listed in project deviation form, attach to this TCL, signed and stamped. Notes: 1-Any exception and deviation from the requirements of MR and the related specifications including referenced codes & standards shall be clearly declared by vendor in writing for purchaser’s approval at bid stage by using of the format included in “Letter Of Conformity” attached to material requisition. Raising no deviation to any single requirement means full compliance of vendor quotation to that requirement. Purchaser has no responsibility to discover deviations or alternatives scattered over the proposal in addition to what proclaimed explicitly in deviation list; hence such deviation (if any) is disregarded and shall not be considered for vendor claim in future. (Vendor to confirm) 2- Bidder shall confirm that all MR attachments listed in MR are available for her for review at bid stage. 3-Considering items 1 & 2, please note that any deviation at detail stage will not be accepted by purchaser. | Confirmed, please see “Deviation List”. | Noted |  |  |
|  | Documents Priority | Bidder to confirm his compliance with documents priority stated in MR. Any conflict between the documents shall be notified by bidder to purchaser and resolved by his agreement | Confirmed | Closed |  |  |
|  | Requirement For Documents | Bidder to confirm that documents required after purchase order shall be submitted in accordance with “Vendor documents min. requirement list” attached to the MR. | Confirmed | Closed |  |  |
|  | Documents | Bidder shall provide following documents while bidding: 1- Copy of valid quality system certificate(ISO 9001/2 or 3) 2- Sub- vendors / sub-suppliers list. 3- Deviation list with letter of conformity (Refer to item A2 above) 4- Reference and Experience List 5- Joint venture agreement (If any) with any company to design / fabrication of equipment. In case of joint venture ; qualified documents of the company who is part of joint venture shall be also submitted | Confirmed, please see attached | Closed |  |  |
|  | Sub-vendors | Bidder to confirm that Sub-Vendors shall be selected from Project’s AVL. Please refer to purchaser’s comments on vendor’s sub-vendors list. | Confirmed, please see attached “Sub Vendor List” | Noted |  |  |
|  | Painting specification | Bidder to confirm that surface preparation and painting of all equipment material including valves , pipes , fittings and everything within the package are in accordance with project painting specification with ref. "BK-GNRAL-PEDCO-000-PI-SP-0006" attached to MR. | Confirmed | Closed |  |  |
|  | Specification | Bidder to confirm that Packing, Marking, Preservation and Preparation for Shipment will be done in accordance to the project specification with Doc. No; BK-GNRAL-PEDCO-000-QC-PR-0045 attached to the MR. | Confirmed (Non-Wooden Box, enough safe and protection) | The project specification with Doc. No; BK-GNRAL-PEDCO-000-QC-PR-0045 shall be followed by vendor | Confirmed (due to size of Package, no wooden box is needed) | Noted, vendor shall be followed specification with Doc. No; BK-GNRAL-PEDCO-000-QC-PR-0045 |
|  | Inspection and Test Plan | Inspection and Test Plan / Quality Control Plan and their related inspection level (Witness, Hold, Review and Approval) shall be at least in accordance with MR and all project requirement such as NDT PROCEDURE. Any deviation is not accepted unless otherwise specified in deviation list and approved by Purchaser. | Confirmed, see KSP ITP too. | ITP will be finalized during PIM | Confirmed | Noted |
|  | Material Certificate | Bidder to confirm that for all pressure parts and those required special consideration or stainless steel as well as for supports the material certificate should be in accordance with EN 10204, 3.1 Material certificate for non-pressure parts shall be in accordance with EN 10204, 2.2 | Confirmed (Material certificate or Laboratory Analysis Report) | Closed |  |  |
|  | Scope | Bidder shall confirm the scope of work and supply are fully in accordance with MR and its attachment with considering following items: 1- The consequences of HAZOP and SIL study shall be in vendor scope of supply and work. 2- Participation in technical meeting whenever it is necessary by Purchaser/Client to close engineering document after order is in vendor scope 3- Lifting beams ( Spreader beams) for erection with sling shackle and etc shall be designed and supplied by vendor 4- N2 preservation with pressure regulator, valve & flexible hoses during transportation & 18 months of site storage based on requirements in MR shall be in vendor scope of supply / work. 5- All utility conditions shall be as per document "Basic engineering design data (10‐BD‐00‐PR‐SPC‐0001)". Moreover, bidder shall consider utility noted condition as per project design data. If other utility condition is needed, whole required facilities is in vendor scope of work/supply. 6- Anchor bolts | Regarding Scope of Supply, please see our Deviation List. - N.A - Confirmed - N.A (just Enough Lifting Lugs will be considered. - Considering type of Chemical; Corrosion Inhibitor, N2 preservation (& Regulator / Flexible Hose) is Not Required as our Experience. - Confirmed, see KSP P&ID - Anchor Bolt is confirmed (4 No. for Skid) | 1.Not accepted this requirement shall be followed by vendor without any cost impact 2.Closed 3. Any requirement for erection shall be supplied by vendor 4.Closed 5.Noted 6.Closed | - Consequences of HAZOP & SIL study is confirmed to be in scope of KSP supply & work. - --- - Lifting Beam for erection with sling shackle design and supply is confirmed - --- - --- | - Noted - Noted |
|  | Applied standard | Applied Standards for equipment design, fabrication and test shall be listed | Confirmed, see attached “Technical Proposal-Rev1”. | Noted |  |  |
|  | outdoor installation/ operation under project site conditions | Vendor shall confirm Air Compressor Packages, Dryer Packages and N2 generation package are fully suitable for outdoor installation/operation under project site conditions “Process Basis of Design; BK-GNRAL-PEDCO-000-PR-DB-0001 “without requiring any additional protection. | Confirmed, but it is better to be installed under Shelter. | Closed |  |  |
|  | Suitability of utility characteristics available at site | Vendor shall confirm suitability of utility characteristics available at site as per project site conditions “Process Basis of Design; BK-GNRAL-PEDCO-000-PR-DB-0001 “. Supplying all facility for utility distribution and regulation shall be part of the vendor scope. | Confirmed (Chemical/ Utility Supply are not by KSP) | Closed |  |  |
|  | Warranty | Vendor shall warranty the engineering, materials and workmanship of the equipment/goods. Under this warranty, all materials, parts, and assemblies found to be defective during the guarantee period shall be replaced free of charge by VENDOR. Also, The guarantee period shall be eighteen (18) months from the date of delivery or twelve (12) months from the installation date of each equipment/packages at site. | Confirmed, see attached “Guarantee Terms”. | Closed |  |  |
|  | Document | Bidder to confirm that all native files due to all calculations and drawings shall be submitted for purchaser approval after order. | Confirmed | Closed |  |  |
|  | Project’s MR | Vendor shall follow project’s MR (Updated Revision D01 with Attachments) as Attachment#1 of this TCL. | Confirmed, See KSP “Deviation List” too. | Noted |  |  |
| PROCESS |  |  |  |  |  |  |
|  | Process Data | 1- inhibitor injection rates (Pint/MMSCF) shall be specified 2- Solvent specification shall be specified 3- The ratio of solvent to inhibitor shall be specified 4- All Equipment shall be designed as per project design criteria | - By Contractor - By Contractor - By Contractor - Confirmed | Closed |  |  |
|  | Project documents | Latest revision of process documents shall be followed by vendor as Attachment#2 of this TCL. | Confirmed | Closed |  |  |
| MECHANIC |  |  |  |  |  |  |
|  | Document Requirement | Bidder to submit all required documents with tender as per “VDR Form” in the MR. For example below documents: 1-Marked up and Filled out MR scope of supply and works (It shall be signed and stamped) 2-GAD for air compressors, air dryers and nitrogen generators with dimensions, all auxiliaries and heaviest part weight table (by considering air cooler in the package) 3-Cross sectional drawing for compressors with BOM 4-Performance Curves 5-Equipment data sheet in each standard format 6-Utility consumption and load list 7-Spare part list 8-Special tool list 9-P&ID’s dedicated for air compressor, dryer and nitrogen generation packages with all equipment, instruments and lube oil, cooling and control systems. 10-Noise Data 11-Equipment List on detail for mechanical and other items | - Confirmed, considering our “Deviation List” - After PO (GAD for Chemical Injection Package, other items; Air Compressor, Air Dryer, Nitrogen Generator are not in KSP scope. - Air Compressor is out of KSP scope - Performance Curves of Dosing Pumps after PO - Data Sheet after PO - Utility Consumption & Load List is attached - Spare Part List and Special Tools is attached - Just P&ID for Chemical Injection Package (CIP), included in Technical Proposal. - After PO - At this stage, please see “Sub Vendor List”. | - Noted - Noted - Cross sectional drawing pumps with BOM - Noted - Noted - Noted - Noted - Noted - Noted - For noise data, Project requirement shall be considered - Equipment List on detail for mechanical and other items shall be considerd. |  |  |
|  | Ladder and platforms | Access ladder and platforms, stairway shall be studied, designed and supplied by vendor. Relevant detail drawings shall be submitted for approval. They shall comply with project standards attached to MR, as minimum. | Confirmed, Ladder Drawing after PO. | Noted |  |  |
|  | Interconnecting piping | Design and supply of all interconnecting piping between equipment and all instruments, valves and … on them is in the vendor's scope. Also all between skid piping (including supporting) c/w all instrument and valve on them shall be designed, prefabricated and supplied by vendor. Detailed isometric DWGs shall be submitted for them. Note1: On skid piping to be assembled at vendor's shop. Note2; Outside skid piping to be prefabricated and match marked for site assembling. | Confirmed | Closed |  |  |
|  | Heat tracing c/w supports, clips and etc. | Design and supply of heat tracing c/w supports, clips, and other provisions (including panels, heating cables and relevant connectors, electrical cables, junction boxes and thermostats) shall be part of the vendor scope. | Heat Tracing for CIP? Please Clarify, while it was not available in MR. | Noted and Closed |  |  |
|  | Insulation c/w supports, clips and etc. | Design and supply of insulation c/w supports, clips and other provisions shall be part of the vendor scope. | Insulation for CIP? Please Clarify. | Noted and Closed |  |  |
|  | Commissioning spare parts | Pre-commissioning, commissioning and start up spare parts shall be supplied by vendor. Vendor shall confirm to provide all required spare parts as per project spare part list attached to MR. | Commissioning is in scope of KSP, with any required Part, but without any list. Please see “Spare Part List & Special Tools”. | Commissioning spare parts as per project spare part list to be supplied by vendor | Confirmed, see attached Spare Part & Special Tools. | Noted |
|  | Two years spare parts | Two years spare parts shall be submitted by vendor as per project spare part list attached to MR. | Confirmed, Please see “Spare Part List & Special Tools”. | Noted |  |  |
|  | Capital spare part | Capital spare part shall be submitted by vendor as optional offer. | Spare Part for Dosing Pump is included in our List, please clarify. | Noted |  |  |
|  | Vent and Drain of Packages | All vents and drains of the packages to be valved, manifolded and to be routed to the edge of skid. | Confirmed, Drain with valve and manifolded to Skid Edge, but Vent to Atmosphere, without any Valve (see P&ID) | Closed |  |  |
|  | Tie-in | All Tie-in for equipment in the packages shall be equipped will flange and valves | Confirmed (see P&ID) | Closed |  |  |
|  | PMI, pickling and passivation | Positive material identification (PMI), pickling and passivation for non-carbon steel material to be included in vendor’s scope of work. | Considering Material Certificate / Laboratory Test Report, PMI is not Required, Also Electro Polishing for SS Tank is recommended instead of Pickling. For SS Piping, Metal Polishing with Acid Paste will be implemented. | Closed |  |  |
|  | Coupling | Couplings and coupling to shaft junctures shall be rated for at least the maximum driver horsepower plus minimum service factor of 1.5. | Confirmed | Closed |  |  |
|  | Skid of the packages | 1-Vendor shall specify number of skids inside package with dimension and weight and preliminary general arrangement. 2-In case of multi skid, please clarify if there is any special procedure for installing and assembling of loose parts. 3-Each skid shall be furnished with 4-point lifting facility and no. two earth lugs located at opposite sides and nameplate made of SS material. 4-Vendor shall specify space required around the skid for maintenance. | - One number, around 4*3*3.5 m, ~2 Tons. - Single skid - Confirmed - Confirmed | Closed |  |  |
|  | Noise level | If noise enclosure will be required for meeting the project noise limit, it shall be supplied by vendor. Any required facilities for ventilation, fire and gas detection and fire extenuation (if any) for enclosure shall be supplied by vendor. | NA | Not accepted this requirement shall be followed by vendor | the Package is installed outdoor, and no firefighting is by Vendor, for facilities inside package in case of need any noise enclosure, will be supplied by vendor. | Closed |
|  | Material | Below requirements shall be followed by vendor: -Wall/roof Plates of tank(s) shall have a minimum thickness of 5 mm. Plates material shall be stainless steel type 316L. -Material of the Storage Tank(s) and its leg supports shall be S.S 316L -The pumps shall be constructed of 316L stainless steel with components suitable for the specified chemical duty. The material grades shall be as per API 675. -All wetted parts of mixers shall be stainless steel 316L. - The minimum size of the package pipes shall be 3/4” and material used of them shall be S.S316L. - Body material of pulsation dampener to be considered SS316L. -The package material shall be in compliance with the requirements of the project “Corrosion Study & Material Selection Report” and project “Piping Material Specification”. | - Confirmed - Tank: SS316L, Leg: CS (with SS316L Reinforce Pad) - Confirmed - Confirmed - Considering low capacity Pump (2 L/Hr), Piping ½” is enough (SS316L), please Confirm - Pulsation Damper SS316L is confirmed - Confirmed (but see our Deviation List from PMS) | 1-Closed 2- Closed 3- Closed 4- Closed 5- Package Pipes size shall be at least 3/4". 6-Closed 7-Noted | - Piping size ¾” is confirmed | Closed |
|  | Motor Power | For motor driven units, the motor nameplate rating (exclusive of the service factor) shall be a minimum of 110 % of the maximum power (including coupling losses) required for any of the rated conditions. Vendor shall confirm that motors nominal power has been considered with derating factor of site ambient condition. | Confirmed | Closed |  |  |
|  | Corrosion allowance | For all parts made of carbon steel, corrosion allowance of 3.2 mm at least to be considered by vendor. In addition corrosion allowance shall be in accordance with the project "Corrosion Study & Material Selection Report". This requirement shall be followed by vendor. | Confirmed (Tank and Piping: SS316L) | Closed |  |  |
|  | Pump’s Capacity | Capacity required for high and low pressure pumps of package is 2.21 and 3.08 lit/hr respectively. As per API 675 standard, Rated capacity shall be at least 110% of maximum capacity required. This requirement to be followed by vendor and revised his offer accordingly. | Confirmed, please see Technical Proposal-Rev1 | Closed |  |  |
|  | Mixer | - Mixers shall be stable while agitating contents of Tank from 1/3 to full. - Mixer sealing type shall be mechanical seal. - Vendor shall follow all requirements of IPS-M-PM-330(1) standard for mixer. | - Confirmed - Packing as Sealing of Agitator/Mixer - Simple and Small Mixer /Agitator, 24 RPM | 1-Closed 2-Mixer sealing type shall be mechanical seal, Otherwise add it to the deviation List 3-Closed | The mixer is a simple low speed (24 RPM) Agitator, and no Mechanical Seal is needed, just suitable Packing is enough and supplied. | Noted |
|  | Pump’s MAWP | Maximum allowable working pressure for pumps to be submitted by vendor in next revision of technical offer. | Confirmed, after PO | Closed |  |  |
|  | Pulsation damper | Pulsation dampeners shall be provided by vendor to limit pulsation within ±3%. | Confirmed | Closed |  |  |
|  | Applied Standards | Applied Standards for equipment design, fabrication and test shall be listed. -The controlled volume pump shall meet the requirements of API 675 as amended and supplemented by IPS-M-PM-150. -The pulsation dampener and storage tanks shall be designed as per ASME Section Vlll Div. I | Confirmed, see attached “Standard & Reference Code” | Closed |  |  |
|  | Vendor’s Scope | All vents and drains of the package to be valved, manifolded and to be routed to the edge of skid. | Confirmed, Drain with valve and manifolded to Skid Edge, but Vent to Atmosphere, without any Valve (see P&ID) | Closed |  |  |
|  | Inspection and Test | Test procedures and acceptance criteria will be checked after order with project specifications and related standards. Vendor to confirm to follow project standard and spec. | Confirmed, see our ITP too. | ITP will be finalized during PIM | Confirmed | Closed |
|  | Purchaser’s approval | Regarding technical offer, diameter, dimension and thickness of each equipment will be checked after receiving vendor calculation and data after order and comments in detail shall be followed without cost impact. | Confirmed (Tank Thk: 5 mm) | Closed |  |  |
|  | Guarantee | Performance, Process and Mechanical Guarantee shall be in the vendor's scope. | Confirmed, see attached “Guarantee Terms & Condition”. | Closed |  |  |
|  | Project’s MR | Vendor shall follow Project’s MR with its Attachments as Attachment#1 of this TCL. (Updated Revision D01 and attachments will be sent for vendor) | Confirmed, considering KSP “Deviation List” too. | Noted |  |  |
| MECHANIC FIX |  |  |  |  |  |  |
| 1. | Fixed equipment mechanical data sheet such as drum /storage tank | Vendor shall submit relevant mechanical data sheet to define shell/head/nozzle/support type & materials & dimension and designing assumption (J.E. / C.A. & etc.…) | Confirmed, see Technical Proposal-Rev1; “Technical Specification of CIP” | Noted. |  |  |
| 2. | Special tools | Special tools for lifting of vessel (i.e. spread bar and skirt bracing if required based vendor’s lifting calculation & relevant bolts/nuts/gasket, Hydraulic bolt tensioning device for bolts greater than 1-1/2” and etc ), shall be considered in vendor’s scope. | Storage Tank has Two No. Lifting Lugs | Noted and closed. |  |  |
| 3. | Lifting/Shipping | Shipping calculation, hydrotest stability calculation and relevant drawings & temporary shipping support for all equipment shall be considered in vendor’s scope and submitted for purchaser’s review at detail stage. Please confirm | Confirmed | Noted and closed. |  |  |
| 4. | Spare part for Fix equipment | Commissioning spare part shall be based on vendor recommendation considering followings as the minimum: -Gaskets: 100 % -Internal Gaskets (If required): 100% -Bolts & Nuts, minimum two set: 10% for each size/material/type Two years spare part shall be based on vendor recommendation considering followings as the minimum: -Gaskets: 200 % -Internal Gaskets (If required): 200% -Bolts & Nuts, minimum two set: 10% for each size/material/type installed -Internal Bolting (If required): 10% for each size/material/type installed Please submit priced 2 years spare part list (in SPLIR form and as optional) | Commissioning is in scope of KSP, with any required Part, but without any list. Please see “Spare Part List & Special Tools”. | Vendor shall be submitted commissioning spare part list after P.O.. | Confirmed | Closed. |
| 5. | preservation | N2 purging preservation with following conditions shall be consider in vendor’s scope of work & supply (it shall be considered in vendor’s base price not as optional): 1-All opening shall be sealed with a suitable metal cover and gasket. Vessels shall be filled with nitrogen gas at a pressure of 0.5 kg/cm2(g). 2-The equipment shall have a temporary gauge to determine purge pressure. 3-Vendor shall submit N2 filling procedure for purchaser/contractor approval at detail stage. 4-All the required ancillaries such as metal covers, bolts & nuts, gaskets and pressure gauge shall be provided by vendor. Please confirm and modify your offer. (using silica gel instead of N2 purge is not acceptable and shall be omit from your offer.) | N2 is not required for Corrosion Inhibitor Tank. Please Clarify / Confirm. | Which type of preservation has been proposed by vendor after PICKLING and PASSIVATION for long time storage. please clarify. | Filling by N2 is not possible in this case of Storage Tank (Atm.) and Piping. And not required. |  |
| 6. | Access for internal loading/unloading | Manhole / hand hole Access for internal loading/unloading of drums to be considered by vendor. | Manhole 20” dia (with Hatch Door) is considered. | Noted and closed. |  |  |
| 7. | Nozzle WRC | Local stress analysis of shells under external loads from piping system and attachments (platforms, pipe supports, davit, lifting lug) shall be included in strength calculation. And also, nozzle load shall be considered according to Allowable Nozzle loads on static Equipment (as per project specification) should be done. Please note that wherever WRC is not applicable FEA shall be done to evaluate local stresses. The original file (native file) of all analysis including all input data and results should be provided by Vendor. Please confirm. | Considering small size of Piping (1/2”) connected to Storage Tank, it is N.A | Noted and closed. |  |  |
| 8. | Loading Data & Anchor Bolt Data | Anchor bolt data (Size, number, B.C.D. ,material ,…) and final loading data including erection, operating and hydro tests shear and moment due to wind and earthquake shall be provided by vendor 1 month after purchase order. Please confirm. | Confirmed | Noted and closed. |  |  |
| 9. | PWHT | When stress relieving is required, whole equipment in one piece without removable internals shall be located in the Furnace. All surfaces of flanges shall be protected by vendor due to prevent of damage. Use of direct flame is not approved. | Confirmed (N.A) | Noted and closed. |  |  |
| 10. | Material requirement | Raw materials supplied for Wet H2S service and shall be in strict accordance with NACE standard MR0103, latest edition and project specification attached to MR, shall be followed. Please confirm. | NACE MR-0103 is not required, considering type of Chemical; Corrosion Inhibitor. | Noted and closed. |  |  |
| 11. | Project documents | Vendor shall provide compliance with the following project documents: - Specification for pressure vessels [BK-GNRAL-PEDCO-000-ME-SP-0001] - Standard drawing for pressure vessel [BK-GNRAL-PEDCO-000-ME-DW-0001] - Specification for Painting [BK-GNRAL-PEDCO-000-PI-SP-0006] - Specification for insulation [BK-GNRAL-PEDCO-000-PI-SP-0019] - Specification for welding [BK-GNRAL-PEDCO-000-PI-SP-0011] - Mechanical Design Criteria [BK-GNRAL-PEDCO-000-ME-DC-0001] | Confirmed | Noted and closed. |  |  |
| 12. | Guarantee | Vendor shall unconditionally guarantee the materials and workmanship on all material and/or services for a period of 12 months after successful start-up at the Purchaser’s Production Site or 18 months after the date of delivery whichever comes earlier. Bidder to confirm. | Confirmed | Noted and closed. |  |  |
| 13. | Material | Manufacturer of FRP vessels is specified as “Tabarestan” in Tank datasheets, while the same is not found in submitted sub-vendor list. Vendor shall revise accordingly. | FRP Vessel? Proposed tank is SS316L, and Manufacturer is KSP. | Noted and closed. |  |  |
| 14. | Equipment supports | Equipment supports and body local stress analysis at support attachment location shall be performed by Vendor. All required dead/live loading as per project specification shall be considered. Any local increase in body thickness if required due to approved calculation shall be considered by Vendor without any cost impacts. | Confirmed, PVElite Calculation after PO. | Noted and closed. |  |  |
| 15. | Seismic/Wind parameters | Vendor shall follow project specifications for Seismic/Wind parameters. Below data shall be considered as minimum requirements: - For seismic: - Code: ASCE 7-10 - Importance Factor: 1. 25 - Site Class: D - Ss = 0.781 - S1 = 1.25 - Fa = 1 - Fv =1.5 - Design wind speed: 120 km/h @10m - Wind Exposure: D - Wind Imp. Factor: 1 | Confirmed | Noted and closed. |  |  |
| 16. | Native Files | Version for all Software to be considered in accordance with code edition. For PV-Elite: PVElite 2019 should be considered for general mechanical calculation. ASME Material Data Base shall be set to 2019. Version of PVElite can be discussed in K.O.M. For Nozzle Pro: Nozzle Pro V8 (PRG2019) For AutoCAD: Native files of drawings to be compatible with AutoCAD 2014. Vendor to confirm. | Confirmed (Nozzle PRO is Not Required, considering small nozzles of tank) | Noted and closed. |  |  |
| 17. | Accessories | Equipment shall be supplied with all required accessories such as internal parts, nozzles, supports, lifting devices, name plates, earth lugs, insulation supports, fire proofing supports and so on. All internals including vortex breakers are included in the Vendor's scope of supply. | Confirmed, (but Insulation Support, Fire Proofing is N.A) | Noted and closed. |  |  |
| 18. | Attachments | Any attachments which are directly welded internally or externally to the vessel shall be made of the same material grade used for the shell or heads as appropriate. | Confirmed (SS316L as Reinforce Pad) | Noted and closed. |  |  |
| 19. | Minimum Documents | In case of order, the minimum documents which are listed in the table of ATTACHMENT #2 of MR shall be submitted by the Vendor. In addition, if deemed necessary, more specific documents may be required for completion of job and shall be provided by Vendor. | Confirmed (every Applicable Document, as VDR after PO) | Noted and closed. |  |  |
| 20. | Mechanical Calculation | Vendor shall provide mechanical calculation for all components including pressure parts, non-pressure parts, flat cover (if any), structural parts, etc. as per project specifications. | Strength Calculation (PVElite) for Tank will be sent after PO | Noted and closed. |  |  |
| 21. | Agitator | The loads originated by the agitator including axial load, torsion & bending moments, to be taken into consideration for check of stresses in the wall of the top head or shell. | Confirmed (RPM 24) | Noted and closed. |  |  |
| 22. | Detail Drawing | In case of order, complete detail drawing should be provided in separate sheet including but not limited to following list: - General Arrangement - Body detail - Nozzle Detail - External attachment - Internal attachment - Template/Gauge Plate Drawing (for skirt-supported vessels only) - Name Plate - Ladder & platform detail drawing - Development drawing - Transportation Detail Drawing - General Arrangement, complete detail drawings and support drawings related to the removable internals. (if any). All drawings shall be drawn in suitable scale (drawings to be provided in A3 size and shall have weighted part list.) | Confirmed, after PO | Noted and closed. |  |  |
| 23. | Thicknesses after forming | The thicknesses after forming of any pressure retaining parts shall not be less than the calculated thickness (Only positive tolerances on plate thickness are acceptable.) | Confirmed | Noted and closed. |  |  |
| 24. | Foundation loading data | Foundation loading data including erection, operating and hydrostatic test, shear and moment due to wind and earthquake shall be provided 1 month after P.O. It’s the Vendor’s responsibility to verify and finalize the quantity/diameter of setting and anchor bolts. | Confirmed | Noted and closed. |  |  |
| 25. | Native files | All vendor deliverable shall be submitted with its native files for all revisions. All native e-files should be submitted in original format (e.g. AutoCAD, Word, Excel, etc.) along with PDF file. Vendor to Confirm. | Confirmed | Noted and closed. |  |  |
| 26. | Specification for Welded Pressure Vessel | For the vessels which operate under pressure (e.g. with DP: 3.5 barg), project requirements in accordance with Specification for Welded Pressure Vessel: BK-GNRAL-PEDCO-000-ME-SP-0001 shall be fully met by Vendor. Vendor to confirm. | The Storage Tank Design Pressure is ATM + FOL (as Duty Spec for CIP). If 3.5 Barg is required, please clarify. | Noted and closed. |  |  |
| 27. | Type of Flanges | Using Slip-on type flanges are not permitted for pressure vessels. For pressurized vessel just weld neck flange is acceptable. | Confirmed | Noted and closed. |  |  |
| 28. | Ladder and platforms | Ladder and platforms shall be designed by vendor in accordance with “Standard drawing for pressure vessel [BK-GNRAL-PEDCO-000-ME-DW-0001]. Besides, sufficient internal access by using Internal ladder and/or ladder rungs shall be considered by Vendor. | Confirmed (just simple Ladder for small Tank) | Noted and closed. |  |  |
| 29. | Gaskets | Gaskets shall be considered as per project specifications. Vendor to confirm. | Confirmed | Noted and closed. |  |  |
| 30. | Calculation for Platforms | Calculation is required for those platforms either of following specified cases: - Platforms which their geometry (plan) does not covered by L&P standard drawings, - Platforms which having piping loads more than 400kg vertically or 200 kg horizontally. - Whole platform structures for above-mentioned platforms shall be submitted for review/approval. | N.A | Noted and closed. |  |  |
| 31. | Using leg supports | Limitations on weight/size of vertical vessels for using leg supports as per project specification shall be considered by Vendor. In any other cases (except for small vessels mounted on skid) skirt support shall be considered for vertical vessels by Vendor. However, any deviations from project specification shall be approved by purchaser. | Storage Tank (1600 L) will be mounted on 4 Legs (Leg: L shape 120 or 150) | Noted and closed. |  |  |
| 32. | Hydrostatic test | Under the condition of the hydrostatic test, the stress shall not exceed 90 percent of the minimum specified yield strength of the material at testing temperature. For vertical vessels, which are shop-tested in the horizontal position, the shop test pressure must be increased by the hydrostatic head corresponding to the vessel field- tested full of water in the vertical position, in compliance with the calculation note. Vendor to confirm. | Confirmed | Noted and closed. |  |  |
| 33. | MAWP | For pressure vessels, Nozzle necks, nozzle reinforcement and flanges shall not be limiting elements in the determination of maximum allowable working pressure. If nozzle necks, nozzle reinforcement or flanges prove to be limiting elements, the MAWP shall be limited to the design pressure. | Confirmed | Noted and closed. |  |  |
| 34. | Chemical Cleaning | Chemical Cleaning, Pickling & Passivation for Stainless Steel Parts are in Vendor’s scope. | Electro Polishing of SS Storage tank is recommended instead of Pickling/ Passivation. And Metal Polishing with Acid Paste will be implemented for SS Piping. | Noted and closed. |  |  |
| 35. | Surface preparation | Surface preparation and painting shall be in accordance with Specification for Painting Doc. No. BK-GNRAL-PEDCO-000-PI-SP-0006. Vendor to confirm. | Confirmed (RAL 7040; Window Gray is recommended instead of RAL 7047; Tele Gray) | Please consider RAL7047as per project spec. | RAL 7047 is confirmed. | Closed |
| 36. | Inhibition materials | Inhibition materials against internal corrosion for transportation and long-time storage at stockyard is in vendor scope of supply. Requirements of section 8.2 of Project Specification for Pressure Vessels (BK-GNRAL-PEDCO-000-ME-SP-0001) for all vessels shall be followed by vendor. Bidder to Confirm. | N.A (just keep under shelter) | Noted and closed. |  |  |
| 37. | Removable cover | The Tanks which will be filled by solid chemical diluted in place shall have fixed roof with segmental removable cover. | Confirmed | Noted and closed. |  |  |
| 38. | Comments on Vendor’s Proposal and its attachments | Please refer to the Attachment #1 of this TCL for other Mechanical comments on the offer. | Confirmed, see “Deviation List” too | Noted |  |  |
| PIPING & MATERIAL |  |  |  |  |  |  |
|  | General Arrangement | Please send us General Arrangement Drawing of Chemical Injection Package. | Confirmed (after PO) At this stage just sample GAD will be sent. | Noted and closed. |  |  |
|  | PMS | Project PMS shall be followed by vendor. | Confirmed, considering “Deviation List” | Noted and closed. |  |  |
|  | VPIS | VPIS will be finalized later. | Confirmed | Noted and closed. |  |  |
| INSTRUMENT & CONTROL |  |  |  |  |  |  |
|  | Control System Requirements | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0004 and provide the requirement of type A package as mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. Signal details and communications shall be followed as per project P&ID. | Confirmed, please see “P&ID” in KSP Technical Proposal. LCP is including all Start / Stop Push Buttons, and Running / Fault Indicator & LALL. | Noted. |  |  |
|  | Panel Requirements | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP- 004 for panels mounted field panels and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed | Noted. |  |  |
|  | Instrumentation | Regarding supplying instrument on datasheet, bidder is requested to consider all instruments mentioned on project datasheet and Project P&ID. VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0001 for instrumentation devices and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed, considering “Deviation List” | Noted. |  |  |
|  | Instrumentation Valves | VENDOR shall follow below documents for valves (if any) and other documents as mentioned in these documents. - BK-GNRAL-PEDCO-000-IN-SP-0005 - BK-GNRAL-PEDCO-000-IN-SP-0006 - BK-GNRAL-PEDCO-000-IN-SP-0007 VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed | Noted. |  |  |
|  | Connection with MCC | If MCC is supplied by VENDOR all interfaces shall be supplied by VENDOR. Below signals are typically defined in this project to be provided for electrical interfaces: - Start command (2 sec high pulse) - Stop command (2 sec high pulse) - Trip command (low signal for trip) - Run/Stop Status (high signal for run status) - Local/Remote status (high signal for local status) - Tripped on Fault (high signal for fault status) - Available for remote control (high signal for available status) Detail signal requirements as per P&ID shall be followed. | Confirmed (Pump Stroke Adjustment is Local / Manual) | Noted. |  |  |
|  | Cable | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0010 for cables and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed | Noted. |  |  |
|  | Terminals | All terminals shall be screw type. VENDOR shall note that each core/pair/etc. of cable shall be wired to dedicated terminal | Confirmed | Noted. |  |  |
|  | Local Panel Dimension | VENDOR shall specify the estimated dimension of panels. | Appx 650*450*300 mm | Noted. |  |  |
|  | Required Power | VENDOR shall specify the estimated power required (non-UPS and UPS) | As “Utility Consumption List”; Max 3.5 kW | Noted. Only 400VAC is provided by contractor. UPS/NON UPS shall be prepared by vendor if required | UPS is Not Required. |  |
|  | Special Tools | VENDOR shall supply all special tools for compressor and axillary devices. This is included the special cables (not mentioned in BK-GNRAL-PEDCO-000-IN-SP-0010), MODBUS sockets, software and etc. The list shall be provided separately. | Compressor and related Special Tools is out of KSP scope. | Noted. |  |  |
|  | Protection Instruments | The protective instruments shall be according to API-670 and NFPA 20. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed | Noted. |  |  |
|  | Start-up | VENDOR shall clearly state the start-up sequence. It shall be mentioned if any additional instrument or valve is required. Also, the interface signals shall be mentioned (if any) | After Filling the Storage Tank, and enough Liquid Level of Chemical inside Tank, by pushing Start Push Buttons of Electro Pump, Dosing will be started, be noted that related Valves of mentioned pump’s piping must be open, as P&ID. | Noted. |  |  |
|  | Spare part | VENDOR shall supply all commissioning/ 2 years spare part as per NISOC procedure, attached to PMR | Confirmed, considering issued “Spare Part List & Special Tools” | Noted. |  |  |
| SAFETY |  |  |  |  |  |  |
|  | Sound Level | Noise level of Machinery / valves inside the package should not exceed 85 dbA @1m. | Confirmed | Closed |  |  |
| ELECTRICAL |  |  |  |  |  |  |
|  | Specification | All electrical equipment shall comply with electrical references & standard that mentioned on material requisition. IPS-E-EL-100 (1), IPS-M-EL-131 (2) & IPS-M-PM-290(2). | Confirmed | Closed |  |  |
|  | Vendor Scope of Work | All documents related to electrical equipment shall be submitted by vendor such as preliminary load list, electrical equipment list, motor data sheet, motor outline drawing & etc. | after PO | Closed |  |  |
|  | Vendor Scope of Work | Vendor list & sub vendor list of manufacture shall be according to project AVL. | Confirmed, see attached “Sub Vendor List” | Closed |  |  |
|  | Vendor Scope of Work | The cable from switchgear to package panel is by client, but the cable in the package form panel to each equipment is by vendor. In other word, from client point of view, there is only one load which assumed the power demand of package is the total load. (Tie in between client & vendor is main panel). | Confirmed | Closed |  |  |
|  | Vendor Scope of Work | According to item 4, the total electrical demand is required. | Confirmed, see “Utility Consumption List”, indicating required Power. | Closed |  |  |
|  | Vendor Scope of Work | Since the power & control cables in the skid is by vendor, therefore the cable list in the package shall be according to “BK-GNRAL-PEDCO-000-EL-SP-0014”. Interconnection in the skid is by vendor. | Confirmed | Closed |  |  |
|  | Vendor Scope of Work | Supplier shall design the routing of the power & control cable inside package. | Confirmed | Closed |  |  |
|  | Vendor Scope of Work | If AC UPS or DC UPS & batteries are required, vendor shall provide all necessary documents (such as calculation, list etc.) & supply required equipment according to purchaser standards. | N.A | Closed |  |  |
|  | Spare Part | A complete set of spare parts for commissioning shall be supplied for each motor. | N.A (Commissioning is by KSP, with any required Part) | Closed |  |  |
|  | Test & Inspection | The equipment shall be factory tested. Certified copies of test reports shall be submitted to the purchaser. | Confirmed | Closed |  |  |
|  | Vendor Scope of Work | Data sheet & Specification of LV induction motor have been attached. Please follow them. | Confirmed | Closed |  |  |
|  | Vendor Scope of Work | Deviation & clarification form shall be issued by vendor | Confirmed, please see “Deviation List” | Closed |  |  |
|  | Vendor Scope of Work | Deviation list for anti-condensation of motor shall be according to IPS-M-EL-131(2). | Confirmed, Anti Condensate Heater will be considered for Motors. | Closed |  |  |
| STRUCTURE |  |  |  |  |  |  |
|  | Earthquake load | Based on STD2800 this site is located at High Risk Area which is named Seismic Zone 2. also according to ASCE 7 (2010) Seismic Zone is equal to Zone 3 with Seismic Importance Factor 1.25 | Confirmed, will be considered in PVElite Strength Calculation. | Closed. |  |  |
|  | Wind | According to project spec. mean hourly with 50Yr return period speed shall be considered 120 Km/hr. | Confirmed | Closed. |  |  |
|  | Anchor bolt | The size and type and location of the anchor bolts should be provided in a guide map. | Confirmed | Closed. |  |  |
|  | Skid | The exact dimensions & size of the skid and the proper alignment under the skid should be determined. | Confirmed after PO (Appx. 4*3*3.5 m) | Closed. |  |  |
|  | Loads and center mass | The static & dynamic loads of the equipment and the center of gravity of it shall be determined. | Confirmed, after PO | Closed. |  |  |
|  | Room layout | If this equipment must be kept in the room, all the necessary information of the room layout, such as the drain system and the required network, or where the air ducts pass, and any required requirements must be clearly mentioned and presented in a layout. | Package will be installed in outside are, but under Shelter (by Contractor) | Closed. |  |  |
|  | References | The following codes & standards should be considered in the civil & structure section: AWS , ASCE7-10 , INBC6 | The only Structure available in the CIP, is Structural Steel Skid, which is designed enough Safe. | Close |  |  |