| طرح نگهداشت و افزایش تولید 27 مخزن |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CALCULATION NOTE FOR HELI PAD, WELL PAD & DIESEL STORAGE PAD - W028 نگهداشت و افزایش تولید میدان نفتی بینک |  |  |  |  |  |  |  |
| D02 |  | AUG. 2024 | AFC | R.Berlouie | M.Fakharian | M.Sadeghian |  |
| D01 |  | FEB. 2023 | IFC | R.Berlouie | M.Fakharian | M.Mehrshad |  |
| D00 |  | DEC. 2021 | IFC | R.Berlouie | M.Fakharian | M.Mehrshad |  |
| Rev. |  | Date | Purpose of Issue/Status | Prepared by: | Checked by: | Approved by: | CLIENT Approval |
| Class:1 |  |  | COMPANY Doc. Number: F0Z-707550 |  |  |  |  |
| Status: |  | IDC: Inter-Discipline Check IFC: Issued For Comment IFA: Issued For Approval AFD: Approved For Design AFC: Approved For Construction AFP: Approved For Purchase AFQ: Approved For Quotation IFI: Issued For Information AB-R: As-Built for CLIENT Review AB-A: As-Built –Approved |  |  |  |  |  |

| PAGE | D00 | D01 | D02 | D03 | D04 |  | PAGE | D00 | D01 | D02 | D03 | D04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | X | X |  |  |  | 66 |  |  |  |  |  |
| 2 | X | X | X |  |  |  | 67 |  |  |  |  |  |
| 3 | X |  |  |  |  |  | 68 |  |  |  |  |  |
| 4 | X |  |  |  |  |  | 69 |  |  |  |  |  |
| 5 | X |  |  |  |  |  | 70 |  |  |  |  |  |
| 6 | X |  | X |  |  |  | 71 |  |  |  |  |  |
| 7 | X |  |  |  |  |  | 72 |  |  |  |  |  |
| 8 | X |  |  |  |  |  | 73 |  |  |  |  |  |
| 9 | X |  |  |  |  |  | 74 |  |  |  |  |  |
| 10 | X |  | X |  |  |  | 75 |  |  |  |  |  |
| 11 | X |  | X |  |  |  | 76 |  |  |  |  |  |
| 12 | X |  | X |  |  |  | 77 |  |  |  |  |  |
| 13 | X |  |  |  |  |  | 78 |  |  |  |  |  |
| 14 | X |  |  |  |  |  | 79 |  |  |  |  |  |
| 15 | X |  |  |  |  |  | 80 |  |  |  |  |  |
| 16 | X |  |  |  |  |  | 81 |  |  |  |  |  |
| 17 | X |  |  |  |  |  | 82 |  |  |  |  |  |
| 18 | X |  |  |  |  |  | 83 |  |  |  |  |  |
| 19 | X |  |  |  |  |  | 84 |  |  |  |  |  |
| 20 | X |  |  |  |  |  | 85 |  |  |  |  |  |
| 21 | X |  |  |  |  |  | 86 |  |  |  |  |  |
| 22 | X |  |  |  |  |  | 87 |  |  |  |  |  |
| 23 | X |  |  |  |  |  | 88 |  |  |  |  |  |
| 24 | X |  |  |  |  |  | 89 |  |  |  |  |  |
| 25 | X |  |  |  |  |  | 90 |  |  |  |  |  |
| 26 | X |  |  |  |  |  | 91 |  |  |  |  |  |
| 27 | X |  |  |  |  |  | 92 |  |  |  |  |  |
| 28 | X |  |  |  |  |  | 93 |  |  |  |  |  |
| 29 | X |  | X |  |  |  | 94 |  |  |  |  |  |
| 30 | X |  |  |  |  |  | 95 |  |  |  |  |  |
| 31 | X |  |  |  |  |  | 96 |  |  |  |  |  |
| 32 | X |  |  |  |  |  | 97 |  |  |  |  |  |
| 33 | X |  |  |  |  |  | 98 |  |  |  |  |  |
| 34 | X |  | X |  |  |  | 99 |  |  |  |  |  |
| 35 | X |  |  |  |  |  | 100 |  |  |  |  |  |
| 36 | X |  |  |  |  |  | 101 |  |  |  |  |  |
| 37 | X |  |  |  |  |  | 102 |  |  |  |  |  |
| 38 | X |  |  |  |  |  | 103 |  |  |  |  |  |
| 39 | X |  |  |  |  |  | 104 |  |  |  |  |  |
| 40 |  |  |  |  |  |  | 105 |  |  |  |  |  |
| 41 |  |  |  |  |  |  | 106 |  |  |  |  |  |
| 42 |  |  |  |  |  |  | 107 |  |  |  |  |  |
| 43 |  |  |  |  |  |  | 108 |  |  |  |  |  |
| 44 |  |  |  |  |  |  | 109 |  |  |  |  |  |
| 45 |  |  |  |  |  |  | 110 |  |  |  |  |  |
| 46 |  |  |  |  |  |  | 111 |  |  |  |  |  |
| 47 |  |  |  |  |  |  | 112 |  |  |  |  |  |
| 48 |  |  |  |  |  |  | 113 |  |  |  |  |  |
| 49 |  |  |  |  |  |  | 114 |  |  |  |  |  |
| 50 |  |  |  |  |  |  | 115 |  |  |  |  |  |
| 51 |  |  |  |  |  |  | 116 |  |  |  |  |  |
| 52 |  |  |  |  |  |  | 117 |  |  |  |  |  |
| 53 |  |  |  |  |  |  | 118 |  |  |  |  |  |
| 54 |  |  |  |  |  |  | 119 |  |  |  |  |  |
| 55 |  |  |  |  |  |  | 120 |  |  |  |  |  |
| 56 |  |  |  |  |  |  | 121 |  |  |  |  |  |
| 57 |  |  |  |  |  |  | 122 |  |  |  |  |  |
| 58 |  |  |  |  |  |  | 123 |  |  |  |  |  |
| 59 |  |  |  |  |  |  | 124 |  |  |  |  |  |
| 60 |  |  |  |  |  |  | 125 |  |  |  |  |  |
| 61 |  |  |  |  |  |  | 126 |  |  |  |  |  |
| 62 |  |  |  |  |  |  | 127 |  |  |  |  |  |
| 63 |  |  |  |  |  |  | 128 |  |  |  |  |  |
| 64 |  |  |  |  |  |  | 129 |  |  |  |  |  |
| 65 |  |  |  |  |  |  | 130 |  |  |  |  |  |

CONTENTS

1	INTRODUCTION	5

2	scope	5

3	NORMATIVE REFERENCES	5

3.1	Local Codes and Standards	5

3.2	International Codes and Standards	5

3.3	The Project Documents	5

4	MATERIAL PROPERTIES	5

5	computer software	6

6	Soil	6

7	load combination	6

7.1	-Ultimate Load Combinations	6

7.2	Serviceability Load Combinations	7

8	DESIGN OF CELLAR AREA	7

8.1	Loads	7

8.1.1	DEAD load (D)	7

8.1.1.1	Self weight (dead)	7

8.1.1.2	machinery load (M.load)	8

8.1.2	Thermal Load	8

8.1.3	Live load	9

8.1.4	Snow loads	9

8.2	CELLAR AREA DESIGN AND RESULTS	10

8.2.1	Settlement Control	11

9	PIT DESIGN	14

9.1	LOADS	14

9.1.1	DEAD load (D)	14

9.1.2	Thermal Load(T)	14

9.1.3	Live load (L)	15

9.1.4	Snow loads (S):	15

9.1.5	Seismic Load (Ex&Ey)	15

9.1.6	Seismic Load of Wall (Ex&Ey)	15

9.1.7	Soil pressure load (H)	17

9.1.8	Overhead load effect on each wall	18

9.1.9	Seismic design  of walls based on Mononobe-Okabe Method	23

9.2	ANALYSIS AND DESIGN	24

9.2.1	Bottom of Pit design :	24

9.2.2	Wall of Pit design :	25

10	Concrete slab(th=20 cm):	27

10.1	Loads:	27

10.1.1	DEAD load (D)	27

10.1.2	Thermal Load(t)	27

10.1.3	Live load (L):	27

10.1.4	Snow loads(S):	27

10.1.5	machinery load(ML)	28

10.2	Soil Pressure Control	29

10.3	Settlement Control	30

11	GAS OIL STORAGE FOUNDATION (13m x 13m x 0.3m)	32

11.1	Thermal Load (T)	33

11.2	Live load (L)	33

11.3	Settlement Control	34

11.4	Soil Pressure Control	34

12	HELIPAD FOUNDATION (8m x 8m x 0.25m)	36

12.1	Loads on foundation	36

12.1.1	Thermal Load(T)	36

12.1.2	Live load (l)	37

12.2	Settlement Control	38

12.3	Soil Pressure Control	39

12.4	Check Additional Reinforcement	40

13	edge beam Design	40

13.1	Soil Pressure Control	42

13.2	Settlement Control	42

- INTRODUCTION

Binak oilfield in Bushehr province is a part of the southern oilfields of Iran, is located 20 km northwest of Genaveh city.

With the aim of increasing production of oil from Binak oilfield, an EPC/EPD Project has been defined by NIOC/NISOC and awarded to Petro Iran Development Company (PEDCO). Also PEDCO (as General Contractor) has assigned the EPC-packages of the Project to "Hirgan Energy - Design and Inspection" JV.

- scope

This document covers minimum necessary requirements for the check  and design of helipad, well pad & diesel Storage Pad relating to well028..

- NORMATIVE REFERENCES

  - Local Codes and Standards

- IPS-C-CE-200	Construction Standard For concrete structures”.

- IPS-E-CE-500	Engineering standard For loads”.

- Iranian Seismic design code for Petroleum facilities .pub.No.038 (3rd edition)

- Iranian National Building Code .INBC No. 9

- Iranian Code of Practice for seismic Resistant Design of Building Standard No.2800 (4th edition)

  - International Codes and Standards

ACI 318-14 “Building Code Requirements for reinforced concrete “. American Concrete institute.

  - The Project Documents

- BK-GNRAL-PEDCO-000-ST-SP-0001	Specification for Concrete Work

- BK-W028-PEDCO-110-SV-PY-0002          Civil & structural Drawings-W028

- MATERIAL PROPERTIES

Material properties are delivered in the following table :

Table 1.Material Propertie

- computer software

Computer software which is used in analysis & design of structure and foundation of storage tank , is defined in the following table.

D02

Table 2.Computer software:

| software |  |
| --- | --- |
| computer software | sap 21.0.2 |
|  | safe 2016 |

- Soil

D02

soil parameters such as allowable soil bearing capacity , allowable settlement and subgrade Modulus are based on  Geotechnical report according to L/b and type of foundation .

- load combination

The following combinations of loads shall be used in the design of foundations, and shall be used for displacement and soil reaction forces.(according to ASCE07-10)

  - -Ultimate Load Combinations

For the design of the structural elements the following load combinations are considered.

-1.4(D±T)+1.6H

-1.2(D±T)+ 1.6L + 0.5S +1.6H

-1.2(D±T) + 1.6S + L+1.6H

-1.2(D±T)+L+0.5S+1.6H

-1.2(D±T)+ 1.0E + L + 0.2S+1.6H

-0.9(D±T) + 1.0E+1.6H

-0.9(D±T)+W+1.6H

  - Serviceability Load Combinations

To check the deflection and displacements, structural stability and soil pressure, the following load combinations are considered.

- (D±T) + H

-(D±T)+L+ H

-(D±T)+S + H

-(D±T)+0.75L+0.75S + H

-(D±T)+0.7E + H

-0.6(D±T) + H

-0.6(D±T)+E + H

Effect of lateral soil pressure & water pressure (H) applied with coefficient of 1.6 in ultimate load   combinations and coeeficient of 1.0 in service load combination .

- DESIGN OF CELLAR AREA

Figure 1-Cellar pit Area

  - Loads

    - DEAD load (D)

      - Self weight (dead)

Part 1 of Dead loads Include the self -weight of the concrete Pad  .

      - machinery load (M.load)

Part 2 of dead load :Assumed 1000 ton load of machine on concrete pad that distributed along it on point that has been shown at bellow .due to the dynamic nature of machine load ,impact load of 1.25 applied at m load as related load case.

MLoad on each point

Pad Thickness=300mm

Figure 2-Applied Point Load On Soil

    - Thermal Load

The ambient thermal load has been assumed based on the thermal variation for structure of 28 ºC.

    - Live load

According to relevant specifications live load about 600 kg/m2 ,has been considered for design of concrete pad.

    - Snow loads

According to Iranian National Building Code No.6  table 6-7-1 this site location is in Zone 1 :   so    Ps=0.25 KN/m2

  - CELLAR AREA DESIGN AND RESULTS

## According to geotechnical report  soil parameters are as follows:

## Soil parameters from geotechnical report

## allowable bearing capacity :2.3 kg/cm2

## allowable settlement :2.5 cm

D02

According to above table from geotechnical report  Ks=0.9 kg/cm3.

    - Settlement Control

Settlement in different service load combinations should be checked by allowable settlement

Figure 3 –Settlement Of foundation under service Load Combination(mm)

D02

Maximum settlement of foundation (δ max) under critical load combination is equals to 12.9 mm, which is less than 25mm and that’s acceptable.

Figure 4 –Soil Pressure under service Load Combination(0.118 N/mm2)

Maximum soil pressure  of foundation (q max) under critical load combination is equals to 0.1039 N/mm2, which is less qall=0.23 N/mm2  and that’s acceptable.

D02

Figure 5 –Punching Shear Control

With assumed loading and with above output from safe 2016 software punching shear  checked and result is less than 1 and is acceptable.

Figure 6 –Additional to Ø12@200 (top & bott) reinforcement

According to above output from safe 2016 software there is no need to additional reinforcement.

- PIT DESIGN

The pit under the rig assumed buried concrete pit that effect of rig loads on this pit considered as follows .This pit has been modeled at sap software..

  - LOADS

    - DEAD load (D)

Dead loads Include the self -weight of the structure

Reinforced Concrete Density	:              2500 kg/m³

Soil density 				:              1800 kg/m³

    - Thermal Load(T)

The ambient thermal load has been assumed based on the thermal variation for structure of 28 ºC.

    - Live load (L)

For bottom distributed load about 500 kg/m2  ,has been considered for design of structure

    - Snow loads (S):

According to Iranian National Building Code No.6  table 6-7-1 this site location is in Zone 1 so

Pg = 0.25 KN/m2

    - Seismic Load (Ex&Ey)

Due to the burial pit the earthquake effects ignored.

    - Seismic Load of Wall (Ex&Ey)

Wall gravity seismic load according to code 038. Section 7 for non -building structures applied on each wall  .

cu=Sa*IRw

Wall 3

L=1.5m

Wall 5   L=2.5 m

Wall 1

L=2.5m

Wall 4   L=3.65m

Wall 2   L=3.65m

Sa=0.75

I=1.25

Rw=3.5

cu=Sa*IRw=0.75*1.253.5=0.27

Wall 1 :             Ewall=0.27*2.5*1000*2.5*0.3*h=506.25h

Wall 2,4 :             Ewall=0.27*2.5*1000*3.65*0.45*h=1108.3h

Wall 3 :             Ewall=0.27*1.8*1000*1.5*0.3*h=302.7h

Wall 5 :             Ewall=0.27*2.5*1000*2.1*0.45*h=637 h

    - Soil pressure load (H)

Soil pressure load applied at each area mesh base on joint pattern load .

Soil density =1800 kg/m³

φ=33

K0=1-sinφ=0.455

σ=K0γh

Figure 7-assign joint pattern data

According to above figure static soil pressure applied to model with this pattern values.

Figure 8 –Applied Soil Pressure Load

    - Overhead load effect on each wall

For North and south wall due to each point load on 5 point we should distributed load on bottom of pit :

Figure 9 –Plan of Load Area

For determine the lateral pressure of soil due to different overheads on the wall according to the theory of elasticity ,horizontal pressure on the wall at depth z due to linear load q with unlimited length calculate from the following equation:

loading area is assumed about  15m x2.5m thus

Figure 10–parameters for equation(a & b)

∆σz=qπ(α+sinα cos⁡(α+2β))

For determine the lateral pressure of soil due to overheads on north & South wall calculation is:

Wall 5   L=2.5 m

Wall 3

L=1.5m

Wall 1

Wall 4

Wall 2

horizontal pressure on wall 2 due to overhead load :

| z= | 0 | alpha | 0 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | beta | 90 |  |  |
|  |  | q | 13.33 | vertical pressure(t/m2) | 0 |
|  |  | pi | 3.1416 | horizontal pressure(t/m2) | 0 |
| z= | 0.5 | alpha | 35.53768 |  |  |
|  |  | beta | 45 |  |  |
|  |  | q | 13.33 | vertical pressure(t/m2) | 1.198592 |
|  |  | pi | 3.1416 | horizontal pressure(t/m2) | 0.359578 |
| z= | 1 | alpha | 45 |  |  |
|  |  | beta | 26.56505 |  |  |
|  |  | q | 13.33 | vertical pressure(t/m2) | 2.90892 |
|  |  | pi | 3.1416 | horizontal pressure(t/m2) | 0.872676 |
| z= | 1.5 | alpha | 45 |  |  |
|  |  | beta | 18.43495 |  |  |
|  |  | q | 13.33 | vertical pressure(t/m2) | 3.757747 |
|  |  | pi | 3.1416 | horizontal pressure(t/m2) | 1.127324 |
| z= | 2 | alpha | 42.27369 |  |  |
|  |  | beta | 14.03624 |  |  |
|  |  | q | 13.33 | vertical pressure(t/m2) | 4.091595 |
|  |  | pi | 3.1416 | horizontal pressure(t/m2) | 1.227479 |
| z= | 2.5 | alpha | 38.8845 |  |  |
|  |  | beta | 11.30993 |  |  |
|  |  | q | 13.33 | vertical pressure(t/m2) | 4.151432 |
|  |  | pi | 3.1416 | horizontal pressure(t/m2) | 1.24543 |

According to above data overhead load applied at model with these joint patterns.

Joint pattern definition for walls

For wall 2               0<h<1.5  V (Q1 wall 2)= -751.3Z+1878.5   h : from top of soil

For wall 2                 01.5<h<2.5  V (Q1 wall 2)= -118Z+1245

horizontal pressure on wall 1 due to overhead load :

| z= | 0 | alpha | 4.09E-08 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | beta | 9.00E+01 |  |  |
|  |  | q | 1.33E+01 | vertical pressure(t/m2) | 0 |
|  |  | pi | 3.14E+00 | horizontal pressure(t/m2) | 0 |
| z= | 0.5 | alpha | 1.84E+01 |  |  |
|  |  | beta | 6.34E+01 |  |  |
|  |  | q | 1.33E+01 | vertical pressure(t/m2) | 0.262077 |
|  |  | pi | 3.14E+00 | horizontal pressure(t/m2) | 0.078623 |
| z= | 1 | alpha | 2.91E+01 |  |  |
|  |  | beta | 4.50E+01 |  |  |
|  |  | q | 1.33E+01 | vertical pressure(t/m2) | 1.151218 |
|  |  | pi | 3.14E+00 | horizontal pressure(t/m2) | 0.345366 |
| z= | 1.5 | alpha | 3.31E+01 |  |  |
|  |  | beta | 3.37E+01 |  |  |
|  |  | q | 1.33E+01 | vertical pressure(t/m2) | 2.03053 |
|  |  | pi | 3.14E+00 | horizontal pressure(t/m2) | 0.609159 |
| z= | 2 | alpha | 3.37E+01 |  |  |
|  |  | beta | 2.66E+01 |  |  |
|  |  | q | 1.33E+01 | vertical pressure(t/m2) | 2.626149 |
|  |  | pi | 3.14E+00 | horizontal pressure(t/m2) | 0.787845 |
| z= | 2.5 | alpha | 3.27E+01 |  |  |
|  |  | beta | 2.18E+01 |  |  |
|  |  | q | 1.33E+01 | vertical pressure(t/m2) | 2.963193 |
|  |  | pi | 3.14E+00 | horizontal pressure(t/m2) | 0.888958 |

For wall 1              0<h<1.5  V (Q1 wall 1)= -406Z+1015   h : from top of soil

For wall 1                 01.5<h<2.5  V (Q1 wall 1)= -279Z+888

Figure 11–apply overhead load

    - Seismic design  of walls based on Mononobe-Okabe Method

According to above calculations Pactive based on change in height is as below:

V=-1027.97Z+2569.9

Figure 12–Apply seismic load of soil on model

  - ANALYSIS AND DESIGN

Model analysis is done by Sap 2000 software.in model loads are applied ,some graphical outputs from modeling are shown as follows.:

    - Bottom of Pit design :

According to Sap 2000 results Maximum  moment for bottom slab is :   6.84ton.m

So for bottom  slab  ∅12@200 mm   is used.

Figure 13–M22 Diagram on bottom of Pit

According to M22 diagram on bottom of pit reinforcement calculation is as follow:

Mn=Muφ=755555

Rn1=Mn1bd2=5.373

m1=fy0.85fc=18.824

ρreq=1m11-1-2m1Rn1fy=0.0013

As=ρreq.b.d=5.12   cm2

Used As used=∅12@200 mm  ok

    - Wall of Pit design :

According to Sap 2000 results Maximum  moment for wall of slab is :   6.8 ton.m

So for bottom  slab  ∅12@200 mm  is used.

Figure14 –Wall 2 of Pit

According to M22 diagram on bottom of pit reinforcement calculation is as follow:

Mn=Mu∅=582222.22

Rn1=Mn1bd2=4.140

m1=fy0.85fc=18.824

ρreq=1m11-1-2m1Rn1fy=0.00109

As=ρreq.b.d=3.92  cm2

Used As used=∅12@200 mm  ok

- Concrete slab(th=20 cm):

One of the panel (15.9x20m)which the  wellhead machine are located on, has been modeled in safe 2016 software and the software output have been shown in the following :

Figure 15-plan of all panels

  - Loads:

    - DEAD load (D)

Overhead caused by equipment located during drilling assumed 500kg/m2.

    - Thermal Load(t)

The ambient thermal load has been assumed based on the thermal variation for structure of 28 ºC.

    - Live load (L):

For bottom distributed load about 200kg/m2  ,has been considered for design of structure

    - Snow loads(S):

According to Iranian National Building Code No.6  table 6-7-1 this site location is in Zine 1 so

Pg = 0.25 KN/m2 = 25 kg/m2

    - machinery load(ML)

Apply 45 ton of truck load with 10.1 m x 2.99 m  dimension on concrete pad .

ML=4500010.1x2.9=1536kg/m2=1.536e-2 N/mm2

Figure16 –loading on Pads(th=200 mm)

  - Soil Pressure Control

## According to geotechnical report allowable bearing capacity for mat foundation is as follows :

## qa=2.25 ≈2.3 kg/cm2=0.23N/mm2

D02

Soil pressures in different service load combinations should be checked by allowable value. The following figure is extracted from “SAFE” model.

Figure17 –Soil Pressure diagram(0.0354N/mm2)

Maximum soil pressure of foundation under critical load combination is equals to 0.0354 N/mm2, which is less than allowable bearing capacity of soil.

  - Settlement Control

Settlement in different service load combinations should be checked by allowable value. The following figure is extracted from “SAFE” model.

Figure18 –Displacement under service load combination(3.78  mm)

Figure19 –Slab Reinforcement

According to above figure use φ10@200 for this slab and design out puts shows as below:

Figure20 –Slab Resultant Reinforcement(Mmax=0.1089 ton-m/m=1067.22 N-mm/mm)

Mn=Mu∅=12100

Rn1=Mn1bd2=0.774

m1=fy0.85fc=156.865

ρreq=1m11-1-2m1Rn1fy=0.000197

As=ρreq.b.d=0.246 cm2

Used As used=∅10@200 mm=3.9cm2 >As=0.246 cm2 ok

Due to the minimum required amount of reinforcement , minimum reinforcement has been used .

- GAS OIL STORAGE FOUNDATION (13m x 13m x 0.3m)

Gas Oil Storage foundation has been modeled as a concrete pad with thickness 0.3 m in safe 2016 software.

Figure21–Gas Oil Storage Foundation (13mx13mx0.3m)

A concrete pad with thickness 0.3m has been modelled in safe software 2016 as bellow:

Figure22 –Slab Property Data(th=0.3m)

  - Thermal Load (T)

The ambient thermal load has been assumed based on the thermal variation for structure of 28 ºC.

  - Live load (L)

For bottom distributed load about 600kg/m2  ,has been considered for design of structure

Figure23–Load applied on Pad

  - Settlement Control

Settlement in different service load combinations should be checked by allowable value. The following figure is extracted from “SAFE” model.

D02

Figure24 –Displacement under service load combination(1.31 mm)

  - Soil Pressure Control

Soil pressures in different service load combinations should be checked by allowable value. The following figure is extracted from “SAFE” model.

D02

Figure25–soil Pressure under service load combination(max=0.0178 N/mm2)

Figure26–slab Resultant Mmax(=0.26 t-m/m,2548.23 N-mm/mm)

Mn=Mu∅=28888.889

Rn1=Mn1bd2=0.571

m1=fy0.85fc=156.865

ρreq=1m11-1-2m1Rn1fy=0.000144

As=ρreq.b.d=0.325  cm2

Used As used=∅10@200 mm=3.29 cm2>As=0.325   ok

Due to the minimum required amount of reinforcement , minimum reinforcement has been used .

- HELIPAD FOUNDATION (8m x 8m x 0.25m)

Helipad  foundation has been modeled as a concrete pad with thickness 0.25 m in safe 2016 software.

Figure27–Helipad Foundation (8mx8mx0.25m)

A concrete pad with thickness 0.25m has been modelled in safe software 2016 as bellow:

Figure28 –Slab Property Data(th=0.25m)

  - Loads on foundation

    - Thermal Load(T)

The ambient thermal load has been assumed based on the thermal variation for structure of 28 ºC.

    - Live load (l)

For Helipad Live load due to table 6-5-1 code No.6 assumed 3KN/m2 (300kg/m2)

Figure29 –Load applied on Heli Pad

Figure 30–Load applied on Heli Pad

According to Iranian national building code (NO.6) table6-5-1 note 12 two point load with distance of 2.45 m has been applied on helipad model .Each operating load that added to live load is about 750 kg

Figure 31–apply operating load

  - Settlement Control

Settlement in different service load combinations should be checked by allowable value. The following figure is extracted from “SAFE” model.

Figure 32  –Displacement under service load combination(1.07 mm)

Figure33–slab Resultant Mmax(=0.297 t-m/m,2913.64 N-mm/mm)

Mn=Mu∅=33000

Rn1=Mn1bd2=1.078

m1=fy0.85fc=156.863

ρreq=1m11-1-2m1Rn1fy=0.000275

As=ρreq.b.d=0.482 cm2

Used As used=∅10@200 mm=3.29 cm2>As=0.482   ok

Due to the minimum required amount of reinforcement , minimum reinforcement has been used .

  - Soil Pressure Control

Soil pressures in different service load combinations should be checked by allowable value. The following figure is extracted from “SAFE” model

Figure34 –soil Pressure under service load combination(max=0.0151N/mm2)

  - Check Additional Reinforcement

Figure35–Additional reinforcement

According to above output from safe 2016 software there is not need for additional reinforcement to existing reinforcement(∅10@200 top & bott)

- edge beam Design

Edge beam has been modeled as a concrete beam with width of 0.5 m and height of 0.45 in safe 2016 software .load of two wheels of truck applied on edge beam .

Figure36 –edge beam model

Each wheel load assumed about 7.5ton(7500kg) and apply in 2 points with distance of 3m.

  - Soil Pressure Control

Figure37 –soil pressure under service load combination(q=0.821kg /cm2)

  - Settlement Control

Figure38 –soil displacement under service load combination(7.13mm)