| Row | Clarification Title | Bidder reply (1st round) | Conclusion (2nd round) | Bidder reply (2nd round) | Conclusion (final/TCM) |
| --- | --- | --- | --- | --- | --- |
| GENERAL |  |  |  |  |  |
|  | Vendor to confirm that quotation is in full compliance with scope of supply, scope of work, all requirements of the MR No. BK-GCS-PEDCO-120-ME-MR-0003_D03 and all applicable codes, standards and specifications (and other attachments) there mentioned. | Tehran Javan Confirm |  |  |  |
|  | Commercial proposal should be based on ex-work. | Tehran Javan Confirm |  |  |  |
|  | Vendor shall mention any deviation / alternative compared with requirements of MR and its attachments on the signed and stamped Deviation Form. (attachment #3 MR) Note: Raising no deviation to any single requirement means full compliance of vendor quotation to that requirement. Purchaser has no responsibility to discover deviations or alternatives scattered over the proposal in addition to what proclaimed explicitly in deviation list; hence such deviation (if any) is disregarded and shall not be considered for vendor claim in future. | Tehran Javan Confirm |  |  |  |
|  | Vendor is sole responsible to consider this clarification and modify his offer based on this TCL. All modifications which need to be considered on the next revision of offer based on this clarification should be clearly marked with revision and cloud marks. All consequences due to changes without cloud marks are in vendor's responsibility. Anyway, this clarification has priority over the offer. | Tehran Javan Confirm |  |  |  |
|  | Sub-vendors/Sub-suppliers shall be in acc. with MOP-AVL (Latest Ed.) Note: Vendor shall have overall responsibility for the coordination with his Sub-Vendors, and for the proper execution of any accessory item assembly, testing, delivery and any other service defined herein. | Tehran Javan Confirm |  |  |  |
|  | It is the responsibility of the vendor to ensure that there is no inconsistency in proposal and related attachments. So, any discrepancy between Vendor proposal attachments shall be removed. | Tehran Javan Confirm |  |  |  |
|  | All documents required after purchase order shall be submitted in accordance with vendor documents requirements of the PMR. | Tehran Javan Confirm |  |  |  |
|  | Vendor shall guarantee that the materials still meet all mechanical properties after all fabrication heat treatments foreseen and repairs to the materials. Note : All materials shall be new and un-corroded | Tehran Javan Confirm |  |  |  |
|  | Guarantee period shall be considered as 18 months after successful start-up at the purchaser’s production site or 24 months after the date of delivery whichever comes earlier. | Tehran Javan Confirm |  |  |  |
| MECHANICAL |  |  |  |  |  |
|  | Project spare part list shall be followed by vendor for commissioning, start up & two years spare parts: - Project spare part supply procedure (document name : E&C-QC-SP-1) | Tehran Javan Confirm |  |  |  |
|  | Mechanical guarantee shall be in vendor scope of work. | Tehran Javan Confirm |  |  |  |
|  | Hydraulic / performance guarantee for internals is in vendor scope. In case, based on removable internal design, changing in vessel or internal size is required, it shall be clearly mentioned in vendor proposal. No cost impact is accepted after PO. | Tehran Javan Confirm |  |  |  |
|  | All required special tools shall be considered in vendor main offer (they shall be supplied by vendor without any cost impact).A qualified and complete list has to be included in the bid documentation. | Not Required |  |  |  |
|  | Vendor shall prepare material and material grade as per data sheets of equipment. Any substitution shall be expressed in deviation form. | Tehran Javan Confirm |  |  |  |
|  | Painting, RAL code and surface preparation on vessels shall be performed based on relevant project specification. Primer, intermediate and final coat should be carried out based on approved painting procedure in vendor’s shop before shipment. Supply of paint material is in vendor scope. Touch up paint for field repair works is in vendor’s scope of supply. | Tehran Javan Confirm |  |  |  |
|  | All internal & external welded parts are in vendor’s scope of work / supply. | Tehran Javan Confirm |  |  |  |
|  | In case of order, Purchaser would not review documents sent by vendor before receiving Vendor Print Index and Schedule. Vendor Print Index and Schedule (VPIS) shall be prepared as first document after PO. All documents shall be issued after getting approval of this document. | Tehran Javan Confirm |  |  |  |
|  | PWHT, PFHT shall be performed by vendor where required according to relevant codes, standards and specifications & mechanical data sheets even though it has not been specified in mechanical data sheets. | Tehran Javan Confirm |  |  |  |
|  | Hardness test on sample and production shall be done after PWHT with purchaser inspection witness as per project welding specification with witness level on both PQR and production. | Tehran Javan Confirm |  |  |  |
|  | Vendor’s commercial and technical proposal shall be in accordance with the latest revision of mechanical data sheets. Please find attached file | Tehran Javan Confirm |  |  |  |
|  | No fabrication shall be commenced until complete calculations and shop drawings have been received and approved or Approved as Noted (with minor comments) by purchaser. | Tehran Javan Confirm |  |  |  |
|  | Trial Assembly of all removable Internals is in vessel vendor’s scope of work. | Tehran Javan Confirm |  |  |  |
|  | All devices required for transport and lifting are in vendor scope of work/supply. | Tehran Javan Confirm |  |  |  |
|  | For all pressure parts and supports, the material certificate should be in accordance with EN 10204, 3.1. Material certificate for non-pressure parts shall be in accordance with EN 10204, 2.2. Please confirm. | Tehran Javan Confirm |  |  |  |
|  | All raw materials including support saddle/skirt/lugs/legs shall have Original mill certificate as per EN 10204 3.1. and shall include: - Impact test (as per ASME code and purchaser requirements) - Supplementary UT test as per SA 578 level B or C (required for CS thickness above and equal to 30mm) Non-standard forging shall be examined by UT at mill. | Tehran Javan Confirm |  |  |  |
|  | Design & supply of all non-removable/ removable internals (& their supports) are in vendor scope of supply. | Tehran Javan Confirm |  |  |  |
|  | The cost of all repairs, subsequent inspection and electrodes or paint (if required at site) shall be in vendor responsibility. | Tehran Javan Confirm |  |  |  |
|  | N2 purging for long time preservation with following conditions shall be considered in vendor’s scope of work & supply. 1-All opening shall be sealed with a suitable metal cover and gasket. Vessels shall be filled with nitrogen gas at a pressure of 0.5 kg/cm2(g). 2-The equipment shall have a temporary gauge to determine purge pressure. 3-Vendor shall submit N2 filling procedure for purchaser/contractor approval at detail stage. 4-All the required ancillaries such as metal covers, bolts & nuts, gaskets and pressure gauge shall be provided by vendor. | Normally N2 Purging Used For Sea Shipment And This Item Not Required So It Should Be Finalized By Client. |  |  |  |
|  | Any clash between seam welds, nozzle openings, nozzle reinforcement pads, insulation rings, stiffening rings, pipe support clips, lifting devices and relevant pads, ladder and platform lugs/pads and any internal and external attachment shall be checked by vendor. Any unreported clash will not be accepted. | Tehran Javan Confirm |  |  |  |
|  | Design (calc. & drawings) & supply of ladders & platforms is in vendor scope. Grating shall be hot dip galvanized. | Tehran Javan Confirm |  |  |  |
|  | Blind flange with gaskets, caps, bolts & nuts, temporary saddles for hydrotest & shipment (if required) shall be included vendor’s scope of supply. | Tehran Javan Confirm |  |  |  |
|  | All gaskets to be used for hydrostatic test shall be same as operating case. | Tehran Javan Confirm |  |  |  |
|  | Local stress analysis of shells under external loads from piping system and attachments (platforms, pipe supports, davit, and lifting lug) shall be included in strength calculation and shall be performed by vendor. Please note that wherever WRC is not applicable FEA shall be done to evaluate local stresses. | Tehran Javan Confirm |  |  |  |
|  | All Native files of drawings & calculations (like PVElite, CodeCalc, Ansys, FEA, NozzlePro, ACAD DWG,..) shall be provided by Vendor at detail stage. | Tehran Javan Confirm |  |  |  |
|  | Anchor bolt data (Size, number, B.C.D., material,…) and final loading data including erection, operating and hydro tests shear and moment due to wind and earthquake shall be provided by vendor 1 month after purchase order. | Tehran Javan Confirm |  |  |  |
|  | To account for piping loads, 20% of fab. Weight shall be considered at operating COG. | Tehran Javan Confirm |  |  |  |
|  | Stress ratio due to combined loads shall be limited to 0.9 max. | Tehran Javan Confirm |  |  |  |
|  | For nozzle necks made by plate full radiography shall be considered. | Tehran Javan Confirm |  |  |  |
|  | Lifting lugs (or trunnion), tailing lug shall be analyzed by vendor with a safety factor 2. | Tehran Javan Confirm |  |  |  |
|  | Shipping calculation, and relevant drawings for all equipment shall be considered in vendor’s scope and submitted for purchaser’s approval at detail stage. Please confirm | Tehran Javan Confirm |  |  |  |
|  | In addition to ASME Sec. VIII Div.1, IPS-G-ME-150 shall be considered for design of pressure vessels. | Tehran Javan Confirm |  |  |  |
|  | Upper section of skirt with minimum length if 500 mm shall have same material as shell. | Tehran Javan Confirm |  |  |  |
|  | Hydrostatic test shall be in accordance with the requirements of ASME code Sec. VIII Div.1. UG 99b Note 35 | Tehran Javan Confirm |  |  |  |
|  | Based on design code, considering material, thickness and MDMT, impact test for all parts shall be considered in vendor’s scope, if required. | Tehran Javan Confirm |  |  |  |
| PIPING & MATERIAL |  |  |  |  |  |
|  | Nozzle orientation/elevation will be finalized later. Please confirm that the orientation/elevation of Nozzles has no effect on the price and the vendor’s claim will not be accepted. | Tehran Javan Confirm If Nozzle Orientation Or Elevation Do Not Affect Dimension Of Vessel. |  |  |  |
|  | Vendor shall consider purchaser's information document/drawing regarding nozzle orientation, access platform, pipe support clip and so on before finalizing engineering phase. Also vendor is to get purchaser’s approval in this regard before construction phase. | Tehran Javan Confirm |  |  |  |
| ELECTRICAL |  |  |  |  |  |
|  | 3 pcs earth lug shall be considered on each equipment. | Tehran Javan Confirm |  |  |  |
| STRUCTURE |  |  |  |  |  |
|  | The static and dynamic loads of the equipment shall be presented at the foundation level. The elevation and location of the equipment center of mass shall be determined. | Tehran Javan Confirm |  |  |  |
| QC |  |  |  |  |  |
|  | Vendor to issue all evidence to prove it is in MOP AVL for this pressure vessels and also ISO 9001 and ISO 3834 is required. | Tehran Javan Confirm |  |  |  |
|  | Vendor shall review and confirm project ITP attached to PMR which is issued as minimum requirements for preparation of detailed ITP after PO and finalizing in PIM. | Tehran Javan Confirm |  |  |  |
|  | Vendor shall confirm to supply raw materials from approved vendor list and submit all required material certificates for TPI approval before manufacturing. | Tehran Javan Confirm |  |  |  |
|  | Welding book (including WPS, PQR, weld map and NDT map) shall be prepared by vendor and submitted for purchaser approval (pressure parts, and structural). No welding shall be commenced before receiving approved welding book. | Tehran Javan Confirm |  |  |  |
|  | Vendor shall confirm to employ certified and expert workers and inspectors and to use calibrated measuring instruments according to regulations of ISO 3834 and ISO 9001. | Tehran Javan Confirm |  |  |  |
|  | Vendor shall confirm to perform and report all required inspection and tests according to project specification and datasheets and reference standards and approved ITP including visual, dimensional, NDT, PMI, PWHT, Hardness, Hydrotest, Performance, Running, … | Tehran Javan Confirm |  |  |  |
|  | All NDT including radiographic, ultrasonic, magnetic particle or liquid penetrate examinations shall be performed before and after Post Weld Heat Treatment of the vessel. This will be performed according to code and specification. | Tehran Javan Confirm |  |  |  |
|  | All WPS, PQR, WPQT and NDT map shall be prepared before pre-inspections meeting. No welding shall be commenced until complete welding documents (specially welding map, NDE map, WPS, PQR) have been received and approved by purchaser. | Tehran Javan Confirm |  |  |  |