| Row | Clarification Title | Bidder reply (1st round) | Conclusion (2nd round) | Bidder reply (2nd round) | Conclusion (final/TCM) |
| --- | --- | --- | --- | --- | --- |
| GENERAL |  |  |  |  |  |
|  | Vendor to confirm that quotation is in full compliance with scope of supply, scope of work, all requirements of the MR No. BK-GCS-PEDCO-120-ME-MR-0005_D03 and all applicable codes, standards and specifications (and other attachments) there mentioned. | confirm |  |  |  |
|  | Commercial proposal shall be based on following main scope of work and supply: TK-2209: Detailed Mechanical/Structural Design, Supply of Material (Tanks & Stairways, platforms, permanent ladder, platforms for access to the tank from the roof access platform) and shop fabrication & test -EXW TK-2301 A/B & TK-2102: Detailed Mechanical/Structural Design, Supply of Material (Tanks & Stairways, platforms, permanent ladder, platforms for access to the tank from the roof access platform) and pre-fabricated & shop tests –EXW | Confirm. We are unable to do basic design. We will only provide workshop drawings. |  |  |  |
|  | Following items shall be included in vendor proposal as option: DDP delivery- fabrication, site assembly and erection, all required tests such as hydro test, PT and vacuum test for TK-2301 A/B & TK-2102. | confirm |  |  |  |
|  | Any deviations shall be listed in project deviation form (Material requisition attachment #3), signed and stamped. | confirm |  |  |  |
|  | Sub-vendors/Sub-suppliers shall be in acc. with MOP-AVL (Latest Ed.) | confirm |  |  |  |
|  | All devices required for transport and lifting are in vendor scope of work/supply. (shall be quoted separately as option ) | confirm |  |  |  |
| MECHANIC |  |  |  |  |  |
|  | All gaskets to be used for hydrostatic test shall be same as operating case. | confirm |  |  |  |
|  | Local stress analysis of shells under external loads from piping system and attachments (platforms, pipe supports, davit, and lifting lug) shall be included in strength calculation and shall be performed by vendor. Please note that wherever WRC is not applicable FEA shall be done to evaluate local stresses. | confirm |  |  |  |
|  | All Native files of calculations, design, documents, drawings (AME Tank/Tank Design, CAD, Word, Excel..) shall be submitted by Vendor at detail stag. | This company provides the native drawing files and sends examples of native files in other cases |  |  |  |
|  | Anchor bolt data (Size, number, B.C.D., material,…) and final loading data including erection, operating and hydro tests shear and moment due to wind and earthquake shall be provided by vendor 1 month after purchase order | confirm |  |  |  |
|  | All external attachments such as Wind girder, Clips for Piping Supports, Ladders, Platforms, stairs, Lifting Devices, and Relevant Supports, Insulation Supports, Name Plate and Relevant Supports, Earth Lugs and etc., shall be in vendors Scope of work and supply. | confirm |  |  |  |
|  | Roof main structure for supported cone roofs shall be designed in accordance with applicable standards. | confirm |  |  |  |
|  | Blind Flanges with Gaskets, Bolts and Nuts, shall be in vendors Scope of work and supply. | confirm |  |  |  |
|  | All necessary devices for transportation (including adequate saddles, nozzle covers, bolts, nuts, temporary gaskets for shipping, cradle, wooden belts for sling the equipment, packing boxes, fitting, etc.) shall be included in main offer . | confirm |  |  |  |
| PIPING & MATERIAL |  |  |  |  |  |
|  | All materials shall be new and un-corroded. | confirm |  |  |  |
|  | Nozzle orientation/elevation will be finalized later. Please confirm that the orientation/elevation of Nozzles has no effect on the price and the vendor’s claim will not be accepted. | confirm |  |  |  |
|  | Vendor shall consider purchaser's information document/drawing regarding nozzle orientation, access platform, pipe support clip and so on before finalizing engineering phase. Also vendor is to get purchaser’s approval in this regard before construction phase. | confirm |  |  |  |
| STRUCTURE |  |  |  |  |  |
|  | The exact dimensions & size of the bottom plate and the proper alignment under bottom plate shall be determined. | confirm |  |  |  |
|  | The static & dynamic loads of the equipment and the center of gravity of it shall be determined. | confirm |  |  |  |