| # | Clarification Title | Clarification Description (Rev.00) | Conclusion (Rev.00) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description (Rev.01) | Conclusion (Rev.01) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description (Rev.02) | Conclusion (Rev.02) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description (Rev.03) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GENERAL |  |  |  |  |  |  |  |  |
|  | Documents Priority | Vendor to confirm that in case of contradiction between this TCL and offer; vendor confirmation of compliance through this technical clarification is governed and shall be followed by vendor after order. In such case offer won’t be considered as basis of any claim after order. | CONFIRMED | CLOSED |  |  |  |  |
|  | Deviation List | Bidder shall confirm his full compliance to meet all purchaser documents listed and attached to MR, code / standards listed in MR with ref. No. "BK-GCS-PEDCO-120-ME-MR-0006" as well as MR requirements. All deviations shall be listed in project deviation form, attach to this TCL, signed and stamped. Notes: 1-Any exception and deviation from the requirements of MR and the related specifications including referenced codes & standards shall be clearly declared by vendor in writing for purchaser’s approval at bid stage by using of the format included in “Letter Of Conformity” attached to material requisition. Raising no deviation to any single requirement means full compliance of vendor quotation to that requirement. Purchaser has no responsibility to discover deviations or alternatives scattered over the proposal in addition to what proclaimed explicitly in deviation list; hence such deviation (if any) is disregarded and shall not be considered for vendor claim in future. (Vendor to confirm) 2- Bidder shall confirm that all MR attachments listed in MR are available for her for review at bid stage. 3-Considering items 1 & 2, please note that any deviation at detail stage will not be accepted by purchaser. | CONFIRMED | CLOSED |  |  |  |  |
|  | Documents Priority | Bidder to confirm his compliance with documents priority stated in MR. Any conflict between the documents shall be notified by bidder to purchaser and resolved by his agreement | Will be submit according our document list | Not accepted, if there is any deviation will be specified. | No deviation, some of the documents for this project cannot be produced, so will be finalize after order. | Noted |  |  |
|  | Requirement For Documents | Bidder to confirm that documents required after purchase order shall be submitted in accordance with “Vendor documents min. requirement list” attached to the MR. | Will be doing , IF REQUIRED | Closed. Attachment #2 of relevant MR will be checked, singed and stamped by the vendor and if there is any deviation will be specified. | No deviation, All documents (if required) will be Produce. | CONFIRMED |  |  |
|  | Documents | Bidder shall provide following documents while bidding: 1- Copy of valid quality system certificate(ISO 9001/2 or 3) 2- Sub- vendors / sub-suppliers list 3- Deviation list with letter of conformity (Refer to item A2 above) 4- Reference and Experience List 5- Joint venture agreement (If any) with any company to design / fabrication of equipment. In case of joint venture ; qualified documents of the company who is part of joint venture shall be also submitted | Please check attached documents | Documents will be final after P.O. | CONFIRMED | Closed |  |  |
|  | Sub-vendors | Bidder to confirm that Sub-Vendors shall be selected from Project’s AVL. | confirmed | Closed |  |  |  |  |
|  | Painting specification | Bidder to confirm that surface preparation and painting of all equipment material including valves , pipes , fittings and everything within the package are in accordance with project painting specification with ref. "BK-GNRAL-PEDCO-000-PI-SP-0006" attached to MR. | confirmed | Closed |  |  |  |  |
|  | Specification | Bidder to confirm that Packing, Marking, Preservation and Preparation for Shipment will be done in accordance to the project specification with Doc. No; BK-GNRAL-PEDCO-000-QC-PR-0045 attached to the MR. | confirmed | Closed |  |  |  |  |
|  | Inspection and Test Plan | Inspection and Test Plan / Quality Control Plan and their related inspection level (Witness, Hold, Review and Approval) shall be at least in accordance with MR and all project requirement such as NDT PROCEDURE. Any deviation is not accepted unless otherwise specified in deviation list and approved by Purchaser. | ITP & QCP will be finalize after order in KOM meeting | Closed |  |  |  |  |
|  | Material Certificate | Bidder to confirm that for all pressure parts and those required special consideration or stainless steel as well as for supports the material certificate should be in accordance with EN 10204, 3.1 Material certificate for non-pressure parts shall be in accordance with EN 10204, 2.2 | confirmed | Closed |  |  |  |  |
|  | Scope | Bidder shall confirm the scope of work and supply are fully in accordance with MR and its attachment with considering following items: 1- The consequences of HAZOP and SIL study shall be in vendor scope of supply and work. 2- Participation in technical meeting whenever it is necessary by Purchaser/Client to close engineering document after order is in vendor scope 3- Lifting beams ( Spreader beams) for erection with sling shackle and etc shall be designed and supplied by vendor 4- N2 preservation with pressure regulator, valve & flexible hoses during transportation & 18 months of site storage based on requirements in MR shall be in vendor scope of supply / work. 5- All utility conditions shall be as per document "Basic engineering design data (10‐BD‐00‐PR‐SPC‐0001)". Moreover bidder shall consider utility noted condition as per project design data. If other utility condition is needed, whole required facilities is in vendor scope of work/supply. 6- Anchor bolts | confirmed | Closed |  |  |  |  |
|  | Applied standard | Applied Standards for equipment design, fabrication and test shall be listed | Confirmed, API standard is base of activity . | Closed |  |  |  |  |
|  | outdoor installation/ operation under project site conditions | Vendor shall confirm Air Compressor Packages, Dryer Packages and N2 generation package are fully suitable for outdoor installation/operation under project site conditions “Process Basis of Design; BK-GNRAL-PEDCO-000-PR-DB-0001 “without requiring any additional protection. | Oil free Air cooled of screw compressor is suitable only for INDOOR condition. | Air Compressor Packages, Dryer Packages and N2 generation package shall be suitable for outdoor installation-under shelter under project site conditions without requiring any additional protection. | CONFIRMED for ambient temperature maximum 55 .C | Closed |  |  |
|  | Suitability of utility characteristics available at site | Vendor shall confirm suitability of utility characteristics available at site as per project site conditions “Process Basis of Design; BK-GNRAL-PEDCO-000-PR-DB-0001 “. Supplying all facility for utility distribution and regulation shall be part of the vendor scope. | Confirmed | Closed |  |  |  |  |
|  | Warranty | Vendor shall warranty the engineering, materials and workmanship of the equipment/goods. Under this warranty, all materials, parts, and assemblies found to be defective during the guarantee period shall be replaced free of charge by VENDOR. Also, The guarantee period shall be eighteen (18) months from the date of delivery or twelve (12) Months from the installation date of each equipment/packages at site. | Confirmed | Closed |  |  |  |  |
|  | Project’s MR | Vendor shall follow project’s MR with its Attachments. | Confirmed | Closed |  |  |  |  |
|  | Document | Bidder to confirm that all native files due to all calculations and drawings shall be submitted for purchaser approval after order. | Confirmed | Closed |  |  |  |  |
|  | Material | Material and corrosion allowances shall be in accordance with data sheets. |  |  | CONFIRMED | Closed |  |  |
|  | Material | - Internal coating is required for Air and Nitrogen receivers |  |  | CONFIRMED | Closed |  |  |
| PROCESS |  |  |  |  |  |  |  |  |
|  | General comment | Considering that in vendor’s proposal, Compressors of Nitrogen and Air production packages have been considered the same, therefore the proposal shall be totally revised by vendor based on project requirements. | Confirmed | Closed |  |  |  |  |
|  | General comment | Vendor shall follow Latest revision of project documents as Attachment#1 of this TCL. | Confirmed | Noted | CONFIRMED | Noted |  |  |
|  | General comment | Vendor shall be capable to generate nitrogen with purity of 99.9% | Will be deliver 99.5% UP 99.9% | Purity 99.9 is accepted | CONFIRMED | Closed |  |  |
|  | General comment | Utility consumption list and equipment list will be submitted | Confirmed | Closed |  |  |  |  |
| MECHANIC |  |  |  |  |  |  |  |  |
|  | Document Requirement | Bidder to submit all required documents with tender as per “VDR Form” in the MR. For example below documents: 1-Marked up and Filled out MR scope of supply and works (It shall be signed and stamped) 2-GAD for air compressors, air dryers and nitrogen generators with dimensions, all auxiliaries and heaviest part weight table (by considering air cooler in the package) 3-Cross sectional drawing for compressors with BOM 4-Performance Curves 5-Equipment data sheet in each standard format 6-Utility consumption and load list 7-Spare part list 8-Special tool list 9-P&ID’s dedicated for air compressor, dryer and nitrogen generation packages with all equipment, instruments and lube oil, cooling (if required) and control systems. 10-Noise Data 11-Equipment List on detail for mechanical and other items | Confirmed | Closed |  |  |  |  |
|  | Major comment | Air compressed supplied for instrument / plant air and nitrogen package shall be separate from each other. 2 Oil-free screw compressors, one in duty operation and one as standby mode to supply compressed air for instrument and plant air (PK-C-2203 A/B) and 1 Oil-free screw compressor to supply compressed air for nitrogen package (PK-C-2204) shall be considered by vendor as per project requirement. | Confirmed | Closed |  |  |  |  |
|  | Interconnecting piping | Design and supply of all interconnecting piping between equipment and all instruments, valves and … on them is in the vendor's scope. Also all between skid piping (including supporting) c/w all instrument and valve on them shall be designed, prefabricated and supplied by vendor. Detailed isometric DWGs shall be submitted for them. Note1: On skid piping to be assembled at vendor's shop. Note2; outside skid piping to be prefabricated and match marked for site assembling. | Confirmed, all on skid will be supply | Closed |  |  |  |  |
|  | Heat tracing c/w supports, clips and etc. | Design and supply of heat tracing c/w supports, clips, and other provisions (including panels, heating cables and relevant connectors, electrical cables, junction boxes and thermostats) shall be part of the vendor scope. | Confirmed, all on skid will be supply | Closed |  |  |  |  |
|  | Insulation c/w supports, clips and etc. | Design and supply of insulation c/w supports, clips shall be in vendor’s scope of supply and work. | Confirmed, all on skid will be supply | Closed |  |  |  |  |
|  | Commissioning spare parts | Pre-commissioning, commissioning and start up spare parts shall be supplied by vendor. Vendor shall confirm to provide all required spare parts as per project spare part list attached to MR. | Confirmed, please check spare part list | Closed |  |  |  |  |
|  | Two years spare part | Two years spare parts shall be considered in vendor’s base offer as per project spare part list attached to MR. | Confirmed, please check spare part list | Closed |  |  |  |  |
|  | Capital spare part | Capital spare parts to be considered as optional offer. | Confirmed | Closed |  |  |  |  |
|  | Vent and Drain of Packages | All vents and drains of the packages to be valved, manifolded and to be routed to the edge of skid. | Confirmed | Closed |  |  |  |  |
|  | Tie-in | All Tie-in for equipment in the packages shall be equipped will flange and valves | Confirmed | Closed |  |  |  |  |
|  | Coupling | The coupling shall be of the API671 forged-steel, non-lubricated, flexible-element spacer type. The flexible elements shall be stainless steel metallic disk. | Confirmed, if required | Closed |  |  |  |  |
|  | Coupling balancing | Vendor shall specify the coupling balancing grade and balancing method as per API671. | Confirmed, if required | Closed |  |  |  |  |
|  | PMI, pickling and passivation | Positive material identification (PMI), pickling and passivation for non-carbon steel material to be included in vendor’s scope of work. | Confirmed | Closed |  |  |  |  |
|  | Skid of the packages | Vendor shall specify number of skid inside package with dimension and weight and preliminary general arrangement. 2-In case of multi skid, please clarify if there is any special procedure for installing and assembling of loose parts. 3-Each skid shall be furnished with 4-point lifting facility and no. two earth lugs located at opposite sides and nameplate made of SS material. 4-Vendor shall specify space required around the skid for maintenance. | Confirmed | Closed |  |  |  |  |
|  | Inlet condition of compressor | Inlet condition of compressor shall be considered as ambient condition of site as per relevant project specification for Site Condition “Process Basis of Design; BK-GNRAL-PEDCO-000-PR-DB-0001 ". In this regard, air compressor shall be sized for worse case condition of AMB site condition and in any condition the outlet condition as per project duty spec. shall be met. | Confirmed | Closed |  |  |  |  |
|  | Lube oil system for air compressor | Lube oil system for bearings of air compressors shall be in vendor’s scope of supply and work and its dedicated P&ID with all auxiliaries such as pumps, oil cooler, reservoir and instruments shall be submitted by vendor in next revision of technical offer. | Confirmed | Closed |  |  |  |  |
|  | Noise level | If noise enclosure will be required for meeting the project noise limit, it shall be supplied by vendor. Any required facilities for ventilation, fire and gas detection and fire extenuation (if any) for enclosure shall be supplied by vendor. | Confirmed | Closed |  |  |  |  |
|  | Material | Below requirements shall be followed by vendor: -Material ASTM No. to be submitted for compressor and its auxiliaries. -Compressor casing shall be made of cast iron. -Rotors and shafts shall be forged and heat‐treated steel. -Timing gear shall be forged steel. -Gear box housing material shall be C.S. -The instrument air lines and the other air piping shall be galvanized CS based on project requirement “Piping Corrosion study and material selection report; BK-GCS-PEDCO-120-PI-RT-0001”. -Oil pumps casing shall be C.S. -Oil pumps impellers shall be S.S. -Oil cooler, shell shall be C.S and tube shall be SS 316 L -Oil piping shall be SS 316L. | Confirmed | Closed |  |  |  |  |
|  | Motor Power | The air compressor motor name plate power (exclusive of the service factor) and after derating shall be more than 110% of maximum required power by air compressor (including all losses such as gear box coupling and etc.) in all cases including PSV setting and etc. Vendor to confirm this requirement. | Confirmed | Closed |  |  |  |  |
|  | Applied Standards | Applied Standards for equipment design, fabrication and test shall be listed. ISO10440-2 standard shall be followed by vendor for air compressors. | Confirmed | Closed |  |  |  |  |
|  | Vendor’s Scope | All vents and drains of the package to be valved, manifolded and to be routed to the edge of skid. | Confirmed | Closed |  |  |  |  |
|  | Inspection and Test | Test procedures and acceptance criteria will be checked after order with project specifications and related standards. Vendor to confirm to follow project standard and spec. | Confirmed | Closed |  |  |  |  |
|  | Purchaser’s approval | Regarding technical offer, diameter, dimension and thickness of each equipment will be checked after receiving vendor calculation and data after order and comments in detail shall be followed without cost impact. | Confirmed | Closed |  |  |  |  |
|  | Guarantee | Performance, Process and Mechanical Guarantee shall be in the vendor's scope. | Confirmed | Closed |  |  |  |  |
|  | Corrosion allowance | For all parts made of carbon steel, corrosion allowance of 3.2 mm shall be considered by vendor. Also vendor shall follow project document “Piping Corrosion study and material selection report; BK-GCS-PEDCO-120-PI-RT-0001” attached to MR. | Confirmed | Closed |  |  |  |  |
|  | Air filter | Inlet air filter for compressors shall be considered and indicated on P&ID | Confirmed, P&ID of project in attached of MR confirmed | Closed |  |  |  |  |
|  | Compressor stages, Inter coolers and Separators | Vendor shall specify and indicate below items on PFD & P&ID for Air and Nitrogen production packages: -The number of screw compressor stages -Inter/After coolers -Separators between and after stages | Confirmed | Closed |  |  |  |  |
|  | Calculation report | Calculation report for each of compressor stage including temperature, pressure, capacity at suction and discharge of compressor and shaft power required at each stage to be specified by vendor. | Confirmed | Closed |  |  |  |  |
|  | MAWP & PSV setting | Compressor's MAWP and PSV setting pressure on discharge line shall be specified by vendor. | Confirmed | Closed |  |  |  |  |
|  | Ladder and platforms | Access ladder and platforms, stairway shall be studied, designed and supplied by vendor. Relevant detail drawings shall be submitted for approval. They shall comply with project standards attached to MR, as minimum. | Confirmed, if required | Closed |  |  |  |  |
| MECHANIC FIX |  |  |  |  |  |  |  |  |
|  | Fixed equipment mechanical data sheet such as drum | Vendor shall submit relevant mechanical data sheet to define shell/head/nozzle/support type & materials & dimension and designing assumption (J.E. / C.A. & etc.…) | Confirmed, if required | Closed |  |  |  |  |
|  | Special tools | Special tools for lifting of vessel (i.e. spread bar and skirt bracing if required based vendor’s lifting calculation & relevant bolts/nuts/gasket, Hydraulic bolt tensioning device for bolts greater than 1-1/2” and etc ), shall be considered in vendor’s scope. | Confirmed, if required | Closed |  |  |  |  |
|  | Lifting/Shipping | Shipping calculation, hydrotest stability calculation and relevant drawings & temporary shipping support for all equipment shall be considered in vendor’s scope and submitted for purchaser’s review at detail stage. Please confirm. | Confirmed | Closed |  |  |  |  |
|  | preservation | N2 purging preservation with following conditions shall be consider in vendor’s scope of work & supply (it shall be considered in vendor’s base price not as optional): 1-All opening shall be sealed with a suitable metal cover and gasket. Vessels shall be filled with nitrogen gas at a pressure of 0.5 kg/cm2(g). 2-The equipment shall have a temporary gauge to determine purge pressure. 3-Vendor shall submit N2 filling procedure for purchaser/contractor approval at detail stage. 4-All the required ancillaries such as metal covers, bolts & nuts, gaskets and pressure gauge shall be provided by vendor. Please confirm and modify your offer.( using silica gel instead of N2 purge is not acceptable and shall be omit from your offer.) | Confirmed | Closed |  |  |  |  |
|  | Access for internal loading/unloading | Manhole / hand hole Access for internal loading/unloading of drums | Confirmed, if required | Closed |  |  |  |  |
|  | Nozzle WRC | Local stress analysis of shells under external loads from piping system and attachments (platforms, pipe supports, davit, and lifting lug) shall be included in strength calculation. And also, nozzle load shall be considered according to Allowable Nozzle loads on static Equipment (as per project specification) should be done. Please note that wherever WRC is not applicable FEA shall be done to evaluate local stresses. The original file (native file) of all analysis including all input data and results should be provided by Vendor. Please confirm. | N/A | Closed |  |  |  |  |
|  | Loading Data & Anchor Bolt Data | Anchor bolt data (Size, number, B.C.D. ,material ,…) and final loading data including erection, operating and hydro tests shear and moment due to wind and earthquake shall be provided by vendor 1 month after purchase order. Please confirm. | Confirmed | Closed |  |  |  |  |
|  | PWHT | When stress relieving is required, whole equipment in one piece without removable internals shall be located in the Furnace. All surfaces of flanges shall be protected by vendor due to prevent of damage. Use of direct flame is not approved. | Confirmed | Closed |  |  |  |  |
|  | Referenced documents | Referenced documents, specifications and STDs specified in project MR shall be fully followed by vendor. In addition, for major components within the package, following codes and standards (Latest Editions) shall be considered by vendor: 1- Pressure Vessel Design: ASME BPVC VIII Div. 1 2- Materials: ASME BPVC Sec. II, ASTM 4- Nozzle and External Load Design: WRC 537 5- Wind: ASCE 7 6- Seismic: ASCE 7 | Confirmed | Closed |  |  |  |  |
|  | Material | Material ASTM No. of all package parts including grade and designation shall be specified in equipment datasheets. Furthermore, MDMT, design temp and pressures shall be specified as well as requirement of impact test and heat treatment. | Confirmed, impact test and heat treatment N/A | Closed |  |  |  |  |
|  | Plates | Sliding plates, templates, liners, insert plate & shims (whichever is applicable) are in vendor’s scope of supply. | Confirmed, if required | Closed |  |  |  |  |
|  | Piping Support | All internal piping, valves and fittings with their supports for cooling, sealing, purging, vent, drain and lubrication shall be provided and installed by Vendor. Any auxiliary connection inside B.L shall be flanged type and extended up to the skid's boundary. | Confirmed, if required | Closed |  |  |  |  |
|  | Walk way | In order to provide access to all parts of package, Walkways, platforms, ladders & handrails shall be considered by Vendor | Confirmed | Closed |  |  |  |  |
|  | Seismic/Wind parameters | Vendor shall follow project specifications for Seismic/Wind parameters. Below data shall be considered as minimum requirements: - For seismic: - Code: ASCE 7-10 - Importance Factor: 1. 25 - Site Class: D - Ss = 0.781 - S1 = 1.25 - Fa = 1 - Fv =1.5 - Design wind speed: 120 km/h @10m - Wind Exposure: C | Confirmed, if required | Closed |  |  |  |  |
|  | Grid plate and Grating | The floor shall be covered with grid plate which is suitably supported to prevent excessive deflection. Checkered plate for walkway or other places that end user may use, is not acceptable. Hot dip galvanized grating with four clips per each piece of grating shall be considered for bottom of all walkways. | N/A | Closed |  |  |  |  |
|  | Drain nozzle height | Drain valve height shall have enough space for operation and maintenance. | Confirmed, if required | Closed |  |  |  |  |
|  | Passages and all open surfaces | Passages and all open surfaces around the equipment shall be covered with gratings. All the grating penetrations (piping, cables, etc.) shall be bordered with a flat bar which will act as a foot guard. | Confirmed, if required | Closed |  |  |  |  |
|  | Commissioning & Start-up | All items required for commissioning & start-up inside the package shall be supplied by vendor. Necessary quantities of catalysts, cartridges, chemicals, desiccant, molecular sieve, etc., whichever is applicable and required for start-up and initial operation of the package shall be supplied by vendor. | Confirmed, if required | Closed |  |  |  |  |
|  | P&IDs | Purchaser P&IDs to be fully reviewed by vendor and if any extra provisions for control and reliable operation is needed, to be announced by vendor ay bid stage. | Confirmed | Closed |  |  |  |  |
|  | Shop-assembled | The unit shall be shop-assembled to the maximum extent practical to minimize on-site erection work. All skids and equipment that mate directly (without interconnecting piping by the purchaser) shall be trial fit in the vendor’s shop. The vendor shall identify all loose-shipped components requiring field installation in the proposal. | Confirmed | Closed |  |  |  |  |
|  | pressure vessels Specification | All pressure vessels inside the package shall comply with requirements explained in Specification for Welded Pressure Vessel BK-GNRAL-PEDCO-000-ME-SP-0001 attached to material requisition. | Confirmed | Closed |  |  |  |  |
|  | Nozzle Flange | All nozzles shall be flanged with Welding neck flanges. For nozzle sizes smaller than 2”, long-welding-neck flanges are required. | Confirmed | Closed |  |  |  |  |
|  | Standard Drawings | All details of pressure vessels designed according to ASME BPVC VIII shall follow Standard Drawings for Fixed Equipment attached to this file. | Confirmed | Closed |  |  |  |  |
|  | Vessel Support type | Vessels with diameters (OD) and weights greater than 1000 mm and 1200 kg, respectively; are not allowed to be supported by legs. | Confirmed | Closed |  |  |  |  |
|  | Lifting Lug | The skid shall be provided with lifting lugs for a four-point lift. Lifting and transport of the skid, complete with all equipment mounted, shall not permanently distort or otherwise damage the skid or the machinery mounting surfaces. | Confirmed | Closed |  |  |  |  |
|  | Overall plot plan | A preliminary overall plot plan shall be added to the next revision by bidder to show approximate arrangement and dimensions of the package. | Will be specify in GA document | Closed |  |  |  |  |
|  | Documents | Bidder to submit all required documents with tender as per “RFD Form” in the MR. For example below documents: 1-Marked up and Filled out MR scope of supply and works (It shall be signed and stamped) 2-GA DWG with outline dimensions and weights related to this project 3-A complete Equipment List 4-Noise Data 5-Utility Consumption and load List 6-Spare Part List 7-NDT 8-Hydrostatic test report 9-PWHT report (if required by code) 10-VPIS 11-Marking, packing, shipping procedure 12-Manufacturing & assembly 13-Name plate 14-ITP 15-Painting procedure and sand blast 16-Final book | Confirmed | Closed |  |  |  |  |
|  | Access for internal loading/unloading | Manhole / hand hole Access for internal loading/unloading of drums | Confirmed | Closed |  |  |  |  |
| PIPING & MATERIAL |  |  |  |  |  |  |  |  |
|  | PMS | Project PMS shall be followed by vendor. | N/A | Not accepted, Project PMS shall be followed by the vendor | CONFIRMED | Closed |  |  |
|  | General arrangement drawing | Air & N2 Package General arrangement drawing shall be issued by vendor. | Confirmed | Closed |  |  |  |  |
|  | 3D model | PDMS 3D model shall be issued by vendor. | N/A | Not accepted | CONFIRMED | Closed |  |  |
|  | VPIS | VPIS will be finalized after P.O. (Piping Doc. shall be added to document which listed to document list) | N/A | Not accepted, VPIS will be finalized after P.O. | CONFIRMED | Closed |  |  |
|  | PLOT PLAN | Please confirm arrangement of packages on plot plan drawing | Will be submit only GA for specify piping | Closed |  |  |  |  |
|  | Material | Material and corrosion allowances shall be in accordance with data sheets. | Confirmed | Closed |  |  |  |  |
|  | Material | - Internal coating is required for Air and Nitrogen receivers | Confirmed | Closed |  |  |  |  |
| INSTRUMENT & CONTROL |  |  |  |  |  |  |  |  |
|  | Control System Requirements | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0004 and provide the requirement of type C package for nitrogen package, air compressor and sufficient controller for air dryer in field as mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. As minimum: 1-UCP with redundant CPU/PS dedicated for each train to be provided by vendor. 2-SIL rated loops for protection and trip to be merged inside UCP panel for each train. (redundancy for IO is required) 3- Monitoring of machine protection signals could be also merged inside UCP panel due to vendor design (required transmitters/transducer for temperature/vibration and etc. sensors to be considered for UCP connection otherwise separate panel, at least VMS to be provided.) | Confirmed | Closed |  |  |  |  |
|  | Instrumentation | Regarding supplying instrument on datasheet), bidder is requested to consider all instruments mentioned on project datasheet. VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0001 for instrumentation devices and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed | Closed. AVL for sub vendors shall be followed | CONFIRMED | Closed |  |  |
|  | Instrumentation Valves | VENDOR shall follow below documents for valves and other documents as mentioned in these documents. - BK-GNRAL-PEDCO-000-IN-SP-0005 - BK-GNRAL-PEDCO-000-IN-SP-0006 - BK-GNRAL-PEDCO-000-IN-SP-0007 VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Will be prepare and submit documents according attached document list | OPEN. Valves inside package shall be prepared as per attached Spec of PMR. Vendor to confirm. AVL for sub vendors shall be followed. | CONFIRMED | Closed |  |  |
|  | Connection with MCC | If MCC is supplied by VENDOR all interfaces shall be supplied by VENDOR. Below signals are typically defined in this project to be provided for electrical interfaces: - Start command (2 sec high pulse) - Stop command (2 sec high pulse) - Trip command (low signal for trip) - Run/Stop Status (high signal for run status) - Local/Remote status (high signal for local status) - Tripped on Fault (high signal for fault status) Available for remote control (high signal for available status) | Confirmed, if required Commercial Proposal will be include JB ,LCP ,control panel & electrical panel for every compressor package Dryer will be control in compressor panel | OPEN. Control Block diagram and P&ID to be prepared by vendor | CONFIRMED | Refer to block diagram & P&ID comments | confirmed and corrected in related document. P&ID | Closed |
|  | Cable | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0010 for cables and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed | Closed. AVL for sub vendors shall be followed | CONFIRMED |  |  |  |
|  | Terminals | All terminals shall be screw type. VENDOR shall note that each core/pair/etc. of cable shall be wired to dedicated terminal | Confirmed | Closed. |  |  |  |  |
|  | Panel Dimension | VENDOR shall specify the estimated number and dimension of panels. | For every package of compressor : JB,LCP on skid Control panel : 600*600*2000 mm Electrical panel : 600*600*2000 mm | OPEN. Electrical panel shall be implemented on local panel. Please clarify. | CONFIRMED | Refer to block diagram & P&ID comments | confirmed and corrected in related document | Closed |
|  | Required Power | VENDOR shall specify the estimated power required (non-UPS and UPS) | 2*55kw for instrument compressor package 1*45kw for PSA compressor package | OPEN. PSA to be clarified. Number of panels shall be dedicated. | For PSA , one panel for control panel and one panel for electrical panel. | Closed |  |  |
|  | Special Tools | VENDOR shall supply all special tools for compressor and axillary devices. This is included the special cables (not mentioned in BK-GNRAL-PEDCO-000-IN-SP-0010), MODBUS sockets, software and etc. The list shall be provided separately. | Confirmed | Closed. AVL for sub vendors shall be followed | Confirmed | Closed |  |  |
|  | Protection Instruments | The protective instruments shall be according to API-670. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed | OPEN. Please clarify regarding required protection sensors . AVL for sub vendors shall be followed | protection sensors will be according attached P&ID . Confirmed | Refer to block diagram & P&ID comments. | confirmed and corrected in related document | Closed |
|  | Software | VENDOR shall supply all the software (including the passwords, licenses and program sources). The programs shall be supplied with all tags, descriptions and etc. The software programs shall be installed on engineering laptops and also supplied on CD/DVD. | Confirmed | Closed. |  |  |  |  |
|  | Sequence controller | VENDOR shall clearly specify the scenario for sequence controlling of compressor and dryer also nitrogen package, the interfaces signals, required hardware (e.g. panels, if any) and additional control or on/off valves. | Confirmed | OPEN. To be clarified | After order | Refer to block diagram & P&ID comments. | confirmed and corrected in related document | Closed |
|  | Start-up | VENDOR shall clearly state the start-up sequence. It shall be mentioned if any additional instrument or valve is required. Also, the interface signals shall be mentioned (if any) | Confirmed | OPEN. To be clarified | After order | Refer to block diagram & P&ID comments. | confirmed and corrected in related document | Closed |
|  | Spare part | VENDOR shall supply all commissioning/ 2 years spare part as per NISOC procedure, attached to PMR | Confirmed | OPEN. List to be prepared | Please check attached file. | Refer to block diagram & P&ID comments. | confirmed and corrected in related document | Closed |
|  | Control & Safety | As per project specification, PLC based UCP (for control and safety) dedicated to each compressor is requested for both package (N2 and air): -One PLC based UCP for each compressor (totally 2.Nos. located on control room) for Air compressor package (type C). -One PLC based UCP for nitrogen package (located on control room) (type C). -Redundancy for CPU/power supply /critical and safety I/Os and communication link to be considered in each UCP. Separate SIL rated system for ESD signals (could be relay based ESD part inside UCP) shall be considered in parallel as safety back up logic. - LCP is required for each compressors for on/off and commissioning/startup and gauge boarding. -Project Spec to be respected for packages type C. -Microprocessor based LCP is required for each dryer. -Regarding Machine monitoring at list minimum requirements as per API to be respected (vibration for trip…) | Confirmed | OPEN. Control Block diagram and P&ID to be prepared by vendor | Please check attached documents. | Refer to block diagram & P&ID comments. | confirmed and corrected in related document | Closed |
|  | Signals | Since MCC is supplied by VENDOR : - Interface signals between MCC and UCP (VIA IRP) and LCP shall be considered in vendor scope. Below signals are typically defined in this project: - Start command - Stop command - Trip command - Run/Stop Status - Local/Remote status Fault status |  | Open- shall be confirm by vendor. | Confirmed | Refer to block diagram & P&ID comments. | confirmed and corrected in related document | Closed |
|  | Hook UP drawing | Project Instrument Hook UP drawing (BK-GCS-PEDCO-120-IN-DG-0002), Shall be Followed by vendor | Confirmed | Closed |  |  |  |  |
| SAFETY |  |  |  |  |  |  |  |  |
|  | Noise Level | Noise levels of items should not exceed 85 dbA @ 1m. | Confirmed | Closed |  |  |  |  |
| ELECTRICAL |  |  |  |  |  |  |  |  |
|  | Specification | All electrical equipment shall comply with electrical references & standard that mentioned on material requisition. IPS-E-EL-100 (1), IPS-M-EL-131 (2) & IPS-M-PM-290(2). | Confirmed | Closed |  |  |  |  |
|  | Vendor Scope of Work | All documents related to electrical equipment shall be submitted by vendor such as preliminary load list, electrical equipment list, motor data sheet, motor outline drawing & etc. | Confirmed | Closed |  |  |  |  |
|  | Vendor Scope of Work | Vendor list & sub vendor list of manufacture shall be according to project AVL. | Confirmed | Closed |  |  |  |  |
|  | Vendor Scope of Work | The cable from switchgear to package panel is by client, but the cable in the package form panel to each equipment is by vendor. In other word, from client point of view, there is only one load which assumed the power demand of package is the total load. This note is valid for both nitrogen & air package. (Tie in between client & vendor is main panel). | Confirmed , total power=4*55+1*45=265 kw Needs to specify distance of panels up skid of compressor | More information, The cable & power from switchgear (400V, 3Phase & 50Hz), to main package panel (in field) is by contractor, but the cable in the package from main panel to each equipment (on skid) is by vendor. In other word, from contractor point of view, there are 3 below loads: - Instrument & Plant Air Package (PK-C-2203A) - Instrument & Plant Air Package (PK-C-2203B) - Nitrogen Package (PK-2204) (Tie in between contractor & vendor is main panel which panel will be provided by vendor). Therefore the total power demand shall be specified by vendor. It seems total power demand shall be revised. | Confirmed , for the cable in the package from main local panel to each equipment. and All wiring from control room up skid of package will be by contractor. | Closed |  |  |
|  | Vendor Scope of Work | According to item 4, the total electrical load is required not each one separately. | 265 kw | Please consider above item. | Confirmed, for the cable in the package from main local panel to each equipment. and All wiring from control room up skid of package will be by contractor. | Closed |  |  |
|  | Vendor Scope of Work | Since the power & control cables in the skid is by vendor, therefore the cable list in the package shall be according to “BK-GNRAL-PEDCO-000-EL-SP-0014”. Interconnection in the skid is by vendor. | All wiring on skid acceptable | Closed |  |  |  |  |
|  | Vendor Scope of Work | Supplier shall design the routing of the power & control cable inside package. | Confirmed | Closed |  |  |  |  |
|  | Vendor Scope of Work | If AC UPS or DC UPS & batteries are required, vendor shall provide all necessary documents (such as calculation, list etc.) & supply required equipment according to purchaser standards. | Confirmed, if required | Closed |  |  |  |  |
|  | Spare Part | A complete set of spare parts for commissioning shall be supplied for each motor. | Confirmed | Closed |  |  |  |  |
|  | Test & Inspection | The equipment shall be factory tested. Certified copies of test reports shall be submitted to the purchaser. | Confirmed | Closed |  |  |  |  |
|  | Vendor Scope of Work | Data sheet & Specification of LV induction motor have been attached. Please follow them. | Confirmed | Closed |  |  |  |  |
|  | Deviation | Deviation of power cables is not acceptable & according to item 6, cables shall be XLPE & armored. | Confirmed | Closed |  |  |  |  |
|  | Vendor Scope of Work | The engineering & procurement of package shall be according to “BK-GNRAL-PEDCO-000-EL-SP-0011”. | Confirmed | Closed |  |  |  |  |
|  | Vendor Scope of Work | The method of start & stop of air compressor & interlock with other equipment shall be specified. | Confirmed | Closed |  |  |  |  |
|  | Vendor Scope of Work | The method of start & stop of nitrogen package & interlock between main motor & auxiliary loads of N2 package shall be specified. | Confirmed | Closed |  |  |  |  |
|  | Vendor Scope of Work | I/O List shall be issued by vendor. | Confirmed | Closed |  |  |  |  |
| STRUCTURE |  |  |  |  |  |  |  |  |
|  | Earthquake load | Use ASCE7-10 for earthquake load | Confirmed | Closed |  |  |  |  |
|  | Anchor bolt | The type & location of the anchor bolt should be provided. | Confirmed | Closed |  |  |  |  |
|  | Skid | The exact dimensions & size of the skid and the proper alignment under the skid should be determined. | Confirmed | Closed |  |  |  |  |
|  | Loads and center mass | The static & dynamic loads of the equipment and the level of the center mass the device should be determined. | Confirmed | Closed |  |  |  |  |

| # | Clarification Description NISOC Letter 1403/06/27 03/2294/242202 | Conclusion (Rev.00) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description (Rev.01) | Conclusion (Rev.01) (VENDOR to Confirm & Consider in the Final Offers) |
| --- | --- | --- | --- | --- |
| مکانیک |  |  |  |  |
| 1. | استاندارد طراحی کمپرسورها، ISO-10440-2 میباشد، سازندگان میبایست ضمن تایید این مورد، انحرافات Major خود با این استاندارد را بطور مشخص اعلام نمایند. | در صورت پذیرش فشار عملکردی ، مورد تایید می باشد . |  |  |
| 2. | جدول تغییرات دما و فشار با دبی برای کمپرسور هوا و نیتروژن از سوی سازندگان ارائه گردد | شرکت آصا صاحب نرم افزارطراحی می باشد و کلیه محاسبات بر اساس دیتاهای تقاضا صورت خواهد گرفت و انتخاب از جداول انجام نخواهد شد |  |  |
| 4. | کلیه سازندگان دست دوم کالا از جمله سازنده Air End و فیلتر روغن میبایست از لیست بلند وزارت نفت انتخاب گردند. | مورد تایید |  |  |
| 5. | کالای یدکی میبایست بر اساس پیشنهاد سازنده و با در نظر گرفتن حداقلهای مندرج در پیوست 11 اسناد پیمان در نظر گرفته شود. | مورد تایید و پیشنهاد بر اساس مدرک مربوطه انجام شده است |  |  |
| 6. | مدرک ASA-LR-02-263: در جداول سازنده صفحه 55/18 باید محصول پیشنهاد شده از لیست مجموعه محصولات مارک گردد. | به پیوست ارایه شده است |  |  |
| برق |  |  |  |  |
| 1. | آیتم های مرتبط با تستهای کارخانه ای از دیدگاه الکتریکال ناقص است لذا ضمن تامین /تکمیل دستگاه های تست الکتریکال، در خصوص آیتم های ITP بر اساس ملاحظات تست کارخانه ای هر تجهیز متعاقبا اظهار نظر خواهد شد. | کلیه تستهای مورد نیاز انجام خواهد پذیرفت و هیچ محدودیتی در این خصوص وجود ندارد |  |  |
| 2. | سیستم حفاظت الکتریکی فارغ از حفاظت های پیش فرض مربوط به راه انداز نرم الکتروموتورها، باید در نظر گرفته شود. | مورد تایید می باشد |  |  |
| 3. | آیتم 2-3 Instrument Plant Airمدرک TBE مربوط به Grounding Bosses جهت هر پکیج بصورت مجزا ضروری است . | مورد تایید می باشد |  |  |
| 4. | چنانچه پکیج ها به صورت آماده(وارداتی) تامین خواهد گردید نسبت به ارائه مدارک کامل اقدام شود، بدیهی است تجهیزات پروژه باید بر اساس نیازمندیهای خاص پروژه تامین گردند. | مورد تایید می باشد و صرفاً ایر اند وارداتی می باشد |  |  |
| 5. | تجهیزات تست تجهیزات الکتریکال روئیت نگردید. | کلیه تجهیزات مورد نیاز موجود می باشد و هیچ محدودیتی در این خصوص وجود ندارد |  |  |
| 6. | مدرک Ir-02-265 در خصوص قطعات یدکی به قطعات الکتریکال اشاره ای نشده است. | قطعات یدکی طبق مدرک پروژه یه پیوست ارایه شده است |  |  |
| ابزار دقیق |  |  |  |  |
| 1. | SUB-VENDORهای معرفی شده بخصوص در بخش برند سیستم کنترل و بستن کنترل پنل، لازم است از لیست بلند وزارت نفت باشند. | مورد تایید می باشد |  |  |
| 2. | در مورد پایش عملکرد ماشین MMS الزامات SPEC و استاندارد API 670 رعایت گردد. در نظر گرفتن سنسور لرزش مطابق P&ID و SPEC الزامی است. | مورد تایید می باشد و این نوع پکیج نیازی به سنسور لرزش ندارد |  |  |
| 3. | تمامی نرم افزار های مورد نیاز باید بصورت اصلی و مطابق SPEC پروژه ارائه گردد. | مورد تایید میباشد |  |  |
| 4. | در ITP ارائه شده از سازندگان، در نظر گرفتن موارد بازرسی سیستم کنترل و پنل کنترل محلی و تست های FAT و SAT آنها دیده شود. | کلیه تست های مورد اشاره در استاندارد پروژه، بدون هیچگونه محدودیتی انجام خواهد پذیرفت |  |  |

| # | Clarification Description PEDCO Letter 1403/05/14 BK-SUR-ENG-MEC-03-09433 | Conclusion (Rev.00) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description (Rev.01) | Conclusion (Rev.01) (VENDOR to Confirm & Consider in the Final Offers) |
| --- | --- | --- | --- | --- |
| Mechanical Comments : |  |  |  |  |
| 1. | صرفا Deviation List هر سازنده مورد بررسي قرار مي گيرد و هيجگونه انحراف از مدارك بسته خريد كه در Deviation list مطرح نگرديده قابل بررسي نبوده و مورد پذيرش نخواهد بود و مسئوليت كامل تطابق كالا با مدارك بسته خريد بر عهده آن سازنده مي باشد. | مورد تایید می باشد |  |  |
| Process comments : |  |  |  |  |
| 1. | تمامی تجهیزات مورد نیاز پکیج های هوا و نیتروژن از قبیل بافرها، فیلترها و ... در شرح کار سازنده می باشد بجز V-2203 و V-2204 که موقعیت آنها در پایین دست پکیج طراحی شده است و قابلیت استفاده به عنوان نوسان گیر کمپرسور یا Surge Drum را ندارد. | مورد تایید می باشد |  |  |
| Instrument comments : |  |  |  |  |
| 1. | شرکت آراز صنعت برای پکیج هوا می بایست دیاگرام مربوطه اصلاح گردد و متناسب با نام کمپرسورها ارایه گردد. | هر گونه لاجیک مورد درخواست قابل اجرا می باشد و در صورت برنده شده این شرکت متعهد با تامین نیاز آن مجموعه محترم طبق لاجیک کارفرما خواهد بود |  |  |
| 2. | در شماتیک ارایه شده برای شرکت آراز صنعت ارتباط بین UCP پکیج ها و سیستم ESD/DCS ایستگاه نمایش داده شود. | بعد سفارش گذاری ارایه خواهد شد و در این خصوص هیچ محدودیتی وجود ندارد |  |  |
| 3. | در شماتیک ارایه شده برای شرکت آراز صنعت ارتباط MCC و UCP/LCP ها نمایش داده شود. | بعد سفارش گذاری ارایه خواهد شد و در این خصوص هیچ محدودیتی وجود ندار |  |  |
| 4. | شرکت آراز صنعت No-Deviate را تایید نموده است این درحالیست که تناقضات یا نقایص زیادی با شرح فعالیت پروژه دارد. | در بخش برق و کنترل و ابزار دقیق، هیچ محدودیتی وجود ندارد و مطابق اسناد و استاندارد و نیاز پروژه (بدون مغایرت ) انجام خواهد شد . |  |  |