| # | Clarification Title | Clarification Description (Rev.00) | Conclusion (Rev.00) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description (Rev.01) | Conclusion (Rev.02-Final) (VENDOR to Confirm & Consider in the Final Offers) |
| --- | --- | --- | --- | --- | --- |
| GENERAL |  |  |  |  |  |
|  | Documents Priority | Vendor to confirm that in case of contradiction between this TCL and offer; vendor confirmation of compliance through this technical clarification is governed and shall be followed by vendor after order. In such case offer won’t be considered as basis of any claim after order. | Confirmed. | Closed |  |
|  | Deviation List | Bidder shall confirm his full compliance to meet all purchaser documents listed and attached to MR, code / standards listed in MR with ref. No. "BK-GCS-PEDCO-120-ME-MR-0006" as well as MR requirements. All deviations shall be listed in project deviation form, attach to this TCL, signed and stamped. Notes: 1-Any exception and deviation from the requirements of MR and the related specifications including referenced codes & standards shall be clearly declared by vendor in writing for purchaser’s approval at bid stage by using of the format included in “Letter Of Conformity” attached to material requisition. Raising no deviation to any single requirement means full compliance of vendor quotation to that requirement. Purchaser has no responsibility to discover deviations or alternatives scattered over the proposal in addition to what proclaimed explicitly in deviation list; hence such deviation (if any) is disregarded and shall not be considered for vendor claim in future. (Vendor to confirm) 2- Bidder shall confirm that all MR attachments listed in MR are available for her for review at bid stage. 3-Considering items 1 & 2, please note that any deviation at detail stage will not be accepted by purchaser. | Confirmed, by considering deviation list. | Deviation list will be finalized in clarification meeting. | Meeting conclusion-05.08.2023: Please refer to agreed deviation list. HY: The deviation list has not been finalized. |
|  | Documents Priority | Bidder to confirm his compliance with documents priority stated in MR. Any conflict between the documents shall be notified by bidder to purchaser and resolved by his agreement | Confirmed, by considering deviation list. | Deviation list will be finalized in clarification meeting. | Meeting conclusion-05.08.2023: All agreements on this TCL and deviation list priority are higher than MR. Vendor Confirmed. Closed. HY: Deviation list shall be finalized |
|  | Requirement For Documents | Bidder to confirm that documents required after purchase order shall be submitted in accordance with “Vendor documents min. requirement list” attached to the MR. | VPIS will be finalized in the KOM after P.O. | Closed | Meeting conclusion-05.08.2023: Attachment #2 of relevant MR will be checked, singed and stamped by the vendor and if there is any deviation will be specified. Closed. |
|  | Documents | Bidder shall provide following documents while bidding: 1- Copy of valid quality system certificate (ISO 9001/2 or 3) 2- Deviation list with letter of conformity (Refer to item A2 above) 3- Reference and Experience List 4- Joint venture agreement (If any) with any company to design / fabrication of equipment. In case of joint venture; qualified documents of the company who is part of joint venture shall be also submitted | - Noted. - Noted. - Noted. - Will be submitted (if any) | Closed |  |
|  | Sub-vendors | Bidder to confirm that Sub-Vendors shall be selected from Project’s AVL. Please refer to purchaser’s comments on Vendor’s sub-vendor list. | Project’s AVL has not been received. Please send us AVL in this regard. Please, see HAVAYAR submitted sub-vendor list and let us know if you have any comment. | Project AVL shall be followed by vendor as attachment#1 | Meeting conclusion-05.08.2023: Sub- Vendor list shall be according to Project’s AVL (MOP AVL) and shall be approved by the client’s after P.O. Closed. HY reply: on/off valves inter skid of dryer and dewpoint transmitter, are as per HAVAYAR sub vendor list. These items are not in MOP/ NPC AVL and we cannot change them. |
|  | Painting specification | Bidder to confirm that surface preparation and painting of all equipment material including valves, pipes, fittings and everything within the package are in accordance with project painting specification with ref. "BK-GNRAL-PEDCO-000-PI-SP-0006" attached to MR. | Project painting specification and also RAL code will be followed for all equipment except compressor, filters, water trap, chiller and valves which are based on manufacturer standard. Thickness and type of paint is according to client spec, procedure will be followed according to Havayar's procedure which will be sent in detail design stage for client review. | Project painting specification with ref. "BK-GNRAL-PEDCO-000-PI-SP-0006" Shall be followed by vendor. | Meeting conclusion-05.08.2023: Vendor confirmed for between skids. Closed. |
|  | Specification | Bidder to confirm that Packing, Marking, Preservation and Preparation for Shipment will be done in accordance to the project specification with Doc. No; BK-GNRAL-PEDCO-000-QC-PR-0045 attached to the MR. | Confirmed, by considering deviation list. | Deviation list will be finalized in clarification meeting. | Meeting conclusion-05.08.2023: Please refer to agreed deviation list. |
|  | Inspection and Test Plan | Inspection and Test Plan / Quality Control Plan and their related inspection level (Witness, Hold, Review and Approval) shall be at least in accordance with MR and all project requirement such as NDT PROCEDURE. Any deviation is not accepted unless otherwise specified in deviation list and approved by Purchaser. | Project ITP will be followed for receivers and vessel of dryers and N2 generators. ITP of compressor, chiller, filters and water trap are based on manufacturer standard. ITP will be finalized after P.O. and in PIM. | ITP will be finalized during PIM | Meeting conclusion-05.08.2023: Vendor confirmed. Closed. |
|  | Material Certificate | Bidder to confirm that for all pressure parts and those required special consideration or stainless steel as well as for supports the material certificate should be in accordance with EN 10204, 3.1 Material certificate for non-pressure parts shall be in accordance with EN 10204, 2.2 | Confirmed, except for compressor, chiller, filters and water trap. | Not accepted, this requirement shall be followed by vendor | Meeting conclusion-05.08.2023: Material certificated will be submitted by the vendor according to approved ITP. Closed. HY reply: Confirmed except for compressor and chiller. |
|  | Scope | Bidder shall confirm the scope of work and supply are fully in accordance with MR and its attachment with considering following items: 1- The consequences of HAZOP and SIL study shall be in vendor scope of supply and work. 2- Participation in technical meeting whenever it is necessary by Purchaser/Client to close engineering document after order is in vendor scope 3- Lifting beams (Spreader beams) for erection with sling shackle and etc. shall be designed and supplied by vendor 4- N2 preservation with pressure regulator, valve & flexible hoses during transportation & 18 months of site storage based on requirements in MR shall be in vendor scope of supply / work. 5- All utility conditions shall be as per document "Basic engineering design data (10‐BD‐00‐PR‐SPC‐0001)". Moreover, bidder shall consider utility noted condition as per project design data. If other utility condition is needed, whole required facilities is in vendor scope of work/supply. 6- Anchor bolts | Confirmed, by considering deviation list. - Please, be noted that any change due to this meeting which cause significant cost impact is out of HAVAYAR scope. But the items which is related to safety of the package or lack of engineering design will be followed and applied by HAVAYAR. HAVAYAR will only attend to HAZOP and SIL meeting. Be noted that the package is non-SIL and any changes in this regard after P.O. is out of HAVAYAR’s scope. - Confirmed. - Not applicable, lifting lug for equipment has been considered. - N2 filling for shipment of practical equipment will be considered, (Method and equipment of filling is vendor responsibility and design) except for filters, water trap. - Confirmed. - Confirmed. | 1.Not accepted, this requirement shall be followed by vendor without any cost impact 2. Closed 3. Closed 4. This requirement shall be confirmed by vendor. 5.Closed 6.Closed | Meeting conclusion-05.08.2023: - Project specification will be followed by the Vendor. - Closed. - Closed. - N2 preservation will be done according to approved “Preservation procedures” that will be submitted by the Vendor on detail stage. - Closed. - Closed. HY reply: HAVAYAR will only attend to HAZOP and SIL meeting. Be noted that the package is non-SIL and any changes in this regard after P.O. is out of HAVAYAR’s scope. |
|  | Applied standard | Applied Standards for equipment design, fabrication and test shall be listed | Havayar applicable Codes and Standards have been submitted previously. | Vendor shall follow project specification ; "Specification for Air Compressor Package with Doc. No. BK-GCS-PEDCO-120-ME-SP-0006". As per project specification, Oil-free screw compressors shall be compatible with requirements of ISO 10440-2 standard otherwise shall be mentioned as a deviation. | Meeting conclusion-05.08.2023: All project specification shall be followed by the Vendor if there is any deviation shall be specified clearly. HY reply: Please be noted that compressor and chiller package will be design based on manufacturer standard. |
|  | outdoor installation/ operation under project site conditions | Vendor shall confirm Air Compressor Packages, Dryer Packages and N2 generation package are fully suitable for outdoor installation/operation under project site conditions “Process Basis of Design; BK-GNRAL-PEDCO-000-PR-DB-0001 “without requiring any additional protection. | Confirmed, package has been considered outdoor and under shelter, supply of shelter and crane are out of Havayar scope. | Closed |  |
|  | Suitability of utility characteristics available at site | Vendor shall confirm suitability of utility characteristics available at site as per project site conditions “Process Basis of Design; BK-GNRAL-PEDCO-000-PR-DB-0001 “. Supplying all facility for utility distribution and regulation shall be part of the vendor scope. | Confirmed with considering deviation list. | Deviation list will be finalized in clarification meeting. | Meeting conclusion-05.08.2023: Please refer to agreed deviation list. |
|  | Warranty | Vendor shall warranty the engineering, materials and workmanship of the equipment/goods. Under this warranty, all materials, parts, and assemblies found to be defective during the guarantee period shall be replaced free of charge by VENDOR. Also, the guarantee period shall be eighteen (18) months from the date of delivery or twelve (12) months from the installation date of each equipment/package at site. | This item will be finalized in commercial offer. | Noted | Meeting conclusion-05.08.2023: Vendor Confirmed to following “GUARANTEE AND WARRANTY” according to clause 9 of relevant MR. Closed. |
|  | Document | Bidder to confirm that all native files due to all calculations and drawings shall be submitted for purchaser approval after order. | Confirmed. | Closed |  |
|  | Deviation list | Vendor’s deviation list is not accepted and will be finalized in Clarification Meeting. | Will be finalized in Clarification Meeting. | Noted | Meeting conclusion-05.08.2023: Please refer to agreed deviation list. HY: Deviation list has not been finalized. |
| PROCESS |  |  |  |  |  |
|  | Process Data | Air compressor package, air dryer packages, nitrogen package ... shall be revised as per project specification/data sheet. Latest revision of project documents has been attached as Attachment#2 of this TCL. Please follow them. | The design of the packages has been revised based on Attachment#2 of the TCL, however, the considered deviations have been mentioned in revised deviation list. Also, these documents are in conflict with project duty speciation and revised offer are proposed based on duty specification Doc. requirements. |  | Meeting conclusion-05.08.2023: Mechanical design pressure for pressure part is 12.5 bar g. Also Hydro test will be done corrected on technical offer. Vendor confirmed. Closed. |
|  | General comment | Vendor shall be capable to generate nitrogen with purity of 99.9% | Based on the attached documents, the design of the nitrogen package has been done with the purity of 98% of produced N2. If 99.9% of N2 is required, please uniform that for all bidders. As per Doc “DUTY SPECIFICATION FOR INSTRUMENT/PLANT AIR & NITROGEN PACKAGES” client requested for N2 with 98% purity. | As per client request, nitrogen purity 99.9 shall be follow. | Meeting conclusion-05.08.2023: Vendor confirmed. Closed. |
| MECHANIC |  |  |  |  |  |
|  | Document Requirement | Bidder to submit all required documents with tender as per “VDR Form” in the MR. For example, below documents: 1-Marked up and Filled out MR scope of supply and works (It shall be signed and stamped) 2-GAD for air compressors, air dryers and nitrogen generators with dimensions, all auxiliaries and heaviest part weight table (by considering air cooler in the package) 3-Cross sectional drawing for compressors with BOM 4-Performance Curves 5-Equipment data sheet in each standard format 6-Utility consumption and load list 7-Spare part list 8-Special tool list 9-P&ID’s dedicated for air compressor, dryer and nitrogen generation packages with all equipment, instruments and lube oil, cooling and control systems. 10-Noise Data 11-Equipment List on detail for mechanical and other items | - Scope of work and supply has been filled in “TBE for Instrument/Plant Air & Nitrogen Packages”. Any deviation has been submitted. - Package preliminary G.A. for client review will be submitted with the next revision of proposal. Weight of each equipment has been specified in relevant data sheet and will be specified in G.A. - Cross sectional drawing for compressors is not applicable. - Will be submitted after P.O. - Equipment data sheet has been submitted previously. - Electrical load list has been submitted previously. - Spare part list will be submitted with the next revision of proposal. - Special tool is not applicable. - Overall PID of this package has been submitted previously, sample PID of compressor will be submitted with the next revision of technical proposal. - Will be submitted after P.O. - Has been submitted in technical proposal. | 1. Project's MR shall be signed and stamped by vendor at this stage. 2. Noted 3. Not accepted, to be submitted by vendor. 4.Noted 5.Noted 6. Noted 7. Noted 8. Noted 9.Noted 10. Noted 11. Noted | Meeting conclusion-05.08.2023: - Project's MR will be signed and stamped by vendor and will be submitted. - Closed. - VPIS shall be submitted by the Vendor and shall be approved by the Client. - Closed. - Closed. - Closed. - Closed. - Closed. - Closed. - Closed. - Closed. HY: Cross section drawing is not applicable. |
|  | Lube oil system | Vendor shall submit all of the components required in compressor’s bearings lube oil system with related P&ID in next revision of technical offer. | Sample PID of compressor will be submitted with next revision of proposal. | Pending to vendor submission | Meeting conclusion-05.08.2023: Manufacture STD is Confirmed. Closed. |
|  | Air filter | Inlet air filter for compressors shall be considered and indicated on P&ID. | Inlet air filter has been considered inside of compressor skid (inside of enclosure), sample of compressor PID will be submitted with next revision of proposal. | Pending to vendor submission | Meeting conclusion-05.08.2023: Manufacture STD is Confirmed. Closed. |
|  | Compressor stages, Inter coolers and Separators | Vendor shall specify and indicate below items on PFD & P&ID for Air and Nitrogen production packages: -The number of screw compressor stages -Inter/After coolers -Separators between and after stages | Sample PID of compressor will be submitted with next revision of proposal and all of these items will be specified. PFD of whole package has been submitted and PFD of compressor is not applicable. | Vendor shall specify and indicate below items on PFD & P&ID for Air and Nitrogen production packages: -The number of screw compressor stages -Inter/After coolers -Separators between and after stages | Meeting conclusion-05.08.2023: VPIS shall be submitted by the Vendor and shall be approved by the Client. Closed. |
|  | Calculation report | Calculation report for each of compressor stage including temperature, pressure, capacity at suction and discharge of compressor and shaft power required at each stage to be specified by vendor. | Compressor requested data (FAD, shaft power, Site condition) are already specified in compressor data sheet and process deviation list. Interstage information con not be supplied and are vendor know-how. | Noted | Meeting conclusion-05.08.2023: Closed. |
|  | MAWP & PSV setting | Compressor's MAWP and PSV setting pressure on discharge line shall be specified by vendor. | Compressor's MAWP will be specified after P.O. and PSV setting pressure on discharge line is equal to package design pressure. | Noted, but package design pressure shall be revised to 12.5 barg as per project requirement. | Meeting conclusion-05.08.2023: Mechanical design pressure for pressure part is 12.5 bar g. Vendor confirmed. Closed. |
|  | Ladder and platforms | Access ladder and platforms, stairway shall be studied, designed and supplied by vendor. Relevant detail drawings shall be submitted for approval. They shall comply with project standards attached to MR, as minimum. | Ladder and platform are not applicable due to the small size of package equipment. Stairway is not applicable. | Closed |  |
|  | Interconnecting piping | Design and supply of all interconnecting piping between equipment and all instruments, valves and … on them is in the vendor's scope. Also, all between skid piping (including supporting) c/w all instrument and valve on them shall be designed, prefabricated and supplied by vendor. Detailed isometric DWGs shall be submitted for them. Note1: On skid piping to be assembled at vendor's shop. Note2; Outside skid piping to be prefabricated and match marked for site assembling. | Be noted that piping between skids/Equipment will be supplied as loose item by Havayar, installation and pipe work is out of Havayar scope. Detailed isometric DWGs for inter-skid of air compressors, chiller, dryer, and N2 generators skid is not applicable. Instruments are as per HAVAYAR submitted PID. Note 1: Confirmed. Note 2: Be noted that piping between skids/Equipment will be supplied as loose item by Havayar, installation and pipe work is out of Havayar scope. | Detailed isometric DWGs shall be submitted by vendor. Note2: Loose items shall be prefabricated and match marked for site assembling. Vendor shall confirm this requirement. | Meeting conclusion-05.08.2023: Vendor Confirmed to preparation isometric drawing and loose item. Closed. |
|  | Heat tracing c/w supports, clips and etc. | Design and supply of heat tracing c/w supports, clips, and other provisions (including panels, heating cables and relevant connectors, electrical cables, junction boxes and thermostats) shall be part of the vendor scope. | Be noted that since minimum ambient temperature is +5C, so heat tracing is not applicable. | Closed |  |
|  | Insulation c/w supports, clips and etc. | Design and supply of insulation c/w supports, clips shall be in vendor’s scope of supply and work. | Insulation is not required. | Closed |  |
|  | Commissioning spare parts | Pre-commissioning, commissioning and start-up spare parts shall be supplied by vendor as per project spare part list. | Confirmed. Please, check and let us know if you have any comment. | Vendor shall follow project spare part list as Attachment#1. | Meeting conclusion-05.08.2023: Spare part list will be submitted according reference document on next rev. of technical offer. Closed. |
|  | Two years spare parts | Two years spare parts shall be in vendor’s base offer. | Confirmed. Please, check and let us know if you have any comment. | Vendor shall follow project spare part list as Attachment#1. | Meeting conclusion-05.08.2023: Spare part list will be submitted according reference document on next rev. of technical offer. Closed. |
|  | Capital spare part | Capital spare part to be submitted as optional offer. | Capital spare part is not applicable. | Closed. |  |
|  | Material | ASTM material shall be used and submitted for Air dryers, Nitrogen generators and compressors. | Confirmed. For others, ASTM Equivalent material will be submitted after P.O. | Noted, equivalent materials shall be with purchaser's approval. | Meeting conclusion-05.08.2023: Before P.O. will be submitted by the Vendor. Refer to Clouse 24. Closed. |
|  | Vent and Drain of Packages | All vents and drains of the packages to be valved, manifolded and to be routed to the edge of skid. | Drain tie-in point is at the edge of skids (if required and practical), be noted that since vent fluid is air and is routed to ATM, so it is not required to gather vent line in one point. | Closed |  |
|  | Tie-in | All Tie-in for equipment in the packages shall be equipped will flange and valves | Confirmed. Please, see HAVAYAR submitted PID. | Closed |  |
|  | Coupling | The coupling shall be of the API671 forged-steel, non-lubricated, flexible-element spacer type. The flexible elements shall be stainless steel metallic disk. | Coupling is based on manufacturer standard. | Not accepted, this requirement shall be followed by vendor. | Meeting conclusion-05.08.2023: This item will be added to the deviation list. Coupling will be dry flexible Spacer with Non sparking guard. Vendor Confirmed. Closed. |
|  | Coupling balancing | Vendor shall specify the coupling balancing grade and balancing method as per API671. | Coupling balancing grade and balancing method is based on manufacturer standard. | Not accepted, this requirement shall be followed by vendor. | Meeting conclusion-05.08.2023: This item will be added to the deviation list. Coupling Balanced will be according to ISO 1940-1 G2.5. Closed. |
|  | PMI, pickling and passivation | Positive material identification (PMI), pickling and passivation for non-carbon steel material to be included in vendor’s scope of work. | Will be considered if required. | Closed |  |
|  | Skid of the packages | Vendor shall specify number of skids inside package with dimension and weight and preliminary general arrangement. 2-In case of multi skid, please clarify if there is any special procedure for installing and assembling of loose parts. 3-Each skid shall be furnished with 4-point lifting facility and no. two earth lugs located at opposite sides and nameplate made of SS material. 4-Vendor shall specify space required around the skid for maintenance. | - Be noted that the whole package will not be delivered as one skid, each compressor, Nitrogen generator, dryer and chiller has its own skid, Filters will be installed on piping, Air and N2 buffer will be installed on foundation by leg, number of skids inside package with dimension and weight will be specified in preliminary general arrangement and will be submitted in the next revision of technical proposal. - Installing and assembling procedure will be submitted after P.O. - Confirmed. - Confirmed, will be specified in the next revision of proposal. | - Closed - Closed - Closed - Pending to vendor submission | Meeting conclusion-05.08.2023: Closed. |
|  | Inlet condition of compressor | Inlet condition of compressor shall be considered as ambient condition of site as per relevant project specification for Site Condition “Process Basis of Design; BK-GNRAL-PEDCO-000-PR-DB-0001 ". In this regard, air compressor shall be sized for worse case condition of AMB site condition and in any condition the outlet condition as per project duty spec. shall be met. | Confirmed. Please, see process deviation list. | Deviation list will be finalized in clarification meeting. | Meeting conclusion-05.08.2023: Please refer to agreed deviation list. |
|  | Lube oil system for air compressor | Lube oil system for bearings of air compressors shall be in vendor’s scope of supply and work and its dedicated P&ID with all auxiliaries such as pumps, oil cooler, reservoir and instruments shall be submitted by vendor in next revision of technical offer. | Confirmed, be noted that compressor oil system design is by vendor and is included inside of the compressor skid. Sample PID of compressor will be submitted in next revision of proposal. | Pending to vendor submission | Meeting conclusion-05.08.2023: Manufacture STD is Confirmed. Closed. |
|  | Noise level | If noise enclosure will be required for meeting the project noise limit, it shall be supplied by vendor. Any required facilities for ventilation, fire and gas detection and fire extenuation (if any) for enclosure shall be supplied by vendor. | Enclosure will be considered only for compressor and chiller. Fire and gas detection and fire extenuation for enclosure is not applicable and required. | Closed |  |
|  | Material | Below requirements shall be followed by vendor: -Compressor casing shall be made of cast iron. -Rotors and shafts shall be forged and heat‐treated steel. -Timing gear shall be made of forged steel. -Gear box housing material shall be C.S. -The instrument air lines and the other air piping shall be galvanized CS based on project requirement “Piping Corrosion study and material selection report; BK-GCS-PEDCO-120-PI-RT-0001”. -Oil pumps casing shall be C.S. -Oil pumps impellers shall be S.S. -Oil cooler, shell shall be C.S and tube shall be SS 316 L -Oil piping shall be SS 316L. | - Compressor casing material: EN-GJR-200 - Male/Female Rotor material: 1ST:C35+N/2ND: X12Cr13+QT650 - Timing Gear Material: 16MnCr35 - Gear box housing material: Cast Iron. - Piping inter skid of compressor is based on manufacturer standard which is C.S. - Oil pump material: Cast Iron. - Type of Oil pump is Screw/Gear type which is based on manufacturer standard. - Oil cooler inter skid of compressor which is based on manufacturer standard and radiator type and the material is aluminum. - Oil piping is rubber hose and SS. | - Noted - Vendor shall confirm that Rotors and Shafts will be forged. - Vendor shall confirm that Timing gear will be forged. - Gear box housing shall be made of C.S. This requirement shall be followed by vendor. - Vendor shall confirm that all air pipes will be galvanized. - Oil pump casing shall be made of C.S. - Shell and tube of oil cooler shall be made of C.S and S.S 316 L respectively. - Rubber hose pipe is not acceptable. Oil piping shall be S.S 316L. The manufacturer’s standard is not acceptable | Meeting conclusion-05.08.2023: All relevant items will be added to the deviation/clarification. Closed. HY: Compressor and its accessories are completely based on manufacturer Std. Please refer to our agreement on items 2, 3, 22, 25 of TCL. |
|  | Applied Standards | Applied Standards for equipment design, fabrication and test shall be listed. ISO10440-2 standard shall be followed by vendor for air compressors. | Compressor will be designed, fabricated and test based on manufacturer standard not ISO10440-2. | The manufacturer’s standard is not acceptable and It's considered as a deviation. | Meeting conclusion-05.08.2023: Manufacture STD is Confirmed. Closed. |
|  | Vendor’s Scope | All vents and drains of the package to be valved, manifolded and to be routed to the edge of skid. | Drain tie-in point is at the edge of skids (if required and practical), be noted that since vent fluid is air and is routed to ATM, so it is not required to gather vent line in one point. | Closed |  |
|  | Inspection and Test | Test procedures and acceptance criteria will be checked after order with project specifications and related standards. Vendor to confirm to follow project standard and spec. | Project ITP will be followed for receivers and vessel of dryers and N2 generators. ITP of compressor, chiller, filters and water trap are based on manufacturer standard. ITP will be finalized after P.O. and in PIM. | Noted but ITP of compressor, chiller, filters and water traps shall be with purchaser's approval. | Meeting conclusion-05.08.2023: ITP will be finalized during PIM and subjected to the client’s approval. Closed. |
|  | Purchaser’s approval | Regarding technical offer, diameter, dimension and thickness of each equipment will be checked after receiving vendor calculation and data after order and comments in detail shall be followed without cost impact. | Confirmed. | Closed |  |
|  | Guarantee | Performance, Process and Mechanical Guarantee shall be in the vendor's scope. | Confirmed. | Closed |  |
|  | Project’s MR | Vendor shall follow project’s MR (Rev. D01) with its Attachments. Updated Attachments will be sent as Attachment#1 of this TCL. | Confirmed, by considering deviation list. | Project's MR shall be signed and stamped by vendor and submitted in next revision. Vendor's deviation list will be finalized before P.O in clarification meeting. | Meeting conclusion-05.08.2023: Vendor confirmed. Closed. |
|  | New Item |  |  | Please refer to Atachment#1 of this TCL for project spare part list and AVL. | Meeting conclusion-05.08.2023: Vendor confirmed. Closed. |
|  | New Item |  |  | For other mechanical comments refer to Attachment#1 of this TCL. | Meeting conclusion-05.08.2023: Vendor confirmed. Closed. |
| MECHANIC FIX |  |  |  |  |  |
|  | Fixed equipment mechanical data sheet such as drum | Vendor shall submit relevant mechanical data sheet to define shell/head/nozzle/support type & materials & dimension and designing assumption (J.E. / C.A. & etc.…) | Relevant mechanical data sheet has been submitted previously and mentioned items have been specified in data sheets. | Noted and closed. |  |
|  | Special tools | Special tools for lifting of vessel (i.e. spread bar and skirt bracing if required based vendor’s lifting calculation & relevant bolts/nuts/gasket, Hydraulic bolt tensioning device for bolts greater than 1-1/2” and etc), shall be considered in vendor’s scope. | Lifting lug for equipment has been considered. Special tools are not applicable. | Noted and closed. |  |
|  | Lifting/Shipping | Shipping calculation, hydrotest stability calculation and relevant drawings & temporary shipping support for all equipment shall be considered in vendor’s scope and submitted for purchaser’s review at detail stage. Please confirm. | Confirmed. | Noted and closed. |  |
|  | preservation | N2 purging preservation with following conditions shall be consider in vendor’s scope of work & supply (it shall be considered in vendor’s base price not as optional): 1-All opening shall be sealed with a suitable metal cover and gasket. Vessels shall be filled with nitrogen gas at a pressure of 0.5 kg/cm2(g). 2-The equipment shall have a temporary gauge to determine purge pressure. 3-Vendor shall submit N2 filling procedure for purchaser/contractor approval at detail stage. 4-All the required ancillaries such as metal covers, bolts & nuts, gaskets and pressure gauge shall be provided by vendor. Please confirm and modify your offer. (Using silica gel instead of N2 purge is not acceptable and shall be omit from your offer.) | N2 filling for shipment will be considered, (Method and equipment of filling is vendor responsibility) except for filters and water trap which N2 filling is not applicable. - Confirmed as per vendor procedure. - Confirmed. - Confirmed. - Confirmed. | Noted and closed. - Noted and closed. - Noted and closed. - Noted and closed. 4-Noted and closed. |  |
|  | Access for internal loading/unloading | Manhole / hand hole Access for internal loading/unloading of drums | Diameter of air dryer tower is so small to install of manhole/ hand hole. Therefore, there is not any opening such as handhole or manhole on towers. Body flange will not be considered, Two connections for charging and discharging molecular sieve at the top and bottom of the tower are considered. For nitrogen generator towers, handhole 8" will be considered. Also, manhole 18" will be considered for Air and N2 Receivers which is sufficient since there is no internal inside of receiver. | Noted and closed. |  |
|  | Nozzle WRC | Local stress analysis of shells under external loads from piping system and attachments (platforms, pipe supports, davit, lifting lug) shall be included in strength calculation. And also, nozzle load shall be considered according to Allowable Nozzle loads on static Equipment (as per project specification) should be done. Please note that wherever WRC is not applicable FEA shall be done to evaluate local stresses. The original file (native file) of all analysis including all input data and results should be provided by Vendor. Please confirm. | Confirmed, client external nozzle load will be followed at skid battery limit. Nozzle load will not be considered for inter skid of compressor, chiller, dryers, N2 generator, and filters and water trap. | Noted and closed. |  |
|  | Loading Data & Anchor Bolt Data | Anchor bolt data (Size, number, B.C.D., material,) and final loading data including erection, operating and hydro tests shear and moment due to wind and earthquake shall be provided by vendor 1 month after purchase order. Please confirm. | Confirmed. | Noted and closed. |  |
|  | PWHT | When stress relieving is required, whole equipment in one piece without removable internals shall be located in the Furnace. All surfaces of flanges shall be protected by vendor due to prevent of damage. Use of direct flame is not approved. | PWHT is not applicable le in this package. | Noted and closed. |  |
|  | Corrosion allowance | For all parts made of carbon steel, corrosion allowance of 3.2 mm to be considered by vendor. In addition, corrosion allowance shall be in accordance with the project "Corrosion Study & Material Selection Report" attached to MR. This requirement shall be followed by vendor. | As per this document clause 10.3.4 C.A of 3 mm shall be considered for C.S parts and this value is confirmed., except inter skid of equipment which C.A. is based on manufacturer design. Filters and water trap is based on their manufacturer standard (Omega company) which is aluminum and C.A. is zero. | Noted and closed. |  |
|  | Referenced documents | Referenced documents, specifications and STDs specified in project MR shall be fully followed by vendor. In addition, for major components within the package, following codes and standards (Latest Editions) shall be considered by vendor: 1- Pressure Vessel Design: ASME BPVC VIII Div. 1 2- Materials: ASME BPVC Sec. II, ASTM 4- Nozzle and External Load Design: WRC 537 5- Wind: ASCE 7 6- Seismic: ASCE 7 | Confirmed, by considering deviation list. 1~6: Confirmed. | Noted and Vendor’s deviation list is not accepted. | Meeting conclusion-05.08.2023: Please refer to agreed deviation list. |
|  | Material | Material ASTM No. of all package parts including grade and designation shall be specified in equipment datasheets. Furthermore, MDMT, design temp. and pressures shall be specified as well as requirement of impact test and heat treatment. | All material will be based on ASME, unless otherwise Equivalent material will be submitted after contract. MDMT, design temp. and pressures has been specified in relevant equipment data sheet. | Noted and closed. |  |
|  | Plates | Sliding plates, templates, liners, insert plate & shims (whichever is applicable) are in vendor’s scope of supply. | Not applicable. Installation works are out of HAVAYAR scope of supply and work. | Sliding plates, templates, liners, insert plate & shims (whichever is applicable) are in vendor’s scope of supply. | Meeting conclusion-05.08.2023: Sliding plate and shims will be not prepared by the vendor. Closed. |
|  | Piping Support | All internal piping, valves and fittings with their supports for cooling, sealing, purging, vent, drain and lubrication shall be provided and installed by Vendor. Any auxiliary connection inside B.L shall be flanged type and extended up to the skid's boundary. | Confirmed for inter-skid of equipment with considering deviation list. | Noted and Vendor’s deviation list is not accepted. | Meeting conclusion-05.08.2023: Please refer to agreed deviation list. |
|  | Walk way | In order to provide access to all parts of package, Walkways, platforms, ladders & handrails shall be considered by Vendor | Ladder and platform are not applicable due to the small size. | Noted and closed. | Meeting conclusion-05.08.2023: Closed. |
|  | Seismic/Wind parameters | Vendor shall follow project specifications for Seismic/Wind parameters. Below data shall be considered as minimum requirements: - For seismic: - Code: ASCE 7-10 - Importance Factor: 1. 25 - Site Class: D - Ss = 0.781 - S1 = 1.25 - Fa = 1 - Fv =1.5 - Design wind speed: 120 km/h @10m - Wind Exposure: C | Confirmed. | Noted and closed. | Meeting conclusion-05.08.2023: Closed. |
|  | Grid plate and Grating | The floor shall be covered with grid plate which is suitably supported to prevent excessive deflection. Checkered plate for walkway or other places that end user may use, is not acceptable. Hot dip galvanized grating with four clips per each piece of grating shall be considered for bottom of all walkways. | Not applicable in this package. | Noted and closed. |  |
|  | Drain nozzle height | Drain valve height shall have enough space for operation and maintenance. | Confirmed. | Noted and closed. |  |
|  | Passages and all open surfaces | Passages and all open surfaces around the equipment shall be covered with gratings. All the grating penetrations (piping, cables, etc.) shall be bordered with a flat bar which will act as a foot guard. | Not applicable in this package. Inter-skids design is as per manufacturer design and standard. | Noted and closed. |  |
|  | Commissioning & Start-up | All items required for commissioning & start-up inside the package shall be supplied by vendor. Necessary quantities of catalysts, cartridges, chemicals, desiccant, molecular sieve, etc., whichever is applicable and required for start-up and initial operation of the package shall be supplied by vendor. | Confirmed. | Noted and closed. |  |
|  | P&IDs | Purchaser P&IDs to be fully reviewed by vendor and if any extra provisions for control and reliable operation is needed, to be announced by vendor ay bid stage. | All requirement has been shown in Havayar submitted PID and any deviation from Purchaser P&IDs has been specified in deviation list. | Noted and closed. |  |
|  | Shop-assembled | The unit shall be shop-assembled to the maximum extent practical to minimize on-site erection work. All skids and equipment that mate directly (without interconnecting piping by the purchaser) shall be trial fit in the vendor’s shop. The vendor shall identify all loose-shipped components requiring field installation in the proposal. | Be noted that the whole package will not be delivered as one skid, each compressor, Nitrogen generator, dryer and chiller has its own skid, Filters and water trap will be installed on piping, Air and N2 buffer will be installed on foundation by leg, number of skids inside package with dimension and weight will be specified in preliminary general arrangement and will be submitted in the next revision of technical proposal. | The unit shall be shop-assembled to the maximum extent practical to minimize on-site erection work. All skids and equipment that mate directly (without interconnecting piping by the purchaser) shall be trial fit in the vendor’s shop. The vendor shall identify all loose-shipped components requiring field installation in the proposal. | Meeting conclusion-05.08.2023: Vendor Confirmed. Closed. |
|  | pressure vessels Specification | All pressure vessels inside the package shall comply with requirements explained in Specification for Welded Pressure Vessel BK-GNRAL-PEDCO-000-ME-SP-0001 attached to material requisition. | Confirmed by considering deviation list. | Noted and Vendor’s deviation list is not accepted. | Meeting conclusion-05.08.2023: Please refer to agreed deviation list. |
|  | Nozzle Flange | All nozzles shall be flanged with Welding neck flanges. For nozzle sizes smaller than 2”, long-welding-neck flanges are required. | Confirmed, connection of filters and water trap is based on manufacturer standard which is Threaded. | Noted and Threaded.is not applicable for pressure vessel. | Meeting conclusion-05.08.2023: Vendor Confirmed. Closed. |
|  | Standard Drawings | All details of pressure vessels designed according to ASME BPVC VIII shall follow Standard Drawings for Fixed Equipment attached to this file. | Standard drawing will only be followed for air, N2 receiver, and heat exchanger. Towers of dryer and N2 generator design is based on HAVAYAR design. | For every pressure vessel shall be followed. | Meeting conclusion-05.08.2023: Please refer to agreed deviation list. |
|  | Vessel Support type | Vessels with diameters (OD) and weights greater than 1000 mm and 1200 kg, respectively; are not allowed to be supported by legs. | Please be noted that all vessels will be supported by leg, due to the small size of vessels and vendor design requirement. | Noted and closed. |  |
|  | Lifting Lug | The skid shall be provided with lifting lugs for a four-point lift. Lifting and transport of the skid, complete with all equipment mounted, shall not permanently distort or otherwise damage the skid or the machinery mounting surfaces. | Confirmed. | Noted and closed. |  |
|  | Overall plot plan | A preliminary overall plot plan shall be added to the next revision by bidder to show approximate arrangement and dimensions of the package. | Confirmed, will be submitted with the next revision of technical proposal. | Noted and closed. |  |
|  | Comments on Vendor’s Proposal and its attachments | Please refer to the Attachment #1 of this TCL for Mechanical comments on the offer. | Please see reply to commented offer. | Noted and closed. |  |
| PIPING & MATERIAL |  |  |  |  |  |
|  | PMS | Project PMS shall be followed by vendor. | Please be noted that project PMS is not applicable for piping inside of the air compressor skid, chiller, air dryer skid and nitrogen generator skid. Piping inside of skid will be based on manufacturing standard. PMS will only be followed at B.L. of skids. Be noted that piping between skids/Equipment will be supplied as loose item by Havayar, installation and pipe work is out of Havayar scope. Please be noted that valves including ball valves and check valves inside the air dryer skid and also VIP valves and angle seat valves inside the N2 generator skid will be selected based on HAVAYAR's Know-How and valve spec and PMS can't be followed. Valve material of instrument air has been considered same as plant air (piping class AN04). | Please be notice that according to PMS instrument air<4” piping class is AZ00 and nitrogen piping class is AN01. | Meeting conclusion-05.08.2023: PMS is Confirm to perform HY reply: PMS will be followed for between skid piping. |
|  | General arrangement drawing | Air & N2 Package General arrangement drawing shall be issued by vendor. | Confirmed, will be submitted with the next revision of technical proposal. | CLOSED | Meeting conclusion-05.08.2023: Vendor Confirmed. Closed. |
|  | Equipment Catalogue | Catalogue of All equipment shall be issued by vendor. | Confirmed, will be submitted with the next revision of technical proposal. | CLOSED | Meeting conclusion-05.08.2023: Vendor Confirmed. Closed. |
|  | 3D model | PDMS 3D model shall be issued by vendor. |  |  | Meeting conclusion-05.08.2023: Vendor Confirmed. Closed. HY reply: 3D model for compressor skid and chiller not applicable and will be shown as a box. For dryer and N2 generator STP file will be submitted. |
|  | VPIS | VPIS will be finalized after P.O. |  |  | Meeting conclusion-05.08.2023: Attachment #2 of relevant MR will be checked, singed and stamped by the vendor and if there is any deviation will be specified. Closed. |
|  | PLOT PLAN | Please confirm arrangement of packages on plot plan drawing |  |  | Meeting conclusion-05.08.2023: Vendor Confirmed. Closed. |
|  | New Item |  |  | Internal Epoxy coating is required for Air and Nitrogen Receivers. | Confirmed |
|  | New Item |  |  | Corrosion allowance shall be in accordance with data sheets | Confirmed, 3.2mm has been considered. |
|  | New Item |  |  | “Specification for the design of piping in mechanical package”, document No.: BK-GNRAL_PEDCO-000-PI-SP-0003 shall be met for piping design. | Confirmed for between skid piping. For inter skid, manufacturer std. will be considered. Also, Please refer to our agreement on item 1 of TCL, in PIPING & MATERIAL Sec. |
| INSTRUMENT & CONTROL |  |  |  |  |  |
|  | Control System Requirements | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0004 and provide the requirement of type C package for nitrogen package, air compressor and sufficient controller for air dryer in field as mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. As minimum: 1-UCP with redundant CPU/PS dedicated for each train to be provided by vendor. 2-SIL rated loops for protection and trip to be merged inside UCP panel for each train. (redundancy for IO is required) 3- Monitoring of machine protection signals could be also merged inside UCP panel due to vendor design (required transmitters/transducer for temperature/vibration and etc. sensors to be considered for UCP connection otherwise separate panel, at least VMS to be provided. ) | N2 and Air package compressors are non-SIL and we don’t have SIL rated loop for these packages. Please be noted that MMS & Machinery sensors are not applicable for this type of compressor. Air package: Compressors and dryers have a LCP and a microprocessor based controller (without redundancy) as per manufacturer design. Since arrangement of compressors and dryers is 1+1 (one duty & one standby) therefore UCP is not required for this package. If UCP is mandatory we can consider a UCP based on PLC for Air package but please uniform PLC model, UCP characteristics and quantity between all bidders and be noted that we cannot omit the local microprocessor of compressor. N2 package: Compressor, dryer and chiller have a LCP and a microprocessor based controller (without redundancy) also, N2 generator has a LCP based on PLC S7-1200 (without redundancy) as per manufacturer design. Since all skid are unique and each skid has LCP therefore UCP is not required. If UCP is mandatory we can consider a UCP based on PLC for N2 package but please uniform PLC model, UCP characteristics and quantity between all bidders and be noted that we cannot omit the local microprocessor of compressor and chiller. | As per project specification, PLC based UCP (for control and safety) dedicated to each compressor is requested for both package (N2 and air): -One PLC based UCP for each compressor (totally 2.Nos. located on control room) for Air compressor package (type C). one of UCP shall be considered as master. -One PLC based UCP for nitrogen package (located on control room) (type C). -Redundancy for /power supply /critical and safety I/Os and communication link to be considered in each UCP. Separate SIL rated system for ESD signals (could be relay based ESD part inside UCP) shall be considered in parallel as safety back up logic. - LCP is required for each compressors for on/off and commissioning/startup and gauge boarding. -Project Spec to be respected for packages type C. -Microprocessor based LCP is required for each dryer. -Regarding Machine monitoring at list minimum requirements as per API to be respected (vibration for trip…) | Meeting conclusion-05.08.2023: Noted and closed. Control Configuration Block diagram shall be provided by vendor in Bid stage. HY Reply: SIL for UCP (Safety and ESD Loop) will be considered. Gauge boarding and MMS & Machinery sensors are not applicable for this type of compressor. |
|  | Instrumentation | Regarding supplying instrument on datasheet), bidder is requested to consider all instruments mentioned on project datasheet. VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0001 for instrumentation devices and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed with considering deviation list. | OPEN-Final deviation list to be provided or Spec shall be confirmed by vendor if there is no deviation. | Meeting conclusion-05.08.2023: Confirmed except for machineries (compressor and chillers) which shall be added in deviation list. |
|  | Instrumentation Valves | VENDOR shall follow below documents for valves and other documents as mentioned in these documents. - BK-GNRAL-PEDCO-000-IN-SP-0005 - BK-GNRAL-PEDCO-000-IN-SP-0006 - BK-GNRAL-PEDCO-000-IN-SP-0007 VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | Confirmed with considering deviation list. | OPEN-Final deviation list to be provided or Spec shall be confirmed by vendor if there is no deviation. | Meeting conclusion-05.08.2023: Confirmed for between skids (special items which shall be added in deviation list) |
|  | Connection with MCC | If MCC is supplied by VENDOR all interfaces shall be supplied by VENDOR. Below signals are typically defined in this project to be provided for electrical interfaces: - Start command (2 sec high pulse) - Stop command (2 sec high pulse) - Trip command (low signal for trip) - Run/Stop Status (high signal for run status) - Local/Remote status (high signal for local status) - Tripped on Fault (high signal for fault status) Available for remote control (high signal for available status) | Since motor is LV and starter of motor is by HAVAYAR (inside LCP) therefore we don’t have any signal to/from client MCC. | Since MCC is supplied by VENDOR : Interface signals between MCC and UCP (VIA IRP) and LCP shall be considered in vendor scope. Below signals are typically defined in this project: - Start command - Stop command - Trip command - Run/Stop Status - Local/Remote status Fault status | Meeting conclusion-05.08.2023: Noted and closed. HY Reply: There is no signal to/from client MCC and cabling between UCP and LCP/JB is out of Havayar’s Scope. |
|  | Cable | VENDOR shall follow BK-GNRAL-PEDCO-000-IN-SP-0010 for cables and all other details mentioned in this document. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | On skid cable is as per manufacture standard. PVC, unarmored. Between skid cable is out of HAVAYAR scope. | OPEN-Project spec to be followed for cabling inside package battery limit(between skid) . All instrument signals on package skid shall be terminated on JBs by vendor. | Meeting conclusion-05.08.2023: Confirmed. Closed |
|  | Terminals | All terminals shall be screw type. VENDOR shall note that each core/pair/etc. of cable shall be wired to dedicated terminal | Confirmed. | Closed. |  |
|  | Panel Dimension | VENDOR shall specify the estimated number and dimension of panels. | Each skid of equipment has a LCP and already we don’t have UCP. Please specify which panel estimated dimension is required? | OPEN-To be updated as per project request in item no. 1 | Meeting conclusion-05.08.2023: Confirmed. Closed |
|  | Required Power | VENDOR shall specify the estimated power required (non-UPS and UPS) | Confirmed. Please refer to HAVAYAR Load list. All power consumption (non-UPS and UPS) has been specified in load list. | OPEN-To be updated as per project request in item no. 1 | Meeting conclusion-05.08.2023: Confirmed. Will be added in new load list. Closed |
|  | Special Tools | VENDOR shall supply all special tools for compressor and axillary devices. This is included the special cables (not mentioned in BK-GNRAL-PEDCO-000-IN-SP-0010), MODBUS sockets, software and etc. The list shall be provided separately. | Special cable (if any) will be supplied by HAVAYAR. We don’t have any other special tools. | Closed. |  |
|  | Protection Instruments | The protective instruments shall be according to API-670. VENDOR shall comply with all the requirements and mentioned any mismatch in deviation list which is attached to the PMR. | API-670 is not applicable for this type of compressor. | As per project specification: 5.4.6.2 (Mod.) Based on the vendor's experience and recommendation, non‐contacting vibration and axial transducers shall be supplied, installed and calibrated in accordance with API 670. 5.4.6.4 (Mod.) Based on the vendor's experience and recommendation, vibration and axial position monitors shall be supplied and calibrated in accordance with API 670. | Meeting conclusion-05.08.2023: Project Spec is requested by contractor. It will be followed by vendor if applicable HY reply: Not applicable for air and N2 package. |
|  | Software | VENDOR shall supply all the software (including the passwords, licenses and program sources). The programs shall be supplied with all tags, descriptions and etc. The software programs shall be installed on engineering laptops and also supplied on CD/DVD. | Original software and license are not applicable for local microprocessors and also PLC S7-1200. Laptop is not required already. | Not accepted- To be updated as per project request in item no. 1. Lap top and software licenses shall be provided by vendor | Meeting conclusion-05.08.2023: Confirmed. Closed |
|  | Sequence controller | VENDOR shall clearly specify the scenario for sequence controlling of compressor and dryer also nitrogen package,t he interface signals, required hardware (e.g. panels, if any) and additional control or on/off valves. | For Air package Compressors and dryers are 1+1 (one duty & one standby) therefore sequencer panel for compressors is not required and sequence is by client DCS. Dryer valves sequence (for each skid) is fully automatic and by local controller. All signals will be as per HAVAYAR submitted P&ID. For nitrogen generator valves sequence is fully automatic by LCP (PLC S7-1200). | Not accepted- To be updated as per project request in item no. 1 | Meeting conclusion-05.08.2023: Confirmed after Po will be submitted. Closed |
|  | Start-up | VENDOR shall clearly state the start-up sequence. It shall be mentioned if any additional instrument or valve is required. Also, the interface signals shall be mentioned (if any) | There is no need to additional instrument or signal for start-up sequence. All required instrument and signals have been specified in HAVAYAR P&ID. | OPEN-To be updated as per project request in item no. 1 | Meeting conclusion-05.08.2023: Confirmed as per Project P&ID. Closed |
|  | Spare part | VENDOR shall supply all commissioning/ 2 years spare part as per NISOC procedure, attached to PMR. | Noted. Please refer to submitted spare part list, all applicable items have been specified. | OPEN_ be updated as per project request in item no. 1 | Meeting conclusion-05.08.2023: Confirmed. Closed |
|  |  |  |  | Project Instrument Hook UP drawing (BK-GCS-PEDCO-120-IN-DG-0002), Shall be Followed by vendor | Meeting conclusion-05.08.2023: Noted except items specified on deviation list |
|  |  |  |  | Analyzers as minimum as per project P&ID for Air and nitrogen package is required | Meeting conclusion-05.08.2023: Noted. Closed. |
| SAFETY |  |  |  |  |  |
|  | Noise Level | Noise levels of items should not exceed 85 dbA @ 1m. | Confirmed. | Closed | Meeting conclusion-05.08.2023: Noted. Closed. |
| ELECTRICAL |  |  |  |  |  |
|  | Specification | All electrical equipment shall comply with electrical references & standard that mentioned on material requisition. IPS-E-EL-100 (1), IPS-M-EL-131 (2) & IPS-M-PM-290(2). | Electrical deviations from client specifications have been submitted previously. Please be noted that LCP on skid of compressor and chiller is as per manufacturer design. If there is any special item from IPS or other standard, please specify that. | Not Accepted. Shall be clarify in MOM. In addition mentioned standards have been attached | Meeting conclusion-05.08.2023: For Motor IPS-M-EL-131 (2) will be followed by the vendor. LCP (distribution Panel) is based on manufacturer STD. |
|  | Vendor Scope of Work | All documents related to electrical equipment shall be submitted by vendor such as preliminary load list, electrical equipment list, motor data sheet, motor outline drawing & etc. | Preliminary load list has been submitted previously. electrical equipment list, motor data sheet, motor outline drawing & etc. will be submitted after P.O. | Closed |  |
|  | Vendor Scope of Work | Vendor list & sub vendor list of manufacture shall be according to project AVL. | HAVAYAR sub-vendor list has been submitted previously. Please refer to HAVAYAR sub vendor list and let us to know your comments. | Closed |  |
|  | Vendor Scope of Work | The cable from switchgear to package panel is by client, but the cable in the package form panel to each equipment is by vendor. In other word, from client point of view, there is only one load which assumed the power demand of package is the total load. This note is valid for both nitrogen & air package. (Tie in between client & vendor is main panel). | All required feeder is as per HAVAYAR load list. Please be noted that already we don’t have separate distribution panel for N2/Air package. All cable inter skid of equipment is by HAVAYAR. Battery limit for electrical cable is at the edge of LCPs. | Not accepted. The cable & power from switchgear (400V, 3Phase & 50Hz), to main package panel (in field) is by contractor, but the cable in the package from main panel to each equipment (in skid) is by vendor. In other word, from contractor point of view, there are 3 below loads: - Instrument & Plant Air Package (PK-C-2203A) - Instrument & Plant Air Package (PK-C-2203B) - Nitrogen Package (PK-2204) (Tie in between contractor & vendor is main panel which panel will be provided by vendor). Therefore the total power demand shall be specified by vendor. | Meeting conclusion-05.08.2023: - Instrument & Plant Air Package (PK-C-2203A) - Instrument & Plant Air Package (PK-C-2203B) - Nitrogen Package (PK-2204) Tie in between contractor & vendor is main panel which panel will be provided by vendor. Closed. HY reply: Interface with Client’s Cable is LCP at edge of the skid compressor and chiller. as per HAVAYAR load list. |
|  | Vendor Scope of Work | According to item 4, the total electrical load is required not each one separately. | Please refer to our reply on item 4. | Not accepted. Please refer to above item | Meeting conclusion-05.08.2023: Refer to item 4. Acceptable. Closed. |
|  | Vendor Scope of Work | Since the power & control cables in the skid is by vendor, therefore the cable list in the package shall be according to “BK-GNRAL-PEDCO-000-EL-SP-0014”. Interconnection in the skid is by vendor. | On skid cable is as per manufacturer standard. PVC, unarmoured. | Not accepted. | Meeting conclusion-05.08.2023: Cable shall be according to project specification between skid. Closed. |
|  | Vendor Scope of Work | Supplier shall design the routing of the power & control cable inside package. | Routing of the power & control cable only on skid is by HAVAYAR. | Closed |  |
|  | Vendor Scope of Work | If AC UPS or DC UPS & batteries are required, vendor shall provide all necessary documents (such as calculation, list etc.) & supply required equipment according to purchaser standards. | UPS or DC UPS & batteries are out of HAVAYAR scope and we only specify consumption in load list if any. | Closed |  |
|  | Spare Part | A complete set of spare parts for commissioning shall be supplied for each motor. | Commissioning spare part for motor is not required. | Closed |  |
|  | Test & Inspection | The equipment shall be factory tested. Certified copies of test reports shall be submitted to the purchaser. | Noted. Please specify your required certificates (if any). ITP items will be finalized in PIM. | Closed |  |
|  | Vendor Scope of Work | Data sheet & Specification of LV induction motor have been attached. Please follow them. | Confirmed with considering deviation list. | Deviation list is not accepted | Meeting conclusion-05.08.2023: Please refer to agreed deviation list. |
|  | Vendor Scope of Work | Deviation & clarification form shall be checked by vendor. Please refer to Attachment#3 of this TCL for electrical comment on vendor’s deviation list. | Noted. Please refer to our replies on Attachment#3. | Closed |  |
|  | Specification | All electrical equipment shall comply with electrical references & standard that mentioned on material requisition. IPS-E-EL-100 (1), IPS-M-EL-131 (2) & IPS-M-PM-290(2). | Electrical deviations from client specifications have been submitted previously. Please be noted that LCP on skid of compressor and chiller is as per manufacturer design. If there is any special item from IPS or other standard, please specify that. | Closed |  |
|  | Vendor Scope of Work | The engineering & procurement of package shall be according to “BK-GNRAL-PEDCO-000-EL-SP-0011”. |  |  | Meeting conclusion-05.08.2023: Vendor Confirmed for between skid items. Closed. |
|  | Vendor Scope of Work | Data sheet & Specification of LV induction motor have been attached. Please follow them. |  |  | Meeting conclusion-05.08.2023: Attached data sheet will be filled in by the vendor. Specification will be followed by the Vendor accept specified on deviation list. Closed. |
|  | Vendor Scope of Work | The method of start & stop of air compressor & interlock with other equipment shall be specified. |  |  | Meeting conclusion-05.08.2023: Will be specified after P.O. by the Vendor. |
|  | Vendor Scope of Work | The interlock between main motor & auxiliary loads of N2 package shall be specified. |  |  | Meeting conclusion-05.08.2023: Will be specified after P.O. by the Vendor. |
|  | Vendor Scope of Work | I/O List shall be issued by vendor. |  |  | Meeting conclusion-05.08.2023: Vendor confirmed. After P.O. Closed. |
| STRUCTURE |  |  |  |  |  |
|  | Earthquake load | Use ASCE7-10 for earthquake load | Confirmed. | Closed | Meeting conclusion-05.08.2023: Noted. Closed. |
|  | Anchor bolt | The type & location of the anchor bolt should be provided. | Confirmed., will be specified in detail drawing after P.O. | Closed | Meeting conclusion-05.08.2023: Noted. Closed. |
|  | Skid | The exact dimensions & size of the skid and the proper alignment under the skid should be determined. | Confirmed., will be specified in detail drawing after P.O. Installation and alignment is out of HAVAYAR scope. | Closed | Meeting conclusion-05.08.2023: Noted. Closed. |
|  | Loads and center mass | The static & dynamic loads of the equipment and the level of the center mass the device should be determined. | Static load including Earthquake load will be submitted according to client site condition based on manufacturer calculation. Required input data for foundation design will be submitted by HAVAYAR after P.O. | Closed | Meeting conclusion-05.08.2023: Noted. Closed. |

| # | Clarification Description NISOC Letter 1403/06/27 03/2294/242202 | Conclusion (Rev.00) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description (Rev.01) | Conclusion (Rev.01) (VENDOR to Confirm & Consider in the Final Offers) |
| --- | --- | --- | --- | --- |
| مکانیک |  |  |  |  |
| 1. | استاندارد طراحی کمپرسورها، ISO-10440-2 میباشد، سازندگان میبایست ضمن تایید این مورد، انحرافات Major خود با این استاندارد را بطور مشخص اعلام نمایند. | قابل قبول نمی باشد و کمپرسور بر اساس استاندارد سازنده طراحی شده است. |  |  |
| 2. | جدول تغییرات دما و فشار با دبی برای کمپرسور هوا و نیتروژن از سوی سازندگان ارائه گردد. | منحنی عملکرد کمپرسورهای واحد هوا و نیتروژن ارسال خواهد شد. |  |  |
| 4. | کلیه سازندگان دست دوم کالا از جمله سازنده Air End و فیلتر روغن میبایست از لیست بلند وزارت نفت انتخاب گردند. | در این خصوص به Sub vendor list ارائه شده رجوع شود. |  |  |
| 5. | کالای یدکی میبایست بر اساس پیشنهاد سازنده و با در نظر گرفتن حداقلهای مندرج در پیوست 11 اسناد پیمان در نظر گرفته شود. | مورد تائید است |  |  |
| 6. | در پیشنهاد فنی سازنده تعداد درایرها 2 عدد در نظر گرفته شده است که میبایست 2 عدد 2 قلو باشند. | مورد تائید است. در پیشنهاد فنی ارائه شده نیز این این موضوع در نظر گرفته شده بود. |  |  |
| 7. | ظرفیت کمپرسور هوا با محاسبات و داده های فرآیندی پروژه مغایرت دارد. | مغایرتی وجود نداشته و تمامی محاسبات بر اساس مدارک کارفرما می باشد. در نظر داشته باشید بر اساس داده های ورودی پروژه ظرفیت پکیج هوا (خروجی درایر) معادل 119.7 Nm3/hr در نظر گرفته شده است و ظرفیت PA معادل 30 Nm3/hr. بر اساس محاسبات وندور ظرفیت کمپرسور مورد نیاز معادل 170.8 Nm3/hr می باشد. در نظر داشته باشید بر اساس داده های ورودی پروژه 10% over design در طراحی درایر و کمپرسور لحاظ می گردد. |  |  |
| 8. | ظرفیت کمپرسور نیتروژن با محاسبات و داده های فرآیندی پروژه مغایرت دارد. همچنین عنوان کمپرسور به اشتباه هوا درج شده که میبایست اصلاح گردد. | مغایرتی وجود نداشته و تمامی محاسبات بر اساس مدارک کارفرما می باشد. در نظر داشته باشید بر اساس داده های ورودی پروژه ظرفیت پکیج نیتروژن معادل 34 Nm3/hr در نظر گرفته شده است و بر اساس محاسبات وندور ظرفیت کمپرسور مورد نیاز معادل 194 Nm3/hr می باشد. |  |  |
| برق |  |  |  |  |
| 1. | آیتم های مرتبط با تستهای کارخانه ای از دیدگاه الکتریکال ناقص است لذا ضمن تامین /تکمیل دستگاه های تست الکتریکال، در خصوص آیتم های ITP بر اساس ملاحظات تست کارخانه ای هر تجهیز متعاقبا اظهار نظر خواهد شد. | ITP در جلسه PIM نهایی خواهد شد. |  |  |
| 2. | سیستم حفاظت الکتریکی فارغ از حفاظت های پیش فرض مربوط به راه انداز نرم الکتروموتورها، باید در نظر گرفته شود. | سیستم های حفاظت الکتریکی مطابق مدارک ارسال شده و بر اساس طراحی و استاندارد طراح می باشد. در نظر داشته باشید راه اندازی موتورها ستاره مثلث می باشد. |  |  |
| 3. | آیتم 2-3 Instrument Plant Air TBE مربوط به Grounding Bosses جهت هر پکیج بصورت مجزا ضروری است . | مورد تاید است |  |  |
| 4. | چنانچه پکیج ها به صورت آماده(وارداتی) تامین خواهد گردید نسبت به ارائه مدارک کامل اقدام شود، بدیهی است تجهیزات پروژه باید بر اساس نیازمندیهای خاص پروژه تامین گردند. | کلیه پکیج ها در کارخانه هوایار ساخنه خواهد شد و منطبق بر نیازمندی های پروژه و مدارک فنی ارائه شده تسط هوایار میباشد |  |  |
| 5. | تعداد ژنراتور نیتروژن با اسناد مهندسی تفضیلی مغایرت دارد.. | بر اساس مدارک ورودی پروژه یک عدد نیتروژن ساز برای پکیج نیتروژن در نظر گرفته شده است. |  |  |
| 6. | دلیل نحوه راه اندازی الکتروموتور چیلر 4 کیلووات بصورت ستاره مثلث چیست؟ | بر اساس طراحی وندور مربوط به چیلر در نظر گرفته شده است و مغایرتی با استانداردهای موجود ندارد. |  |  |
| 7. | ITP فاقد تست های کارخانه تجهیزات الکتریکال بوده لذا در این خصوص و بر اساس مفاد پیمان متعاقبا اظهار نظر خواهد شد. | ITP در جلسه PIM نهایی خواهد شد. |  |  |
| 8. | با توجه به اظهار بدون آرمور بودن کابل های درون پکیج در لیست انحرافات، جهت حفاظت مکانیکال کابلها جه تمهیداتی اندیشیده شده است. | با توجه به اینکه موقعیت پکیج under shelter می باشد و همچنین کمپرسور ها داخل کابینت قرار گرفته اند، کابل Unarmored در نظر گرفته شده است. |  |  |
| 9. | لیست انحرافات: درجه حفاظت IP تجهیزات الکتریکی (تابلو و ...) باید بر اساس ملاحظات استاندارد IPS انتخاب گردد و در غیر اینصورت مورد پذیرش نیست. | IP تجهیزات الکتریکی مطابق مدارک ارسال شده می باشد و غیر قابل تغییر می باشد. |  |  |
| ابزار دقیق |  |  |  |  |
| 1. | SUB-VENDORهای معرفی شده بخصوص در بخش برند سیستم کنترل و بستن کنترل پنل، لازم است از لیست بلند وزارت نفت باشند. | در خصوص panel integrator امکان تغییر وندور مربوطه وجود ندارد و تمامی پروژه های موجود در شرکت هوایار توسط وندور مشخص شده در sub vendor list انجام می گیرد. |  |  |
| 2. | در مورد پایش عملکرد ماشین MMS الزامات SPEC و استاندارد API 670 رعایت گردد. در نظر گرفتن سنسور لرزش مطابق P&ID و SPEC الزامی است. | MMS برای این تایپ کمپرسور امکان پذیر نمی باشد. |  |  |
| 3. | تمامی نرم افزار های مورد نیاز باید بصورت اصلی و مطابق SPEC پروژه ارائه گردد. | Original software برای PLC ارائه خواهد شد |  |  |
| 4. | در ITP ارائه شده از سازندگان، در نظر گرفتن موارد بازرسی سیستم کنترل و پنل کنترل محلی و تست های FAT و SAT آنها دیده شود. | LCP به صورت جداگانه تست نخواهد شد و همراه با کمپرسور تست functional انجام خواهد گردید. FAT برای UCP انجام خواهد شد. |  |  |

| # | Clarification Description PEDCO Letter 1403/05/14 BK-SUR-ENG-MEC-03-09433 | Conclusion (Rev.00) (VENDOR to Confirm & Consider in the Final Offers) | Clarification Description (Rev.01) | Conclusion (Rev.01) (VENDOR to Confirm & Consider in the Final Offers) |
| --- | --- | --- | --- | --- |
| Mechanical Comments : |  |  |  |  |
| 1. | صرفا Deviation List هر سازنده مورد بررسي قرار مي گيرد و هيجگونه انحراف از مدارك بسته خريد كه در Deviation list مطرح نگرديده قابل بررسي نبوده و مورد پذيرش نخواهد بود و مسئوليت كامل تطابق كالا با مدارك بسته خريد بر عهده آن سازنده مي باشد. | مدرکDeviation List نهایی نشده است. لذا نیاز است موارد اعلام شده مورد توافق طرفین قرار گیرد و نهایی شود. |  |  |
| 2. | مطابق Spec و Duty spec پروژه فيلتر ورودي هوا مي بايست auto cleaning type باشد و توضيح سازنده مورد پذيرش نمي باشد. | مورد قبول نمیباشد. فیلتر از نوعauto cleaning type برای این نوع کمپرسور (Screw Type) کاربردی نیست. فیلتر مورد استفاده در این نوع از کمپرسور به صورت کارتریجی است و در زمان خاموش شدن کمپرسور قابل تعویض خواهد بود. |  |  |
| Process comments : |  |  |  |  |
| 1. | تمامی تجهیزات مورد نیاز پکیج های هوا و نیتروژن از قبیل بافرها، فیلترها و ... در شرح کار سازنده می باشد بجز V-2203 و V-2204 که موقعیت آنها در پایین دست پکیج طراحی شده است و قابلیت استفاده به عنوان نوسان گیر کمپرسور یا Surge Drum را ندارد. | مورد تایید است در خصوص اسکوپ کاری به مدرک PID ارسالی هوایار برای پکیج های هوا و نیتروژن رجوع شود. |  |  |
| Instrument comments : |  |  |  |  |
| 1. | برای تمام ادوات ابزاردقیق برای شرکت هوایار برای تجهیزات و پنل های ابزاردقیقی در فیلد می بایست 65 باشد. | تمامی تجهیزات IP65 میباشد به جز O2 ترنسمیتر که داخلJB با IP65 قرار گرفته است و پنل ها مطابق مدارک ارسال شده میباشد |  |  |
| 2. | P&ID می بایست در طول مهندسی تفصیلی توسط سازنده تکمیل گردد و موارد درخواست کارفرما و ذینفع اصلی مرتفع گردد. | مورد تاید میباشد |  |  |