| طرح نگهداشت و افزایش تولید 27 مخزن |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CALCULATION NOTE FOR LEAN GLYCOL STORAGE TANK(TK-2102) نگهداشت و افزایش تولید میدان نفتی بینک |  |  |  |  |  |  |  |
| D00 |  | JAN. 2024 | IFC | R.Berlouie | M.Fakharian | S.Faramarzpour |  |
| Rev. |  | Date | Purpose of Issue/Status | Prepared by: | Checked by: | Approved by: | CLIENT Approval |
| Class:2 |  |  | COMPANY Doc. Number: F0Z-709135 |  |  |  |  |
| Status: |  | IDC: Inter-Discipline Check IFC: Issued For Comment IFA: Issued For Approval AFD: Approved For Design AFC: Approved For Construction AFP: Approved For Purchase AFQ: Approved For Quotation IFI: Issued For Information AB-R: As-Built for CLIENT Review AB-A: As-Built –Approved |  |  |  |  |  |

REVISION RECORD SHEET

| PAGE | D00 | D01 | D02 | D03 | D04 |  | PAGE | D00 | D01 | D02 | D03 | D04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X |  |  |  |  |  | 66 |  |  |  |  |  |
| 2 | X |  |  |  |  |  | 67 |  |  |  |  |  |
| 3 | X |  |  |  |  |  | 68 |  |  |  |  |  |
| 4 | X |  |  |  |  |  | 69 |  |  |  |  |  |
| 5 | X |  |  |  |  |  | 70 |  |  |  |  |  |
| 6 | X |  |  |  |  |  | 71 |  |  |  |  |  |
| 7 | X |  |  |  |  |  | 72 |  |  |  |  |  |
| 8 | X |  |  |  |  |  | 73 |  |  |  |  |  |
| 9 | X |  |  |  |  |  | 74 |  |  |  |  |  |
| 10 | X |  |  |  |  |  | 75 |  |  |  |  |  |
| 11 | X |  |  |  |  |  | 76 |  |  |  |  |  |
| 12 | X |  |  |  |  |  | 77 |  |  |  |  |  |
| 13 | X |  |  |  |  |  | 78 |  |  |  |  |  |
| 14 | X |  |  |  |  |  | 79 |  |  |  |  |  |
| 15 | X |  |  |  |  |  | 80 |  |  |  |  |  |
| 16 | X |  |  |  |  |  | 81 |  |  |  |  |  |
| 17 | X |  |  |  |  |  | 82 |  |  |  |  |  |
| 18 | X |  |  |  |  |  | 83 |  |  |  |  |  |
| 19 | X |  |  |  |  |  | 84 |  |  |  |  |  |
| 20 | X |  |  |  |  |  | 85 |  |  |  |  |  |
| 21 | X |  |  |  |  |  | 86 |  |  |  |  |  |
| 22 |  |  |  |  |  |  | 87 |  |  |  |  |  |
| 23 |  |  |  |  |  |  | 88 |  |  |  |  |  |
| 24 |  |  |  |  |  |  | 89 |  |  |  |  |  |
| 25 |  |  |  |  |  |  | 90 |  |  |  |  |  |
| 26 |  |  |  |  |  |  | 91 |  |  |  |  |  |
| 27 |  |  |  |  |  |  | 92 |  |  |  |  |  |
| 28 |  |  |  |  |  |  | 93 |  |  |  |  |  |
| 29 |  |  |  |  |  |  | 94 |  |  |  |  |  |
| 30 |  |  |  |  |  |  | 95 |  |  |  |  |  |
| 31 |  |  |  |  |  |  | 96 |  |  |  |  |  |
| 32 |  |  |  |  |  |  | 97 |  |  |  |  |  |
| 33 |  |  |  |  |  |  | 98 |  |  |  |  |  |
| 34 |  |  |  |  |  |  | 99 |  |  |  |  |  |
| 35 |  |  |  |  |  |  | 100 |  |  |  |  |  |
| 36 |  |  |  |  |  |  | 101 |  |  |  |  |  |
| 37 |  |  |  |  |  |  | 102 |  |  |  |  |  |
| 38 |  |  |  |  |  |  | 103 |  |  |  |  |  |
| 39 |  |  |  |  |  |  | 104 |  |  |  |  |  |
| 40 |  |  |  |  |  |  | 105 |  |  |  |  |  |
| 41 |  |  |  |  |  |  | 106 |  |  |  |  |  |
| 42 |  |  |  |  |  |  | 107 |  |  |  |  |  |
| 43 |  |  |  |  |  |  | 108 |  |  |  |  |  |
| 44 |  |  |  |  |  |  | 109 |  |  |  |  |  |
| 45 |  |  |  |  |  |  | 110 |  |  |  |  |  |
| 46 |  |  |  |  |  |  | 111 |  |  |  |  |  |
| 47 |  |  |  |  |  |  | 112 |  |  |  |  |  |
| 48 |  |  |  |  |  |  | 113 |  |  |  |  |  |
| 49 |  |  |  |  |  |  | 114 |  |  |  |  |  |
| 50 |  |  |  |  |  |  | 115 |  |  |  |  |  |
| 51 |  |  |  |  |  |  | 116 |  |  |  |  |  |
| 52 |  |  |  |  |  |  | 117 |  |  |  |  |  |
| 53 |  |  |  |  |  |  | 118 |  |  |  |  |  |
| 54 |  |  |  |  |  |  | 119 |  |  |  |  |  |
| 55 |  |  |  |  |  |  | 120 |  |  |  |  |  |
| 56 |  |  |  |  |  |  | 121 |  |  |  |  |  |
| 57 |  |  |  |  |  |  | 122 |  |  |  |  |  |
| 58 |  |  |  |  |  |  | 123 |  |  |  |  |  |
| 59 |  |  |  |  |  |  | 124 |  |  |  |  |  |
| 60 |  |  |  |  |  |  | 125 |  |  |  |  |  |
| 61 |  |  |  |  |  |  | 126 |  |  |  |  |  |
| 62 |  |  |  |  |  |  | 127 |  |  |  |  |  |
| 63 |  |  |  |  |  |  | 128 |  |  |  |  |  |
| 64 |  |  |  |  |  |  | 129 |  |  |  |  |  |
| 65 |  |  |  |  |  |  | 130 |  |  |  |  |  |

CONTENTS

1.0	INTRODUCTION	7

2.0	Scope	7

3.0	NORMATIVE REFERENCE	7

4.0	Material properties	8

5.0	Computer software	8

6.0	Geometry	9

7.0	DESIGN LOAD	10

8.0	Design Load Combinations	17

9.0	Tank ANALYSIS AND DESIGN	18

10.0	ATTACHMENTS	23

- INTRODUCTION

Binak oilfield in Bushehr province is a part of the southern oilfields of Iran, is located 20 km northwest of Genaveh city.

With the aim of increasing production of oil from Binak oilfield, an EPC/EPD Project has been defined by NIOC/NISOC and awarded to Petro Iran Development Company (PEDCO). Also PEDCO (as General Contractor) has assigned the EPC-packages of the Project to "Hirgan Energy - Design and Inspection" JV.

- Scope

This report covers design of Lean Glycol Storage Tank (TK-2102). The calculation of foundation is performed using “SAP” software.

- NORMATIVE REFERENCE

  - Local Codes and Standards

- INBC Part 6 “Iranian National Building Code

- INBC Part 7 “Iranian National Building Code

- INBC Part 9 “Iranian National Building Code

- INBC Part 10 “Iranian National Building Code

- Iranian Seismic Design Code for Petroleum Facilities(3rd edition)

  - International Codes and Standards

- ASCE 7-10 “Minimum Design Loads and Associated Criteria for Buildings and Other Structures-American Society of Civil Engineers”.

- ACI 318. “Building Code Requirements for Reinforced Concrete”, American Concrete Institute.

- AISC 358 “Prequalified Connections for Special and Intermediate Steel Moment Frames for Seismic Applications.” American Institute of Steel Construction, Inc.

- AISC 360 - “Specification for Structural Steel Buildings”. American Institute of Steel Construction, Inc.

  - The Project Documents

- BK-GNRAL-PEDCO-000-ST-SP-0001 SPECIFICATION FOR CONCRETE WORK

- BK-GNRAL-PEDCO-000-ST-DC-0001 Structural Design Criteria

- BK-GNRAL-PEDCO-000-CV-SP-0004 Specification For Earth Work

- BK-GCS-PEDCO-120-GT-RT-0001 Geotechnical Investigation Report for Compressor Station

- Material properties

Material properties are delivered in the following table.

| Material properties |  |
| --- | --- |
| Structure and Foundation concrete | F’c=300kg/cm²(28 days cylindrical sample) |
| Long. Reinforcement | Fy=4000 kg/cm² (AIII) |
| Trans. Reinforcement | Fy=4000 kg/cm² (AIII) |

- Computer software

Computer’s Software, which is used in structure and foundation analysis and design, are defined in the following table.

| Computer software |  |
| --- | --- |
| analysis and design of structure and foundation | SAP 21.1.0 |

- Geometry

- foundation and pedestal plan for TK-2102

- DESIGN LOAD

  - Load case for tank foundation design

The loads that apply on the foundation, according to tank design report (annex I), are as following:

| Anchor Bolt data |  |  |  |  |
| --- | --- | --- | --- | --- |
| anchor bolt dia. | M36 |  |  |  |
| No. of anchor bolt | 8 |  |  |  |
| BCD | 4200 | mm |  |  |
| foundation load data |  |  |  |  |
| Seismic | Shear (N) |  | 135,800 | N |
|  | Moment (N.m) | Ring wall | 311,500 | N.m |
|  |  | Slab | 348,600 | N.m |
| wind | Shear (N) |  | 53,900 | N |
|  | Moment (N.m) |  | 120,600 | N.m |
| Weight | Empty |  | 6,700 | Kg |
|  | Operating |  | 55,100 | Kg |
|  | Hydrotest |  | 68,700 | Kg |

- foundation and pedestal plan for TK-2102

  - Dead Load

Based on mechanical data sheet fabrication load applied as Dead load on foundation as follows:

Dead Weight/Area: [6700/(3.1415x4*4)]*4=534 Kg /m2

- Dead Load On Pedestal

  - Operation Load

Normal Fluid Load on ring foundation:

F = (Ope load / area of ring foundation) → area of ring foundation: (𝜋 × 4²)/4=12.56

55100/12.56=4.387 ton/m²

- Area uniform F load on the inside ring foundation

  - HT (Test Fluid Load)

Test Load on ring foundation: HT = Hydro test load / area of ring foundation → area of ring foundation: (𝜋 × 4²)/4=12.56 m2 → 68700/12.56= 5.470 ton/m²

- area uniform HT load on the inside ring foundation

  - E (Earthquake Load)

Shear load = 135800 N (According to General Arrangement Lean Glycol Storage Tank (TK-2102).

This shear is based on the datasheet, but we used API650 with ASD behavior coefficient so its value is multiplied by 1.4 → 135800  1.4 = 190120 N

Moment = 311500 N.m (According to General Arrangement Lean Glycol Storage Tank (TK-2102).

This moment is based on the datasheet, but we used API650 with ASD behavior coefficient so its value is multiplied by 1.4 → 311500  1.4 = 436100 N.m

- Earthquake Load (EX)

  - W (Wind Load)

Shear load = 53900 N (According to General Arrangement Lean Glycol Storage Tank (TK-2102).

This shear is based on the datasheet, but we used API650 with ASD behavior coefficient so its value is multiplied by 1.4 → 53900  1.4 = 75460 N

Moment = 120600 N.m (According to General Arrangement Lean Glycol Storage Tank (TK-2102).

This moment is based on the datasheet, but we used API650 with ASD behavior coefficient so its value is multiplied by 1.4 → 120600  1.4 = 168840 N.m

- wind load (WX)

  - Soil Pressure (DS)

Due to soil weight on foundation apply Soil Load on foundation as follows :

ƔH=1800 x 0.80 = 1440 kg/m2

- SOIL PRESSURE

- Design Load Combinations

- Tank ANALYSIS AND DESIGN

  - Tank geometry

As noted before, the structure has modelled and analysed by SAP2000. This figure shows a 3D view of the model in software.

- 3D model

  - Stability Check

  - Soil Pressure Check

| L / C | Pu | Mu | e | e/Df | L z-z | SOIL BEARING ( TON / M2 ) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Qmax | Qmin | ALLOW. |  | Uplift Length |
| LC / 1 | 191 | 0 | 0.00 | 0.00 | 0.0 | 5.448 | 5.448 | 10.000 |  | 0.00 |
| LC / 2 | 191 | 31.53444 | 0.17 | 0.03 | 0.0 | 6.584 | 4.313 | 10.000 |  | 0.00 |
| LC / 3 | 191 | 23.65083 | 0.12 | 0.02 | 0.0 | 6.300 | 4.597 | 10.000 |  | 0.00 |
| LC / 4 | 114 | 31.53444 | 0.28 | 0.04 | 0.0 | 4.405 | 2.133 | 10.000 |  | 0.00 |
| LC / 5 | 204 | 7.88361 | 0.04 | 0.01 | 0.0 | 6.121 | 5.553 | 10.000 |  | 0.00 |
| LC / 6 | 123 | 7.88361 | 0.06 | 0.01 | 0.0 | 3.786 | 3.218 | 10.000 |  | 0.00 |
| LC / 7 | 191 | 60.044012 | 0.31 | 0.05 | 0.0 | 7.611 | 3.286 | 10.000 |  | 0.00 |
| LC / 8 | 191 | 45.033009 | 0.24 | 0.04 | 0.0 | 7.070 | 3.826 | 10.000 |  | 0.00 |
| LC / 9 | 191 | -45.4618948 | -0.24 | 0.04 | 0.0 | 3.811 | 7.086 | 10.000 |  | 0.00 |
| LC / 10 | 114 | 60.044012 | 0.52 | 0.08 | 0.0 | 5.432 | 1.106 | 10.000 |  | 0.00 |
| LC / 11 | 114 | -60.044012 | -0.52 | 0.08 | 0.0 | 1.106 | 5.432 | 10.000 |  | 0.00 |
| LC / 12 | 0 | 0 | 0.00 | 0.00 | 0.0 | 0.000 | 0.000 | 10.000 |  | 0.00 |
| LC / 13 | 0 | 0 | 0.00 | 0.00 | 0.0 | 0.000 | 0.000 | 10.000 |  | 0.00 |
| LC / 14 | 0 | 0 | 0.00 | 0.00 | 0.0 | 0.000 | 0.000 | 10.000 |  | 0.00 |

| MAXIMUM SOIL STRESS = |  |  | 7.611 | TON / M2 |  |
| --- | --- | --- | --- | --- | --- |

  - Reinforcing

  - Pedestal reinforcement

As Req=T / F. fy=2328 / 0.9x4000=0.64cm2

Min As=0.0015x Aped=0.0015x0.8284xDp2x10000=310.65 cm2

Max (0.64 , 310.65)=310.65

USE 72 T25 AS used=353.42 cm2

  - Foundation reinforcement

According to ACI-318-08:

AS,min=0.0018×60×100=10.8cm2 for top and bottom

→ If use Φ 16@200 → A = 10.05cm² >  5.4 cm² → ok

- M11 Max - M11 Min

- M22 Max   -    M22 Min

- ATTACHMENTS

  - SOWFTWARE FILE

“SAP” software file is attached.

  - MECHANICAL DATA SHEET

Mechanical data sheet is attached.