| Row | Clarification Title | Bidder reply (1st round) | Conclusion (2nd round) | Bidder reply (2nd round) | Conclusion (final/TCM) |
| --- | --- | --- | --- | --- | --- |
| GENERAL |  |  |  |  |  |
|  | Vendor to confirm that in case of contradiction between this TCL and offer; vendor confirmation of compliance through this technical clarification is governed and shall be followed by vendor after order. In such case offer won’t be considered as basis of any claim after order. | Confirmed | Closed. |  |  |
|  | Bidder shall confirm his full compliance to meet all purchaser documents listed and attached to MR, code / standards listed in MR with ref. No. “BK-GCS-PEDCO-120-ME-MR-0013 “as well as MR requirements. All deviations shall be listed in project deviation form, attach to this TCL, signed and stamped. Notes: 1-Any exception and deviation from the requirements of MR and the related specifications including referenced codes & standards shall be clearly declared by vendor in writing for purchaser’s approval at bid stage by using of the format included in “Letter Of Conformity” attached to material requisition. Raising no deviation to any single requirement means full compliance of vendor quotation to that requirement. Purchaser has no responsibility to discover deviations or alternatives scattered over the proposal in addition to what proclaimed explicitly in deviation list; hence such deviation (if any) is disregarded and shall not be considered for vendor claim in future. (Vendor to confirm) 2- Bidder shall confirm that all MR attachments listed in MR are available for her for review at bid stage. 3-Considering items 1 & 2, please note that any deviation at detail stage will not be accepted by purchaser. 4- MR scope of work and supply shall be signed and stamped by vendor. | Confirmed, by considering deviation list. | Deviation list will be finalized in clarification meeting. |  |  |
|  | Bidder to confirm his compliance with documents priority stated in MR. Any conflict between the documents shall be notified by bidder to purchaser and resolved by his agreement | Confirmed | Closed. |  |  |
|  | Bidder shall provide following documents while bidding: 1- Copy of valid quality system certificate(ISO 9001/2 or 3) 2- Sub- vendors / sub-suppliers list. 3- Deviation list with letter of conformity (Refer to item A2 above) 4- Reference and Experience List 5- Joint venture agreement (If any) with any company to design / fabrication of equipment. In case of joint venture ; qualified documents of the company who is part of joint venture shall be also submitted | 1.The certificate of ISO 9001:2008 has not been gotten yet, we have organized office and manufacturing department and we have prepared the pre-requirements for ISO and preliminary stages for the contact has been done, we will submit you the certificate in the next stages of project. 2, 3, 4, 5.confirmed. | 1.Noted 2, 3, 4, 5. Closed. |  |  |
|  | Bidder shall submit selected Sub-Vendor List based on Project AVL, before proceeding with any purchasing activity. Vendors shall be selected from A.V.L Based On Products. Please confirm. | Confirmed please see attached sub-vendor list | Closed. Sub-Vendor List shall be selected from AVL. |  |  |
|  | The Bidder shall confirm that the surface preparation and painting of all equipment and materials are in accordance with the project painting specification referenced as 'BK-GNRAL-PEDCO-000-PI-SP-0006,' which is attached to the MR. | Confirmed, by considering deviation list. | Deviation list will be finalized in clarification meeting. |  |  |
|  | Bidder to confirm that Packing, Marking, transporting procedure (BK-GNRAL-PEDCO-000-QC-PR-0045) for Shipment will be done in accordance to the project specification attached to the MR. | Confirmed, by considering deviation list. | Deviation list will be finalized in clarification meeting. |  |  |
|  | Inspection and Test Plan / Quality Control Plan and their related inspection level (Witness, Hold, Review and Approval) shall be at least in accordance with MR and all project requirement such as NDT PROCEDURE. Inspection and testing of cranes shall be performed as per IPS-I-GN-350. Any deviation is not accepted unless otherwise specified in deviation list and approved by Purchaser. | Confirmed, by considering deviation list. | Deviation list will be finalized in clarification meeting. |  |  |
|  | Bidder to confirm that all native files due to all calculations and drawings shall be submitted for purchaser approval after order. | 1.Confirmed except for the CAD files. 2.Just submit PDF from your calculation | Not accepted, all native files due to all calculations and drawings shall be submitted for purchaser approval after order. |  |  |
|  | Following documents shall be submitted at bid stage (as a minimum requirement): 1. Vendor Catalogue 2. Quality Assurance Manual /Quality Management System Certificate (according to latest rev. of ISO) 3. Electrical Load list (Bridge driver, operator pendant, Hoist Motor and etc) 4. Manufacturing Master Time Schedule Per Equipment (Including Design, Procurement, Fabrication, Test, Packing, Delivery) 5-Sectional drawing with BOM shall be submitted by vendor. 6-Noise Data 7-GA DWG with outline dimension, weights, load table and etc. shall be submitted by vendor in next revision. 8. Sub vendor list 9. Spare part list for startup and commissioning 10. Two years spare part list 11. Special tools list | 1.confirmed 2. The certificate of ISO 9001:2008 has not been gotten yet, we have organized office and manufacturing department and we have prepared the pre-requirements for ISO and preliminary stages for the contact has been done, we will submit you the certificate in the next stages of project. 3.Confirmed . it will be submitted during detail design. 4. confirmed. Time schedule is attached. 5. Confirmed . it will be submitted during detail design. 6.N.A. 7.confirmed. 8. Confirmed. 9. • No spare parts Required for Start-up and Commissioning. Please refer to deviation list. 10. • A List of Two years spare parts based on experiences of similar projects is submitted as attached. Please refer to deviation list. 11. No especial tools required for erection and maintenance. The cranes will be delivered ready to use. Please refer to deviation list. | 1.Closed 2.Noted 3.Closed 4.Closed 5.closed 6. closed 7.closed 8.closed 9. closed 10. Deviation list will be finalized in clarification meeting. 11. Closed. |  |  |
|  | Bidder shall confirm the scope of work and supply are fully in accordance with MR and its attachment. If there is any deviation shall be declared at this stage. | Confirmed, by considering deviation list. | Deviation list will be finalized in clarification meeting. |  |  |
|  | Vendor shall confirm Crane and Equipment’s are fully suitable for outdoor installation/operation under project site conditions “Process Basis of Design; BK-GNRAL-PEDCO-000-PR-DB-0001 “without requiring any additional protection. | This document was not sent to this company with this number. However, according to the client's documents, the cranes are considered to be outdoor /under shelter. And its temperature, humidity and altitude conditions are considered in accordance with TBE. | All the documents mentioned are attached to the MR document. Please check again. |  |  |
|  | First filling of lubricant (Oil and grease) is in vendor scope of supply. | Our systems are life time lubricated. Please refer to deviation list. | Closed. |  |  |
|  | Pre-commissioning, commissioning and start up spare parts shall be supplied by vendor. Vendor shall confirm to provide all required spare parts as per project spare part list attached to MR. | In case of necessity, spare parts for pre-commissioning, commissioning and start-up will be supplied without extra charge but will not be delivered separately. The crane will be delivered ready to use. | Noted |  |  |
|  | Following items shall be considered (supplied) and included in the bid documentation: • Spare parts for commissioning and start-up • Spare parts for two years operation; a qualified and complete list based on E&C-QC-SP-1 • Capital spare parts (as option / if any) | - No spare parts Required for Start-up and Commissioning - A List of Two years spare parts based on experiences of similar projects is submitted as attached. - will be submitted after PO (optional) | - Noted. - Noted, Project Documents shall be followed by vendor. - Closed. |  |  |
|  | The paint of crane’s structure shall be resistant to chemical corrosion. | Confirmed, by considering deviation list. | Deviation list will be finalized in clarification meeting. |  |  |
|  | Vendor shall not offer prototype design or a design with less than 2 years of successful operation in similar service. | Confirmed | Closed. |  |  |
|  | List of special tools required for the quoted cranes Necessary special tools for erection/ installation and maintenance, if any, shall be included in supply. | No special tools required for erection and maintenance. The cranes will be delivered ready to use. | Closed. |  |  |
|  | The crane manufacturer representative shall be informed about the installation location before any crane manufacturing activity. | Confirmed | Closed. |  |  |
|  | The crane shall be guaranteed for satisfactory performance for the rated working capacity, maximum lifting and other motion limitation. Vendor shall warranty the engineering, materials and workmanship of the equipment/goods. Under this warranty, all materials, parts, and assemblies found to be defective during the guarantee period shall be replaced free of charge by VENDOR. | Confirmed | Closed. |  |  |
|  | The guarantee period shall be eighteen (18) months from the date of delivery or twelve (12) months from the date of mechanical completion & start-up, whichever occurs later. Also bidder to confirm performance guarantee requirements based on relevant data sheets & specifications (attached to the equipment PMR). | Confirmed | Closed. |  |  |
| MECHANIC |  |  |  |  |  |
|  | The vendor should fill all the datasheets of the cranes with doc No: BK-GCS-PEDCO-120-ME-DT-0027 | It will be submitted during detail design. | Closed. |  |  |
|  | Specification For Overhead Travelling Cranes with doc No. “BK-GCS-PEDCO-120-ME-SP-0009” shall be followed by vendor. | Confirmed, by considering deviation list. | Deviation list will be finalized in clarification meeting. |  |  |
|  | ASTM number and grade of all supplied material to be specified. | We Comply with the European Standards ATEX, FEM and DIN. | Vendor shall be followed project requirements. |  |  |
|  | Test procedures shall be Submitted by vendor based on project´s document. | It will not be possible for us nor for the German supplier to provide all these documents and perform all these tests which are specially related to motor & LV DISTRIBUTION panels. Please see deviation list. ITP and FAT procedure (Factory acceptance test) & SAT procedure (Site Test) is submitted. Please consider. | Deviation list will be finalized in clarification meeting. ITP will be finalized during PIM . |  |  |
|  | Bidder to consider the stainless steel name plate (including information as per PURCHASER advices) for the equipment. | Acc. to Manufacturer/Supplier Standards, it is acceptable. | Noted. |  |  |
|  | Controls complete with remote controller and safe interlocks shall be considered by vendor. | Push button is confirmed. Remote control can be supplied at additional cost. | Closed. |  |  |
|  | The crane shall be designed with facilities for easy inspection and maintenance. Filling and drain plugs on gear boxes shall be easily accessible and to be provided with the necessary means to avoid spillage. | - Confirmed. - All mechanisms are life time lubricated thanks to KUHNEZUG Environmentally friendly German cranes design. | Closed. |  |  |
|  | Easy access shall be provided for brake adjustment and replacement and the required ladder for access to crane shall be provided by carne vendor | confirmed | Closed. |  |  |
|  | First filling of lubricant (Oil and grease) is in vendor scope of supply. | Thanks to environmentally-friendly KUHNEZUG German Cranes’ design, the cranes will be Life Time Lubricated. | Closed. |  |  |
|  | Elevation of crane installation shall such that the truck loading/unloading will be performed easily. Height from crane hook to finish floor level shall not be less than following: Truck (Bogie) height + Highest package height + 1.5m Vertical free board. | confirmed | Closed. |  |  |
|  | Brakes shall be DC Electromagnetic and Fail-Safe type. | confirmed | Closed. |  |  |
|  | The electrical power supply to the trolley and end carriages should be realized by festooned cable supplied complete with all suspension accessories and pay-out system. | confirmed | Closed. |  |  |
|  | All overhead cranes and associated metal structures and equipment shall be suitably connected to the Main earthing network of the plant. The crane wheels shall not be used as means of earthing. | confirmed | Closed. |  |  |
| SAFETY |  |  |  |  |  |
|  | Vendor confirms that the Noise level @ 1m will not exceed 85 dB | confirmed | Closed. |  |  |
| ELECTRICAL |  |  |  |  |  |
|  | All electrical equipment shall comply with electrical references & standard that mentioned on material requisition. IPS-E-EL-100 (1). IPS-M-El-271 (2), IPS-M-El-161 (2), IPS-M-EL-131 (2) | We follow ATEX/FEM & DIN standards which cover BS and IPS standards. | All electrical equipment shall comply with electrical references & standard that mentioned on material requisition |  |  |
|  | All documents related to electrical equipment shall be submitted by vendor such as preliminary load list, electrical equipment list, cable list & . . . | Please see to deviation list. | Deviation list will be finalized in clarification meeting. |  |  |
|  | The rated voltage of project is 3pahse, 400/230 V, 50 Hz. Please consider them for detail design. | Confirmed | Closed. |  |  |
|  | Vendor list & sub vendor list of manufacture shall be according to project AVL. | The closest offer to the Project Demands and According to the updated Technology has been Submitted. KUHNEZUG German Cranes company design and fabricate his products based on FEM, CMAA, DIN, Atex and other international standards in Germany and has his own standards for the products. Some technical specifications may be technically better/ higher than request or standard. The technical advantages mentioned in deviations We, FarabarSanat Company design and fabricate the cranes’ steel structure based on FEM, CMAA, DIN, standards. Also, please see technical Offers, Deviations List and attachments | Noted. Vendor list & sub vendor list of manufacture shall be according to project AVL. |  |  |
|  | Motor shall be manufactured & tested in accordance with the latest applicable sections of the IEC standards. Project specification shall be applied as well. | It will not be possible for us nor for the German supplier to provide all these documents and perform all these tests which are specially related to motor & LV DISTRIBUTION panels. Please see deviation list. | Deviation list will be finalized in clarification meeting. |  |  |
|  | Environmental criteria for design of electrical systems, shall be considered by vendor (According to Electrical System Design Criteria) | This document was not sent to this company with this number. | All the documents mentioned are attached to the MR document. Please check again. |  |  |
|  | - Vendor shall issue required cable list. Cables shall be according to “BK-GNRAL-PEDCO-000-EL-SP-0014”. | This document was not sent to this company with this number. Some deviations for cable, which have been common in projects, are listed in the deviation list. Please refer to the deviation list. | All the documents mentioned are attached to the MR document. Please check again. |  |  |
|  | Required Electrical specifications have been attached. Vendor shall follow them. | Confirmed, by considering deviation list. | Deviation list will be finalized in clarification meeting. |  |  |
|  | A complete set of spare parts for commissioning shall be supplied for each electrical equipment | No spare parts Required for Start-up and Commissioning. Please see to deviation list. | Deviation list will be finalized in clarification meeting. |  |  |
|  | Data Sheets for LV Induction Motors shall be filled for all motors and submitted. | It will be submitted during detail design. | Closed. |  |  |
| STRUCTURE |  |  |  |  |  |
|  | All cranes must comply with the specifications stated in the architectural and structural documents of the units. | Confirmed. | Closed. |  |  |