| طرح نگهداشت و افزایش تولید 27 مخزن |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Thermal/Mechnical Calculation Book نگهداشت و افزایش تولید میدان نفتی بینک |  |  |  |  |  |  |  |
| V03 | Oct.2024 | AFC | AAC | M.FAKHARIAN | M.SADEGHIAN |  |
| V02 | Jul.2024 | IFA | AAC | M.FAKHARIAN | M.SADEGHIAN |  |
| V01 | Mar.2024 | IFA | AAC | M.FAKHARIAN | S.FRAMARZPOUR |  |
| V00 | Nov.2023 | IFA | AAC | M.FAKHARIAN | S.FRAMARZPOUR |  |
| Rev. | Date | Purpose of Issue/Status | Prepared by: | Checked by: | Approved by: | CLIENT Approval |
| Status: | IFA: Issued For Approval IFI: Issued For Information AFC: Approved For Construction |  |  |  |  |  |

REVISION RECORD SHEET

| PAGE | V00 | V01 | V02 | V03 | V04 |  | PAGE | V00 | V01 | V02 | V03 | V04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X | X | X | X |  |  | 66 |  |  | X | X |  |
| 2 | X | X | X | X |  |  | 67 |  |  | X | X |  |
| 3 | X |  | X | X |  |  | 68 |  |  | X | X |  |
| 4 | X |  | X | X |  |  | 69 |  |  | X | X |  |
| 5 | X | X | X |  |  |  | 70 |  |  | X | X |  |
| 6 | X | X | X | X |  |  | 71 |  |  | X | X |  |
| 7 | X | X | X | X |  |  | 72 |  |  | X | X |  |
| 8 | X | X | X | X |  |  | 73 |  |  | X | X |  |
| 9 |  |  | X | X |  |  | 74 |  |  | X | X |  |
| 10 |  |  | X | X |  |  | 75 |  |  | X |  |  |
| 11 |  |  | X | X |  |  | 76 |  |  | X |  |  |
| 12 |  |  | X | X |  |  | 77 |  |  | X |  |  |
| 13 |  |  | X | X |  |  | 78 |  |  | X |  |  |
| 14 |  |  | X | X |  |  | 79 |  |  | X |  |  |
| 15 |  |  | X | X |  |  | 80 |  |  | X |  |  |
| 16 |  |  | X | X |  |  | 81 |  |  | X |  |  |
| 17 |  |  | X | X |  |  | 82 |  |  | X |  |  |
| 18 |  |  | X | X |  |  | 83 |  |  | X |  |  |
| 19 |  |  | X | X |  |  | 84 |  |  | X |  |  |
| 20 |  |  | X | X |  |  | 85 |  |  | X |  |  |
| 21 |  |  | X | X |  |  | 86 |  |  | X |  |  |
| 22 |  |  | X | X |  |  | 87 |  |  | X |  |  |
| 23 |  |  | X | X |  |  | 88 |  |  | X |  |  |
| 24 |  |  | X | X |  |  | 89 |  |  | X |  |  |
| 25 |  |  | X | X |  |  | 90 |  |  | X |  |  |
| 26 |  |  | X | X |  |  | 91 |  |  | X |  |  |
| 27 |  |  | X | X |  |  | 92 |  |  | X |  |  |
| 28 |  |  | X | X |  |  | 93 |  |  |  |  |  |
| 29 |  |  | X | X |  |  | 94 |  |  |  | X |  |
| 30 |  |  | X | X |  |  | 95 |  |  |  | X |  |
| 31 |  |  | X | X |  |  | 96 |  |  |  | X |  |
| 32 |  |  | X | X |  |  | 97 |  |  |  | X |  |
| 33 |  |  | X | X |  |  | 98 |  |  |  | X |  |
| 34 |  |  | X |  |  |  | 99 |  |  |  | X |  |
| 35 |  |  | X |  |  |  | 100 |  |  |  | X |  |
| 36 |  |  | X |  |  |  | 101 |  |  |  | X |  |
| 37 |  |  | X |  |  |  | 102 |  |  |  | X |  |
| 38 |  |  | X |  |  |  | 103 |  |  |  | X |  |
| 39 |  |  | X |  |  |  | 104 |  |  |  | X |  |
| 40 |  |  | X |  |  |  | 105 |  |  |  | X |  |
| 41 |  |  | X |  |  |  | 106 |  |  |  | X |  |
| 42 |  |  | X |  |  |  | 107 |  |  |  | X |  |
| 43 |  |  | X |  |  |  | 108 |  |  |  | X |  |
| 44 |  |  | X |  |  |  | 109 |  |  |  | X |  |
| 45 |  |  | X |  |  |  | 110 |  |  |  | X |  |
| 46 |  |  | X |  |  |  | 111 |  |  |  | X |  |
| 47 |  |  | X |  |  |  | 112 |  |  |  | X |  |
| 48 |  |  | X |  |  |  | 113 |  |  |  | X |  |
| 49 |  |  | X |  |  |  | 114 |  |  |  | X |  |
| 50 |  |  | X |  |  |  | 115 |  |  |  | X |  |
| 51 |  |  | X |  |  |  | 116 |  |  |  | X |  |
| 52 |  |  | X |  |  |  | 117 |  |  |  | X |  |
| 53 |  |  | X |  |  |  | 118 |  |  |  | X |  |
| 54 |  |  | X | X |  |  | 119 |  |  |  | X |  |
| 55 |  |  | X | X |  |  | 120 |  |  |  | X |  |
| 56 |  |  | X | X |  |  | 121 |  |  |  | X |  |
| 57 |  |  | X | X |  |  | 122 |  |  |  | X |  |
| 58 |  |  | X | X |  |  | 123 |  |  |  | X |  |
| 59 |  |  | X | X |  |  | 124 |  |  |  | X |  |
| 60 |  |  | X | X |  |  | 125 |  |  |  | X |  |
| 61 |  |  | X | X |  |  | 126 |  |  |  | X |  |
| 62 |  |  | X | X |  |  | 127 |  |  |  | X |  |
| 63 |  |  | X | X |  |  | 128 |  |  |  | X |  |
| 64 |  |  | X | X |  |  | 129 |  |  |  | X |  |
| 65 |  |  | X | X |  |  | 130 |  |  |  | X |  |

REVISION RECORD SHEET

| PAGE | V00 | V01 | V02 | V03 | V04 |  | PAGE | V00 | V01 | V02 | V03 | V04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 130 |  |  |  | X |  |  | 196 |  |  |  |  |  |
| 131 |  |  |  | X |  |  | 197 |  |  |  |  |  |
| 132 |  |  |  | X |  |  | 198 |  |  |  |  |  |
| 133 |  |  |  | X |  |  | 199 |  |  |  |  |  |
| 134 |  |  |  | X |  |  | 200 |  |  |  |  |  |
| 135 |  |  |  | X |  |  | 201 |  |  |  |  |  |
| 136 |  |  |  | X |  |  | 202 |  |  |  |  |  |
| 137 |  |  |  | X |  |  | 203 |  |  |  |  |  |
| 138 |  |  |  | X |  |  | 204 |  |  |  |  |  |
| 139 |  |  |  | X |  |  | 205 |  |  |  |  |  |
| 140 |  |  |  | X |  |  | 206 |  |  |  |  |  |
| 141 |  |  |  | X |  |  | 207 |  |  |  |  |  |
| 142 |  |  |  | X |  |  | 208 |  |  |  |  |  |
| 143 |  |  |  | X |  |  | 209 |  |  |  |  |  |
| 144 |  |  |  | X |  |  | 210 |  |  |  |  |  |
| 145 |  |  |  | X |  |  | 211 |  |  |  |  |  |
| 146 |  |  |  | X |  |  | 212 |  |  |  |  |  |
| 147 |  |  |  | X |  |  | 213 |  |  |  |  |  |
| 148 |  |  |  | X |  |  | 214 |  |  |  |  |  |
| 149 |  |  |  | X |  |  | 215 |  |  |  |  |  |
| 150 |  |  |  | X |  |  | 216 |  |  |  |  |  |
| 151 |  |  |  |  |  |  | 217 |  |  |  |  |  |
| 152 |  |  |  |  |  |  | 218 |  |  |  |  |  |
| 153 |  |  |  |  |  |  | 219 |  |  |  |  |  |
| 154 |  |  |  |  |  |  | 220 |  |  |  |  |  |
| 155 |  |  |  |  |  |  | 221 |  |  |  |  |  |
| 156 |  |  |  |  |  |  | 222 |  |  |  |  |  |
| 157 |  |  |  |  |  |  | 223 |  |  |  |  |  |
| 158 |  |  |  |  |  |  | 224 |  |  |  |  |  |
| 159 |  |  |  |  |  |  | 225 |  |  |  |  |  |
| 160 |  |  |  |  |  |  | 226 |  |  |  |  |  |
| 161 |  |  |  |  |  |  | 227 |  |  |  |  |  |
| 162 |  |  |  |  |  |  | 228 |  |  |  |  |  |
| 163 |  |  |  |  |  |  | 229 |  |  |  |  |  |
| 164 |  |  |  |  |  |  | 230 |  |  |  |  |  |
| 165 |  |  |  |  |  |  | 231 |  |  |  |  |  |
| 166 |  |  |  |  |  |  | 232 |  |  |  |  |  |
| 167 |  |  |  |  |  |  | 233 |  |  |  |  |  |
| 168 |  |  |  |  |  |  | 234 2 |  |  |  |  |  |
| 169 |  |  |  |  |  |  | 235 |  |  |  |  |  |
| 170 |  |  |  |  |  |  | 236 |  |  |  |  |  |
| 171 |  |  |  |  |  |  | 237 |  |  |  |  |  |
| 172 |  |  |  |  |  |  | 238 |  |  |  |  |  |
| 173 |  |  |  |  |  |  | 239 |  |  |  |  |  |
| 174 |  |  |  |  |  |  | 240 |  |  |  |  |  |
| 175 |  |  |  |  |  |  | 241 |  |  |  |  |  |
| 176 |  |  |  |  |  |  | 242 |  |  |  |  |  |
| 177 |  |  |  |  |  |  | 243 |  |  |  |  |  |
| 178 |  |  |  |  |  |  | 244 |  |  |  |  |  |
| 179 |  |  |  |  |  |  | 245 |  |  |  |  |  |
| 180 |  |  |  |  |  |  | 246 |  |  |  |  |  |
| 181 |  |  |  |  |  |  | 247 |  |  |  |  |  |
| 182 |  |  |  |  |  |  | 248 |  |  |  |  |  |
| 183 |  |  |  |  |  |  | 249 |  |  |  |  |  |
| 184 |  |  |  |  |  |  | 250 |  |  |  |  |  |
| 185 |  |  |  |  |  |  | 251 |  |  |  |  |  |
| 186 |  |  |  |  |  |  | 252 |  |  |  |  |  |
| 187 |  |  |  |  |  |  | 253 |  |  |  |  |  |
| 188 |  |  |  |  |  |  | 254 |  |  |  |  |  |
| 189 |  |  |  |  |  |  | 255 |  |  |  |  |  |
| 190 |  |  |  |  |  |  | 256 |  |  |  |  |  |
| 191 |  |  |  |  |  |  | 257 |  |  |  |  |  |
| 192 |  |  |  |  |  |  | 258 |  |  |  |  |  |
| 193 |  |  |  |  |  |  | 259 |  |  |  |  |  |
| 194 |  |  |  |  |  |  | 260 |  |  |  |  |  |
| 195 |  |  |  |  |  |  |  |  |  |  |  |  |

03

03

Contents

1.0	INTRODUCTION	5

2.0	Thermal/Mechanical Calculation Book	6

3.0	Mechanical Calculation Book	14

3.1	Stationary Header Calculation @ Design Pressure for ae-2101.	14

3.2	Stationary Header Calculation @ test Pressure for ae-2101.	24

3.3	floating Header Calculation @ Design Pressure for ae-2101.	34

3.4	floating Header Calculation @ test Pressure for ae-2101.	44

3.5	Stationary Header Calculation @ Design Pressure for ae-2102.	54

3.6	Stationary Header Calculation @ test Pressure for ae-2102.	64

3.7	floating Header Calculation @ Design Pressure for ae-2102.	75

3.8	floating Header Calculation @ test Pressure for ae-2102.	85

3.9	FINITE ELEMENT FOR AE-2101	95

3.10	FINITE ELEMENT FOR AE-2102	123

- INTRODUCTION

Binak oilfield in Bushehr province is a part of the southern oilfields of Iran, is located 20 km northwest of Genaveh city.

With the aim of increasing production of oil from Binak oilfield, an EPC/EPD Project has been defined by NIOC/NISOC and awarded to Petro Iran Development Company (PEDCO). Also PEDCO (as General Contractor) has assigned the EPC-packages of the Project to "Hirgan Energy - Design and Inspection" JV.

GENERAL DEFINITION

The following terms shall be used in this document.

| CLIENT: | National Iranian South Oilfields Company (NISOC) |
| --- | --- |
| PROJECT: | Binak Oilfield Development – Manufacturing (w/Engineering & Material Supply) of Air Coolers |
| EPD/EPC CONTRACTOR (GC): | Petro Iran Development Company (PEDCO) |
| OWNER: | OWNER is collectively refer to National Iranian South Oil Company (NISOC) and Petro Iran Development Company (PEDCO) |
| EPC CONTRACTOR: | Joint Venture of : Hirgan Energy – Design & Inspection(D&I) Companies |
| VENDOR: | Aban Air Cooler (AAC) |
| EXECUTOR: | Executor is the party which carries out all or part of construction and/or commissioning for the project. |
| THIRD PARTY INSPECTOR (TPI): | Third Party Inspector |
| SHALL: | Is used where a provision is mandatory. |
| SHOULD: | Is used where a provision is advisory only. |
| WILL: | Is normally used in connection with the action by CLIENT rather than by an EPC/EPD CONTRACTOR, supplier or VENDOR. |
| MAY: | Is used where a provision is completely discretionary. |

03

03

Thermal/Mechanical Calculation Book

- Mechanical Calculation Book

## Stationary Header Calculation @ Design Pressure for ae-2101.

Input Echo, COMPONENT      1,         Description: St.AE-2101

Figure Number Analyzed                                    A8

03

03

Design Internal Pressure                   P         22.0000  bars

Design Temperature                      Temp        155.0000  C

VESSEL MATERIAL DATA:

Material Specification                           SA-240 316L

Shell Allowable Stress at Design Temp      S        114.3898  N./mm^2

Shell Allowable Stress at Ambient         SA        115.1465  N./mm^2

Shell Yield Stress at Design Temperature  Sy        129.8699  N./mm^2

SHORT-SIDE VESSEL DATA:

Short-side Length Dimension                H        160.0000  mm.

Minimum Thickness of Short-side Plates    t1         20.0000  mm.

Mid-side Joint Efficiency on Short-side    E          0.8500

Corner Joint Efficiency on Short-side     EC          0.8500

LONG-SIDE VESSEL DATA:

Long-side Length Dimension                 h        122.0000  mm.

Minimum Thickness of Long-side Plates     t2         20.0000  mm.

Mid-side Joint Efficiency on Long-side     E          0.8500

ADDITIONAL VESSEL DATA:

Minimum Thickness of End Plate            t5         20.0000  mm.

C-Factor for End Plate                Cf_Epl          0.2000

Long-side Plate # 1,

Pitch Distance                          p         61.0000  mm.

Uniform Hole Diameter                  d0         25.6500  mm.

Depth of Holes                         T0         20.0000  mm.

Long-side Plate # 2,

Pitch Distance                          p         61.0000  mm.

# 1: Hole Diameter                     d0         36.3000  mm.

Hole Depth                        T0          2.7000  mm.

# 2: Hole Diameter                     d1         28.5750  mm.

Hole Depth                        T1         17.3000  mm.

STAY PLATE MATERIAL DATA:

Stay Material Specification                      SA-240 316L

Stay Allowable Stress at Design Temp      Sr        114.3898  N./mm^2

Stay Allowable Stress at Ambient          SA        115.1465  N./mm^2

Stay Yield Stress at Design Temp          Sy        129.8699  N./mm^2

STAY PLATE DATA:

Minimum Thickness of Stay                 t3         10.0000  mm.

Minimum Thickness of Stay                 t4         10.0000  mm.

The Stay(s) Are Not Welded to the End  Plate

Rectangular Vessel Results, Item number     1, Desc: St.AE-2101

ASME Code, Section VIII, Division 1, 2019 App. 13

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

03

03

Short-side 1 Calculations

Membrane Ligament Efficiency [Em]:

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Short-side 2 Calculations

Membrane Ligament Efficiency [Em]:

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Long-side 1 Calculations

Effective Diameter [De]: 25.650 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 61.000 - 25.650 / 61.000

= 0.580

Bending Ligament Efficiency [Eb]:

As diameter holes are uniform Eb = Em

= 0.580

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Long-side 2 Calculations

Effective Diameter [De]:

03

03

= ( d0 * T0 + d1 * T1 + d2 * T2 ) / ( t1 - CA )

= ( 36.30 * 2.70 + 28.57 * 17.30 + 0.00 * 0.00 ) /

( 20.00 - 0.00 )

= 29.618 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 61.000 - 29.618 / 61.000

= 0.514

Dist from Neutral axis of c/s to extreme fibers [Ci & Co]:

Calculation of Xbar:

= (( b0 * T0 * ( T0/2 + T1 + T2 )) + ( b1 * T1 *

( T1/2 + T2 )) + ( b2 * T2 * ( T2/2 ))) /

( b0 * T0 + b1 * T1 + b2 * T2 )

= (( 0.97 * 2.70 * ( 2.70 /2 + 17.30 + 0.00 )) + ( 1.28* 17.30 *

( 17.30 /2 + 0.00 )) + ( 2.40 * 0.00 * ( 0.00 /2 ))) /

( 0.97 * 2.70 + 1.28 * 17.30 + 2.40 * 0.00 )

= 9.713 mm.

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

Ci = Xbar

= 9.713 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t - CA - Xbar )

= -( 20.000 - 0.000 - 9.713 )

= -10.287 mm.

Moment of Inertia (Section 13-6, Equation (5)) [I]:

= 0.079 cm**4

Effective Diameter [De]:

= Pitch - (( 6 * I ) / (( t - CA)^2 * (-Co)))

= 61.00 - (( 6 * 0.08 ) / ((20.00 - 0.00 )^2 * (10.29 )))

= 31.851 mm.

Bending Ligament Efficiency [Eb]:

= Pitch - De / Pitch

= 61.000 - 31.851 / 61.000

= 0.478

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Em          Eb          Ci          Co

Short-side 1       0.850       0.850      10.000     -10.000

2       0.850       0.850      10.000     -10.000

Long-side  1       0.580       0.580      10.000     -10.000

2       0.514       0.478       9.713     -10.287

Moment of Inertia of a Strip of the Vessel Wall:

Thickness  t1, I1  = 0.0667 cm**4

Thickness  t2, I2  = 0.0667 cm**4

Rectangular Vessel Parameters:

Alpha =      H / h    = 1.3115

03

03

K     = (I2/I1)*Alpha = 1.3115

Membrane Stress Calculations per Section 13-9

Membrane Stresses at Short-side 1

Membrane Stress at Short-side 1 [Sms]:

= P * h / ( 2 * t1 ) * { 3 - [( 6 + K *

( 11 - alpha2) / ( 3 + 5 * K )]}

= 22.00 * 122.00 / ( 2 * 20.00 ) * { 3- [( 6 + 1.31 *

( 11 - 1.312) / ( 3 + 5 * 1.31 )]}

= 7.37 N./mm^2

Membrane Stresses at Short-side 2

Membrane Stress at Short-side 2 [Sms]:

= P * h / ( 2 * t1 ) * { 3 - [( 6 + K *

( 11 - alpha2) / ( 3 + 5 * K )]}

= 22.00 * 122.00 / ( 2 * 20.00 ) * { 3- [( 6 + 1.31 *

( 11 - 1.312) / ( 3 + 5 * 1.31 )]}

= 7.37 N./mm^2

Membrane Stresses at Long-side 1

Membrane Stress at Long-side 1 at A [Sml]:

= P * H / 2 * t2

= 22.00 * 160.00 / 2 * 20.00

= 8.80 N./mm^2

If Em( 0.580 ) < E( 0.850 ) and Eb( 0.580 ) < E( 0.850 ) then

Sml =  Sml / Em

= 8.80 / 0.58

= 15.19 N./mm^2

Membrane Stresses at Long-side 2

Membrane Stress at Long-side 2 at A [Sml]:

= P * H / 2 * t2

= 22.00 * 160.00 / 2 * 20.00

= 8.80 N./mm^2

If Em( 0.514 ) < E( 0.850 ) and Eb( 0.478 ) < E( 0.850 ) then

Sml =  Sml / Em

= 8.80 / 0.51

= 17.11 N./mm^2

Membrane Stresses at Stay Plate

Membrane Stress at Stay Plate [t3]:

= P * h / ( 2 * t3 ) * [( 6 + K * ( 11 -

alpha2) / ( 3 + 5 * K )]

= 22.00 * 122.00 / ( 2 * 10.00 ) * [( 6 + 1.31 * ( 11 -

1.312) / ( 3 + 5 * 1.31 )]

= 25.52 N./mm^2

Membrane Stress at Stay Plate [t4]:

= P * h / ( 2 * t4 ) * [( 6 + K * ( 11 -

alpha2) / ( 3 + 5 * K )]

= 22.00 * 122.00 / ( 2 * 10.00 ) * [( 6 + 1.31 * ( 11 -

1.312) / ( 3 + 5 * 1.31 )]

= 25.52 N./mm^2

03

03

MEMBRANE STRESSES:  Membrane Stress Calculations per Section 13-9,

Equations (13-15). (N./mm^2) :

STRESS LOCATIONS                                Actual    Allowable

----------------------------------------------------------------------

Short-side 1                                      7.37        97.23

Short-side 2                                      7.37        97.23

Short-side Corner                                 7.37        97.23

Long-side  1 at A                                15.19       114.39

Long-side  2 at A                                17.11       114.39

Long-side Corner                                  8.80        97.23

Stay Plate (t3)                                  25.52       114.39

Stay Plate (t4)                                  25.52       114.39

Bending Stress Calculations per Section 13-9

Bending Stresses at Short-side 1

Bending Stress at Short-side 1 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 22.00 * 10.00 / ( 24 * 0.07 ) * [ -3 * 160.002 + 2 * 122.002 *

(( 3 + 5 * 1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))]

= -44.45 N./mm^2

Bending Stress at Short-side 1 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 22.00 * -10.00 /( 24 * 0.07 ) * [ -3 * 160.002 + 2 * 122.002 *

(( 3 + 5 * 1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))]

= 44.45 N./mm^2

Bending Stress at Short-side 1 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 22.00 * 122.002 * 10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))

= 61.15 N./mm^2

Bending Stress at Short-side 1 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 22.00 * 122.002 * -10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))

= -61.15 N./mm^2

Bending Stresses at Short-side 2

Bending Stress at Short-side 2 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 22.00 * 10.00 / ( 24 * 0.07 ) * [ -3 * 160.002 + 2 * 122.002 *

(( 3 + 5 * 1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))]

= -44.45 N./mm^2

Bending Stress at Short-side 2 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

03

03

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 22.00 * -10.00 /( 24 * 0.07 ) * [ -3 * 160.002 + 2 * 122.002 *

(( 3 + 5 * 1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))]

= 44.45 N./mm^2

Bending Stress at Short-side 2 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 22.00 * 122.002 * 10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))

= 61.15 N./mm^2

Bending Stress at Short-side 2 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 22.00 * 122.002 * -10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))

= -61.15 N./mm^2

Bending Stresses at Long-side 1

Bending Stress at Long-side 1 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 22.00 * 122.002 * 10.00 / ( 12 * 0.07 ) * [( 3 + 1.31 *

( 6 - 1.312)) / ( 3 + 5 * 1.31 )]

= 36.89 N./mm^2

If Em( 0.580 ) < E( 0.850 ) and Eb( 0.580 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 36.89 / 0.58

= 63.66 N./mm^2

Bending Stress at Long-side 1 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 22.00 * 122.002 * -10.00 / ( 12 * 0.07 ) * [( 3 + 1.31 *

( 6 - 1.312)) / ( 3 + 5 * 1.31 )]

= -36.89 N./mm^2

If Em( 0.580 ) < E( 0.850 ) and Eb( 0.580 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -36.89 / 0.58

= -63.66 N./mm^2

Bending Stress at Long-side 1 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 22.00 * 122.002 * 10.00 / ( 12 * 0.07 ) * [( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 )]

= 61.15 N./mm^2

Bending Stress at Long-side 1 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 22.00 * 122.002 * -10.00 / ( 12 * 0.07 ) * [( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 )]

= -61.15 N./mm^2

Bending Stresses at Long-side 2

03

03

Bending Stress at Long-side 2 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 22.00 * 122.002 * 9.71 / ( 12 * 0.07 ) * [( 3 + 1.31 *

( 6 - 1.312)) / ( 3 + 5 * 1.31 )]

= 35.83 N./mm^2

If Em( 0.514 ) < E( 0.850 ) and Eb( 0.478 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 35.83 / 0.48

= 74.98 N./mm^2

Bending Stress at Long-side 2 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 22.00 * 122.002 * -10.29 / ( 12 * 0.07 ) * [( 3 + 1.31 *

( 6 - 1.312)) / ( 3 + 5 * 1.31 )]

= -37.95 N./mm^2

If Em( 0.514 ) < E( 0.850 ) and Eb( 0.478 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -37.95 / 0.48

= -79.42 N./mm^2

Bending Stress at Long-side 2 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 22.00 * 122.002 * 9.71 / ( 12 * 0.07 ) * [( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 )]

= 59.40 N./mm^2

Bending Stress at Long-side 2 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 22.00 * 122.002 * -10.29 / ( 12 * 0.07 ) * [( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 )]

= -62.91 N./mm^2

BENDING STRESSES:   Bending Stress Calculations per Section 13-9,

Equations (16-19). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -44.45       44.45       145.85

at Q                    61.15      -61.15       145.85

Short-side 2 at N                   -44.45       44.45       145.85

at Q                    61.15      -61.15       145.85

Long-side  1 at M                    63.66      -63.66       171.58

at Q                    61.15      -61.15       145.85

Long-side  2 at M                    74.98      -79.42       171.58

at Q                    59.40      -62.91       145.85

Total Stress Calculations per Section 13-9

Total Stresses at Short-side 1

Total Stress at short side 1 at N inner [STS_Ni]:

= Sms + SbsNi

03

03

= 7.37 + -44.45

= -37.08 N./mm^2

Total Stress at short side 1 at N outer [STS_No]:

= Sms + SbsNo

= 7.37 + 44.45

= 51.83 N./mm^2

Total Stress at short side 1 at Q inner [STS_Qi]:

= Sms + SbsQi

= 7.37 + 61.15

= 68.53 N./mm^2

Total Stress at short side 1 at Q outer [STS_Qo]:

= Sms + SbsQo

= 7.37 + -61.15

= -53.78 N./mm^2

Total Stresses at Short-side 2

Total Stress at short side 2 at N inner [STS_Ni]:

= Sms + SbsNi

= 7.37 + -44.45

= -37.08 N./mm^2

Total Stress at short side 2 at N outer [STS_No]:

= Sms + SbsNo

= 7.37 + 44.45

= 51.83 N./mm^2

Total Stress at short side 2 at Q inner [STS_Qi]:

= Sms + SbsQi

= 7.37 + 61.15

= 68.53 N./mm^2

Total Stress at short side 2 at Q outer [STS_Qo]:

= Sms + SbsQo

= 7.37 + -61.15

= -53.78 N./mm^2

Total Stresses at Long-side 1

Total Stress at long side 1 at M inner [STL_Mi]:

= Sml + SblMi

= 15.19 + 63.66

= 78.84 N./mm^2

Total Stress at long side 1 at M outer [STL_Mo]:

= Sml + SblMo

= 15.19 + -63.66

= -48.47 N./mm^2

Total Stress at long side 1 at Q inner [STL_Qi]:

= Sml + SblQi

= 8.80 + 61.15

= 69.95 N./mm^2

Total Stress at long side 1 at Q outer [STL_Qo]:

= SmlB + SblQo

= 8.80 + -61.15

= -52.35 N./mm^2

03

03

Total Stresses at Long-side 2

Total Stress at long side 2 at M inner [STL_Mi]:

= Sml + SblMi

= 17.11 + 74.98

= 92.08 N./mm^2

Total Stress at long side 2 at M outer [STL_Mo]:

= Sml + SblMo

= 17.11 + -79.42

= -62.31 N./mm^2

Total Stress at long side 2 at Q inner [STL_Qi]:

= Sml + SblQi

= 8.80 + 59.40

= 68.20 N./mm^2

Total Stress at long side 2 at Q outer [STL_Qo]:

= SmlB + SblQo

= 8.80 + -62.91

= -54.11 N./mm^2

TOTAL STRESSES:     Total Stress Calculations per Section 13-9,

Equations (20-24). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -37.08       51.83       145.85

at Q                    68.53      -53.78       145.85

Short-side 2 at N                   -37.08       51.83       145.85

at Q                    68.53      -53.78       145.85

Long-side  1 at M                    78.84      -48.47       171.58

at Q                    69.95      -52.35       145.85

Long-side  2 at M                    92.08      -62.31       171.58

at Q                    68.20      -54.11       145.85

End Plate Stresses (N./mm^2):

Actual    Allowable

End Plate                                        67.73       114.39

Required End Plate thickness due to Internal Pressure [trEP]:

= d * sqrt( Z * C * P / ( SE ) ) + ca

= 160.000 * sqrt( 2.405 * 0.200 * 22.000 / ( 114.390 ) ) + 0.000

= 15.390 mm.

End Plate MAWP at given Thickness [MAWPEP]:

= ((T-ca)/d)^2 * ((SE)/(C*Z)) per UG-34 (c)(3)

= ((20.0000-0.0000)/160.0000)^2*((114)/(.20*2.41))

= 37.154 bars

where Z is:

= min( 3.4 - 2.4( d/D ), 2.5 )

= min( 3.4 - 2.4( 160.000 / 386.000 ), 2.5 )

= 2.405

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

03

03

High Stress (Highest % of Allowable)             25.52   N./mm^2

High Stress Percentage                           22.31   %

M.A.W.P. for Membrane Stresses                   98.63   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)            -79.42   N./mm^2

High Stress Percentage                           46.28   %

M.A.W.P. for Bending Stresses                    47.53   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)             92.08   N./mm^2

High Stress Percentage                           53.67   %

M.A.W.P. for Total Stresses                      40.99   bars

Rectangular Vessel Results For Item 1 : A8

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)             25.52   N./mm^2

High Stress Percentage                           22.31   %

M.A.W.P. for Membrane Stresses                   98.63   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)            -79.42   N./mm^2

High Stress Percentage                           46.28   %

M.A.W.P. for Bending Stresses                    47.53   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)             92.08   N./mm^2

High Stress Percentage                           53.67   %

M.A.W.P. for Total Stresses                      40.99   bars

## Stationary Header Calculation @ test Pressure for ae-2101.

03

03

Input Echo, COMPONENT      1,         Description: St.AE-2101

Figure Number Analyzed                                    A8

Design Internal Pressure                   P         28.6000  bars

Design Temperature                      Temp         25.0000  C

VESSEL MATERIAL DATA:

Material Specification                           SA-240 316L

Shell Allowable Stress at Design Temp      S        155.0000  N./mm^2

Shell Allowable Stress at Ambient         SA        155.0000  N./mm^2

Shell Yield Stress at Design Temperature  Sy        172.3750  N./mm^2

SHORT-SIDE VESSEL DATA:

Short-side Length Dimension                H        160.0000  mm.

Minimum Thickness of Short-side Plates    t1         20.0000  mm.

Mid-side Joint Efficiency on Short-side    E          0.8500

Corner Joint Efficiency on Short-side     EC          0.8500

LONG-SIDE VESSEL DATA:

Long-side Length Dimension                 h        122.0000  mm.

Minimum Thickness of Long-side Plates     t2         20.0000  mm.

Mid-side Joint Efficiency on Long-side     E          0.8500

ADDITIONAL VESSEL DATA:

Minimum Thickness of End Plate            t5         20.0000  mm.

C-Factor for End Plate                Cf_Epl          0.2000

Long-side Plate # 1,

Pitch Distance                          p         61.0000  mm.

Uniform Hole Diameter                  d0         25.6500  mm.

Depth of Holes                         T0         20.0000  mm.

Long-side Plate # 2,

Pitch Distance                          p         61.0000  mm.

# 1: Hole Diameter                     d0         36.3000  mm.

Hole Depth                        T0          2.7000  mm.

# 2: Hole Diameter                     d1         28.5750  mm.

Hole Depth                        T1         17.3000  mm.

STAY PLATE MATERIAL DATA:

Stay Material Specification                      SA-240 316L

Stay Allowable Stress at Design Temp      Sr        114.3898  N./mm^2

Stay Allowable Stress at Ambient          SA        115.1465  N./mm^2

Stay Yield Stress at Design Temp          Sy        129.8699  N./mm^2

STAY PLATE DATA:

Minimum Thickness of Stay                 t3         10.0000  mm.

Minimum Thickness of Stay                 t4         10.0000  mm.

The Stay(s) Are Not Welded to the End  Plate

Rectangular Vessel Results, Item number     1, Desc: St.AE-2101

ASME Code, Section VIII, Division 1, 2019 App. 13

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Short-side 1 Calculations

Membrane Ligament Efficiency [Em]:

03

03

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Short-side 2 Calculations

Membrane Ligament Efficiency [Em]:

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Long-side 1 Calculations

Effective Diameter [De]: 25.650 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 61.000 - 25.650 / 61.000

= 0.580

Bending Ligament Efficiency [Eb]:

As diameter holes are uniform Eb = Em

= 0.580

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Long-side 2 Calculations

Effective Diameter [De]:

= ( d0 * T0 + d1 * T1 + d2 * T2 ) / ( t1 - CA )

= ( 36.30 * 2.70 + 28.57 * 17.30 + 0.00 * 0.00 ) /

( 20.00 - 0.00 )

03

03

= 29.618 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 61.000 - 29.618 / 61.000

= 0.514

Dist from Neutral axis of c/s to extreme fibers [Ci & Co]:

Calculation of Xbar:

= (( b0 * T0 * ( T0/2 + T1 + T2 )) + ( b1 * T1 *

( T1/2 + T2 )) + ( b2 * T2 * ( T2/2 ))) /

( b0 * T0 + b1 * T1 + b2 * T2 )

= (( 0.97 * 2.70 * ( 2.70 /2 + 17.30 + 0.00 )) + ( 1.28* 17.30 *

( 17.30 /2 + 0.00 )) + ( 2.40 * 0.00 * ( 0.00 /2 ))) /

( 0.97 * 2.70 + 1.28 * 17.30 + 2.40 * 0.00 )

= 9.713 mm.

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

Ci = Xbar

= 9.713 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t - CA - Xbar )

= -( 20.000 - 0.000 - 9.713 )

= -10.287 mm.

Moment of Inertia (Section 13-6, Equation (5)) [I]:

= 0.079 cm**4

Effective Diameter [De]:

= Pitch - (( 6 * I ) / (( t - CA)^2 * (-Co)))

= 61.00 - (( 6 * 0.08 ) / ((20.00 - 0.00 )^2 * (10.29 )))

= 31.851 mm.

Bending Ligament Efficiency [Eb]:

= Pitch - De / Pitch

= 61.000 - 31.851 / 61.000

= 0.478

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Em          Eb          Ci          Co

Short-side 1       0.850       0.850      10.000     -10.000

2       0.850       0.850      10.000     -10.000

Long-side  1       0.580       0.580      10.000     -10.000

2       0.514       0.478       9.713     -10.287

Moment of Inertia of a Strip of the Vessel Wall:

Thickness  t1, I1  = 0.0667 cm**4

Thickness  t2, I2  = 0.0667 cm**4

Rectangular Vessel Parameters:

Alpha =      H / h    = 1.3115

K     = (I2/I1)*Alpha = 1.3115

03

03

Membrane Stress Calculations per Section 13-9

Membrane Stresses at Short-side 1

Membrane Stress at Short-side 1 [Sms]:

= P * h / ( 2 * t1 ) * { 3 - [( 6 + K *

( 11 - alpha2) / ( 3 + 5 * K )]}

= 28.60 * 122.00 / ( 2 * 20.00 ) * { 3- [( 6 + 1.31 *

( 11 - 1.312) / ( 3 + 5 * 1.31 )]}

= 9.59 N./mm^2

Membrane Stresses at Short-side 2

Membrane Stress at Short-side 2 [Sms]:

= P * h / ( 2 * t1 ) * { 3 - [( 6 + K *

( 11 - alpha2) / ( 3 + 5 * K )]}

= 28.60 * 122.00 / ( 2 * 20.00 ) * { 3- [( 6 + 1.31 *

( 11 - 1.312) / ( 3 + 5 * 1.31 )]}

= 9.59 N./mm^2

Membrane Stresses at Long-side 1

Membrane Stress at Long-side 1 at A [Sml]:

= P * H / 2 * t2

= 28.60 * 160.00 / 2 * 20.00

= 11.44 N./mm^2

If Em( 0.580 ) < E( 0.850 ) and Eb( 0.580 ) < E( 0.850 ) then

Sml =  Sml / Em

= 11.44 / 0.58

= 19.74 N./mm^2

Membrane Stresses at Long-side 2

Membrane Stress at Long-side 2 at A [Sml]:

= P * H / 2 * t2

= 28.60 * 160.00 / 2 * 20.00

= 11.44 N./mm^2

If Em( 0.514 ) < E( 0.850 ) and Eb( 0.478 ) < E( 0.850 ) then

Sml =  Sml / Em

= 11.44 / 0.51

= 22.24 N./mm^2

Membrane Stresses at Stay Plate

Membrane Stress at Stay Plate [t3]:

= P * h / ( 2 * t3 ) * [( 6 + K * ( 11 -

alpha2) / ( 3 + 5 * K )]

= 28.60 * 122.00 / ( 2 * 10.00 ) * [( 6 + 1.31 * ( 11 -

1.312) / ( 3 + 5 * 1.31 )]

= 33.17 N./mm^2

Membrane Stress at Stay Plate [t4]:

= P * h / ( 2 * t4 ) * [( 6 + K * ( 11 -

alpha2) / ( 3 + 5 * K )]

= 28.60 * 122.00 / ( 2 * 10.00 ) * [( 6 + 1.31 * ( 11 -

1.312) / ( 3 + 5 * 1.31 )]

= 33.17 N./mm^2

MEMBRANE STRESSES:  Membrane Stress Calculations per Section 13-9,

Equations (13-15). (N./mm^2) :

03

03

STRESS LOCATIONS                                Actual    Allowable

----------------------------------------------------------------------

Short-side 1                                      9.59       131.75

Short-side 2                                      9.59       131.75

Short-side Corner                                 9.59       131.75

Long-side  1 at A                                19.74       155.00

Long-side  2 at A                                22.24       155.00

Long-side Corner                                 11.44       131.75

Stay Plate (t3)                                  33.17       114.39

Stay Plate (t4)                                  33.17       114.39

Bending Stress Calculations per Section 13-9

Bending Stresses at Short-side 1

Bending Stress at Short-side 1 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 28.60 * 10.00 / ( 24 * 0.07 ) * [ -3 * 160.002 + 2 * 122.002 *

(( 3 + 5 * 1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))]

= -57.79 N./mm^2

Bending Stress at Short-side 1 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 28.60 * -10.00 /( 24 * 0.07 ) * [ -3 * 160.002 + 2 * 122.002 *

(( 3 + 5 * 1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))]

= 57.79 N./mm^2

Bending Stress at Short-side 1 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 28.60 * 122.002 * 10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))

= 79.50 N./mm^2

Bending Stress at Short-side 1 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 28.60 * 122.002 * -10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))

= -79.50 N./mm^2

Bending Stresses at Short-side 2

Bending Stress at Short-side 2 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 28.60 * 10.00 / ( 24 * 0.07 ) * [ -3 * 160.002 + 2 * 122.002 *

(( 3 + 5 * 1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))]

= -57.79 N./mm^2

Bending Stress at Short-side 2 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 28.60 * -10.00 /( 24 * 0.07 ) * [ -3 * 160.002 + 2 * 122.002 *

(( 3 + 5 * 1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))]

= 57.79 N./mm^2

03

03

Bending Stress at Short-side 2 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 28.60 * 122.002 * 10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))

= 79.50 N./mm^2

Bending Stress at Short-side 2 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 28.60 * 122.002 * -10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 ))

= -79.50 N./mm^2

Bending Stresses at Long-side 1

Bending Stress at Long-side 1 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 28.60 * 122.002 * 10.00 / ( 12 * 0.07 ) * [( 3 + 1.31 *

( 6 - 1.312)) / ( 3 + 5 * 1.31 )]

= 47.96 N./mm^2

If Em( 0.580 ) < E( 0.850 ) and Eb( 0.580 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 47.96 / 0.58

= 82.75 N./mm^2

Bending Stress at Long-side 1 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 28.60 * 122.002 * -10.00 / ( 12 * 0.07 ) * [( 3 + 1.31 *

( 6 - 1.312)) / ( 3 + 5 * 1.31 )]

= -47.96 N./mm^2

If Em( 0.580 ) < E( 0.850 ) and Eb( 0.580 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -47.96 / 0.58

= -82.75 N./mm^2

Bending Stress at Long-side 1 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 28.60 * 122.002 * 10.00 / ( 12 * 0.07 ) * [( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 )]

= 79.50 N./mm^2

Bending Stress at Long-side 1 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 28.60 * 122.002 * -10.00 / ( 12 * 0.07 ) * [( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 )]

= -79.50 N./mm^2

Bending Stresses at Long-side 2

Bending Stress at Long-side 2 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 28.60 * 122.002 * 9.71 / ( 12 * 0.07 ) * [( 3 + 1.31 *

03

03

( 6 - 1.312)) / ( 3 + 5 * 1.31 )]

= 46.58 N./mm^2

If Em( 0.514 ) < E( 0.850 ) and Eb( 0.478 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 46.58 / 0.48

= 97.47 N./mm^2

Bending Stress at Long-side 2 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 28.60 * 122.002 * -10.29 / ( 12 * 0.07 ) * [( 3 + 1.31 *

( 6 - 1.312)) / ( 3 + 5 * 1.31 )]

= -49.33 N./mm^2

If Em( 0.514 ) < E( 0.850 ) and Eb( 0.478 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -49.33 / 0.48

= -103.24 N./mm^2

Bending Stress at Long-side 2 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 28.60 * 122.002 * 9.71 / ( 12 * 0.07 ) * [( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 )]

= 77.21 N./mm^2

Bending Stress at Long-side 2 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 28.60 * 122.002 * -10.29 / ( 12 * 0.07 ) * [( 3 + 5 *

1.312 * 1.31 ) / ( 3 + 5 * 1.31 )]

= -81.78 N./mm^2

BENDING STRESSES:   Bending Stress Calculations per Section 13-9,

Equations (16-19). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -57.79       57.79       197.62

at Q                    79.50      -79.50       197.62

Short-side 2 at N                   -57.79       57.79       197.62

at Q                    79.50      -79.50       197.62

Long-side  1 at M                    82.75      -82.75       232.50

at Q                    79.50      -79.50       197.62

Long-side  2 at M                    97.47     -103.24       232.50

at Q                    77.21      -81.78       197.62

Total Stress Calculations per Section 13-9

Total Stresses at Short-side 1

Total Stress at short side 1 at N inner [STS_Ni]:

= Sms + SbsNi

= 9.59 + -57.79

= -48.20 N./mm^2

03

03

Total Stress at short side 1 at N outer [STS_No]:

= Sms + SbsNo

= 9.59 + 57.79

= 67.37 N./mm^2

Total Stress at short side 1 at Q inner [STS_Qi]:

= Sms + SbsQi

= 9.59 + 79.50

= 89.08 N./mm^2

Total Stress at short side 1 at Q outer [STS_Qo]:

= Sms + SbsQo

= 9.59 + -79.50

= -69.91 N./mm^2

Total Stresses at Short-side 2

Total Stress at short side 2 at N inner [STS_Ni]:

= Sms + SbsNi

= 9.59 + -57.79

= -48.20 N./mm^2

Total Stress at short side 2 at N outer [STS_No]:

= Sms + SbsNo

= 9.59 + 57.79

= 67.37 N./mm^2

Total Stress at short side 2 at Q inner [STS_Qi]:

= Sms + SbsQi

= 9.59 + 79.50

= 89.08 N./mm^2

Total Stress at short side 2 at Q outer [STS_Qo]:

= Sms + SbsQo

= 9.59 + -79.50

= -69.91 N./mm^2

Total Stresses at Long-side 1

Total Stress at long side 1 at M inner [STL_Mi]:

= Sml + SblMi

= 19.74 + 82.75

= 102.50 N./mm^2

Total Stress at long side 1 at M outer [STL_Mo]:

= Sml + SblMo

= 19.74 + -82.75

= -63.01 N./mm^2

Total Stress at long side 1 at Q inner [STL_Qi]:

= Sml + SblQi

= 11.44 + 79.50

= 90.94 N./mm^2

Total Stress at long side 1 at Q outer [STL_Qo]:

= SmlB + SblQo

= 11.44 + -79.50

= -68.06 N./mm^2

Total Stresses at Long-side 2

03

03

Total Stress at long side 2 at M inner [STL_Mi]:

= Sml + SblMi

= 22.24 + 97.47

= 119.71 N./mm^2

Total Stress at long side 2 at M outer [STL_Mo]:

= Sml + SblMo

= 22.24 + -103.24

= -81.00 N./mm^2

Total Stress at long side 2 at Q inner [STL_Qi]:

= Sml + SblQi

= 11.44 + 77.21

= 88.65 N./mm^2

Total Stress at long side 2 at Q outer [STL_Qo]:

= SmlB + SblQo

= 11.44 + -81.78

= -70.34 N./mm^2

TOTAL STRESSES:     Total Stress Calculations per Section 13-9,

Equations (20-24). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -48.20       67.37       197.62

at Q                    89.08      -69.91       197.62

Short-side 2 at N                   -48.20       67.37       197.62

at Q                    89.08      -69.91       197.62

Long-side  1 at M                   102.50      -63.01       232.50

at Q                    90.94      -68.06       197.62

Long-side  2 at M                   119.71      -81.00       232.50

at Q                    88.65      -70.34       197.62

End Plate Stresses (N./mm^2):

Actual    Allowable

End Plate                                        88.05       155.00

Required End Plate thickness due to Internal Pressure [trEP]:

= d * sqrt( Z * C * P / ( SE ) ) + ca

= 160.000 * sqrt( 2.405 * 0.200 * 28.600 / ( 155.000 ) ) + 0.000

= 15.074 mm.

End Plate MAWP at given Thickness [MAWPEP]:

= ((T-ca)/d)^2 * ((SE)/(C*Z)) per UG-34 (c)(3)

= ((20.0000-0.0000)/160.0000)^2*((155)/(.20*2.41))

= 50.344 bars

where Z is:

= min( 3.4 - 2.4( d/D ), 2.5 )

= min( 3.4 - 2.4( 160.000 / 386.000 ), 2.5 )

= 2.405

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)             33.17   N./mm^2

High Stress Percentage                           29.00   %

M.A.W.P. for Membrane Stresses                   98.63   bars

03

03

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)           -103.24   N./mm^2

High Stress Percentage                           44.40   %

M.A.W.P. for Bending Stresses                    64.41   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)            119.71   N./mm^2

High Stress Percentage                           51.49   %

M.A.W.P. for Total Stresses                      55.55   bars

Rectangular Vessel Results For Item 1 : A8

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)             33.17   N./mm^2

High Stress Percentage                           29.00   %

M.A.W.P. for Membrane Stresses                   98.63   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)           -103.24   N./mm^2

High Stress Percentage                           44.40   %

M.A.W.P. for Bending Stresses                    64.41   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)            119.71   N./mm^2

High Stress Percentage                           51.49   %

M.A.W.P. for Total Stresses                      55.55   bars

## floating Header Calculation @ Design Pressure for ae-2101.

Input Echo, COMPONENT      1,         Description: Fl.AE-2101

Figure Number Analyzed                                    A7

Design Internal Pressure                   P         22.0000  bars

Design Temperature                      Temp        155.0000  C

VESSEL MATERIAL DATA:

Material Specification                           SA-240 316L

Shell Allowable Stress at Design Temp      S        114.3898  N./mm^2

Shell Allowable Stress at Ambient         SA        115.1465  N./mm^2

Shell Yield Stress at Design Temperature  Sy        129.8699  N./mm^2

SHORT-SIDE VESSEL DATA:

Short-side Length Dimension                H        100.0000  mm.

Minimum Thickness of Short-side Plates    t1         15.0000  mm.

Mid-side Joint Efficiency on Short-side    E          0.8500

Corner Joint Efficiency on Short-side     EC          0.8500

LONG-SIDE VESSEL DATA:

Long-side Length Dimension                 h        122.0000  mm.

Minimum Thickness of Long-side Plates     t2         20.0000  mm.

Mid-side Joint Efficiency on Long-side     E          0.8500

ADDITIONAL VESSEL DATA:

Minimum Thickness of End Plate            t5         20.0000  mm.

C-Factor for End Plate                Cf_Epl          0.2000

Long-side Plate # 1,

Pitch Distance                          p         61.0000  mm.

Uniform Hole Diameter                  d0         25.6500  mm.

Depth of Holes                         T0         20.0000  mm.

Long-side Plate # 2,

Pitch Distance                          p         61.0000  mm.

# 1: Hole Diameter                     d0         36.3000  mm.

Hole Depth                        T0          2.7000  mm.

# 2: Hole Diameter                     d1         28.5750  mm.

Hole Depth                        T1         17.3000  mm.

STAY PLATE MATERIAL DATA:

Stay Material Specification                      SA-240 316L

Stay Allowable Stress at Design Temp      Sr        114.3898  N./mm^2

Stay Allowable Stress at Ambient          SA        115.1465  N./mm^2

Stay Yield Stress at Design Temp          Sy        129.8699  N./mm^2

STAY PLATE DATA:

Minimum Thickness of Stay                 t3         10.0000  mm.

The Stay(s) Are Not Welded to the End  Plate

Rectangular Vessel Results, Item number     1, Desc: Fl.AE-2101

ASME Code, Section VIII, Division 1, 2019 App. 13

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Short-side 1 Calculations

Membrane Ligament Efficiency [Em]:

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 15.000 - 0.000 / 2

= 7.500 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 15.000 - 0.000 ) / 2

= -7.500 mm.

Short-side 2 Calculations

Membrane Ligament Efficiency [Em]:

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 15.000 - 0.000 / 2

= 7.500 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 15.000 - 0.000 ) / 2

= -7.500 mm.

Long-side 1 Calculations

Effective Diameter [De]: 25.700 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 61.000 - 25.700 / 61.000

= 0.579

Bending Ligament Efficiency [Eb]:

As diameter holes are uniform Eb = Em

= 0.579

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Long-side 2 Calculations

Effective Diameter [De]:

= ( d0 * T0 + d1 * T1 + d2 * T2 ) / ( t1 - CA )

= ( 36.30 * 2.70 + 28.57 * 17.30 + 0.00 * 0.00 ) /

( 20.00 - 0.00 )

= 29.618 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 61.000 - 29.618 / 61.000

= 0.514

Dist from Neutral axis of c/s to extreme fibers [Ci & Co]:

Calculation of Xbar:

= (( b0 * T0 * ( T0/2 + T1 + T2 )) + ( b1 * T1 *

( T1/2 + T2 )) + ( b2 * T2 * ( T2/2 ))) /

( b0 * T0 + b1 * T1 + b2 * T2 )

= (( 0.97 * 2.70 * ( 2.70 /2 + 17.30 + 0.00 )) + ( 1.28* 17.30 *

( 17.30 /2 + 0.00 )) + ( 2.40 * 0.00 * ( 0.00 /2 ))) /

( 0.97 * 2.70 + 1.28 * 17.30 + 2.40 * 0.00 )

= 9.713 mm.

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

Ci = Xbar

= 9.713 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t - CA - Xbar )

= -( 20.000 - 0.000 - 9.713 )

= -10.287 mm.

Moment of Inertia (Section 13-6, Equation (5)) [I]:

= 0.079 cm**4

Effective Diameter [De]:

= Pitch - (( 6 * I ) / (( t - CA)^2 * (-Co)))

= 61.00 - (( 6 * 0.08 ) / ((20.00 - 0.00 )^2 * (10.29 )))

= 31.851 mm.

Bending Ligament Efficiency [Eb]:

= Pitch - De / Pitch

= 61.000 - 31.851 / 61.000

= 0.478

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Em          Eb          Ci          Co

Short-side 1       0.850       0.850       7.500      -7.500

2       0.850       0.850       7.500      -7.500

Long-side  1       0.579       0.579      10.000     -10.000

2       0.514       0.478       9.713     -10.287

Moment of Inertia of a Strip of the Vessel Wall:

Thickness  t1, I1  = 0.0281 cm**4

Thickness  t2, I2  = 0.0667 cm**4

Rectangular Vessel Parameters:

Alpha =      H / h    = 0.8197

K     = (I2/I1)*Alpha = 1.9429

Membrane Stress Calculations per Section 13-9

Membrane Stresses at Short-side 1

Membrane Stress at Short-side 1 [Sms]:

= p * h / ( 4 * t1 ) * { 4 - [( 2 + K *

( 5 - alpha2)) / ( 1 + 2 * K )]}

= 22.00 * 122.00 / ( 4 * 15.00 ) * { 4 - [( 2 + 1.94 *

( 5 - 0.822)) / ( 1 + 2 * 1.94 )]}

= 8.36 N./mm^2

Membrane Stresses at Short-side 2

Membrane Stress at Short-side 2 [Sms]:

= p * h / ( 4 * t1 ) * { 4 - [( 2 + K *

( 5 - alpha2)) / ( 1 + 2 * K )]}

= 22.00 * 122.00 / ( 4 * 15.00 ) * { 4 - [( 2 + 1.94 *

( 5 - 0.822)) / ( 1 + 2 * 1.94 )]}

= 8.36 N./mm^2

Membrane Stresses at Long-side 1

Membrane Stress at Long-side 1 at A [Sml]:

= P * H / 2 * t2

= 22.00 * 100.00 / 2 * 20.00

= 5.50 N./mm^2

If Em( 0.579 ) < E( 0.850 ) and Eb( 0.579 ) < E( 0.850 ) then

Sml =  Sml / Em

= 5.50 / 0.58

= 9.50 N./mm^2

Membrane Stresses at Long-side 2

Membrane Stress at Long-side 2 at A [Sml]:

= P * H / 2 * t2

= 22.00 * 100.00 / 2 * 20.00

= 5.50 N./mm^2

If Em( 0.514 ) < E( 0.850 ) and Eb( 0.478 ) < E( 0.850 ) then

Sml =  Sml / Em

= 5.50 / 0.51

= 10.69 N./mm^2

Membrane Stresses at Stay Plate

Membrane Stress at Stay Plate [Smsp]:

= P * h / ( 2 * t3 ) * [( 2 + K * ( 5 -

alpha2)) / ( 1 + 2 * K )]

= 22.00 * 122.00 / ( 2 * 10.00 ) * [( 2 + 1.94 * ( 5 -

0.822)) / ( 1 + 2 * 1.94 )]

= 28.59 N./mm^2

MEMBRANE STRESSES:  Membrane Stress Calculations per Section 13-9,

Equations (1-3). (N./mm^2) :

STRESS LOCATIONS                                Actual    Allowable

----------------------------------------------------------------------

Short-side 1                                      8.36        97.23

Short-side 2                                      8.36        97.23

Short-side Corner                                 8.36        97.23

Long-side  1 at A                                 9.50       114.39

Long-side  2 at A                                10.69       114.39

Long-side Corner                                  5.50        97.23

Stay Plate (t3)                                  28.59       114.39

Bending Stress Calculations per Section 13-9

Bending Stresses at Short-side 1

Bending Stress at Short-side 1 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 1 + 2 * Alpha2 * K ) / ( 1 + 2 * K ))]

= 22.00 * 7.50 / ( 24 * 0.03 ) * [ -3 * 100.002 + 2 * 122.002 *

(( 1 + 2 * 0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))]

= -19.56 N./mm^2

Bending Stress at Short-side 1 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 1 + 2 * Alpha2 * K ) / ( 1 + 2 * K ))]

= 22.00 * -7.50 / ( 24 * 0.03 ) * [ -3 * 100.002 + 2 * 122.002 *

(( 1 + 2 * 0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))]

= 19.56 N./mm^2

Bending Stress at Short-side 1 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K ))

= 22.00 * 122.002 * 7.50 / ( 12 * 0.03 ) * (( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))

= 53.78 N./mm^2

Bending Stress at Short-side 1 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K ))

= 22.00 * 122.002 * -7.50 / ( 12 * 0.03 ) * (( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))

= -53.78 N./mm^2

Bending Stresses at Short-side 2

Bending Stress at Short-side 2 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 1 + 2 * Alpha2 * K ) / ( 1 + 2 * K ))]

= 22.00 * 7.50 / ( 24 * 0.03 ) * [ -3 * 100.002 + 2 * 122.002 *

(( 1 + 2 * 0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))]

= -19.56 N./mm^2

Bending Stress at Short-side 2 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 1 + 2 * Alpha2 * K ) / ( 1 + 2 * K ))]

= 22.00 * -7.50 / ( 24 * 0.03 ) * [ -3 * 100.002 + 2 * 122.002 *

(( 1 + 2 * 0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))]

= 19.56 N./mm^2

Bending Stress at Short-side 2 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K ))

= 22.00 * 122.002 * 7.50 / ( 12 * 0.03 ) * (( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))

= 53.78 N./mm^2

Bending Stress at Short-side 2 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K ))

= 22.00 * 122.002 * -7.50 / ( 12 * 0.03 ) * (( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))

= -53.78 N./mm^2

Bending Stresses at Long-side 1

Bending Stress at Long-side 1 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + K *

( 3 - Alpha2)) / ( 1 + 2 * K )]

= 22.00 * 122.002 * 10.00 / ( 12 * 0.07 ) * [( 1 + 1.94 *

( 3 - 0.822)) / ( 1 + 2 * 1.94 )]

= 46.27 N./mm^2

If Em( 0.579 ) < E( 0.850 ) and Eb( 0.579 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 46.27 / 0.58

= 79.96 N./mm^2

Bending Stress at Long-side 1 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + K *

( 3 - Alpha2)) / ( 1 + 2 * K )]

= 22.00 * 122.002 * -10.00 / ( 12 * 0.07 ) * [( 1 + 1.94 *

( 3 - 0.822)) / ( 1 + 2 * 1.94 )]

= -46.27 N./mm^2

If Em( 0.579 ) < E( 0.850 ) and Eb( 0.579 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -46.27 / 0.58

= -79.96 N./mm^2

Bending Stress at Long-side 1 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K )]

= 22.00 * 122.002 * 10.00 / ( 12 * 0.07 ) * [( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 )]

= 30.25 N./mm^2

Bending Stress at Long-side 1 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K )]

= 22.00 * 122.002 * -10.00 / ( 12 * 0.07 ) * [( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 )]

= -30.25 N./mm^2

Bending Stresses at Long-side 2

Bending Stress at Long-side 2 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + K *

( 3 - Alpha2)) / ( 1 + 2 * K )]

= 22.00 * 122.002 * 9.71 / ( 12 * 0.07 ) * [( 1 + 1.94 *

( 3 - 0.822)) / ( 1 + 2 * 1.94 )]

= 44.94 N./mm^2

If Em( 0.514 ) < E( 0.850 ) and Eb( 0.478 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 44.94 / 0.48

= 94.05 N./mm^2

Bending Stress at Long-side 2 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + K *

( 3 - Alpha2)) / ( 1 + 2 * K )]

= 22.00 * 122.002 * -10.29 / ( 12 * 0.07 ) * [( 1 + 1.94 *

( 3 - 0.822)) / ( 1 + 2 * 1.94 )]

= -47.60 N./mm^2

If Em( 0.514 ) < E( 0.850 ) and Eb( 0.478 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -47.60 / 0.48

= -99.62 N./mm^2

Bending Stress at Long-side 2 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K )]

= 22.00 * 122.002 * 9.71 / ( 12 * 0.07 ) * [( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 )]

= 29.38 N./mm^2

Bending Stress at Long-side 2 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K )]

= 22.00 * 122.002 * -10.29 / ( 12 * 0.07 ) * [( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 )]

= -31.12 N./mm^2

BENDING STRESSES:   Bending Stress Calculations per Section 13-9,

Equations (4-7). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -19.56       19.56       145.85

at Q                    53.78      -53.78       145.85

Short-side 2 at N                   -19.56       19.56       145.85

at Q                    53.78      -53.78       145.85

Long-side  1 at M                    79.96      -79.96       171.58

at Q                    30.25      -30.25       145.85

Long-side  2 at M                    94.05      -99.62       171.58

at Q                    29.38      -31.12       145.85

Total Stress Calculations per Section 13-9

Total Stresses at Short-side 1

Total Stress at short side 1 at N inner [STS_Ni]:

= Sms + SbsNi

= 8.36 + -19.56

= -11.20 N./mm^2

Total Stress at short side 1 at N outer [STS_No]:

= Sms + SbsNo

= 8.36 + 19.56

= 27.92 N./mm^2

Total Stress at short side 1 at Q inner [STS_Qi]:

= Sms + SbsQi

= 8.36 + 53.78

= 62.14 N./mm^2

Total Stress at short side 1 at Q outer [STS_Qo]:

= Sms + SbsQo

= 8.36 + -53.78

= -45.42 N./mm^2

Total Stresses at Short-side 2

Total Stress at short side 2 at N inner [STS_Ni]:

= Sms + SbsNi

= 8.36 + -19.56

= -11.20 N./mm^2

Total Stress at short side 2 at N outer [STS_No]:

= Sms + SbsNo

= 8.36 + 19.56

= 27.92 N./mm^2

Total Stress at short side 2 at Q inner [STS_Qi]:

= Sms + SbsQi

= 8.36 + 53.78

= 62.14 N./mm^2

Total Stress at short side 2 at Q outer [STS_Qo]:

= Sms + SbsQo

= 8.36 + -53.78

= -45.42 N./mm^2

Total Stresses at Long-side 1

Total Stress at long side 1 at M inner [STL_Mi]:

= Sml + SblMi

= 9.50 + 79.96

= 89.47 N./mm^2

Total Stress at long side 1 at M outer [STL_Mo]:

= Sml + SblMo

= 9.50 + -79.96

= -70.46 N./mm^2

Total Stress at long side 1 at Q inner [STL_Qi]:

= Sml + SblQi

= 5.50 + 30.25

= 35.75 N./mm^2

Total Stress at long side 1 at Q outer [STL_Qo]:

= Sml + SblQo

= 5.50 + -30.25

= -24.75 N./mm^2

Total Stresses at Long-side 2

Total Stress at long side 2 at M inner [STL_Mi]:

= Sml + SblMi

= 10.69 + 94.05

= 104.75 N./mm^2

Total Stress at long side 2 at M outer [STL_Mo]:

= Sml + SblMo

= 10.69 + -99.62

= -88.93 N./mm^2

Total Stress at long side 2 at Q inner [STL_Qi]:

= Sml + SblQi

= 5.50 + 29.38

= 34.88 N./mm^2

Total Stress at long side 2 at Q outer [STL_Qo]:

= Sml + SblQo

= 5.50 + -31.12

= -25.62 N./mm^2

TOTAL STRESSES:     Total Stress Calculations per Section 13-9,

Equations (8-12). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -11.20       27.92       145.85

at Q                    62.14      -45.42       145.85

Short-side 2 at N                   -11.20       27.92       145.85

at Q                    62.14      -45.42       145.85

Long-side  1 at M                    89.47      -70.46       171.58

at Q                    35.75      -24.75       145.85

Long-side  2 at M                   104.75      -88.93       171.58

at Q                    34.88      -25.62       145.85

End Plate Stresses (N./mm^2):

Actual    Allowable

End Plate                                        30.85       114.39

Required End Plate thickness due to Internal Pressure [trEP]:

= d * sqrt( Z * C * P / ( SE ) ) + ca

= 110.000 * sqrt( 2.318 * 0.200 * 22.000 / ( 114.390 ) ) + 0.000

= 10.387 mm.

End Plate MAWP at given Thickness [MAWPEP]:

= ((T-ca)/d)^2 * ((SE)/(C*Z)) per UG-34 (c)(3)

= ((20.0000-0.0000)/110.0000)^2*((114)/(.20*2.32))

= 81.562 bars

where Z is:

= min( 3.4 - 2.4( d/D ), 2.5 )

= min( 3.4 - 2.4( 110.000 / 244.000 ), 2.5 )

= 2.318

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)             28.59   N./mm^2

High Stress Percentage                           25.00   %

M.A.W.P. for Membrane Stresses                   88.01   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)            -99.62   N./mm^2

High Stress Percentage                           58.06   %

M.A.W.P. for Bending Stresses                    37.89   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)            104.75   N./mm^2

High Stress Percentage                           61.05   %

M.A.W.P. for Total Stresses                      36.04   bars

Rectangular Vessel Results For Item 1 : A7

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)             28.59   N./mm^2

High Stress Percentage                           25.00   %

M.A.W.P. for Membrane Stresses                   88.01   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)            -99.62   N./mm^2

High Stress Percentage                           58.06   %

M.A.W.P. for Bending Stresses                    37.89   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)            104.75   N./mm^2

High Stress Percentage                           61.05   %

M.A.W.P. for Total Stresses                      36.04   bars

## floating Header Calculation @ test Pressure for ae-2101.

Input Echo, COMPONENT      1,         Description: Fl.AE-2101

Figure Number Analyzed                                    A7

Design Internal Pressure                   P         28.6000  bars

Design Temperature                      Temp         25.0000  C

VESSEL MATERIAL DATA:

Material Specification                           SA-240 316L

Shell Allowable Stress at Design Temp      S        155.0000  N./mm^2

Shell Allowable Stress at Ambient         SA        155.0000  N./mm^2

Shell Yield Stress at Design Temperature  Sy        172.3750  N./mm^2

SHORT-SIDE VESSEL DATA:

Short-side Length Dimension                H        100.0000  mm.

Minimum Thickness of Short-side Plates    t1         15.0000  mm.

Mid-side Joint Efficiency on Short-side    E          0.8500

Corner Joint Efficiency on Short-side     EC          0.8500

LONG-SIDE VESSEL DATA:

Long-side Length Dimension                 h        122.0000  mm.

Minimum Thickness of Long-side Plates     t2         20.0000  mm.

Mid-side Joint Efficiency on Long-side     E          0.8500

ADDITIONAL VESSEL DATA:

Minimum Thickness of End Plate            t5         20.0000  mm.

C-Factor for End Plate                Cf_Epl          0.2000

Long-side Plate # 1,

Pitch Distance                          p         61.0000  mm.

Uniform Hole Diameter                  d0         25.6500  mm.

Depth of Holes                         T0         20.0000  mm.

Long-side Plate # 2,

Pitch Distance                          p         61.0000  mm.

# 1: Hole Diameter                     d0         36.3000  mm.

Hole Depth                        T0          2.7000  mm.

# 2: Hole Diameter                     d1         28.5750  mm.

Hole Depth                        T1         17.3000  mm.

STAY PLATE MATERIAL DATA:

Stay Material Specification                      SA-240 316L

Stay Allowable Stress at Design Temp      Sr        114.3898  N./mm^2

Stay Allowable Stress at Ambient          SA        115.1465  N./mm^2

Stay Yield Stress at Design Temp          Sy        129.8699  N./mm^2

STAY PLATE DATA:

Minimum Thickness of Stay                 t3         10.0000  mm.

The Stay(s) Are Not Welded to the End  Plate

Rectangular Vessel Results, Item number     1, Desc: Fl.AE-2101

ASME Code, Section VIII, Division 1, 2019 App. 13

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Short-side 1 Calculations

Membrane Ligament Efficiency [Em]:

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 15.000 - 0.000 / 2

= 7.500 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 15.000 - 0.000 ) / 2

= -7.500 mm.

Short-side 2 Calculations

Membrane Ligament Efficiency [Em]:

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 15.000 - 0.000 / 2

= 7.500 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 15.000 - 0.000 ) / 2

= -7.500 mm.

Long-side 1 Calculations

Effective Diameter [De]: 25.650 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 61.000 - 25.650 / 61.000

= 0.580

Bending Ligament Efficiency [Eb]:

As diameter holes are uniform Eb = Em

= 0.580

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Long-side 2 Calculations

Effective Diameter [De]:

= ( d0 * T0 + d1 * T1 + d2 * T2 ) / ( t1 - CA )

= ( 36.30 * 2.70 + 28.57 * 17.30 + 0.00 * 0.00 ) /

( 20.00 - 0.00 )

= 29.618 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 61.000 - 29.618 / 61.000

= 0.514

Dist from Neutral axis of c/s to extreme fibers [Ci & Co]:

Calculation of Xbar:

= (( b0 * T0 * ( T0/2 + T1 + T2 )) + ( b1 * T1 *

( T1/2 + T2 )) + ( b2 * T2 * ( T2/2 ))) /

( b0 * T0 + b1 * T1 + b2 * T2 )

= (( 0.97 * 2.70 * ( 2.70 /2 + 17.30 + 0.00 )) + ( 1.28* 17.30 *

( 17.30 /2 + 0.00 )) + ( 2.40 * 0.00 * ( 0.00 /2 ))) /

( 0.97 * 2.70 + 1.28 * 17.30 + 2.40 * 0.00 )

= 9.713 mm.

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

Ci = Xbar

= 9.713 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t - CA - Xbar )

= -( 20.000 - 0.000 - 9.713 )

= -10.287 mm.

Moment of Inertia (Section 13-6, Equation (5)) [I]:

= 0.079 cm**4

Effective Diameter [De]:

= Pitch - (( 6 * I ) / (( t - CA)^2 * (-Co)))

= 61.00 - (( 6 * 0.08 ) / ((20.00 - 0.00 )^2 * (10.29 )))

= 31.851 mm.

Bending Ligament Efficiency [Eb]:

= Pitch - De / Pitch

= 61.000 - 31.851 / 61.000

= 0.478

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Em          Eb          Ci          Co

Short-side 1       0.850       0.850       7.500      -7.500

2       0.850       0.850       7.500      -7.500

Long-side  1       0.580       0.580      10.000     -10.000

2       0.514       0.478       9.713     -10.287

Moment of Inertia of a Strip of the Vessel Wall:

Thickness  t1, I1  = 0.0281 cm**4

Thickness  t2, I2  = 0.0667 cm**4

Rectangular Vessel Parameters:

Alpha =      H / h    = 0.8197

K     = (I2/I1)*Alpha = 1.9429

Membrane Stress Calculations per Section 13-9

Membrane Stresses at Short-side 1

Membrane Stress at Short-side 1 [Sms]:

= p * h / ( 4 * t1 ) * { 4 - [( 2 + K *

( 5 - alpha2)) / ( 1 + 2 * K )]}

= 28.60 * 122.00 / ( 4 * 15.00 ) * { 4 - [( 2 + 1.94 *

( 5 - 0.822)) / ( 1 + 2 * 1.94 )]}

= 10.87 N./mm^2

Membrane Stresses at Short-side 2

Membrane Stress at Short-side 2 [Sms]:

= p * h / ( 4 * t1 ) * { 4 - [( 2 + K *

( 5 - alpha2)) / ( 1 + 2 * K )]}

= 28.60 * 122.00 / ( 4 * 15.00 ) * { 4 - [( 2 + 1.94 *

( 5 - 0.822)) / ( 1 + 2 * 1.94 )]}

= 10.87 N./mm^2

Membrane Stresses at Long-side 1

Membrane Stress at Long-side 1 at A [Sml]:

= P * H / 2 * t2

= 28.60 * 100.00 / 2 * 20.00

= 7.15 N./mm^2

If Em( 0.580 ) < E( 0.850 ) and Eb( 0.580 ) < E( 0.850 ) then

Sml =  Sml / Em

= 7.15 / 0.58

= 12.34 N./mm^2

Membrane Stresses at Long-side 2

Membrane Stress at Long-side 2 at A [Sml]:

= P * H / 2 * t2

= 28.60 * 100.00 / 2 * 20.00

= 7.15 N./mm^2

If Em( 0.514 ) < E( 0.850 ) and Eb( 0.478 ) < E( 0.850 ) then

Sml =  Sml / Em

= 7.15 / 0.51

= 13.90 N./mm^2

Membrane Stresses at Stay Plate

Membrane Stress at Stay Plate [Smsp]:

= P * h / ( 2 * t3 ) * [( 2 + K * ( 5 -

alpha2)) / ( 1 + 2 * K )]

= 28.60 * 122.00 / ( 2 * 10.00 ) * [( 2 + 1.94 * ( 5 -

0.822)) / ( 1 + 2 * 1.94 )]

= 37.17 N./mm^2

MEMBRANE STRESSES:  Membrane Stress Calculations per Section 13-9,

Equations (1-3). (N./mm^2) :

STRESS LOCATIONS                                Actual    Allowable

----------------------------------------------------------------------

Short-side 1                                     10.87       131.75

Short-side 2                                     10.87       131.75

Short-side Corner                                10.87       131.75

Long-side  1 at A                                12.34       155.00

Long-side  2 at A                                13.90       155.00

Long-side Corner                                  7.15       131.75

Stay Plate (t3)                                  37.17       114.39

Bending Stress Calculations per Section 13-9

Bending Stresses at Short-side 1

Bending Stress at Short-side 1 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 1 + 2 * Alpha2 * K ) / ( 1 + 2 * K ))]

= 28.60 * 7.50 / ( 24 * 0.03 ) * [ -3 * 100.002 + 2 * 122.002 *

(( 1 + 2 * 0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))]

= -25.43 N./mm^2

Bending Stress at Short-side 1 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 1 + 2 * Alpha2 * K ) / ( 1 + 2 * K ))]

= 28.60 * -7.50 / ( 24 * 0.03 ) * [ -3 * 100.002 + 2 * 122.002 *

(( 1 + 2 * 0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))]

= 25.43 N./mm^2

Bending Stress at Short-side 1 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K ))

= 28.60 * 122.002 * 7.50 / ( 12 * 0.03 ) * (( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))

= 69.91 N./mm^2

Bending Stress at Short-side 1 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K ))

= 28.60 * 122.002 * -7.50 / ( 12 * 0.03 ) * (( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))

= -69.91 N./mm^2

Bending Stresses at Short-side 2

Bending Stress at Short-side 2 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 1 + 2 * Alpha2 * K ) / ( 1 + 2 * K ))]

= 28.60 * 7.50 / ( 24 * 0.03 ) * [ -3 * 100.002 + 2 * 122.002 *

(( 1 + 2 * 0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))]

= -25.43 N./mm^2

Bending Stress at Short-side 2 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 1 + 2 * Alpha2 * K ) / ( 1 + 2 * K ))]

= 28.60 * -7.50 / ( 24 * 0.03 ) * [ -3 * 100.002 + 2 * 122.002 *

(( 1 + 2 * 0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))]

= 25.43 N./mm^2

Bending Stress at Short-side 2 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K ))

= 28.60 * 122.002 * 7.50 / ( 12 * 0.03 ) * (( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))

= 69.91 N./mm^2

Bending Stress at Short-side 2 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K ))

= 28.60 * 122.002 * -7.50 / ( 12 * 0.03 ) * (( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 ))

= -69.91 N./mm^2

Bending Stresses at Long-side 1

Bending Stress at Long-side 1 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + K *

( 3 - Alpha2)) / ( 1 + 2 * K )]

= 28.60 * 122.002 * 10.00 / ( 12 * 0.07 ) * [( 1 + 1.94 *

( 3 - 0.822)) / ( 1 + 2 * 1.94 )]

= 60.16 N./mm^2

If Em( 0.580 ) < E( 0.850 ) and Eb( 0.580 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 60.16 / 0.58

= 103.81 N./mm^2

Bending Stress at Long-side 1 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + K *

( 3 - Alpha2)) / ( 1 + 2 * K )]

= 28.60 * 122.002 * -10.00 / ( 12 * 0.07 ) * [( 1 + 1.94 *

( 3 - 0.822)) / ( 1 + 2 * 1.94 )]

= -60.16 N./mm^2

If Em( 0.580 ) < E( 0.850 ) and Eb( 0.580 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -60.16 / 0.58

= -103.81 N./mm^2

Bending Stress at Long-side 1 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K )]

= 28.60 * 122.002 * 10.00 / ( 12 * 0.07 ) * [( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 )]

= 39.33 N./mm^2

Bending Stress at Long-side 1 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K )]

= 28.60 * 122.002 * -10.00 / ( 12 * 0.07 ) * [( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 )]

= -39.33 N./mm^2

Bending Stresses at Long-side 2

Bending Stress at Long-side 2 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + K *

( 3 - Alpha2)) / ( 1 + 2 * K )]

= 28.60 * 122.002 * 9.71 / ( 12 * 0.07 ) * [( 1 + 1.94 *

( 3 - 0.822)) / ( 1 + 2 * 1.94 )]

= 58.43 N./mm^2

If Em( 0.514 ) < E( 0.850 ) and Eb( 0.478 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 58.43 / 0.48

= 122.27 N./mm^2

Bending Stress at Long-side 2 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + K *

( 3 - Alpha2)) / ( 1 + 2 * K )]

= 28.60 * 122.002 * -10.29 / ( 12 * 0.07 ) * [( 1 + 1.94 *

( 3 - 0.822)) / ( 1 + 2 * 1.94 )]

= -61.89 N./mm^2

If Em( 0.514 ) < E( 0.850 ) and Eb( 0.478 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -61.89 / 0.48

= -129.51 N./mm^2

Bending Stress at Long-side 2 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K )]

= 28.60 * 122.002 * 9.71 / ( 12 * 0.07 ) * [( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 )]

= 38.20 N./mm^2

Bending Stress at Long-side 2 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 1 + 2 *

Alpha2 * K ) / ( 1 + 2 * K )]

= 28.60 * 122.002 * -10.29 / ( 12 * 0.07 ) * [( 1 + 2 *

0.822 * 1.94 ) / ( 1 + 2 * 1.94 )]

= -40.46 N./mm^2

BENDING STRESSES:   Bending Stress Calculations per Section 13-9,

Equations (4-7). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -25.43       25.43       197.62

at Q                    69.91      -69.91       197.62

Short-side 2 at N                   -25.43       25.43       197.62

at Q                    69.91      -69.91       197.62

Long-side  1 at M                   103.81     -103.81       232.50

at Q                    39.33      -39.33       197.62

Long-side  2 at M                   122.27     -129.51       232.50

at Q                    38.20      -40.46       197.62

Total Stress Calculations per Section 13-9

Total Stresses at Short-side 1

Total Stress at short side 1 at N inner [STS_Ni]:

= Sms + SbsNi

= 10.87 + -25.43

= -14.55 N./mm^2

Total Stress at short side 1 at N outer [STS_No]:

= Sms + SbsNo

= 10.87 + 25.43

= 36.30 N./mm^2

Total Stress at short side 1 at Q inner [STS_Qi]:

= Sms + SbsQi

= 10.87 + 69.91

= 80.79 N./mm^2

Total Stress at short side 1 at Q outer [STS_Qo]:

= Sms + SbsQo

= 10.87 + -69.91

= -59.04 N./mm^2

Total Stresses at Short-side 2

Total Stress at short side 2 at N inner [STS_Ni]:

= Sms + SbsNi

= 10.87 + -25.43

= -14.55 N./mm^2

Total Stress at short side 2 at N outer [STS_No]:

= Sms + SbsNo

= 10.87 + 25.43

= 36.30 N./mm^2

Total Stress at short side 2 at Q inner [STS_Qi]:

= Sms + SbsQi

= 10.87 + 69.91

= 80.79 N./mm^2

Total Stress at short side 2 at Q outer [STS_Qo]:

= Sms + SbsQo

= 10.87 + -69.91

= -59.04 N./mm^2

Total Stresses at Long-side 1

Total Stress at long side 1 at M inner [STL_Mi]:

= Sml + SblMi

= 12.34 + 103.81

= 116.15 N./mm^2

Total Stress at long side 1 at M outer [STL_Mo]:

= Sml + SblMo

= 12.34 + -103.81

= -91.47 N./mm^2

Total Stress at long side 1 at Q inner [STL_Qi]:

= Sml + SblQi

= 7.15 + 39.33

= 46.48 N./mm^2

Total Stress at long side 1 at Q outer [STL_Qo]:

= Sml + SblQo

= 7.15 + -39.33

= -32.18 N./mm^2

Total Stresses at Long-side 2

Total Stress at long side 2 at M inner [STL_Mi]:

= Sml + SblMi

= 13.90 + 122.27

= 136.17 N./mm^2

Total Stress at long side 2 at M outer [STL_Mo]:

= Sml + SblMo

= 13.90 + -129.51

= -115.61 N./mm^2

Total Stress at long side 2 at Q inner [STL_Qi]:

= Sml + SblQi

= 7.15 + 38.20

= 45.35 N./mm^2

Total Stress at long side 2 at Q outer [STL_Qo]:

= Sml + SblQo

= 7.15 + -40.46

= -33.31 N./mm^2

TOTAL STRESSES:     Total Stress Calculations per Section 13-9,

Equations (8-12). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -14.55       36.30       197.62

at Q                    80.79      -59.04       197.62

Short-side 2 at N                   -14.55       36.30       197.62

at Q                    80.79      -59.04       197.62

Long-side  1 at M                   116.15      -91.47       232.50

at Q                    46.48      -32.18       197.62

Long-side  2 at M                   136.17     -115.61       232.50

at Q                    45.35      -33.31       197.62

End Plate Stresses (N./mm^2):

Actual    Allowable

End Plate                                        40.11       155.00

Required End Plate thickness due to Internal Pressure [trEP]:

= d * sqrt( Z * C * P / ( SE ) ) + ca

= 110.000 * sqrt( 2.318 * 0.200 * 28.600 / ( 155.000 ) ) + 0.000

= 10.174 mm.

End Plate MAWP at given Thickness [MAWPEP]:

= ((T-ca)/d)^2 * ((SE)/(C*Z)) per UG-34 (c)(3)

= ((20.0000-0.0000)/110.0000)^2*((155)/(.20*2.32))

= 110.518 bars

where Z is:

= min( 3.4 - 2.4( d/D ), 2.5 )

= min( 3.4 - 2.4( 110.000 / 244.000 ), 2.5 )

= 2.318

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)             37.17   N./mm^2

High Stress Percentage                           32.49   %

M.A.W.P. for Membrane Stresses                   88.01   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)           -129.51   N./mm^2

High Stress Percentage                           55.70   %

M.A.W.P. for Bending Stresses                    51.34   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)            136.17   N./mm^2

High Stress Percentage                           58.57   %

M.A.W.P. for Total Stresses                      48.83   bars

Rectangular Vessel Results For Item 1 : A7

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)             37.17   N./mm^2

High Stress Percentage                           32.49   %

M.A.W.P. for Membrane Stresses                   88.01   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)           -129.51   N./mm^2

High Stress Percentage                           55.70   %

M.A.W.P. for Bending Stresses                    51.34   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)            136.17   N./mm^2

High Stress Percentage                           58.57   %

M.A.W.P. for Total Stresses                      48.83   bars

## Stationary Header Calculation @ Design Pressure for ae-2102.

03

03

Input Echo, COMPONENT      1,         Description: St.AE-2102

Figure Number Analyzed                                    A8

Design Internal Pressure                   P         62.0000  bars

Design Temperature                      Temp        175.0000  C

VESSEL MATERIAL DATA:

Material Specification                           SA-240 316L

Shell Allowable Stress at Design Temp      S        111.9078  N./mm^2

Shell Allowable Stress at Ambient         SA        115.1465  N./mm^2

Shell Yield Stress at Design Temperature  Sy        126.1469  N./mm^2

SHORT-SIDE VESSEL DATA:

Short-side Length Dimension                H        110.0000  mm.

Minimum Thickness of Short-side Plates    t1         20.0000  mm.

Mid-side Joint Efficiency on Short-side    E          0.8500

Corner Joint Efficiency on Short-side     EC          0.8500

LONG-SIDE VESSEL DATA:

Long-side Length Dimension                 h        130.0000  mm.

Minimum Thickness of Long-side Plates     t2         28.0000  mm.

Mid-side Joint Efficiency on Long-side     E          0.8500

ADDITIONAL VESSEL DATA:

Minimum Thickness of End Plate            t5         20.0000  mm.

C-Factor for End Plate                Cf_Epl          0.2000

Long-side Plate # 1,

Pitch Distance                          p         69.0000  mm.

Uniform Hole Diameter                  d0         25.6500  mm.

Depth of Holes                         T0         28.0000  mm.

Long-side Plate # 2,

Pitch Distance                          p         69.0000  mm.

# 1: Hole Diameter                     d0         36.3000  mm.

Hole Depth                        T0          2.7000  mm.

# 2: Hole Diameter                     d1         28.5750  mm.

Hole Depth                        T1         25.3000  mm.

STAY PLATE MATERIAL DATA:

Stay Material Specification                      SA-240 316L

Stay Allowable Stress at Design Temp      Sr        114.3898  N./mm^2

Stay Allowable Stress at Ambient          SA        115.1465  N./mm^2

Stay Yield Stress at Design Temp          Sy        129.8699  N./mm^2

STAY PLATE DATA:

Minimum Thickness of Stay                 t3         10.0000  mm.

Minimum Thickness of Stay                 t4         10.0000  mm.

The Stay(s) Are Not Welded to the End  Plate

Rectangular Vessel Results, Item number     1, Desc: St.AE-2102

ASME Code, Section VIII, Division 1, 2019 App. 13

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Short-side 1 Calculations

03

03

Membrane Ligament Efficiency [Em]:

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Short-side 2 Calculations

Membrane Ligament Efficiency [Em]:

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Long-side 1 Calculations

Effective Diameter [De]: 25.650 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 69.000 - 25.650 / 69.000

= 0.628

Bending Ligament Efficiency [Eb]:

As diameter holes are uniform Eb = Em

= 0.628

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 28.000 - 0.000 / 2

= 14.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 28.000 - 0.000 ) / 2

= -14.000 mm.

Long-side 2 Calculations

03

03

Effective Diameter [De]:

= ( d0 * T0 + d1 * T1 + d2 * T2 ) / ( t1 - CA )

= ( 36.30 * 2.70 + 28.57 * 25.30 + 0.00 * 0.00 ) /

( 28.00 - 0.00 )

= 29.320 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 69.000 - 29.320 / 69.000

= 0.575

Dist from Neutral axis of c/s to extreme fibers [Ci & Co]:

Calculation of Xbar:

= (( b0 * T0 * ( T0/2 + T1 + T2 )) + ( b1 * T1 *

( T1/2 + T2 )) + ( b2 * T2 * ( T2/2 ))) /

( b0 * T0 + b1 * T1 + b2 * T2 )

= (( 1.29 * 2.70 * ( 2.70 /2 + 25.30 + 0.00 )) + ( 1.59* 25.30 *

( 25.30 /2 + 0.00 )) + ( 2.72 * 0.00 * ( 0.00 /2 ))) /

( 1.29 * 2.70 + 1.59 * 25.30 + 2.72 * 0.00 )

= 13.763 mm.

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

Ci = Xbar

= 13.763 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t - CA - Xbar )

= -( 28.000 - 0.000 - 13.763 )

= -14.237 mm.

Moment of Inertia (Section 13-6, Equation (5)) [I]:

= 0.278 cm**4

Effective Diameter [De]:

= Pitch - (( 6 * I ) / (( t - CA)^2 * (-Co)))

= 69.00 - (( 6 * 0.28 ) / ((28.00 - 0.00 )^2 * (14.24 )))

= 31.084 mm.

Bending Ligament Efficiency [Eb]:

= Pitch - De / Pitch

= 69.000 - 31.084 / 69.000

= 0.550

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Em          Eb          Ci          Co

Short-side 1       0.850       0.850      10.000     -10.000

2       0.850       0.850      10.000     -10.000

Long-side  1       0.628       0.628      14.000     -14.000

2       0.575       0.550      13.763     -14.237

Moment of Inertia of a Strip of the Vessel Wall:

Thickness  t1, I1  = 0.0667 cm**4

Thickness  t2, I2  = 0.1829 cm**4

03

03

Rectangular Vessel Parameters:

Alpha =      H / h    = 0.8462

K     = (I2/I1)*Alpha = 2.3218

Membrane Stress Calculations per Section 13-9

Membrane Stresses at Short-side 1

Membrane Stress at Short-side 1 [Sms]:

= P * h / ( 2 * t1 ) * { 3 - [( 6 + K *

( 11 - alpha2) / ( 3 + 5 * K )]}

= 62.00 * 130.00 / ( 2 * 20.00 ) * { 3- [( 6 + 2.32 *

( 11 - 0.852) / ( 3 + 5 * 2.32 )]}

= 19.24 N./mm^2

Membrane Stresses at Short-side 2

Membrane Stress at Short-side 2 [Sms]:

= P * h / ( 2 * t1 ) * { 3 - [( 6 + K *

( 11 - alpha2) / ( 3 + 5 * K )]}

= 62.00 * 130.00 / ( 2 * 20.00 ) * { 3- [( 6 + 2.32 *

( 11 - 0.852) / ( 3 + 5 * 2.32 )]}

= 19.24 N./mm^2

Membrane Stresses at Long-side 1

Membrane Stress at Long-side 1 at A [Sml]:

= P * H / 2 * t2

= 62.00 * 110.00 / 2 * 28.00

= 12.18 N./mm^2

If Em( 0.628 ) < E( 0.850 ) and Eb( 0.628 ) < E( 0.850 ) then

Sml =  Sml / Em

= 12.18 / 0.63

= 19.39 N./mm^2

Membrane Stresses at Long-side 2

Membrane Stress at Long-side 2 at A [Sml]:

= P * H / 2 * t2

= 62.00 * 110.00 / 2 * 28.00

= 12.18 N./mm^2

If Em( 0.575 ) < E( 0.850 ) and Eb( 0.550 ) < E( 0.850 ) then

Sml =  Sml / Em

= 12.18 / 0.58

= 21.18 N./mm^2

Membrane Stresses at Stay Plate

Membrane Stress at Stay Plate [t3]:

= P * h / ( 2 * t3 ) * [( 6 + K * ( 11 -

alpha2) / ( 3 + 5 * K )]

= 62.00 * 130.00 / ( 2 * 10.00 ) * [( 6 + 2.32 * ( 11 -

0.852) / ( 3 + 5 * 2.32 )]

= 82.42 N./mm^2

Membrane Stress at Stay Plate [t4]:

= P * h / ( 2 * t4 ) * [( 6 + K * ( 11 -

03

03

alpha2) / ( 3 + 5 * K )]

= 62.00 * 130.00 / ( 2 * 10.00 ) * [( 6 + 2.32 * ( 11 -

0.852) / ( 3 + 5 * 2.32 )]

= 82.42 N./mm^2

MEMBRANE STRESSES:  Membrane Stress Calculations per Section 13-9,

Equations (13-15). (N./mm^2) :

STRESS LOCATIONS                                Actual    Allowable

----------------------------------------------------------------------

Short-side 1                                     19.24        95.12

Short-side 2                                     19.24        95.12

Short-side Corner                                19.24        95.12

Long-side  1 at A                                19.39       111.91

Long-side  2 at A                                21.18       111.91

Long-side Corner                                 12.18        95.12

Stay Plate (t3)                                  82.42       114.39

Stay Plate (t4)                                  82.42       114.39

Bending Stress Calculations per Section 13-9

Bending Stresses at Short-side 1

Bending Stress at Short-side 1 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 62.00 * 10.00 / ( 24 * 0.07 ) * [ -3 * 110.002 + 2 * 130.002 *

(( 3 + 5 * 0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))]

= -39.25 N./mm^2

Bending Stress at Short-side 1 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 62.00 * -10.00 /( 24 * 0.07 ) * [ -3 * 110.002 + 2 * 130.002 *

(( 3 + 5 * 0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))]

= 39.25 N./mm^2

Bending Stress at Short-side 1 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 62.00 * 130.002 * 10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))

= 101.42 N./mm^2

Bending Stress at Short-side 1 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 62.00 * 130.002 * -10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))

= -101.42 N./mm^2

Bending Stresses at Short-side 2

Bending Stress at Short-side 2 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 62.00 * 10.00 / ( 24 * 0.07 ) * [ -3 * 110.002 + 2 * 130.002 *

(( 3 + 5 * 0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))]

= -39.25 N./mm^2

03

03

Bending Stress at Short-side 2 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 62.00 * -10.00 /( 24 * 0.07 ) * [ -3 * 110.002 + 2 * 130.002 *

(( 3 + 5 * 0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))]

= 39.25 N./mm^2

Bending Stress at Short-side 2 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 62.00 * 130.002 * 10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))

= 101.42 N./mm^2

Bending Stress at Short-side 2 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 62.00 * 130.002 * -10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))

= -101.42 N./mm^2

Bending Stresses at Long-side 1

Bending Stress at Long-side 1 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 62.00 * 130.002 * 14.00 / ( 12 * 0.18 ) * [( 3 + 2.32 *

( 6 - 0.852)) / ( 3 + 5 * 2.32 )]

= 69.84 N./mm^2

If Em( 0.628 ) < E( 0.850 ) and Eb( 0.628 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 69.84 / 0.63

= 111.17 N./mm^2

Bending Stress at Long-side 1 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 62.00 * 130.002 * -14.00 / ( 12 * 0.18 ) * [( 3 + 2.32 *

( 6 - 0.852)) / ( 3 + 5 * 2.32 )]

= -69.84 N./mm^2

If Em( 0.628 ) < E( 0.850 ) and Eb( 0.628 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -69.84 / 0.63

= -111.17 N./mm^2

Bending Stress at Long-side 1 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 62.00 * 130.002 * 14.00 / ( 12 * 0.18 ) * [( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 )]

= 51.74 N./mm^2

Bending Stress at Long-side 1 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 62.00 * 130.002 * -14.00 / ( 12 * 0.18 ) * [( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 )]

= -51.74 N./mm^2

03

03

Bending Stresses at Long-side 2

Bending Stress at Long-side 2 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 62.00 * 130.002 * 13.76 / ( 12 * 0.18 ) * [( 3 + 2.32 *

( 6 - 0.852)) / ( 3 + 5 * 2.32 )]

= 68.66 N./mm^2

If Em( 0.575 ) < E( 0.850 ) and Eb( 0.550 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 68.66 / 0.55

= 124.95 N./mm^2

Bending Stress at Long-side 2 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 62.00 * 130.002 * -14.24 / ( 12 * 0.18 ) * [( 3 + 2.32 *

( 6 - 0.852)) / ( 3 + 5 * 2.32 )]

= -71.03 N./mm^2

If Em( 0.575 ) < E( 0.850 ) and Eb( 0.550 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -71.03 / 0.55

= -129.26 N./mm^2

Bending Stress at Long-side 2 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 62.00 * 130.002 * 13.76 / ( 12 * 0.18 ) * [( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 )]

= 50.87 N./mm^2

Bending Stress at Long-side 2 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 62.00 * 130.002 * -14.24 / ( 12 * 0.18 ) * [( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 )]

= -52.62 N./mm^2

BENDING STRESSES:   Bending Stress Calculations per Section 13-9,

Equations (16-19). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -39.25       39.25       142.68

at Q                   101.42     -101.42       142.68

Short-side 2 at N                   -39.25       39.25       142.68

at Q                   101.42     -101.42       142.68

Long-side  1 at M                   111.17     -111.17       167.86

at Q                    51.74      -51.74       142.68

Long-side  2 at M                   124.95     -129.26       167.86

at Q                    50.87      -52.62       142.68

Total Stress Calculations per Section 13-9

Total Stresses at Short-side 1

03

03

Total Stress at short side 1 at N inner [STS_Ni]:

= Sms + SbsNi

= 19.24 + -39.25

= -20.01 N./mm^2

Total Stress at short side 1 at N outer [STS_No]:

= Sms + SbsNo

= 19.24 + 39.25

= 58.49 N./mm^2

Total Stress at short side 1 at Q inner [STS_Qi]:

= Sms + SbsQi

= 19.24 + 101.42

= 120.66 N./mm^2

Total Stress at short side 1 at Q outer [STS_Qo]:

= Sms + SbsQo

= 19.24 + -101.42

= -82.18 N./mm^2

Total Stresses at Short-side 2

Total Stress at short side 2 at N inner [STS_Ni]:

= Sms + SbsNi

= 19.24 + -39.25

= -20.01 N./mm^2

Total Stress at short side 2 at N outer [STS_No]:

= Sms + SbsNo

= 19.24 + 39.25

= 58.49 N./mm^2

Total Stress at short side 2 at Q inner [STS_Qi]:

= Sms + SbsQi

= 19.24 + 101.42

= 120.66 N./mm^2

Total Stress at short side 2 at Q outer [STS_Qo]:

= Sms + SbsQo

= 19.24 + -101.42

= -82.18 N./mm^2

Total Stresses at Long-side 1

Total Stress at long side 1 at M inner [STL_Mi]:

= Sml + SblMi

= 19.39 + 111.17

= 130.56 N./mm^2

Total Stress at long side 1 at M outer [STL_Mo]:

= Sml + SblMo

= 19.39 + -111.17

= -91.79 N./mm^2

Total Stress at long side 1 at Q inner [STL_Qi]:

= Sml + SblQi

= 12.18 + 51.74

= 63.92 N./mm^2

Total Stress at long side 1 at Q outer [STL_Qo]:

03

03

= SmlB + SblQo

= 12.18 + -51.74

= -39.57 N./mm^2

Total Stresses at Long-side 2

Total Stress at long side 2 at M inner [STL_Mi]:

= Sml + SblMi

= 21.18 + 124.95

= 146.13 N./mm^2

Total Stress at long side 2 at M outer [STL_Mo]:

= Sml + SblMo

= 21.18 + -129.26

= -108.08 N./mm^2

Total Stress at long side 2 at Q inner [STL_Qi]:

= Sml + SblQi

= 12.18 + 50.87

= 63.05 N./mm^2

Total Stress at long side 2 at Q outer [STL_Qo]:

= SmlB + SblQo

= 12.18 + -52.62

= -40.44 N./mm^2

TOTAL STRESSES:     Total Stress Calculations per Section 13-9,

Equations (20-24). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -20.01       58.49       142.68

at Q                   120.66      -82.18       142.68

Short-side 2 at N                   -20.01       58.49       142.68

at Q                   120.66      -82.18       142.68

Long-side  1 at M                   130.56      -91.79       167.86

at Q                    63.92      -39.57       142.68

Long-side  2 at M                   146.13     -108.08       167.86

at Q                    63.05      -40.44       142.68

End Plate Stresses (N./mm^2):

Actual    Allowable

End Plate                                        93.78       111.91

Required End Plate thickness due to Internal Pressure [trEP]:

= d * sqrt( Z * C * P / ( SE ) ) + ca

= 110.000 * sqrt( 2.500 * 0.200 * 62.000 / ( 111.908 ) ) + 0.000

= 18.309 mm.

End Plate MAWP at given Thickness [MAWPEP]:

= ((T-ca)/d)^2 * ((SE)/(C*Z)) per UG-34 (c)(3)

= ((20.0000-0.0000)/110.0000)^2*((111)/(.20*2.50))

= 73.984 bars

where Z is:

= min( 3.4 - 2.4( d/D ), 2.5 )

= min( 3.4 - 2.4( 110.000 / 410.000 ), 2.5 )

= 2.500

03

03

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)             82.42   N./mm^2

High Stress Percentage                           72.06   %

M.A.W.P. for Membrane Stresses                   86.04   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)           -129.26   N./mm^2

High Stress Percentage                           77.00   %

M.A.W.P. for Bending Stresses                    80.52   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)            146.13   N./mm^2

High Stress Percentage                           87.05   %

M.A.W.P. for Total Stresses                      71.22   bars

Rectangular Vessel Results For Item 1 : A8

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)             82.42   N./mm^2

High Stress Percentage                           72.06   %

M.A.W.P. for Membrane Stresses                   86.04   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)           -129.26   N./mm^2

High Stress Percentage                           77.00   %

M.A.W.P. for Bending Stresses                    80.52   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)            146.13   N./mm^2

High Stress Percentage                           87.05   %

M.A.W.P. for Total Stresses                      71.22   bars

## Stationary Header Calculation @ test Pressure for ae-2102.

Input Echo, COMPONENT      1,         Description: St.AE-2102-Test

03

03

Figure Number Analyzed                                    A8

Design Internal Pressure                   P         80.6000  bars

Design Temperature                      Temp         25.0000  C

VESSEL MATERIAL DATA:

Material Specification                           SA-240 316L

Shell Allowable Stress at Design Temp      S        155.1000  N./mm^2

Shell Allowable Stress at Ambient         SA        155.1000  N./mm^2

Shell Yield Stress at Design Temperature  Sy        172.3750  N./mm^2

SHORT-SIDE VESSEL DATA:

Short-side Length Dimension                H        110.0000  mm.

Minimum Thickness of Short-side Plates    t1         20.0000  mm.

Mid-side Joint Efficiency on Short-side    E          0.8500

Corner Joint Efficiency on Short-side     EC          0.8500

LONG-SIDE VESSEL DATA:

Long-side Length Dimension                 h        130.0000  mm.

Minimum Thickness of Long-side Plates     t2         28.0000  mm.

Mid-side Joint Efficiency on Long-side     E          0.8500

ADDITIONAL VESSEL DATA:

Minimum Thickness of End Plate            t5         20.0000  mm.

C-Factor for End Plate                Cf_Epl          0.2000

Long-side Plate # 1,

Pitch Distance                          p         69.0000  mm.

Uniform Hole Diameter                  d0         25.6500  mm.

Depth of Holes                         T0         28.0000  mm.

Long-side Plate # 2,

Pitch Distance                          p         69.0000  mm.

# 1: Hole Diameter                     d0         36.3000  mm.

Hole Depth                        T0          2.7000  mm.

# 2: Hole Diameter                     d1         28.5750  mm.

Hole Depth                        T1         25.3000  mm.

STAY PLATE MATERIAL DATA:

Stay Material Specification                      SA-240 316L

Stay Allowable Stress at Design Temp      Sr        116.9000  N./mm^2

Stay Allowable Stress at Ambient          SA        116.9000  N./mm^2

Stay Yield Stress at Design Temp          Sy        129.8699  N./mm^2

STAY PLATE DATA:

Minimum Thickness of Stay                 t3         10.0000  mm.

Minimum Thickness of Stay                 t4         10.0000  mm.

The Stay(s) Are Not Welded to the End  Plate

03

03

Rectangular Vessel Results, Item number     1, Desc: St.AE-2102-Test

ASME Code, Section VIII, Division 1, 2019 App. 13

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Short-side 1 Calculations

Membrane Ligament Efficiency [Em]:

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Short-side 2 Calculations

Membrane Ligament Efficiency [Em]:

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Long-side 1 Calculations

Effective Diameter [De]: 25.650 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 69.000 - 25.650 / 69.000

= 0.628

Bending Ligament Efficiency [Eb]:

As diameter holes are uniform Eb = Em

= 0.628

03

03

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 28.000 - 0.000 / 2

= 14.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 28.000 - 0.000 ) / 2

= -14.000 mm.

Long-side 2 Calculations

Effective Diameter [De]:

= ( d0 * T0 + d1 * T1 + d2 * T2 ) / ( t1 - CA )

= ( 36.30 * 2.70 + 28.57 * 25.30 + 0.00 * 0.00 ) /

( 28.00 - 0.00 )

= 29.320 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 69.000 - 29.320 / 69.000

= 0.575

Dist from Neutral axis of c/s to extreme fibers [Ci & Co]:

Calculation of Xbar:

= (( b0 * T0 * ( T0/2 + T1 + T2 )) + ( b1 * T1 *

( T1/2 + T2 )) + ( b2 * T2 * ( T2/2 ))) /

( b0 * T0 + b1 * T1 + b2 * T2 )

= (( 1.29 * 2.70 * ( 2.70 /2 + 25.30 + 0.00 )) + ( 1.59* 25.30 *

( 25.30 /2 + 0.00 )) + ( 2.72 * 0.00 * ( 0.00 /2 ))) /

( 1.29 * 2.70 + 1.59 * 25.30 + 2.72 * 0.00 )

= 13.763 mm.

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

Ci = Xbar

= 13.763 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t - CA - Xbar )

= -( 28.000 - 0.000 - 13.763 )

= -14.237 mm.

Moment of Inertia (Section 13-6, Equation (5)) [I]:

= 0.278 cm**4

Effective Diameter [De]:

= Pitch - (( 6 * I ) / (( t - CA)^2 * (-Co)))

= 69.00 - (( 6 * 0.28 ) / ((28.00 - 0.00 )^2 * (14.24 )))

= 31.084 mm.

Bending Ligament Efficiency [Eb]:

= Pitch - De / Pitch

= 69.000 - 31.084 / 69.000

= 0.550

03

03

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Em          Eb          Ci          Co

Short-side 1       0.850       0.850      10.000     -10.000

2       0.850       0.850      10.000     -10.000

Long-side  1       0.628       0.628      14.000     -14.000

2       0.575       0.550      13.763     -14.237

Moment of Inertia of a Strip of the Vessel Wall:

Thickness  t1, I1  = 0.0667 cm**4

Thickness  t2, I2  = 0.1829 cm**4

Rectangular Vessel Parameters:

Alpha =      H / h    = 0.8462

K     = (I2/I1)*Alpha = 2.3218

Membrane Stress Calculations per Section 13-9

Membrane Stresses at Short-side 1

Membrane Stress at Short-side 1 [Sms]:

= P * h / ( 2 * t1 ) * { 3 - [( 6 + K *

( 11 - alpha2) / ( 3 + 5 * K )]}

= 80.60 * 130.00 / ( 2 * 20.00 ) * { 3- [( 6 + 2.32 *

( 11 - 0.852) / ( 3 + 5 * 2.32 )]}

= 25.01 N./mm^2

Membrane Stresses at Short-side 2

Membrane Stress at Short-side 2 [Sms]:

= P * h / ( 2 * t1 ) * { 3 - [( 6 + K *

( 11 - alpha2) / ( 3 + 5 * K )]}

= 80.60 * 130.00 / ( 2 * 20.00 ) * { 3- [( 6 + 2.32 *

( 11 - 0.852) / ( 3 + 5 * 2.32 )]}

= 25.01 N./mm^2

Membrane Stresses at Long-side 1

Membrane Stress at Long-side 1 at A [Sml]:

= P * H / 2 * t2

= 80.60 * 110.00 / 2 * 28.00

= 15.83 N./mm^2

If Em( 0.628 ) < E( 0.850 ) and Eb( 0.628 ) < E( 0.850 ) then

Sml =  Sml / Em

= 15.83 / 0.63

= 25.20 N./mm^2

Membrane Stresses at Long-side 2

Membrane Stress at Long-side 2 at A [Sml]:

= P * H / 2 * t2

= 80.60 * 110.00 / 2 * 28.00

= 15.83 N./mm^2

03

03

If Em( 0.575 ) < E( 0.850 ) and Eb( 0.550 ) < E( 0.850 ) then

Sml =  Sml / Em

= 15.83 / 0.58

= 27.53 N./mm^2

Membrane Stresses at Stay Plate

Membrane Stress at Stay Plate [t3]:

= P * h / ( 2 * t3 ) * [( 6 + K * ( 11 -

alpha2) / ( 3 + 5 * K )]

= 80.60 * 130.00 / ( 2 * 10.00 ) * [( 6 + 2.32 * ( 11 -

0.852) / ( 3 + 5 * 2.32 )]

= 107.15 N./mm^2

Membrane Stress at Stay Plate [t4]:

= P * h / ( 2 * t4 ) * [( 6 + K * ( 11 -

alpha2) / ( 3 + 5 * K )]

= 80.60 * 130.00 / ( 2 * 10.00 ) * [( 6 + 2.32 * ( 11 -

0.852) / ( 3 + 5 * 2.32 )]

= 107.15 N./mm^2

MEMBRANE STRESSES:  Membrane Stress Calculations per Section 13-9,

Equations (13-15). (N./mm^2) :

STRESS LOCATIONS                                Actual    Allowable

----------------------------------------------------------------------

Short-side 1                                     25.01       131.84

Short-side 2                                     25.01       131.84

Short-side Corner                                25.01       131.84

Long-side  1 at A                                25.20       155.10

Long-side  2 at A                                27.53       155.10

Long-side Corner                                 15.83       131.84

Stay Plate (t3)                                 107.15       116.90

Stay Plate (t4)                                 107.15       116.90

Bending Stress Calculations per Section 13-9

Bending Stresses at Short-side 1

Bending Stress at Short-side 1 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 80.60 * 10.00 / ( 24 * 0.07 ) * [ -3 * 110.002 + 2 * 130.002 *

(( 3 + 5 * 0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))]

= -51.03 N./mm^2

Bending Stress at Short-side 1 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 80.60 * -10.00 /( 24 * 0.07 ) * [ -3 * 110.002 + 2 * 130.002 *

(( 3 + 5 * 0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))]

= 51.03 N./mm^2

Bending Stress at Short-side 1 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

03

03

Alpha2 * K ) / ( 3 + 5 * K ))

= 80.60 * 130.002 * 10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))

= 131.85 N./mm^2

Bending Stress at Short-side 1 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 80.60 * 130.002 * -10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))

= -131.85 N./mm^2

Bending Stresses at Short-side 2

Bending Stress at Short-side 2 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 80.60 * 10.00 / ( 24 * 0.07 ) * [ -3 * 110.002 + 2 * 130.002 *

(( 3 + 5 * 0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))]

= -51.03 N./mm^2

Bending Stress at Short-side 2 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 80.60 * -10.00 /( 24 * 0.07 ) * [ -3 * 110.002 + 2 * 130.002 *

(( 3 + 5 * 0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))]

= 51.03 N./mm^2

Bending Stress at Short-side 2 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 80.60 * 130.002 * 10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))

= 131.85 N./mm^2

Bending Stress at Short-side 2 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 80.60 * 130.002 * -10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 ))

= -131.85 N./mm^2

Bending Stresses at Long-side 1

Bending Stress at Long-side 1 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 80.60 * 130.002 * 14.00 / ( 12 * 0.18 ) * [( 3 + 2.32 *

( 6 - 0.852)) / ( 3 + 5 * 2.32 )]

= 90.80 N./mm^2

If Em( 0.628 ) < E( 0.850 ) and Eb( 0.628 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 90.80 / 0.63

= 144.52 N./mm^2

03

03

Bending Stress at Long-side 1 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 80.60 * 130.002 * -14.00 / ( 12 * 0.18 ) * [( 3 + 2.32 *

( 6 - 0.852)) / ( 3 + 5 * 2.32 )]

= -90.80 N./mm^2

If Em( 0.628 ) < E( 0.850 ) and Eb( 0.628 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -90.80 / 0.63

= -144.52 N./mm^2

Bending Stress at Long-side 1 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 80.60 * 130.002 * 14.00 / ( 12 * 0.18 ) * [( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 )]

= 67.27 N./mm^2

Bending Stress at Long-side 1 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 80.60 * 130.002 * -14.00 / ( 12 * 0.18 ) * [( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 )]

= -67.27 N./mm^2

Bending Stresses at Long-side 2

Bending Stress at Long-side 2 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 80.60 * 130.002 * 13.76 / ( 12 * 0.18 ) * [( 3 + 2.32 *

( 6 - 0.852)) / ( 3 + 5 * 2.32 )]

= 89.26 N./mm^2

If Em( 0.575 ) < E( 0.850 ) and Eb( 0.550 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 89.26 / 0.55

= 162.43 N./mm^2

Bending Stress at Long-side 2 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 80.60 * 130.002 * -14.24 / ( 12 * 0.18 ) * [( 3 + 2.32 *

( 6 - 0.852)) / ( 3 + 5 * 2.32 )]

= -92.34 N./mm^2

If Em( 0.575 ) < E( 0.850 ) and Eb( 0.550 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -92.34 / 0.55

= -168.04 N./mm^2

Bending Stress at Long-side 2 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 80.60 * 130.002 * 13.76 / ( 12 * 0.18 ) * [( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 )]

03

03

= 66.13 N./mm^2

Bending Stress at Long-side 2 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 80.60 * 130.002 * -14.24 / ( 12 * 0.18 ) * [( 3 + 5 *

0.852 * 2.32 ) / ( 3 + 5 * 2.32 )]

= -68.41 N./mm^2

BENDING STRESSES:   Bending Stress Calculations per Section 13-9,

Equations (16-19). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -51.03       51.03       197.75

at Q                   131.85     -131.85       197.75

Short-side 2 at N                   -51.03       51.03       197.75

at Q                   131.85     -131.85       197.75

Long-side  1 at M                   144.52     -144.52       232.65

at Q                    67.27      -67.27       197.75

Long-side  2 at M                   162.43     -168.04       232.65

at Q                    66.13      -68.41       197.75

Total Stress Calculations per Section 13-9

Total Stresses at Short-side 1

Total Stress at short side 1 at N inner [STS_Ni]:

= Sms + SbsNi

= 25.01 + -51.03

= -26.01 N./mm^2

Total Stress at short side 1 at N outer [STS_No]:

= Sms + SbsNo

= 25.01 + 51.03

= 76.04 N./mm^2

Total Stress at short side 1 at Q inner [STS_Qi]:

= Sms + SbsQi

= 25.01 + 131.85

= 156.86 N./mm^2

Total Stress at short side 1 at Q outer [STS_Qo]:

= Sms + SbsQo

= 25.01 + -131.85

= -106.83 N./mm^2

Total Stresses at Short-side 2

Total Stress at short side 2 at N inner [STS_Ni]:

= Sms + SbsNi

= 25.01 + -51.03

= -26.01 N./mm^2

03

03

Total Stress at short side 2 at N outer [STS_No]:

= Sms + SbsNo

= 25.01 + 51.03

= 76.04 N./mm^2

Total Stress at short side 2 at Q inner [STS_Qi]:

= Sms + SbsQi

= 25.01 + 131.85

= 156.86 N./mm^2

Total Stress at short side 2 at Q outer [STS_Qo]:

= Sms + SbsQo

= 25.01 + -131.85

= -106.83 N./mm^2

Total Stresses at Long-side 1

Total Stress at long side 1 at M inner [STL_Mi]:

= Sml + SblMi

= 25.20 + 144.52

= 169.72 N./mm^2

Total Stress at long side 1 at M outer [STL_Mo]:

= Sml + SblMo

= 25.20 + -144.52

= -119.32 N./mm^2

Total Stress at long side 1 at Q inner [STL_Qi]:

= Sml + SblQi

= 15.83 + 67.27

= 83.10 N./mm^2

Total Stress at long side 1 at Q outer [STL_Qo]:

= SmlB + SblQo

= 15.83 + -67.27

= -51.44 N./mm^2

Total Stresses at Long-side 2

Total Stress at long side 2 at M inner [STL_Mi]:

= Sml + SblMi

= 27.53 + 162.43

= 189.96 N./mm^2

Total Stress at long side 2 at M outer [STL_Mo]:

= Sml + SblMo

= 27.53 + -168.04

= -140.50 N./mm^2

Total Stress at long side 2 at Q inner [STL_Qi]:

= Sml + SblQi

= 15.83 + 66.13

= 81.96 N./mm^2

Total Stress at long side 2 at Q outer [STL_Qo]:

= SmlB + SblQo

03

03

= 15.83 + -68.41

= -52.58 N./mm^2

TOTAL STRESSES:     Total Stress Calculations per Section 13-9,

Equations (20-24). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -26.01       76.04       197.75

at Q                   156.86     -106.83       197.75

Short-side 2 at N                   -26.01       76.04       197.75

at Q                   156.86     -106.83       197.75

Long-side  1 at M                   169.72     -119.32       232.65

at Q                    83.10      -51.44       197.75

Long-side  2 at M                   189.96     -140.50       232.65

at Q                    81.96      -52.58       197.75

End Plate Stresses (N./mm^2):

Actual    Allowable

End Plate                                       121.91       155.10

Required End Plate thickness due to Internal Pressure [trEP]:

= d * sqrt( Z * C * P / ( SE ) ) + ca

= 110.000 * sqrt( 2.500 * 0.200 * 80.600 / ( 155.100 ) ) + 0.000

= 17.732 mm.

End Plate MAWP at given Thickness [MAWPEP]:

= ((T-ca)/d)^2 * ((SE)/(C*Z)) per UG-34 (c)(3)

= ((20.0000-0.0000)/110.0000)^2*((155)/(.20*2.50))

= 102.540 bars

where Z is:

= min( 3.4 - 2.4( d/D ), 2.5 )

= min( 3.4 - 2.4( 110.000 / 410.000 ), 2.5 )

= 2.500

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)            107.15   N./mm^2

High Stress Percentage                           91.66   %

M.A.W.P. for Membrane Stresses                   87.93   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)           -168.04   N./mm^2

High Stress Percentage                           72.23   %

M.A.W.P. for Bending Stresses                   111.59   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)            189.96   N./mm^2

High Stress Percentage                           81.65   %

M.A.W.P. for Total Stresses                      98.71   bars

03

03

Rectangular Vessel Results For Item 1 : A8

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)            107.15   N./mm^2

High Stress Percentage                           91.66   %

M.A.W.P. for Membrane Stresses                   87.93   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)           -168.04   N./mm^2

High Stress Percentage                           72.23   %

M.A.W.P. for Bending Stresses                   111.59   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)            189.96   N./mm^2

High Stress Percentage                           81.65   %

M.A.W.P. for Total Stresses                      98.71   bars

## floating Header Calculation @ Design Pressure for ae-2102.

Input Echo, COMPONENT      1,         Description: Fl.AE-2101

Figure Number Analyzed                                    A8

Design Internal Pressure                   P         62.0000  bars

Design Temperature                      Temp        175.0000  C

VESSEL MATERIAL DATA:

Material Specification                           SA-240 316L

Shell Allowable Stress at Design Temp      S        111.9078  N./mm^2

Shell Allowable Stress at Ambient         SA        115.1465  N./mm^2

Shell Yield Stress at Design Temperature  Sy        126.1469  N./mm^2

SHORT-SIDE VESSEL DATA:

Short-side Length Dimension                H        100.0000  mm.

Minimum Thickness of Short-side Plates    t1         20.0000  mm.

Mid-side Joint Efficiency on Short-side    E          0.8500

Corner Joint Efficiency on Short-side     EC          0.8500

LONG-SIDE VESSEL DATA:

Long-side Length Dimension                 h         60.0000  mm.

Minimum Thickness of Long-side Plates     t2         20.0000  mm.

Mid-side Joint Efficiency on Long-side     E          0.8500

ADDITIONAL VESSEL DATA:

Minimum Thickness of End Plate            t5         20.0000  mm.

C-Factor for End Plate                Cf_Epl          0.2000

Long-side Plate # 1,

Pitch Distance                          p         69.0000  mm.

Uniform Hole Diameter                  d0         25.6500  mm.

Depth of Holes                         T0         20.0000  mm.

Long-side Plate # 2,

Pitch Distance                          p         69.0000  mm.

# 1: Hole Diameter                     d0         36.3000  mm.

Hole Depth                        T0          2.7000  mm.

# 2: Hole Diameter                     d1         28.5750  mm.

Hole Depth                        T1         17.3000  mm.

STAY PLATE MATERIAL DATA:

Stay Material Specification                      SA-240 316L

Stay Allowable Stress at Design Temp      Sr        114.3898  N./mm^2

Stay Allowable Stress at Ambient          SA        115.1465  N./mm^2

Stay Yield Stress at Design Temp          Sy        129.8699  N./mm^2

STAY PLATE DATA:

Minimum Thickness of Stay                 t3         10.0000  mm.

Minimum Thickness of Stay                 t4         10.0000  mm.

The Stay(s) Are Not Welded to the End  Plate

Rectangular Vessel Results, Item number     1, Desc: Fl.AE-2101

ASME Code, Section VIII, Division 1, 2019 App. 13

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Short-side 1 Calculations

Membrane Ligament Efficiency [Em]:

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Short-side 2 Calculations

Membrane Ligament Efficiency [Em]:

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Long-side 1 Calculations

Effective Diameter [De]: 25.650 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 69.000 - 25.650 / 69.000

= 0.628

Bending Ligament Efficiency [Eb]:

As diameter holes are uniform Eb = Em

= 0.628

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Long-side 2 Calculations

Effective Diameter [De]:

= ( d0 * T0 + d1 * T1 + d2 * T2 ) / ( t1 - CA )

= ( 36.30 * 2.70 + 28.57 * 17.30 + 0.00 * 0.00 ) /

( 20.00 - 0.00 )

= 29.618 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 69.000 - 29.618 / 69.000

= 0.571

Dist from Neutral axis of c/s to extreme fibers [Ci & Co]:

Calculation of Xbar:

= (( b0 * T0 * ( T0/2 + T1 + T2 )) + ( b1 * T1 *

( T1/2 + T2 )) + ( b2 * T2 * ( T2/2 ))) /

( b0 * T0 + b1 * T1 + b2 * T2 )

= (( 1.29 * 2.70 * ( 2.70 /2 + 17.30 + 0.00 )) + ( 1.59* 17.30 *

( 17.30 /2 + 0.00 )) + ( 2.72 * 0.00 * ( 0.00 /2 ))) /

( 1.29 * 2.70 + 1.59 * 17.30 + 2.72 * 0.00 )

= 9.771 mm.

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

Ci = Xbar

= 9.771 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t - CA - Xbar )

= -( 20.000 - 0.000 - 9.771 )

= -10.229 mm.

Moment of Inertia (Section 13-6, Equation (5)) [I]:

= 0.100 cm**4

Effective Diameter [De]:

= Pitch - (( 6 * I ) / (( t - CA)^2 * (-Co)))

= 69.00 - (( 6 * 0.10 ) / ((20.00 - 0.00 )^2 * (10.23 )))

= 31.848 mm.

Bending Ligament Efficiency [Eb]:

= Pitch - De / Pitch

= 69.000 - 31.848 / 69.000

= 0.538

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Em          Eb          Ci          Co

Short-side 1       0.850       0.850      10.000     -10.000

2       0.850       0.850      10.000     -10.000

Long-side  1       0.628       0.628      10.000     -10.000

2       0.571       0.538       9.771     -10.229

Moment of Inertia of a Strip of the Vessel Wall:

Thickness  t1, I1  = 0.0667 cm**4

Thickness  t2, I2  = 0.0667 cm**4

Rectangular Vessel Parameters:

Alpha =      H / h    = 1.6667

K     = (I2/I1)*Alpha = 1.6667

Membrane Stress Calculations per Section 13-9

Membrane Stresses at Short-side 1

Membrane Stress at Short-side 1 [Sms]:

= P * h / ( 2 * t1 ) * { 3 - [( 6 + K *

( 11 - alpha2) / ( 3 + 5 * K )]}

= 62.00 * 60.00 / ( 2 * 20.00 ) * { 3- [( 6 + 1.67 *

( 11 - 1.672) / ( 3 + 5 * 1.67 )]}

= 11.73 N./mm^2

Membrane Stresses at Short-side 2

Membrane Stress at Short-side 2 [Sms]:

= P * h / ( 2 * t1 ) * { 3 - [( 6 + K *

( 11 - alpha2) / ( 3 + 5 * K )]}

= 62.00 * 60.00 / ( 2 * 20.00 ) * { 3- [( 6 + 1.67 *

( 11 - 1.672) / ( 3 + 5 * 1.67 )]}

= 11.73 N./mm^2

Membrane Stresses at Long-side 1

Membrane Stress at Long-side 1 at A [Sml]:

= P * H / 2 * t2

= 62.00 * 100.00 / 2 * 20.00

= 15.50 N./mm^2

If Em( 0.628 ) < E( 0.850 ) and Eb( 0.628 ) < E( 0.850 ) then

Sml =  Sml / Em

= 15.50 / 0.63

= 24.67 N./mm^2

Membrane Stresses at Long-side 2

Membrane Stress at Long-side 2 at A [Sml]:

= P * H / 2 * t2

= 62.00 * 100.00 / 2 * 20.00

= 15.50 N./mm^2

If Em( 0.571 ) < E( 0.850 ) and Eb( 0.538 ) < E( 0.850 ) then

Sml =  Sml / Em

= 15.50 / 0.57

= 27.16 N./mm^2

Membrane Stresses at Stay Plate

Membrane Stress at Stay Plate [t3]:

= P * h / ( 2 * t3 ) * [( 6 + K * ( 11 -

alpha2) / ( 3 + 5 * K )]

= 62.00 * 60.00 / ( 2 * 10.00 ) * [( 6 + 1.67 * ( 11 -

1.672) / ( 3 + 5 * 1.67 )]

= 32.34 N./mm^2

Membrane Stress at Stay Plate [t4]:

= P * h / ( 2 * t4 ) * [( 6 + K * ( 11 -

alpha2) / ( 3 + 5 * K )]

= 62.00 * 60.00 / ( 2 * 10.00 ) * [( 6 + 1.67 * ( 11 -

1.672) / ( 3 + 5 * 1.67 )]

= 32.34 N./mm^2

MEMBRANE STRESSES:  Membrane Stress Calculations per Section 13-9,

Equations (13-15). (N./mm^2) :

STRESS LOCATIONS                                Actual    Allowable

----------------------------------------------------------------------

Short-side 1                                     11.73        95.12

Short-side 2                                     11.73        95.12

Short-side Corner                                11.73        95.12

Long-side  1 at A                                24.67       111.91

Long-side  2 at A                                27.16       111.91

Long-side Corner                                 15.50        95.12

Stay Plate (t3)                                  32.34       114.39

Stay Plate (t4)                                  32.34       114.39

Bending Stress Calculations per Section 13-9

Bending Stresses at Short-side 1

Bending Stress at Short-side 1 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 62.00 * 10.00 / ( 24 * 0.07 ) * [ -3 * 100.002 + 2 * 60.002 *

(( 3 + 5 * 1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))]

= -51.88 N./mm^2

Bending Stress at Short-side 1 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 62.00 * -10.00 /( 24 * 0.07 ) * [ -3 * 100.002 + 2 * 60.002 *

(( 3 + 5 * 1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))]

= 51.88 N./mm^2

Bending Stress at Short-side 1 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 62.00 * 60.002 * 10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))

= 64.37 N./mm^2

Bending Stress at Short-side 1 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 62.00 * 60.002 * -10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))

= -64.37 N./mm^2

Bending Stresses at Short-side 2

Bending Stress at Short-side 2 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 62.00 * 10.00 / ( 24 * 0.07 ) * [ -3 * 100.002 + 2 * 60.002 *

(( 3 + 5 * 1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))]

= -51.88 N./mm^2

Bending Stress at Short-side 2 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 62.00 * -10.00 /( 24 * 0.07 ) * [ -3 * 100.002 + 2 * 60.002 *

(( 3 + 5 * 1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))]

= 51.88 N./mm^2

Bending Stress at Short-side 2 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 62.00 * 60.002 * 10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))

= 64.37 N./mm^2

Bending Stress at Short-side 2 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 62.00 * 60.002 * -10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))

= -64.37 N./mm^2

Bending Stresses at Long-side 1

Bending Stress at Long-side 1 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 62.00 * 60.002 * 10.00 / ( 12 * 0.07 ) * [( 3 + 1.67 *

( 6 - 1.672)) / ( 3 + 5 * 1.67 )]

= 20.61 N./mm^2

If Em( 0.628 ) < E( 0.850 ) and Eb( 0.628 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 20.61 / 0.63

= 32.80 N./mm^2

Bending Stress at Long-side 1 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 62.00 * 60.002 * -10.00 / ( 12 * 0.07 ) * [( 3 + 1.67 *

( 6 - 1.672)) / ( 3 + 5 * 1.67 )]

= -20.61 N./mm^2

If Em( 0.628 ) < E( 0.850 ) and Eb( 0.628 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -20.61 / 0.63

= -32.80 N./mm^2

Bending Stress at Long-side 1 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 62.00 * 60.002 * 10.00 / ( 12 * 0.07 ) * [( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 )]

= 64.37 N./mm^2

Bending Stress at Long-side 1 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 62.00 * 60.002 * -10.00 / ( 12 * 0.07 ) * [( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 )]

= -64.37 N./mm^2

Bending Stresses at Long-side 2

Bending Stress at Long-side 2 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 62.00 * 60.002 * 9.77 / ( 12 * 0.07 ) * [( 3 + 1.67 *

( 6 - 1.672)) / ( 3 + 5 * 1.67 )]

= 20.14 N./mm^2

If Em( 0.571 ) < E( 0.850 ) and Eb( 0.538 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 20.14 / 0.54

= 37.40 N./mm^2

Bending Stress at Long-side 2 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 62.00 * 60.002 * -10.23 / ( 12 * 0.07 ) * [( 3 + 1.67 *

( 6 - 1.672)) / ( 3 + 5 * 1.67 )]

= -21.08 N./mm^2

If Em( 0.571 ) < E( 0.850 ) and Eb( 0.538 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -21.08 / 0.54

= -39.15 N./mm^2

Bending Stress at Long-side 2 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 62.00 * 60.002 * 9.77 / ( 12 * 0.07 ) * [( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 )]

= 62.90 N./mm^2

Bending Stress at Long-side 2 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 62.00 * 60.002 * -10.23 / ( 12 * 0.07 ) * [( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 )]

= -65.85 N./mm^2

BENDING STRESSES:   Bending Stress Calculations per Section 13-9,

Equations (16-19). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -51.88       51.88       142.68

at Q                    64.37      -64.37       142.68

Short-side 2 at N                   -51.88       51.88       142.68

at Q                    64.37      -64.37       142.68

Long-side  1 at M                    32.80      -32.80       167.86

at Q                    64.37      -64.37       142.68

Long-side  2 at M                    37.40      -39.15       167.86

at Q                    62.90      -65.85       142.68

Total Stress Calculations per Section 13-9

Total Stresses at Short-side 1

Total Stress at short side 1 at N inner [STS_Ni]:

= Sms + SbsNi

= 11.73 + -51.88

= -40.15 N./mm^2

Total Stress at short side 1 at N outer [STS_No]:

= Sms + SbsNo

= 11.73 + 51.88

= 63.61 N./mm^2

Total Stress at short side 1 at Q inner [STS_Qi]:

= Sms + SbsQi

= 11.73 + 64.37

= 76.11 N./mm^2

Total Stress at short side 1 at Q outer [STS_Qo]:

= Sms + SbsQo

= 11.73 + -64.37

= -52.64 N./mm^2

Total Stresses at Short-side 2

Total Stress at short side 2 at N inner [STS_Ni]:

= Sms + SbsNi

= 11.73 + -51.88

= -40.15 N./mm^2

Total Stress at short side 2 at N outer [STS_No]:

= Sms + SbsNo

= 11.73 + 51.88

= 63.61 N./mm^2

Total Stress at short side 2 at Q inner [STS_Qi]:

= Sms + SbsQi

= 11.73 + 64.37

= 76.11 N./mm^2

Total Stress at short side 2 at Q outer [STS_Qo]:

= Sms + SbsQo

= 11.73 + -64.37

= -52.64 N./mm^2

Total Stresses at Long-side 1

Total Stress at long side 1 at M inner [STL_Mi]:

= Sml + SblMi

= 24.67 + 32.80

= 57.47 N./mm^2

Total Stress at long side 1 at M outer [STL_Mo]:

= Sml + SblMo

= 24.67 + -32.80

= -8.13 N./mm^2

Total Stress at long side 1 at Q inner [STL_Qi]:

= Sml + SblQi

= 15.50 + 64.37

= 79.88 N./mm^2

Total Stress at long side 1 at Q outer [STL_Qo]:

= SmlB + SblQo

= 15.50 + -64.37

= -48.87 N./mm^2

Total Stresses at Long-side 2

Total Stress at long side 2 at M inner [STL_Mi]:

= Sml + SblMi

= 27.16 + 37.40

= 64.55 N./mm^2

Total Stress at long side 2 at M outer [STL_Mo]:

= Sml + SblMo

= 27.16 + -39.15

= -11.99 N./mm^2

Total Stress at long side 2 at Q inner [STL_Qi]:

= Sml + SblQi

= 15.50 + 62.90

= 78.40 N./mm^2

Total Stress at long side 2 at Q outer [STL_Qo]:

= SmlB + SblQo

= 15.50 + -65.85

= -50.35 N./mm^2

TOTAL STRESSES:     Total Stress Calculations per Section 13-9,

Equations (20-24). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -40.15       63.61       142.68

at Q                    76.11      -52.64       142.68

Short-side 2 at N                   -40.15       63.61       142.68

at Q                    76.11      -52.64       142.68

Long-side  1 at M                    57.47       -8.13       167.86

at Q                    79.88      -48.87       142.68

Long-side  2 at M                    64.55      -11.99       167.86

at Q                    78.40      -50.35       142.68

End Plate Stresses (N./mm^2):

Actual    Allowable

End Plate                                        68.20       111.91

Required End Plate thickness due to Internal Pressure [trEP]:

= d * sqrt( Z * C * P / ( SE ) ) + ca

= 100.000 * sqrt( 2.200 * 0.200 * 62.000 / ( 111.908 ) ) + 0.000

= 15.614 mm.

End Plate MAWP at given Thickness [MAWPEP]:

= ((T-ca)/d)^2 * ((SE)/(C*Z)) per UG-34 (c)(3)

= ((20.0000-0.0000)/100.0000)^2*((111)/(.20*2.20))

= 101.728 bars

where Z is:

= min( 3.4 - 2.4( d/D ), 2.5 )

= min( 3.4 - 2.4( 100.000 / 200.000 ), 2.5 )

= 2.200

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)             32.34   N./mm^2

High Stress Percentage                           28.27   %

M.A.W.P. for Membrane Stresses                  219.31   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)            -65.85   N./mm^2

High Stress Percentage                           46.15   %

M.A.W.P. for Bending Stresses                   134.34   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)             79.88   N./mm^2

High Stress Percentage                           55.98   %

M.A.W.P. for Total Stresses                     110.75   bars

Rectangular Vessel Results For Item 1 : A8

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)             32.34   N./mm^2

High Stress Percentage                           28.27   %

M.A.W.P. for Membrane Stresses                  219.31   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)            -65.85   N./mm^2

High Stress Percentage                           46.15   %

M.A.W.P. for Bending Stresses                   134.34   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)             79.88   N./mm^2

High Stress Percentage                           55.98   %

M.A.W.P. for Total Stresses                     110.75   bars

## floating Header Calculation @ test Pressure for ae-2102.

Input Echo, COMPONENT      1,         Description: Fl.AE-2101-Test

Figure Number Analyzed                                    A8

Design Internal Pressure                   P         80.6000  bars

Design Temperature                      Temp         25.0000  C

VESSEL MATERIAL DATA:

Material Specification                           SA-240 316L

Shell Allowable Stress at Design Temp      S        155.0000  N./mm^2

Shell Allowable Stress at Ambient         SA        155.0000  N./mm^2

Shell Yield Stress at Design Temperature  Sy        172.3750  N./mm^2

SHORT-SIDE VESSEL DATA:

Short-side Length Dimension                H        100.0000  mm.

Minimum Thickness of Short-side Plates    t1         20.0000  mm.

Mid-side Joint Efficiency on Short-side    E          0.8500

Corner Joint Efficiency on Short-side     EC          0.8500

LONG-SIDE VESSEL DATA:

Long-side Length Dimension                 h         60.0000  mm.

Minimum Thickness of Long-side Plates     t2         20.0000  mm.

Mid-side Joint Efficiency on Long-side     E          0.8500

ADDITIONAL VESSEL DATA:

Minimum Thickness of End Plate            t5         20.0000  mm.

C-Factor for End Plate                Cf_Epl          0.2000

Long-side Plate # 1,

Pitch Distance                          p         69.0000  mm.

Uniform Hole Diameter                  d0         25.6500  mm.

Depth of Holes                         T0         20.0000  mm.

Long-side Plate # 2,

Pitch Distance                          p         69.0000  mm.

# 1: Hole Diameter                     d0         36.3000  mm.

Hole Depth                        T0          2.7000  mm.

# 2: Hole Diameter                     d1         28.5750  mm.

Hole Depth                        T1         17.3000  mm.

STAY PLATE MATERIAL DATA:

Stay Material Specification                      SA-240 316L

Stay Allowable Stress at Design Temp      Sr        114.3898  N./mm^2

Stay Allowable Stress at Ambient          SA        115.1465  N./mm^2

Stay Yield Stress at Design Temp          Sy        129.8699  N./mm^2

STAY PLATE DATA:

Minimum Thickness of Stay                 t3         10.0000  mm.

Minimum Thickness of Stay                 t4         10.0000  mm.

The Stay(s) Are Not Welded to the End  Plate

Rectangular Vessel Results, Item number     1, Desc: Fl.AE-2101-Test

ASME Code, Section VIII, Division 1, 2019 App. 13

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Short-side 1 Calculations

Membrane Ligament Efficiency [Em]:

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Short-side 2 Calculations

Membrane Ligament Efficiency [Em]:

= 0.850

Bending Ligament Efficiency [Eb]:

= 0.850

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Long-side 1 Calculations

Effective Diameter [De]: 25.650 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 69.000 - 25.650 / 69.000

= 0.628

Bending Ligament Efficiency [Eb]:

As diameter holes are uniform Eb = Em

= 0.628

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

= t1 - CA / 2

= 20.000 - 0.000 / 2

= 10.000 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t1 - CA ) / 2

= -( 20.000 - 0.000 ) / 2

= -10.000 mm.

Long-side 2 Calculations

Effective Diameter [De]:

= ( d0 * T0 + d1 * T1 + d2 * T2 ) / ( t1 - CA )

= ( 36.30 * 2.70 + 28.57 * 17.30 + 0.00 * 0.00 ) /

( 20.00 - 0.00 )

= 29.618 mm.

Membrane Ligament Efficiency [Em]:

= Pitch - De / Pitch

= 69.000 - 29.618 / 69.000

= 0.571

Dist from Neutral axis of c/s to extreme fibers [Ci & Co]:

Calculation of Xbar:

= (( b0 * T0 * ( T0/2 + T1 + T2 )) + ( b1 * T1 *

( T1/2 + T2 )) + ( b2 * T2 * ( T2/2 ))) /

( b0 * T0 + b1 * T1 + b2 * T2 )

= (( 1.29 * 2.70 * ( 2.70 /2 + 17.30 + 0.00 )) + ( 1.59* 17.30 *

( 17.30 /2 + 0.00 )) + ( 2.72 * 0.00 * ( 0.00 /2 ))) /

( 1.29 * 2.70 + 1.59 * 17.30 + 2.72 * 0.00 )

= 9.771 mm.

Dist from Neutral axis of c/s to inside surface of the vesssel [Ci]:

Ci = Xbar

= 9.771 mm.

Dist from Neutral axis of c/s to extreme outside surface of the section [Co]:

= -( t - CA - Xbar )

= -( 20.000 - 0.000 - 9.771 )

= -10.229 mm.

Moment of Inertia (Section 13-6, Equation (5)) [I]:

= 0.100 cm**4

Effective Diameter [De]:

= Pitch - (( 6 * I ) / (( t - CA)^2 * (-Co)))

= 69.00 - (( 6 * 0.10 ) / ((20.00 - 0.00 )^2 * (10.23 )))

= 31.848 mm.

Bending Ligament Efficiency [Eb]:

= Pitch - De / Pitch

= 69.000 - 31.848 / 69.000

= 0.538

Ligament Efficiency Calculations (Section 13-6, Equations (1)-(6)):

Em          Eb          Ci          Co

Short-side 1       0.850       0.850      10.000     -10.000

2       0.850       0.850      10.000     -10.000

Long-side  1       0.628       0.628      10.000     -10.000

2       0.571       0.538       9.771     -10.229

Moment of Inertia of a Strip of the Vessel Wall:

Thickness  t1, I1  = 0.0667 cm**4

Thickness  t2, I2  = 0.0667 cm**4

Rectangular Vessel Parameters:

Alpha =      H / h    = 1.6667

K     = (I2/I1)*Alpha = 1.6667

Membrane Stress Calculations per Section 13-9

Membrane Stresses at Short-side 1

Membrane Stress at Short-side 1 [Sms]:

= P * h / ( 2 * t1 ) * { 3 - [( 6 + K *

( 11 - alpha2) / ( 3 + 5 * K )]}

= 80.60 * 60.00 / ( 2 * 20.00 ) * { 3- [( 6 + 1.67 *

( 11 - 1.672) / ( 3 + 5 * 1.67 )]}

= 15.25 N./mm^2

Membrane Stresses at Short-side 2

Membrane Stress at Short-side 2 [Sms]:

= P * h / ( 2 * t1 ) * { 3 - [( 6 + K *

( 11 - alpha2) / ( 3 + 5 * K )]}

= 80.60 * 60.00 / ( 2 * 20.00 ) * { 3- [( 6 + 1.67 *

( 11 - 1.672) / ( 3 + 5 * 1.67 )]}

= 15.25 N./mm^2

Membrane Stresses at Long-side 1

Membrane Stress at Long-side 1 at A [Sml]:

= P * H / 2 * t2

= 80.60 * 100.00 / 2 * 20.00

= 20.15 N./mm^2

If Em( 0.628 ) < E( 0.850 ) and Eb( 0.628 ) < E( 0.850 ) then

Sml =  Sml / Em

= 20.15 / 0.63

= 32.07 N./mm^2

Membrane Stresses at Long-side 2

Membrane Stress at Long-side 2 at A [Sml]:

= P * H / 2 * t2

= 80.60 * 100.00 / 2 * 20.00

= 20.15 N./mm^2

If Em( 0.571 ) < E( 0.850 ) and Eb( 0.538 ) < E( 0.850 ) then

Sml =  Sml / Em

= 20.15 / 0.57

= 35.31 N./mm^2

Membrane Stresses at Stay Plate

Membrane Stress at Stay Plate [t3]:

= P * h / ( 2 * t3 ) * [( 6 + K * ( 11 -

alpha2) / ( 3 + 5 * K )]

= 80.60 * 60.00 / ( 2 * 10.00 ) * [( 6 + 1.67 * ( 11 -

1.672) / ( 3 + 5 * 1.67 )]

= 42.04 N./mm^2

Membrane Stress at Stay Plate [t4]:

= P * h / ( 2 * t4 ) * [( 6 + K * ( 11 -

alpha2) / ( 3 + 5 * K )]

= 80.60 * 60.00 / ( 2 * 10.00 ) * [( 6 + 1.67 * ( 11 -

1.672) / ( 3 + 5 * 1.67 )]

= 42.04 N./mm^2

MEMBRANE STRESSES:  Membrane Stress Calculations per Section 13-9,

Equations (13-15). (N./mm^2) :

STRESS LOCATIONS                                Actual    Allowable

----------------------------------------------------------------------

Short-side 1                                     15.25       131.75

Short-side 2                                     15.25       131.75

Short-side Corner                                15.25       131.75

Long-side  1 at A                                32.07       155.00

Long-side  2 at A                                35.31       155.00

Long-side Corner                                 20.15       131.75

Stay Plate (t3)                                  42.04       114.39

Stay Plate (t4)                                  42.04       114.39

Bending Stress Calculations per Section 13-9

Bending Stresses at Short-side 1

Bending Stress at Short-side 1 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 80.60 * 10.00 / ( 24 * 0.07 ) * [ -3 * 100.002 + 2 * 60.002 *

(( 3 + 5 * 1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))]

= -67.45 N./mm^2

Bending Stress at Short-side 1 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 80.60 * -10.00 /( 24 * 0.07 ) * [ -3 * 100.002 + 2 * 60.002 *

(( 3 + 5 * 1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))]

= 67.45 N./mm^2

Bending Stress at Short-side 1 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 80.60 * 60.002 * 10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))

= 83.69 N./mm^2

Bending Stress at Short-side 1 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 80.60 * 60.002 * -10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))

= -83.69 N./mm^2

Bending Stresses at Short-side 2

Bending Stress at Short-side 2 at N Inner[SbsNi]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 80.60 * 10.00 / ( 24 * 0.07 ) * [ -3 * 100.002 + 2 * 60.002 *

(( 3 + 5 * 1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))]

= -67.45 N./mm^2

Bending Stress at Short-side 2 at N Outer[SbsNo]:

= P * c / ( 24 * I1 ) * [ -3 * H2 + 2 * h2 *

(( 3 + 5 * Alpha2 * K ) / ( 3 + 5 * K ))]

= 80.60 * -10.00 /( 24 * 0.07 ) * [ -3 * 100.002 + 2 * 60.002 *

(( 3 + 5 * 1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))]

= 67.45 N./mm^2

Bending Stress at Short-side 2 at Q Inner[SbsQi]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 80.60 * 60.002 * 10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))

= 83.69 N./mm^2

Bending Stress at Short-side 2 at Q Outer[SbsQo]:

= P * h2 * c / ( 12 * I1 ) * (( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K ))

= 80.60 * 60.002 * -10.00 / ( 12 * 0.07 ) * (( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 ))

= -83.69 N./mm^2

Bending Stresses at Long-side 1

Bending Stress at Long-side 1 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 80.60 * 60.002 * 10.00 / ( 12 * 0.07 ) * [( 3 + 1.67 *

( 6 - 1.672)) / ( 3 + 5 * 1.67 )]

= 26.79 N./mm^2

If Em( 0.628 ) < E( 0.850 ) and Eb( 0.628 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 26.79 / 0.63

= 42.64 N./mm^2

Bending Stress at Long-side 1 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 80.60 * 60.002 * -10.00 / ( 12 * 0.07 ) * [( 3 + 1.67 *

( 6 - 1.672)) / ( 3 + 5 * 1.67 )]

= -26.79 N./mm^2

If Em( 0.628 ) < E( 0.850 ) and Eb( 0.628 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -26.79 / 0.63

= -42.64 N./mm^2

Bending Stress at Long-side 1 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 80.60 * 60.002 * 10.00 / ( 12 * 0.07 ) * [( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 )]

= 83.69 N./mm^2

Bending Stress at Long-side 1 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 80.60 * 60.002 * -10.00 / ( 12 * 0.07 ) * [( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 )]

= -83.69 N./mm^2

Bending Stresses at Long-side 2

Bending Stress at Long-side 2 at M Inner[SblMi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 80.60 * 60.002 * 9.77 / ( 12 * 0.07 ) * [( 3 + 1.67 *

( 6 - 1.672)) / ( 3 + 5 * 1.67 )]

= 26.18 N./mm^2

If Em( 0.571 ) < E( 0.850 ) and Eb( 0.538 ) < E( 0.850 ) then

SblMi =  SblMi / Eb

= 26.18 / 0.54

= 48.61 N./mm^2

Bending Stress at Long-side 2 at M Outer[SblMo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + K *

( 6 - Alpha2)) / ( 3 + 5 * K )]

= 80.60 * 60.002 * -10.23 / ( 12 * 0.07 ) * [( 3 + 1.67 *

( 6 - 1.672)) / ( 3 + 5 * 1.67 )]

= -27.40 N./mm^2

If Em( 0.571 ) < E( 0.850 ) and Eb( 0.538 ) < E( 0.850 ) then

SblMo =  SblMo / Eb

= -27.40 / 0.54

= -50.89 N./mm^2

Bending Stress at Long-side 2 at Q Inner[SblQi]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 80.60 * 60.002 * 9.77 / ( 12 * 0.07 ) * [( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 )]

= 81.77 N./mm^2

Bending Stress at Long-side 2 at Q Outer[SblQo]:

= P * h2 * c / ( 12 * I2 ) * [( 3 + 5 *

Alpha2 * K ) / ( 3 + 5 * K )]

= 80.60 * 60.002 * -10.23 / ( 12 * 0.07 ) * [( 3 + 5 *

1.672 * 1.67 ) / ( 3 + 5 * 1.67 )]

= -85.60 N./mm^2

BENDING STRESSES:   Bending Stress Calculations per Section 13-9,

Equations (16-19). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -67.45       67.45       197.62

at Q                    83.69      -83.69       197.62

Short-side 2 at N                   -67.45       67.45       197.62

at Q                    83.69      -83.69       197.62

Long-side  1 at M                    42.64      -42.64       232.50

at Q                    83.69      -83.69       197.62

Long-side  2 at M                    48.61      -50.89       232.50

at Q                    81.77      -85.60       197.62

Total Stress Calculations per Section 13-9

Total Stresses at Short-side 1

Total Stress at short side 1 at N inner [STS_Ni]:

= Sms + SbsNi

= 15.25 + -67.45

= -52.20 N./mm^2

Total Stress at short side 1 at N outer [STS_No]:

= Sms + SbsNo

= 15.25 + 67.45

= 82.70 N./mm^2

Total Stress at short side 1 at Q inner [STS_Qi]:

= Sms + SbsQi

= 15.25 + 83.69

= 98.94 N./mm^2

Total Stress at short side 1 at Q outer [STS_Qo]:

= Sms + SbsQo

= 15.25 + -83.69

= -68.43 N./mm^2

Total Stresses at Short-side 2

Total Stress at short side 2 at N inner [STS_Ni]:

= Sms + SbsNi

= 15.25 + -67.45

= -52.20 N./mm^2

Total Stress at short side 2 at N outer [STS_No]:

= Sms + SbsNo

= 15.25 + 67.45

= 82.70 N./mm^2

Total Stress at short side 2 at Q inner [STS_Qi]:

= Sms + SbsQi

= 15.25 + 83.69

= 98.94 N./mm^2

Total Stress at short side 2 at Q outer [STS_Qo]:

= Sms + SbsQo

= 15.25 + -83.69

= -68.43 N./mm^2

Total Stresses at Long-side 1

Total Stress at long side 1 at M inner [STL_Mi]:

= Sml + SblMi

= 32.07 + 42.64

= 74.71 N./mm^2

Total Stress at long side 1 at M outer [STL_Mo]:

= Sml + SblMo

= 32.07 + -42.64

= -10.57 N./mm^2

Total Stress at long side 1 at Q inner [STL_Qi]:

= Sml + SblQi

= 20.15 + 83.69

= 103.84 N./mm^2

Total Stress at long side 1 at Q outer [STL_Qo]:

= SmlB + SblQo

= 20.15 + -83.69

= -63.54 N./mm^2

Total Stresses at Long-side 2

Total Stress at long side 2 at M inner [STL_Mi]:

= Sml + SblMi

= 35.31 + 48.61

= 83.92 N./mm^2

Total Stress at long side 2 at M outer [STL_Mo]:

= Sml + SblMo

= 35.31 + -50.89

= -15.59 N./mm^2

Total Stress at long side 2 at Q inner [STL_Qi]:

= Sml + SblQi

= 20.15 + 81.77

= 101.92 N./mm^2

Total Stress at long side 2 at Q outer [STL_Qo]:

= SmlB + SblQo

= 20.15 + -85.60

= -65.45 N./mm^2

TOTAL STRESSES:     Total Stress Calculations per Section 13-9,

Equations (20-24). (N./mm^2) :

STRESS LOCATIONS                    Inner       Outer     Allowable

----------------------------------------------------------------------

Short-side 1 at N                   -52.20       82.70       197.62

at Q                    98.94      -68.43       197.62

Short-side 2 at N                   -52.20       82.70       197.62

at Q                    98.94      -68.43       197.62

Long-side  1 at M                    74.71      -10.57       232.50

at Q                   103.84      -63.54       197.62

Long-side  2 at M                    83.92      -15.59       232.50

at Q                   101.92      -65.45       197.62

End Plate Stresses (N./mm^2):

Actual    Allowable

End Plate                                        88.67       155.00

Required End Plate thickness due to Internal Pressure [trEP]:

= d * sqrt( Z * C * P / ( SE ) ) + ca

= 100.000 * sqrt( 2.200 * 0.200 * 80.600 / ( 155.000 ) ) + 0.000

= 15.127 mm.

End Plate MAWP at given Thickness [MAWPEP]:

= ((T-ca)/d)^2 * ((SE)/(C*Z)) per UG-34 (c)(3)

= ((20.0000-0.0000)/100.0000)^2*((155)/(.20*2.20))

= 140.901 bars

where Z is:

= min( 3.4 - 2.4( d/D ), 2.5 )

= min( 3.4 - 2.4( 100.000 / 200.000 ), 2.5 )

= 2.200

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)             42.04   N./mm^2

High Stress Percentage                           36.75   %

M.A.W.P. for Membrane Stresses                  219.31   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)            -85.60   N./mm^2

High Stress Percentage                           43.32   %

M.A.W.P. for Bending Stresses                   186.07   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)            103.84   N./mm^2

High Stress Percentage                           52.54   %

M.A.W.P. for Total Stresses                     153.40   bars

Rectangular Vessel Results For Item 1 : A8

SUMMARY OF RESULTS:

MEMBRANE STRESS SUMMARY,

High Stress (Highest % of Allowable)             42.04   N./mm^2

High Stress Percentage                           36.75   %

M.A.W.P. for Membrane Stresses                  219.31   bars

BENDING STRESS SUMMARY,

High Stress (Highest % of Allowable)            -85.60   N./mm^2

High Stress Percentage                           43.32   %

M.A.W.P. for Bending Stresses                   186.07   bars

TOTAL STRESS SUMMARY,

High Stress (Highest % of Allowable)            103.84   N./mm^2

High Stress Percentage                           52.54   %

M.A.W.P. for Total Stresses                     153.40   bars

## FINITE ELEMENT FOR AE-2101

Tabular Results

Results were generated with the finite element program FE/Pipe®. Stress results are post-processed in accordance with the rules specified in ASME Section III and ASME Section VIII, Division 2.

Analysis Time Stamp: Tue Oct 01 08:56:40 2024.

- Model Notes, Tube Results and API Check

- Load Case Report

- Solution Data

03

03

ASME Code Stress Output Plots

- Stress Results - Notes

- ASME Overstressed Areas

- Highest Primary Stress Ratios

- Highest Secondary Stress Ratios

- Highest Fatigue Stress Ratios

- Highest Stress Ratios Per Region

- Compressive Stress Summary

- Graphical Results

Model Notes, Tube Results and API Check

Model Notes, Tube Results and API Check

661PRO 3.0 - API 661 - Air-Cooled Heat Exchanger Nozzle Model

Input Echo:

Notes:

-  The centerline of the header box is along the -Z axis.

-  Only 1 Nozzle will be modeled at a time. The Nozzle Number

Modeled can be seen below.

-  User Defined Loads API 661 Check is shown below.

-  Sum of ALL Nozzle Loads must not exceed 3*API Allowable

Loads shown in paragraph 7.1.10.2 according to API 661

2013 paragraph 7.1.10.3.

-  Results are given as: Actual Load, Allowable Load,

percentage of allowable for API Check in lbs. and ft-lbs. for

English Units and N. and N-m for SI Units.

Nozzle Number Modelled        : 1

Top or Bottom Nozzle          : Top

Analysis Type                 : User Defined Loads

Nozzle Type                   : Straight

Flange Type                   : Weldneck

Header Box Dimensions:

Centerline on                 :Centerline on LEFT  Side

Height                        :     323.000 mm.

Width                         :     200.000 mm.

Length                        :    1842.000 mm.

Outboard Thickness            :      20.000 mm.

03

03

Tubesheet Side Thickness      :      20.000 mm.

Top Plate Thickness           :      20.000 mm.

Free End Plate Thickness      :      20.000 mm.

Header Box Pressure           :       2.200 MPa

Symmetric Boundary Condition at Centerline

Parition Plates:

Number of Partition Plates    :    2

Partition Plate #1

Plate Elevation from Bottom   :      91.000 mm.

Plate Thickness               :      10.000 mm.

Partition Plate #2

Plate Elevation from Bottom   :     223.000 mm.

Plate Thickness               :      10.000 mm.

Nozzle Dimensions:

Distance to CenterLine        :     921.000 mm.

Nozzle Outside Diameter       :     168.300 mm.

Nozzle Thickness              :       9.600 mm.

Projection to Face of Flange  :     260.000 mm.

Flange Thickness Used         :      31.800 mm.

Flange ID                     :     202.480 mm.

Flange Hub Length Used        :      47.700 mm.

Weld Size and SCF            : 0.000, 1.350

Engineering Details:

Operating  Cycles            :        7000

Occasional Cycles            :           0

Ambient Temperature          :      21.100 deg.

Computation Type             : Gauss Average

Material Properties

Nozzle:

Cold Allowable Stress :     115.100 MPa

Hot  Allowable Stress :      86.600 MPa

Elastic Modulus       :   0.186E+06 MPa

Poissons Ratio        :       0.300

Material ID           :4-Austenitic Steels

Density               :   0.000E+00  N /cu. mm.

Cold Yield Stress     :     172.400 MPa

Hot  Yield Stress     :     129.900 MPa

Cold Tensile Stress   :     482.600 MPa

Density               :   0.000E+00  N /cu. mm.

Header Box:

Cold Allowable Stress :     115.100 MPa

Hot  Allowable Stress :      86.600 MPa

Elastic Modulus       :   0.186E+06 MPa

Poissons Ratio        :       0.300

Material ID           :4-Austenitic Steels

Density               :   0.000E+00  N /cu. mm.

Cold Yield Stress     :     172.400 MPa

Hot  Yield Stress     :     129.900 MPa

Cold Tensile Stress   :     482.600 MPa

Density               :   0.000E+00  N /cu. mm.

Tubesheet was not modeled

03

03

User Defined Loads:

Forces [  N ]       Moments [ mm.-  N ]

FX        FY        FZ        MX        MY         MZ

----------------------------------------------------------------------------

Weight         12000      15090      15090    6420000    9150000    4890000

Operating      12000      15090      15090    6420000    9150000    4890000

Occasional         0          0          0          0          0          0

No Tube Results

API Check for User Defined Loads and API Evaluation

API 661 2013 paragraph 7.1.10.2 and 7.1.10.3.

Results below are shown as:

Actual Load, Allowable Load, % of Allowable

Allowable Load: 3x API Allowable from para. 7.1.10.2

Total Nozzle Loads per DOF in [N and N.m]

Global FX       :       12000, 15015,    79 %

Global FY       :       15090, 30030,    50 %

Global FZ       :       15090, 25020,    60 %

Global MX       :        6420,  9150,    70 %

Global MY       :        9150, 12195,    75 %

Global MZ       :        4890,  6105,    80 %

API 661 para. 7.1.10.3 check PASSED. The summation of

all nozzle loads did NOT exceed the 1.5x API value from

para. 7.1.10.2.

Symmetric boundary condition applied at the center of the

headerbox, the API multiplier from paragraph 7.1.10.3 will

be 1.5 since total loads will be 2x larger due to symmetry.

Table of Contents

Load Case Report

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             8:55am   OCT  1,2024

Load Case Report                                       $X

Inner and outer element temperatures are the same

throughout the model.  No thermal ratcheting

calculations will be performed.

THE   4  LOAD CASES ANALYZED ARE:

1   WEIGHT ONLY    (Wgt Only)

Weight ONLY case run to get the stress range

between the installed and the operating states.

/-------- Loads in Case   1

Loads due to Weight

2   SUSTAINED      (Wgt+Pr)

Sustained case run to satisfy local primary

membrane and bending stress limits.

/-------- Loads in Case   2

Loads due to Weight

03

03

Pressure Case    1

3   OPERATING

Case run to compute the operating stresses used in

secondary, peak and range calculations as needed.

/-------- Loads in Case   3

Pressure Case    1

Loads from (Operating)

4   RANGE          (Fatigue Calc Performed)

Case run to get the RANGE of stresses.

as described in NB-3222.2, 5.5.3.2, 5.5.5.2 or 5.5.6.1.

/-------- Combinations in Range Case   4

Plus  Stress Results from CASE   3

Minus Stress Results from CASE   1

Table of Contents

Solution Data

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             8:55am   OCT  1,2024

Solution Data

Maximum Solution Row Size =    1758

Number of Nodes           =    8591

Number of Elements        =    2923

Number of Solution Cases  =       3

Summation of Loads per Case

Case #               FX                  FY                  FZ

1              12000.              53502.              15090.

2              12000.             -44684.            -104898.

3              12000.             -44684.            -104898.

Table of Contents

ASME Code Stress Output Plots

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             8:56am   OCT  1,2024

ASME Code Stress Output Plots                          $X

1) Pl < SPL (SUS,Membrane) Case 2

2) Qb < SPS (SUS,Bending) Case 2

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

03

03

5) S1+S2+S3 < 4S (SUS,S1+S2+S3) Case 2

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

8) Membrane < User (OPE,Membrane) Case 3

9) Bending < User (OPE,Bending) Case 3

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Table of Contents

Stress Results - Notes

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             8:56am   OCT  1,2024

Stress Results - Notes

- Results  in  this  analysis  were  generated using the finite

element solution method.

- Using 2019 ASME Section VIII Division 2

- Use Polished Bar fatigue curve.

- Ratio between Operating and Design Pressure =   1.000000

Range cases use operating pressure. Primary cases

use design pressure.

- Assume free end displacements of attached pipe

(e.g. thermal loads) are secondary loads.

- Primary bending stresses at discontinuities

are treated like secondary stresses. (Pb=0)

- Use Equivalent Stress (Von Mises).

- TRIAXIAL Stress Guidelines:

S1+S2+S3 evaluation omitted from operating stress.

Include S1+S2+S3 evaluation in primary case evaluation.

Bending stress NOT included for all S1+S2+S3 calculations.

- Use local tensor values for averaged and not averaged stresses.

Table of Contents

ASME Overstressed Areas

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             8:56am   OCT  1,2024

03

03

ASME Overstressed Areas                                $X

*** NO OVERSTRESSED NODES IN THIS MODEL ***

Table of Contents

Highest Primary Stress Ratios

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             8:56am   OCT  1,2024

Highest Primary Stress Ratios                          $X

Circ Plate for Plate # 1

Pl            SPL     Primary Membrane Load Case 2

43            130     Min Prin. Stress = -65. (51% Neg, 8% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

33%

Long Plate for Plate # 1

Pl            SPL     Primary Membrane Load Case 2

103            130     Min Prin. Stress = -107. (97% Neg, 19% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

79%

Circ Plate for Plate # 2

Pl            SPL     Primary Membrane Load Case 2

50            130     Min Prin. Stress = -27. (58% Neg, 9% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

38%

Circ Plate for Plate # 3

Pl            SPL     Primary Membrane Load Case 2

122            130     Min Prin. Stress = -175. (89% Neg, 49% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

93%

Circ Plate for Plate # 4

Pl            SPL     Primary Membrane Load Case 2

93            130     Min Prin. Stress = -113. (76% Neg, 20% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

71%

Long Plate for Plate # 4

Pl            SPL     Primary Membrane Load Case 2

22            130     Min Prin. Stress = -15. (79% Neg, 12% NegHi)

03

03

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

16%

Circ Plate for Plate # 5

Pl            SPL     Primary Membrane Load Case 2

35            130     Min Prin. Stress = -10. (42% Neg, 11% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

26%

Table of Contents

Highest Secondary Stress Ratios

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             8:56am   OCT  1,2024

Highest Secondary Stress Ratios                        $X

In combination case  4 the max range stress divided

by the max component stress is 1.99. The case tensor components

are in some directions additive and so the combination

case will have HIGHER stresses than the largest of any

of the individual cases by more than 50%.

Load      Combined/Max       Combined/Max

Case        (Inside)           (Outside)

------    -------------      -------------

4         1.993               1.970

Circ Plate for Plate # 1

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

88            303     Min Prin. Stress = -65. (51% Neg, 8% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

29%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

88            303     Min Prin. Stress = -65. (51% Neg, 8% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

29%

Long Plate for Plate # 1

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

190            303     Min Prin. Stress = -107. (97% Neg, 19% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

62%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

190            303     Min Prin. Stress = -107. (97% Neg, 19% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

62%

Circ Plate for Plate # 2

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

03

03

75            303     Min Prin. Stress = -27. (58% Neg, 9% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

24%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

75            303     Min Prin. Stress = -27. (58% Neg, 9% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

24%

Circ Plate for Plate # 3

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

245            303     Min Prin. Stress = -175. (89% Neg, 49% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

80%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

245            303     Min Prin. Stress = -175. (89% Neg, 49% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

80%

Circ Plate for Plate # 4

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

157            303     Min Prin. Stress = -113. (76% Neg, 20% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

52%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

157            303     Min Prin. Stress = -113. (76% Neg, 20% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

52%

Long Plate for Plate # 4

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

39            303     Min Prin. Stress = -15. (79% Neg, 12% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

12%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

39            303     Min Prin. Stress = -15. (79% Neg, 12% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

12%

Circ Plate for Plate # 5

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

56            302     Min Prin. Stress = -10. (42% Neg, 11% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

18%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

56            302     Min Prin. Stress = -10. (42% Neg, 11% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

18%

03

03

Table of Contents

Highest Fatigue Stress Ratios

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             8:56am   OCT  1,2024

Highest Fatigue Stress Ratios                          $X

Circ Plate for Plate # 1

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

40     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.052 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 53,800,080.

WRC 474 99% Probability Cycles = 12,498,247.

5%                    WRC 474 95% Probability Cycles = 17,352,244.

BS5500 Allowed Cycles(Curve F) = 17,406,096.

Membrane-to-Bending Ratio = 0.287

Bending-to-PL+PB+Q Ratio = 0.777

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Long Plate for Plate # 1

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

67     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.087 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 12,005,280.

WRC 474 99% Probability Cycles = 2,788,933.

8%                    WRC 474 95% Probability Cycles = 3,872,082.

BS5500 Allowed Cycles(Curve F) = 2,956,780.

Membrane-to-Bending Ratio = 0.249

Bending-to-PL+PB+Q Ratio = 0.800

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Circ Plate for Plate # 2

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

40     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.052 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 53,800,080.

WRC 474 99% Probability Cycles = 12,498,247.

5%                    WRC 474 95% Probability Cycles = 17,352,244.

BS5500 Allowed Cycles(Curve F) = 17,406,096.

Membrane-to-Bending Ratio = 0.287

Bending-to-PL+PB+Q Ratio = 0.777

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Circ Plate for Plate # 3

03

03

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

62     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.079 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 22,928,718.

WRC 474 99% Probability Cycles = 5,326,544.

7%                    WRC 474 95% Probability Cycles = 7,395,236.

BS5500 Allowed Cycles(Curve F) = 3,877,636.

Membrane-to-Bending Ratio = 0.577

Bending-to-PL+PB+Q Ratio = 0.634

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Circ Plate for Plate # 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

29     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.038 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 2.2774E8

WRC 474 99% Probability Cycles = 52,906,380.

3%                    WRC 474 95% Probability Cycles = 73,453,848.

BS5500 Allowed Cycles(Curve F) = 86,924,216.

Membrane-to-Bending Ratio = 0.912

Bending-to-PL+PB+Q Ratio = 0.523

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Long Plate for Plate # 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

10     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.013 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 2.2647E9

WRC 474 99% Probability Cycles = 5.2612E8

1%                    WRC 474 95% Probability Cycles = 7.3045E8

BS5500 Allowed Cycles(Curve F) = 7.5188E9

Membrane-to-Bending Ratio = 0.919

Bending-to-PL+PB+Q Ratio = 0.521

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Circ Plate for Plate # 5

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

29     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.054 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 0.0

530.5                    Markl Fatigue Stress Allowable = 575.0

MPa                    WRC 474 Mean Cycles to Failure = 2.1487E8

WRC 474 99% Probability Cycles = 49,916,348.

5%                    WRC 474 95% Probability Cycles = 69,302,576.

BS5500 Allowed Cycles(Curve F) = 91,314,408.

Membrane-to-Bending Ratio = 9.370

Bending-to-PL+PB+Q Ratio = 0.096

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Table of Contents

03

03

Highest Stress Ratios Per Region

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             8:56am   OCT  1,2024

Highest Stress Ratios Per Region                       $X

Circ Plate for Plate # 1

Pl            SPL     Primary Membrane Load Case 2

43            130     Min Prin. Stress = -65. (51% Neg, 8% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

33%

Qb            SPS     Primary Bending Load Case 2

59            303     Min Prin. Stress = -65. (51% Neg, 8% NegHi)

MPa            MPa     Plot Reference:

2) Qb < SPS (SUS,Bending) Case 2

19%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

88            303     Min Prin. Stress = -65. (51% Neg, 8% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

29%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

72            303     Min Prin. Stress = -65. (51% Neg, 8% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

23%

S1+S2+S3             4S     Part 5 (5.3.2) Load Case 2

46            346     Min Prin. Stress = -65. (51% Neg, 8% NegHi)

MPa            MPa     Plot Reference:

5) S1+S2+S3 < 4S (SUS,S1+S2+S3) Case 2

13%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

88            303     Min Prin. Stress = -65. (51% Neg, 8% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

29%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

72            303     Min Prin. Stress = -65. (51% Neg, 8% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

23%

Membrane           User     Component Evaluation Load Case 3

43            303     Min Prin. Stress = -65. (51% Neg, 8% NegHi)

MPa            MPa     Plot Reference:

8) Membrane < User (OPE,Membrane) Case 3

14%

Bending           User     Component Evaluation Load Case 3

59            303     Min Prin. Stress = -65. (51% Neg, 8% NegHi)

MPa            MPa     Plot Reference:

9) Bending < User (OPE,Bending) Case 3

19%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 4

40            303     Min Prin. Stress = -16. (49% Neg, 7% NegHi)

03

03

MPa            MPa     Plot Reference:

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

13%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 4

35            303     Min Prin. Stress = -16. (49% Neg, 7% NegHi)

MPa            MPa     Plot Reference:

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

11%

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

40     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.052 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 53,800,080.

WRC 474 99% Probability Cycles = 12,498,247.

5%                    WRC 474 95% Probability Cycles = 17,352,244.

BS5500 Allowed Cycles(Curve F) = 17,406,096.

Membrane-to-Bending Ratio = 0.287

Bending-to-PL+PB+Q Ratio = 0.777

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

35     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.045 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 80,855,728.

WRC 474 99% Probability Cycles = 18,783,518.

4%                    WRC 474 95% Probability Cycles = 26,078,552.

BS5500 Allowed Cycles(Curve F) = 34,168,636.

Membrane-to-Bending Ratio = 0.359

Bending-to-PL+PB+Q Ratio = 0.736

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Long Plate for Plate # 1

Pl            SPL     Primary Membrane Load Case 2

103            130     Min Prin. Stress = -107. (97% Neg, 19% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

79%

Qb            SPS     Primary Bending Load Case 2

139            303     Min Prin. Stress = -107. (97% Neg, 19% NegHi)

MPa            MPa     Plot Reference:

2) Qb < SPS (SUS,Bending) Case 2

45%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

138            303     Min Prin. Stress = -107. (97% Neg, 19% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

45%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

190            303     Min Prin. Stress = -107. (97% Neg, 19% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

62%

S1+S2+S3             4S     Part 5 (5.3.2) Load Case 2

122            346     Min Prin. Stress = -107. (97% Neg, 19% NegHi)

MPa            MPa     Plot Reference:

5) S1+S2+S3 < 4S (SUS,S1+S2+S3) Case 2

35%

03

03

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

138            303     Min Prin. Stress = -107. (97% Neg, 19% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

45%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

190            303     Min Prin. Stress = -107. (97% Neg, 19% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

62%

Membrane           User     Component Evaluation Load Case 3

103            303     Min Prin. Stress = -107. (97% Neg, 19% NegHi)

MPa            MPa     Plot Reference:

8) Membrane < User (OPE,Membrane) Case 3

33%

Bending           User     Component Evaluation Load Case 3

139            303     Min Prin. Stress = -107. (97% Neg, 19% NegHi)

MPa            MPa     Plot Reference:

9) Bending < User (OPE,Bending) Case 3

45%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 4

67            303     Min Prin. Stress = -32. (91% Neg, 15% NegHi)

MPa            MPa     Plot Reference:

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

22%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 4

67            303     Min Prin. Stress = -32. (91% Neg, 15% NegHi)

MPa            MPa     Plot Reference:

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

22%

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

67     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.086 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 12,298,224.

WRC 474 99% Probability Cycles = 2,856,986.

8%                    WRC 474 95% Probability Cycles = 3,966,566.

BS5500 Allowed Cycles(Curve F) = 3,014,588.

Membrane-to-Bending Ratio = 0.248

Bending-to-PL+PB+Q Ratio = 0.802

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

67     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.087 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 12,005,280.

WRC 474 99% Probability Cycles = 2,788,933.

8%                    WRC 474 95% Probability Cycles = 3,872,082.

BS5500 Allowed Cycles(Curve F) = 2,956,780.

Membrane-to-Bending Ratio = 0.249

Bending-to-PL+PB+Q Ratio = 0.800

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Circ Plate for Plate # 2

Pl            SPL     Primary Membrane Load Case 2

50            130     Min Prin. Stress = -27. (58% Neg, 9% NegHi)

03

03

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

38%

Qb            SPS     Primary Bending Load Case 2

54            303     Min Prin. Stress = -27. (58% Neg, 9% NegHi)

MPa            MPa     Plot Reference:

2) Qb < SPS (SUS,Bending) Case 2

17%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

57            303     Min Prin. Stress = -27. (58% Neg, 9% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

18%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

75            303     Min Prin. Stress = -27. (58% Neg, 9% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

24%

S1+S2+S3             4S     Part 5 (5.3.2) Load Case 2

28            346     Min Prin. Stress = -27. (58% Neg, 9% NegHi)

MPa            MPa     Plot Reference:

5) S1+S2+S3 < 4S (SUS,S1+S2+S3) Case 2

8%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

57            303     Min Prin. Stress = -27. (58% Neg, 9% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

18%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

75            303     Min Prin. Stress = -27. (58% Neg, 9% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

24%

Membrane           User     Component Evaluation Load Case 3

50            303     Min Prin. Stress = -27. (58% Neg, 9% NegHi)

MPa            MPa     Plot Reference:

8) Membrane < User (OPE,Membrane) Case 3

16%

Bending           User     Component Evaluation Load Case 3

54            303     Min Prin. Stress = -27. (58% Neg, 9% NegHi)

MPa            MPa     Plot Reference:

9) Bending < User (OPE,Bending) Case 3

17%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 4

40            303     Min Prin. Stress = -16. (52% Neg, 9% NegHi)

MPa            MPa     Plot Reference:

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

13%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 4

35            303     Min Prin. Stress = -16. (52% Neg, 9% NegHi)

MPa            MPa     Plot Reference:

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

11%

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

40     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.052 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 53,800,080.

WRC 474 99% Probability Cycles = 12,498,247.

03

03

5%                    WRC 474 95% Probability Cycles = 17,352,244.

BS5500 Allowed Cycles(Curve F) = 17,406,096.

Membrane-to-Bending Ratio = 0.287

Bending-to-PL+PB+Q Ratio = 0.777

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

35     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.045 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 80,855,760.

WRC 474 99% Probability Cycles = 18,783,526.

4%                    WRC 474 95% Probability Cycles = 26,078,564.

BS5500 Allowed Cycles(Curve F) = 34,168,652.

Membrane-to-Bending Ratio = 0.359

Bending-to-PL+PB+Q Ratio = 0.736

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Circ Plate for Plate # 3

Pl            SPL     Primary Membrane Load Case 2

122            130     Min Prin. Stress = -175. (89% Neg, 49% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

93%

Qb            SPS     Primary Bending Load Case 2

203            303     Min Prin. Stress = -175. (89% Neg, 49% NegHi)

MPa            MPa     Plot Reference:

2) Qb < SPS (SUS,Bending) Case 2

67%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

245            303     Min Prin. Stress = -175. (89% Neg, 49% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

80%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

185            303     Min Prin. Stress = -175. (89% Neg, 49% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

61%

S1+S2+S3             4S     Part 5 (5.3.2) Load Case 2

170            346     Min Prin. Stress = -175. (89% Neg, 49% NegHi)

MPa            MPa     Plot Reference:

5) S1+S2+S3 < 4S (SUS,S1+S2+S3) Case 2

49%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

245            303     Min Prin. Stress = -175. (89% Neg, 49% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

80%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

185            303     Min Prin. Stress = -175. (89% Neg, 49% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

61%

Membrane           User     Component Evaluation Load Case 3

122            303     Min Prin. Stress = -175. (89% Neg, 49% NegHi)

MPa            MPa     Plot Reference:

8) Membrane < User (OPE,Membrane) Case 3

40%

03

03

Bending           User     Component Evaluation Load Case 3

203            303     Min Prin. Stress = -175. (89% Neg, 49% NegHi)

MPa            MPa     Plot Reference:

9) Bending < User (OPE,Bending) Case 3

67%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 4

62            303     Min Prin. Stress = -40. (99% Neg, 28% NegHi)

MPa            MPa     Plot Reference:

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

20%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 4

34            303     Min Prin. Stress = -40. (99% Neg, 28% NegHi)

MPa            MPa     Plot Reference:

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

11%

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

62     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.079 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 22,928,718.

WRC 474 99% Probability Cycles = 5,326,544.

7%                    WRC 474 95% Probability Cycles = 7,395,236.

BS5500 Allowed Cycles(Curve F) = 3,877,636.

Membrane-to-Bending Ratio = 0.577

Bending-to-PL+PB+Q Ratio = 0.634

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

34     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.044 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 1.3199E8

WRC 474 99% Probability Cycles = 30,662,112.

4%                    WRC 474 95% Probability Cycles = 42,570,488.

BS5500 Allowed Cycles(Curve F) = 39,697,960.

Membrane-to-Bending Ratio = 4.769

Bending-to-PL+PB+Q Ratio = 0.173

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Circ Plate for Plate # 4

Pl            SPL     Primary Membrane Load Case 2

93            130     Min Prin. Stress = -113. (76% Neg, 20% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

71%

Qb            SPS     Primary Bending Load Case 2

100            303     Min Prin. Stress = -113. (76% Neg, 20% NegHi)

MPa            MPa     Plot Reference:

2) Qb < SPS (SUS,Bending) Case 2

32%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

157            303     Min Prin. Stress = -113. (76% Neg, 20% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

52%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

113            303     Min Prin. Stress = -113. (76% Neg, 20% NegHi)

MPa            MPa     Plot Reference:

03

03

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

37%

S1+S2+S3             4S     Part 5 (5.3.2) Load Case 2

148            346     Min Prin. Stress = -113. (76% Neg, 20% NegHi)

MPa            MPa     Plot Reference:

5) S1+S2+S3 < 4S (SUS,S1+S2+S3) Case 2

42%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

157            303     Min Prin. Stress = -113. (76% Neg, 20% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

52%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

113            303     Min Prin. Stress = -113. (76% Neg, 20% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

37%

Membrane           User     Component Evaluation Load Case 3

93            303     Min Prin. Stress = -113. (76% Neg, 20% NegHi)

MPa            MPa     Plot Reference:

8) Membrane < User (OPE,Membrane) Case 3

30%

Bending           User     Component Evaluation Load Case 3

100            303     Min Prin. Stress = -113. (76% Neg, 20% NegHi)

MPa            MPa     Plot Reference:

9) Bending < User (OPE,Bending) Case 3

32%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 4

29            303     Min Prin. Stress = -19. (60% Neg, 11% NegHi)

MPa            MPa     Plot Reference:

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

9%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 4

28            303     Min Prin. Stress = -19. (60% Neg, 11% NegHi)

MPa            MPa     Plot Reference:

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

9%

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

29     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.038 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 2.2774E8

WRC 474 99% Probability Cycles = 52,906,380.

3%                    WRC 474 95% Probability Cycles = 73,453,848.

BS5500 Allowed Cycles(Curve F) = 86,924,216.

Membrane-to-Bending Ratio = 0.912

Bending-to-PL+PB+Q Ratio = 0.523

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

28     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.036 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 2.5213E8

WRC 474 99% Probability Cycles = 58,572,912.

3%                    WRC 474 95% Probability Cycles = 81,321,112.

BS5500 Allowed Cycles(Curve F) = 1.0450E8

Membrane-to-Bending Ratio = 1.230

Bending-to-PL+PB+Q Ratio = 0.448

03

03

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Long Plate for Plate # 4

Pl            SPL     Primary Membrane Load Case 2

22            130     Min Prin. Stress = -15. (79% Neg, 12% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

16%

Qb            SPS     Primary Bending Load Case 2

26            303     Min Prin. Stress = -15. (79% Neg, 12% NegHi)

MPa            MPa     Plot Reference:

2) Qb < SPS (SUS,Bending) Case 2

8%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

24            303     Min Prin. Stress = -15. (79% Neg, 12% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

7%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

39            303     Min Prin. Stress = -15. (79% Neg, 12% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

12%

S1+S2+S3             4S     Part 5 (5.3.2) Load Case 2

17            346     Min Prin. Stress = -15. (79% Neg, 12% NegHi)

MPa            MPa     Plot Reference:

5) S1+S2+S3 < 4S (SUS,S1+S2+S3) Case 2

5%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

24            303     Min Prin. Stress = -15. (79% Neg, 12% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

7%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

39            303     Min Prin. Stress = -15. (79% Neg, 12% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

12%

Membrane           User     Component Evaluation Load Case 3

22            303     Min Prin. Stress = -15. (79% Neg, 12% NegHi)

MPa            MPa     Plot Reference:

8) Membrane < User (OPE,Membrane) Case 3

7%

Bending           User     Component Evaluation Load Case 3

26            303     Min Prin. Stress = -15. (79% Neg, 12% NegHi)

MPa            MPa     Plot Reference:

9) Bending < User (OPE,Bending) Case 3

8%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 4

10            303     Min Prin. Stress = -4. (51% Neg, 8% NegHi)

MPa            MPa     Plot Reference:

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

3%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 4

7            303     Min Prin. Stress = -4. (51% Neg, 8% NegHi)

MPa            MPa     Plot Reference:

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

2%

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

03

03

10     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.013 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 2.2647E9

WRC 474 99% Probability Cycles = 5.2612E8

1%                    WRC 474 95% Probability Cycles = 7.3045E8

BS5500 Allowed Cycles(Curve F) = 7.5188E9

Membrane-to-Bending Ratio = 0.919

Bending-to-PL+PB+Q Ratio = 0.521

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

7     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.008 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 504.2

777.2                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 9.5210E9

WRC 474 99% Probability Cycles = 2.2118E9

0%                    WRC 474 95% Probability Cycles = 3.0708E9

BS5500 Allowed Cycles(Curve F) = 5.9365E10

Membrane-to-Bending Ratio = 0.913

Bending-to-PL+PB+Q Ratio = 0.523

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Circ Plate for Plate # 5

Pl            SPL     Primary Membrane Load Case 2

35            130     Min Prin. Stress = -10. (42% Neg, 11% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

26%

Qb            SPS     Primary Bending Load Case 2

23            302     Min Prin. Stress = -10. (42% Neg, 11% NegHi)

MPa            MPa     Plot Reference:

2) Qb < SPS (SUS,Bending) Case 2

7%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

47            302     Min Prin. Stress = -10. (42% Neg, 11% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

15%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

56            302     Min Prin. Stress = -10. (42% Neg, 11% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

18%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

47            302     Min Prin. Stress = -10. (42% Neg, 11% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

15%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

56            302     Min Prin. Stress = -10. (42% Neg, 11% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

18%

Membrane           User     Component Evaluation Load Case 3

35            302     Min Prin. Stress = -10. (42% Neg, 11% NegHi)

MPa            MPa     Plot Reference:

03

03

8) Membrane < User (OPE,Membrane) Case 3

11%

Bending           User     Component Evaluation Load Case 3

23            302     Min Prin. Stress = -10. (42% Neg, 11% NegHi)

MPa            MPa     Plot Reference:

9) Bending < User (OPE,Bending) Case 3

7%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 4

29            302     Min Prin. Stress = -3. (17% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

9%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 4

29            302     Min Prin. Stress = -3. (17% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

9%

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

29     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.054 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 0.0

530.5                    Markl Fatigue Stress Allowable = 575.0

MPa                    WRC 474 Mean Cycles to Failure = 2.1487E8

WRC 474 99% Probability Cycles = 49,916,348.

5%                    WRC 474 95% Probability Cycles = 69,302,576.

BS5500 Allowed Cycles(Curve F) = 91,314,408.

Membrane-to-Bending Ratio = 9.370

Bending-to-PL+PB+Q Ratio = 0.096

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

29     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.054 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 0.0

530.5                    Markl Fatigue Stress Allowable = 575.0

MPa                    WRC 474 Mean Cycles to Failure = 2.2377E8

WRC 474 99% Probability Cycles = 51,982,772.

5%                    WRC 474 95% Probability Cycles = 72,171,536.

BS5500 Allowed Cycles(Curve F) = 97,284,968.

Membrane-to-Bending Ratio = 7.767

Bending-to-PL+PB+Q Ratio = 0.114

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Table of Contents

Compressive Stress Summary

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             8:56am   OCT  1,2024

Compressive Stress Summary (MPa)                       $X

Nomenclature:

-------------

Min Stress       - Compressive Membrane and Bending Stress

Pts in Region    - No. of nodes in the model region

>5% Compression  - 5% or more of Compressive Stress Limit

>50% Compression - 50% or more of Compressive Stress Limit

03

03

Compressive Stress Limit = -0.55 Min(Sy, kEt/R), Section

slenderness ratio (elastic buckling) not considered.

>5%    >50%

Min        Pts      Compression

#  Load Type  Case    Stress    in Region   and Bending    Region

--  ---------  ----    ------   ----------  ------------  ----------------

1  SUSTAINED    2        -65.      1856      51%     8%  Circ Plate for Plate # 1

2  OPERATING    3        -65.      1856      51%     8%  Circ Plate for Plate # 1

3  EXPANSION    4        -16.      1856      49%     7%  Circ Plate for Plate # 1

4  SUSTAINED    2       -107.      8072      97%    19%  Long Plate for Plate # 1

5  OPERATING    3       -107.      8072      97%    19%  Long Plate for Plate # 1

6  EXPANSION    4        -32.      8072      91%    15%  Long Plate for Plate # 1

7  SUSTAINED    2        -27.      1504      58%     9%  Circ Plate for Plate # 2

8  OPERATING    3        -27.      1504      58%     9%  Circ Plate for Plate # 2

9  EXPANSION    4        -16.      1504      52%     9%  Circ Plate for Plate # 2

10  SUSTAINED    2       -175.      1600      89%    49%  Circ Plate for Plate # 3

11  OPERATING    3       -175.      1600      89%    49%  Circ Plate for Plate # 3

12  EXPANSION    4        -40.      1600      99%    28%  Circ Plate for Plate # 3

13  SUSTAINED    2       -113.      1440      76%    20%  Circ Plate for Plate # 4

14  OPERATING    3       -113.      1440      76%    20%  Circ Plate for Plate # 4

15  EXPANSION    4        -19.      1440      60%    11%  Circ Plate for Plate # 4

16  SUSTAINED    2        -15.       640      79%    12%  Long Plate for Plate # 4

17  OPERATING    3        -15.       640      79%    12%  Long Plate for Plate # 4

18  EXPANSION    4         -4.       640      51%     8%  Long Plate for Plate # 4

19  SUSTAINED    2        -10.      8272      42%    11%  Circ Plate for Plate # 5

20  OPERATING    3        -10.      8272      42%    11%  Circ Plate for Plate # 5

21  EXPANSION    4         -3.      8272      17%     5%  Circ Plate for Plate # 5

Table of Contents

03

03

03

03

03

03

03

03

03

03

03

03

03

03

## FINITE ELEMENT FOR AE-2102

Tabular Results

Results were generated with the finite element program FE/Pipe®. Stress results are post-processed in accordance with the rules specified in ASME Section III and ASME Section VIII, Division 2.

Analysis Time Stamp: Wed Oct 02 11:48:52 2024.

- Model Notes, Tube Results and API Check

03

03

Load Case Report

- Solution Data

- ASME Code Stress Output Plots

- Stress Results - Notes

- ASME Overstressed Areas

- Highest Primary Stress Ratios

- Highest Secondary Stress Ratios

- Highest Fatigue Stress Ratios

- Highest Stress Ratios Per Region

- Compressive Stress Summary

- Graphical Results

Model Notes, Tube Results and API Check

Model Notes, Tube Results and API Check

661PRO 3.0 - API 661 - Air-Cooled Heat Exchanger Nozzle Model

Input Echo:

Notes:

-  The centerline of the header box is along the -Z axis.

-  Only 1 Nozzle will be modeled at a time. The Nozzle Number

Modeled can be seen below.

-  User Defined Loads API 661 Check is shown below.

-  Sum of ALL Nozzle Loads must not exceed 3*API Allowable

Loads shown in paragraph 7.1.10.2 according to API 661

2013 paragraph 7.1.10.3.

-  Results are given as: Actual Load, Allowable Load,

percentage of allowable for API Check in lbs. and ft-lbs. for

English Units and N. and N-m for SI Units.

Nozzle Number Modelled        : 1

Top or Bottom Nozzle          : Top

Analysis Type                 : User Defined Loads

Nozzle Type                   : Straight

Flange Type                   : Weldneck

Header Box Dimensions:

Centerline on                 :Centerline on LEFT  Side

Height                        :     485.000 mm.

Width                         :     166.000 mm.

Length                        :    2065.000 mm.

03

03

Outboard Thickness            :      28.000 mm.

Tubesheet Side Thickness      :      28.000 mm.

Top Plate Thickness           :      20.000 mm.

Free End Plate Thickness      :      20.000 mm.

Header Box Pressure           :       6.200 MPa

Symmetric Boundary Condition at Centerline

Parition Plates:

Number of Partition Plates    :    2

Partition Plate #1

Plate Elevation from Bottom   :     165.000 mm.

Plate Thickness               :      10.000 mm.

Partition Plate #2

Plate Elevation from Bottom   :     305.000 mm.

Plate Thickness               :      10.000 mm.

Nozzle Dimensions:

Distance to CenterLine        :    1032.500 mm.

Nozzle Outside Diameter       :     114.300 mm.

Nozzle Thickness              :       7.500 mm.

Projection to Face of Flange  :     270.000 mm.

Flange Thickness Used         :      27.300 mm.

Flange ID                     :     154.059 mm.

Flange Hub Length Used        :      40.950 mm.

Weld Size and SCF            : 0.000, 1.350

Engineering Details:

Operating  Cycles            :        7000

Occasional Cycles            :           0

Ambient Temperature          :      21.100 deg.

Computation Type             : Gauss Average

Material Properties

Nozzle:

Cold Allowable Stress :     115.100 MPa

Hot  Allowable Stress :     111.900 MPa

Elastic Modulus       :   0.184E+06 MPa

Poissons Ratio        :       0.300

Material ID           :4-Austenitic Steels

Density               :   0.000E+00  N /cu. mm.

Cold Yield Stress     :     172.400 MPa

Hot  Yield Stress     :     126.100 MPa

Cold Tensile Stress   :     482.700 MPa

Density               :   0.000E+00  N /cu. mm.

Header Box:

Cold Allowable Stress :     115.100 MPa

Hot  Allowable Stress :     111.900 MPa

Elastic Modulus       :   0.184E+06 MPa

Poissons Ratio        :       0.300

Material ID           :4-Austenitic Steels

Density               :   0.000E+00  N /cu. mm.

Cold Yield Stress     :     172.400 MPa

Hot  Yield Stress     :     126.100 MPa

Cold Tensile Stress   :     482.700 MPa

Density               :   0.000E+00  N /cu. mm.

Tubesheet was not modeled

03

03

User Defined Loads:

Forces [  N ]       Moments [ mm.-  N ]

FX        FY        FZ        MX        MY         MZ

----------------------------------------------------------------------------

Weight         10020       8010      10020    2430000    3660000    2430000

Operating      10020       8010      10020    2430000    3660000    2430000

Occasional         0          0          0          0          0          0

No Tube Results

API Check for User Defined Loads and API Evaluation

API 661 2013 paragraph 7.1.10.2 and 7.1.10.3.

Results below are shown as:

Actual Load, Allowable Load, % of Allowable

Allowable Load: 3x API Allowable from para. 7.1.10.2

Total Nozzle Loads per DOF in [N and N.m]

Global FX       :       10020, 15015,    66 %

Global FY       :        8010, 30030,    26 %

Global FZ       :       10020, 25020,    40 %

Global MX       :        2430,  9150,    26 %

Global MY       :        3660, 12195,    30 %

Global MZ       :        2430,  6105,    39 %

API 661 para. 7.1.10.3 check PASSED. The summation of

all nozzle loads did NOT exceed the 1.5x API value from

para. 7.1.10.2.

Symmetric boundary condition applied at the center of the

headerbox, the API multiplier from paragraph 7.1.10.3 will

be 1.5 since total loads will be 2x larger due to symmetry.

Table of Contents

Load Case Report

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             11:46am  OCT  2,2024

Load Case Report                                       $X

Inner and outer element temperatures are the same

throughout the model.  No thermal ratcheting

calculations will be performed.

THE   4  LOAD CASES ANALYZED ARE:

1   WEIGHT ONLY    (Wgt Only)

Weight ONLY case run to get the stress range

between the installed and the operating states.

/-------- Loads in Case   1

Loads due to Weight

2   SUSTAINED      (Wgt+Pr)

Sustained case run to satisfy local primary

membrane and bending stress limits.

/-------- Loads in Case   2

Loads due to Weight

03

03

Pressure Case    1

3   OPERATING

Case run to compute the operating stresses used in

secondary, peak and range calculations as needed.

/-------- Loads in Case   3

Pressure Case    1

Loads from (Operating)

4   RANGE          (Fatigue Calc Performed)

Case run to get the RANGE of stresses.

as described in NB-3222.2, 5.5.3.2, 5.5.5.2 or 5.5.6.1.

/-------- Combinations in Range Case   4

Plus  Stress Results from CASE   3

Minus Stress Results from CASE   1

Table of Contents

Solution Data

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             11:46am  OCT  2,2024

Solution Data

Maximum Solution Row Size =    2466

Number of Nodes           =   13117

Number of Elements        =    4421

Number of Solution Cases  =       3

Summation of Loads per Case

Case #               FX                  FY                  FZ

1              10020.              56025.              10020.

2              10020.            -119625.            -387834.

3              10020.            -119625.            -387834.

Table of Contents

ASME Code Stress Output Plots

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             11:48am  OCT  2,2024

ASME Code Stress Output Plots                          $X

1) Pl < SPL (SUS,Membrane) Case 2

2) Qb < SPS (SUS,Bending) Case 2

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

03

03

5) S1+S2+S3 < 4S (SUS,S1+S2+S3) Case 2

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

8) Membrane < User (OPE,Membrane) Case 3

9) Bending < User (OPE,Bending) Case 3

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Table of Contents

Stress Results - Notes

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             11:48am  OCT  2,2024

Stress Results - Notes

- Results  in  this  analysis  were  generated using the finite

element solution method.

- Using 2019 ASME Section VIII Division 2

- Use Polished Bar fatigue curve.

- Ratio between Operating and Design Pressure =   1.000000

Range cases use operating pressure. Primary cases

use design pressure.

- Assume free end displacements of attached pipe

(e.g. thermal loads) are secondary loads.

- Primary bending stresses at discontinuities

are treated like secondary stresses. (Pb=0)

- Use Equivalent Stress (Von Mises).

- TRIAXIAL Stress Guidelines:

S1+S2+S3 evaluation omitted from operating stress.

Include S1+S2+S3 evaluation in primary case evaluation.

Bending stress NOT included for all S1+S2+S3 calculations.

- Use local tensor values for averaged and not averaged stresses.

Table of Contents

ASME Overstressed Areas

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             11:48am  OCT  2,2024

03

03

ASME Overstressed Areas                                $X

*** NO OVERSTRESSED NODES IN THIS MODEL ***

Table of Contents

Highest Primary Stress Ratios

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             11:48am  OCT  2,2024

Highest Primary Stress Ratios                          $X

Circ Plate for Plate # 1

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

120            340     Min Prin. Stress = -49. (59% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

35%

Long Plate for Plate # 1

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

237            340     Min Prin. Stress = -121. (91% Neg, 29% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

69%

Circ Plate for Plate # 2

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

102            340     Min Prin. Stress = -28. (59% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

30%

Circ Plate for Plate # 3

Pl            SPL     Primary Membrane Load Case 2

143            168     Min Prin. Stress = -192. (77% Neg, 47% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

84%

Circ Plate for Plate # 4

Pl            SPL     Primary Membrane Load Case 2

121            168     Min Prin. Stress = -175. (68% Neg, 22% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

72%

Long Plate for Plate # 4

Pl            SPL     Primary Membrane Load Case 2

03

03

27            168     Min Prin. Stress = -17. (75% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

15%

Circ Plate for Plate # 5

Pl            SPL     Primary Membrane Load Case 2

104            126     Min Prin. Stress = -7. (10% Neg, 3% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

82%

Table of Contents

Highest Secondary Stress Ratios

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             11:48am  OCT  2,2024

Highest Secondary Stress Ratios                        $X

In combination case  4 the max range stress divided

by the max component stress is 1.84. The case tensor components

are in some directions additive and so the combination

case will have HIGHER stresses than the largest of any

of the individual cases by more than 50%.

Load      Combined/Max       Combined/Max

Case        (Inside)           (Outside)

------    -------------      -------------

4         1.838               1.691

Circ Plate for Plate # 1

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

120            340     Min Prin. Stress = -49. (59% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

35%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

120            340     Min Prin. Stress = -49. (59% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

35%

Long Plate for Plate # 1

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

237            340     Min Prin. Stress = -121. (91% Neg, 29% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

69%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

237            340     Min Prin. Stress = -121. (91% Neg, 29% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

03

03

69%

Circ Plate for Plate # 2

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

102            340     Min Prin. Stress = -28. (59% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

30%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

102            340     Min Prin. Stress = -28. (59% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

30%

Circ Plate for Plate # 3

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

271            340     Min Prin. Stress = -192. (77% Neg, 47% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

79%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

271            340     Min Prin. Stress = -192. (77% Neg, 47% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

79%

Circ Plate for Plate # 4

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

210            340     Min Prin. Stress = -175. (68% Neg, 22% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

61%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

210            340     Min Prin. Stress = -175. (68% Neg, 22% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

61%

Long Plate for Plate # 4

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

47            340     Min Prin. Stress = -17. (75% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

13%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

47            340     Min Prin. Stress = -17. (75% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

13%

Circ Plate for Plate # 5

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

115            298     Min Prin. Stress = -7. (10% Neg, 3% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

38%

03

03

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

115            298     Min Prin. Stress = -7. (10% Neg, 3% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

38%

Table of Contents

Highest Fatigue Stress Ratios

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             11:48am  OCT  2,2024

Highest Fatigue Stress Ratios                          $X

Circ Plate for Plate # 1

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

120     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.155 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

771.0                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 1,771,558.

WRC 474 99% Probability Cycles = 411,549.

15%                    WRC 474 95% Probability Cycles = 571,384.

BS5500 Allowed Cycles(Curve F) = 537,311.

Membrane-to-Bending Ratio = 0.385

Bending-to-PL+PB+Q Ratio = 0.722

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Long Plate for Plate # 1

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

169     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.220 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

771.0                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 607,122.

WRC 474 99% Probability Cycles = 141,040.

21%                    WRC 474 95% Probability Cycles = 195,816.

BS5500 Allowed Cycles(Curve F) = 190,383.

Membrane-to-Bending Ratio = 0.303

Bending-to-PL+PB+Q Ratio = 0.768

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Circ Plate for Plate # 2

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

84     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.109 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

770.9                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 4,542,684.

WRC 474 99% Probability Cycles = 1,055,307.

10%                    WRC 474 95% Probability Cycles = 1,465,161.

BS5500 Allowed Cycles(Curve F) = 1,316,472.

Membrane-to-Bending Ratio = 0.150

Bending-to-PL+PB+Q Ratio = 0.869

03

03

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Circ Plate for Plate # 3

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

153     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.198 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

770.9                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 1,478,924.

WRC 474 99% Probability Cycles = 343,567.

19%                    WRC 474 95% Probability Cycles = 477,000.

BS5500 Allowed Cycles(Curve F) = 258,260.

Membrane-to-Bending Ratio = 0.914

Bending-to-PL+PB+Q Ratio = 0.523

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Circ Plate for Plate # 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

108     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.140 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

770.9                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 4,459,746.

WRC 474 99% Probability Cycles = 1,036,040.

13%                    WRC 474 95% Probability Cycles = 1,438,410.

BS5500 Allowed Cycles(Curve F) = 735,938.

Membrane-to-Bending Ratio = 0.790

Bending-to-PL+PB+Q Ratio = 0.559

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Long Plate for Plate # 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

33     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.042 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

770.9                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 61,483,976.

WRC 474 99% Probability Cycles = 14,283,285.

4%                    WRC 474 95% Probability Cycles = 19,830,544.

BS5500 Allowed Cycles(Curve F) = 26,126,478.

Membrane-to-Bending Ratio = 0.826

Bending-to-PL+PB+Q Ratio = 0.548

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Circ Plate for Plate # 5

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

108     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.205 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0145E9

Allowable                    "B31" Fatigue Stress Allowable = 0.0

526.2                    Markl Fatigue Stress Allowable = 575.0

MPa                    WRC 474 Mean Cycles to Failure = 3,397,071.

WRC 474 99% Probability Cycles = 789,170.

20%                    WRC 474 95% Probability Cycles = 1,095,664.

BS5500 Allowed Cycles(Curve F) = 734,195.

Membrane-to-Bending Ratio = 25.637

Bending-to-PL+PB+Q Ratio = 0.038

03

03

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Table of Contents

Highest Stress Ratios Per Region

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             11:48am  OCT  2,2024

Highest Stress Ratios Per Region                       $X

Circ Plate for Plate # 1

Pl            SPL     Primary Membrane Load Case 2

40            168     Min Prin. Stress = -49. (59% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

23%

Qb            SPS     Primary Bending Load Case 2

99            340     Min Prin. Stress = -49. (59% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

2) Qb < SPS (SUS,Bending) Case 2

28%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

92            340     Min Prin. Stress = -49. (59% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

27%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

120            340     Min Prin. Stress = -49. (59% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

35%

S1+S2+S3             4S     Part 5 (5.3.2) Load Case 2

65            448     Min Prin. Stress = -49. (59% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

5) S1+S2+S3 < 4S (SUS,S1+S2+S3) Case 2

14%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

92            340     Min Prin. Stress = -49. (59% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

27%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

120            340     Min Prin. Stress = -49. (59% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

35%

Membrane           User     Component Evaluation Load Case 3

40            340     Min Prin. Stress = -49. (59% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

8) Membrane < User (OPE,Membrane) Case 3

11%

Bending           User     Component Evaluation Load Case 3

99            340     Min Prin. Stress = -49. (59% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

03

03

9) Bending < User (OPE,Bending) Case 3

28%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 4

89            340     Min Prin. Stress = -33. (61% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

26%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 4

120            340     Min Prin. Stress = -33. (61% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

35%

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

89     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.115 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

771.0                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 5,583,941.

WRC 474 99% Probability Cycles = 1,297,200.

11%                    WRC 474 95% Probability Cycles = 1,800,999.

BS5500 Allowed Cycles(Curve F) = 1,320,867.

Membrane-to-Bending Ratio = 0.385

Bending-to-PL+PB+Q Ratio = 0.722

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

120     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.155 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

771.0                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 1,771,558.

WRC 474 99% Probability Cycles = 411,549.

15%                    WRC 474 95% Probability Cycles = 571,384.

BS5500 Allowed Cycles(Curve F) = 537,311.

Membrane-to-Bending Ratio = 0.385

Bending-to-PL+PB+Q Ratio = 0.722

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Long Plate for Plate # 1

Pl            SPL     Primary Membrane Load Case 2

107            168     Min Prin. Stress = -121. (91% Neg, 29% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

63%

Qb            SPS     Primary Bending Load Case 2

190            340     Min Prin. Stress = -121. (91% Neg, 29% NegHi)

MPa            MPa     Plot Reference:

2) Qb < SPS (SUS,Bending) Case 2

55%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

165            340     Min Prin. Stress = -121. (91% Neg, 29% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

48%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

237            340     Min Prin. Stress = -121. (91% Neg, 29% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

69%

03

03

S1+S2+S3             4S     Part 5 (5.3.2) Load Case 2

124            448     Min Prin. Stress = -121. (91% Neg, 29% NegHi)

MPa            MPa     Plot Reference:

5) S1+S2+S3 < 4S (SUS,S1+S2+S3) Case 2

27%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

165            340     Min Prin. Stress = -121. (91% Neg, 29% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

48%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

237            340     Min Prin. Stress = -121. (91% Neg, 29% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

69%

Membrane           User     Component Evaluation Load Case 3

107            340     Min Prin. Stress = -121. (91% Neg, 29% NegHi)

MPa            MPa     Plot Reference:

8) Membrane < User (OPE,Membrane) Case 3

31%

Bending           User     Component Evaluation Load Case 3

190            340     Min Prin. Stress = -121. (91% Neg, 29% NegHi)

MPa            MPa     Plot Reference:

9) Bending < User (OPE,Bending) Case 3

55%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 4

125            340     Min Prin. Stress = -71. (89% Neg, 24% NegHi)

MPa            MPa     Plot Reference:

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

36%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 4

169            340     Min Prin. Stress = -71. (89% Neg, 24% NegHi)

MPa            MPa     Plot Reference:

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

49%

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

125     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.162 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

771.0                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 1,988,003.

WRC 474 99% Probability Cycles = 461,831.

16%                    WRC 474 95% Probability Cycles = 641,194.

BS5500 Allowed Cycles(Curve F) = 474,054.

Membrane-to-Bending Ratio = 0.303

Bending-to-PL+PB+Q Ratio = 0.768

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

169     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.220 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

771.0                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 607,122.

WRC 474 99% Probability Cycles = 141,040.

21%                    WRC 474 95% Probability Cycles = 195,816.

BS5500 Allowed Cycles(Curve F) = 190,383.

Membrane-to-Bending Ratio = 0.303

Bending-to-PL+PB+Q Ratio = 0.768

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

03

03

Circ Plate for Plate # 2

Pl            SPL     Primary Membrane Load Case 2

38            168     Min Prin. Stress = -28. (59% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

22%

Qb            SPS     Primary Bending Load Case 2

87            340     Min Prin. Stress = -28. (59% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

2) Qb < SPS (SUS,Bending) Case 2

25%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

86            340     Min Prin. Stress = -28. (59% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

25%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

102            340     Min Prin. Stress = -28. (59% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

30%

S1+S2+S3             4S     Part 5 (5.3.2) Load Case 2

63            448     Min Prin. Stress = -28. (59% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

5) S1+S2+S3 < 4S (SUS,S1+S2+S3) Case 2

14%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

86            340     Min Prin. Stress = -28. (59% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

25%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

102            340     Min Prin. Stress = -28. (59% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

30%

Membrane           User     Component Evaluation Load Case 3

38            340     Min Prin. Stress = -28. (59% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

8) Membrane < User (OPE,Membrane) Case 3

11%

Bending           User     Component Evaluation Load Case 3

87            340     Min Prin. Stress = -28. (59% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

9) Bending < User (OPE,Bending) Case 3

25%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 4

84            340     Min Prin. Stress = -31. (57% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

24%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 4

79            340     Min Prin. Stress = -31. (57% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

23%

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

84     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.109 Stress   Strain Concentration Factor = 1.000

03

03

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

770.9                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 4,542,684.

WRC 474 99% Probability Cycles = 1,055,307.

10%                    WRC 474 95% Probability Cycles = 1,465,161.

BS5500 Allowed Cycles(Curve F) = 1,316,472.

Membrane-to-Bending Ratio = 0.150

Bending-to-PL+PB+Q Ratio = 0.869

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

79     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.102 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

770.9                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 5,778,046.

WRC 474 99% Probability Cycles = 1,342,292.

10%                    WRC 474 95% Probability Cycles = 1,863,604.

BS5500 Allowed Cycles(Curve F) = 1,577,694.

Membrane-to-Bending Ratio = 0.108

Bending-to-PL+PB+Q Ratio = 0.903

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Circ Plate for Plate # 3

Pl            SPL     Primary Membrane Load Case 2

143            168     Min Prin. Stress = -192. (77% Neg, 47% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

84%

Qb            SPS     Primary Bending Load Case 2

184            340     Min Prin. Stress = -192. (77% Neg, 47% NegHi)

MPa            MPa     Plot Reference:

2) Qb < SPS (SUS,Bending) Case 2

54%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

271            340     Min Prin. Stress = -192. (77% Neg, 47% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

79%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

159            340     Min Prin. Stress = -192. (77% Neg, 47% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

46%

S1+S2+S3             4S     Part 5 (5.3.2) Load Case 2

148            448     Min Prin. Stress = -192. (77% Neg, 47% NegHi)

MPa            MPa     Plot Reference:

5) S1+S2+S3 < 4S (SUS,S1+S2+S3) Case 2

33%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

271            340     Min Prin. Stress = -192. (77% Neg, 47% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

79%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

159            340     Min Prin. Stress = -192. (77% Neg, 47% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

46%

03

03

Membrane           User     Component Evaluation Load Case 3

143            340     Min Prin. Stress = -192. (77% Neg, 47% NegHi)

MPa            MPa     Plot Reference:

8) Membrane < User (OPE,Membrane) Case 3

41%

Bending           User     Component Evaluation Load Case 3

184            340     Min Prin. Stress = -192. (77% Neg, 47% NegHi)

MPa            MPa     Plot Reference:

9) Bending < User (OPE,Bending) Case 3

54%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 4

153            340     Min Prin. Stress = -119. (99% Neg, 68% NegHi)

MPa            MPa     Plot Reference:

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

44%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 4

103            340     Min Prin. Stress = -119. (99% Neg, 68% NegHi)

MPa            MPa     Plot Reference:

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

30%

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

153     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.198 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

770.9                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 1,478,924.

WRC 474 99% Probability Cycles = 343,567.

19%                    WRC 474 95% Probability Cycles = 477,000.

BS5500 Allowed Cycles(Curve F) = 258,260.

Membrane-to-Bending Ratio = 0.914

Bending-to-PL+PB+Q Ratio = 0.523

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

103     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.134 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

770.9                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 4,763,216.

WRC 474 99% Probability Cycles = 1,106,538.

13%                    WRC 474 95% Probability Cycles = 1,536,289.

BS5500 Allowed Cycles(Curve F) = 837,188.

Membrane-to-Bending Ratio = 13.210

Bending-to-PL+PB+Q Ratio = 0.070

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Circ Plate for Plate # 4

Pl            SPL     Primary Membrane Load Case 2

121            168     Min Prin. Stress = -175. (68% Neg, 22% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

72%

Qb            SPS     Primary Bending Load Case 2

128            340     Min Prin. Stress = -175. (68% Neg, 22% NegHi)

MPa            MPa     Plot Reference:

2) Qb < SPS (SUS,Bending) Case 2

37%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

210            340     Min Prin. Stress = -175. (68% Neg, 22% NegHi)

MPa            MPa     Plot Reference:

03

03

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

61%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

149            340     Min Prin. Stress = -175. (68% Neg, 22% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

43%

S1+S2+S3             4S     Part 5 (5.3.2) Load Case 2

151            448     Min Prin. Stress = -175. (68% Neg, 22% NegHi)

MPa            MPa     Plot Reference:

5) S1+S2+S3 < 4S (SUS,S1+S2+S3) Case 2

33%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

210            340     Min Prin. Stress = -175. (68% Neg, 22% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

61%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

149            340     Min Prin. Stress = -175. (68% Neg, 22% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

43%

Membrane           User     Component Evaluation Load Case 3

121            340     Min Prin. Stress = -175. (68% Neg, 22% NegHi)

MPa            MPa     Plot Reference:

8) Membrane < User (OPE,Membrane) Case 3

35%

Bending           User     Component Evaluation Load Case 3

128            340     Min Prin. Stress = -175. (68% Neg, 22% NegHi)

MPa            MPa     Plot Reference:

9) Bending < User (OPE,Bending) Case 3

37%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 4

108            340     Min Prin. Stress = -73. (78% Neg, 22% NegHi)

MPa            MPa     Plot Reference:

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

31%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 4

106            340     Min Prin. Stress = -73. (78% Neg, 22% NegHi)

MPa            MPa     Plot Reference:

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

31%

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

108     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.140 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

770.9                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 4,459,746.

WRC 474 99% Probability Cycles = 1,036,040.

13%                    WRC 474 95% Probability Cycles = 1,438,410.

BS5500 Allowed Cycles(Curve F) = 735,938.

Membrane-to-Bending Ratio = 0.790

Bending-to-PL+PB+Q Ratio = 0.559

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

106     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.138 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

770.9                    Markl Fatigue Stress Allowable = 659.5

03

03

MPa                    WRC 474 Mean Cycles to Failure = 4,608,486.

WRC 474 99% Probability Cycles = 1,070,593.

13%                    WRC 474 95% Probability Cycles = 1,486,384.

BS5500 Allowed Cycles(Curve F) = 770,017.

Membrane-to-Bending Ratio = 1.090

Bending-to-PL+PB+Q Ratio = 0.478

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Long Plate for Plate # 4

Pl            SPL     Primary Membrane Load Case 2

27            168     Min Prin. Stress = -17. (75% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

15%

Qb            SPS     Primary Bending Load Case 2

37            340     Min Prin. Stress = -17. (75% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

2) Qb < SPS (SUS,Bending) Case 2

10%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

38            340     Min Prin. Stress = -17. (75% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

11%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

47            340     Min Prin. Stress = -17. (75% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

13%

S1+S2+S3             4S     Part 5 (5.3.2) Load Case 2

21            448     Min Prin. Stress = -17. (75% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

5) S1+S2+S3 < 4S (SUS,S1+S2+S3) Case 2

4%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

38            340     Min Prin. Stress = -17. (75% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

11%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

47            340     Min Prin. Stress = -17. (75% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

13%

Membrane           User     Component Evaluation Load Case 3

27            340     Min Prin. Stress = -17. (75% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

8) Membrane < User (OPE,Membrane) Case 3

7%

Bending           User     Component Evaluation Load Case 3

37            340     Min Prin. Stress = -17. (75% Neg, 4% NegHi)

MPa            MPa     Plot Reference:

9) Bending < User (OPE,Bending) Case 3

10%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 4

33            340     Min Prin. Stress = -14. (88% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

9%

03

03

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 4

20            340     Min Prin. Stress = -14. (88% Neg, 5% NegHi)

MPa            MPa     Plot Reference:

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

5%

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

33     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.042 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

770.9                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 61,483,976.

WRC 474 99% Probability Cycles = 14,283,285.

4%                    WRC 474 95% Probability Cycles = 19,830,544.

BS5500 Allowed Cycles(Curve F) = 26,126,478.

Membrane-to-Bending Ratio = 0.826

Bending-to-PL+PB+Q Ratio = 0.548

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

20     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.026 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0000E11

Allowable                    "B31" Fatigue Stress Allowable = 567.5

770.9                    Markl Fatigue Stress Allowable = 659.5

MPa                    WRC 474 Mean Cycles to Failure = 3.3834E8

WRC 474 99% Probability Cycles = 78,598,464.

2%                    WRC 474 95% Probability Cycles = 1.0912E8

BS5500 Allowed Cycles(Curve F) = 2.9743E8

Membrane-to-Bending Ratio = 0.854

Bending-to-PL+PB+Q Ratio = 0.539

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Circ Plate for Plate # 5

Pl            SPL     Primary Membrane Load Case 2

104            126     Min Prin. Stress = -7. (10% Neg, 3% NegHi)

MPa            MPa     Plot Reference:

1) Pl < SPL (SUS,Membrane) Case 2

82%

Qb            SPS     Primary Bending Load Case 2

37            298     Min Prin. Stress = -7. (10% Neg, 3% NegHi)

MPa            MPa     Plot Reference:

2) Qb < SPS (SUS,Bending) Case 2

12%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 2

107            298     Min Prin. Stress = -7. (10% Neg, 3% NegHi)

MPa            MPa     Plot Reference:

3) Pl+Pb+Q < SPS (SUS,Inside) Case 2

35%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 2

115            298     Min Prin. Stress = -7. (10% Neg, 3% NegHi)

MPa            MPa     Plot Reference:

4) Pl+Pb+Q < SPS (SUS,Outside) Case 2

38%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 3

107            298     Min Prin. Stress = -7. (10% Neg, 3% NegHi)

MPa            MPa     Plot Reference:

6) Pl+Pb+Q < SPS (OPE,Inside) Case 3

35%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 3

115            298     Min Prin. Stress = -7. (10% Neg, 3% NegHi)

MPa            MPa     Plot Reference:

03

03

7) Pl+Pb+Q < SPS (OPE,Outside) Case 3

38%

Membrane           User     Component Evaluation Load Case 3

104            298     Min Prin. Stress = -7. (10% Neg, 3% NegHi)

MPa            MPa     Plot Reference:

8) Membrane < User (OPE,Membrane) Case 3

34%

Bending           User     Component Evaluation Load Case 3

37            298     Min Prin. Stress = -7. (10% Neg, 3% NegHi)

MPa            MPa     Plot Reference:

9) Bending < User (OPE,Bending) Case 3

12%

Pl+Pb+Q            SPS     Primary+Secondary (Inner) Load Case 4

100            298     Min Prin. Stress = -5. ( 8% Neg, 1% NegHi)

MPa            MPa     Plot Reference:

10) Pl+Pb+Q < SPS (EXP,Inside) Case 4

33%

Pl+Pb+Q            SPS     Primary+Secondary (Outer) Load Case 4

108            298     Min Prin. Stress = -5. ( 8% Neg, 1% NegHi)

MPa            MPa     Plot Reference:

11) Pl+Pb+Q < SPS (EXP,Outside) Case 4

36%

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Inner) Load Case 4

100     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.191 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 4.8949E9

Allowable                    "B31" Fatigue Stress Allowable = 0.0

526.2                    Markl Fatigue Stress Allowable = 575.0

MPa                    WRC 474 Mean Cycles to Failure = 4,277,923.

WRC 474 99% Probability Cycles = 993,800.

19%                    WRC 474 95% Probability Cycles = 1,379,767.

BS5500 Allowed Cycles(Curve F) = 915,978.

Membrane-to-Bending Ratio = 25.639

Bending-to-PL+PB+Q Ratio = 0.038

Plot Reference:

12) Pl+Pb+Q+F < 2Sa (EXP,Inside) Case 4

Pl+Pb+Q+F    Damage Ratio    Primary+Secondary+Peak (Outer) Load Case 4

108     0.000 Life     Stress Concentration Factor = 1.000

MPa     0.205 Stress   Strain Concentration Factor = 1.000

Cycles Allowed for this Stress = 1.0145E9

Allowable                    "B31" Fatigue Stress Allowable = 0.0

526.2                    Markl Fatigue Stress Allowable = 575.0

MPa                    WRC 474 Mean Cycles to Failure = 3,397,071.

WRC 474 99% Probability Cycles = 789,170.

20%                    WRC 474 95% Probability Cycles = 1,095,664.

BS5500 Allowed Cycles(Curve F) = 734,195.

Membrane-to-Bending Ratio = 25.637

Bending-to-PL+PB+Q Ratio = 0.038

Plot Reference:

13) Pl+Pb+Q+F < 2Sa (EXP,Outside) Case 4

Table of Contents

Compressive Stress Summary

FEPipe Version 15.0           Jobname: setup2                 $P

Released Jan. 2021             11:48am  OCT  2,2024

Compressive Stress Summary (MPa)                       $X

03

03

Nomenclature:

-------------

Min Stress       - Compressive Membrane and Bending Stress

Pts in Region    - No. of nodes in the model region

>5% Compression  - 5% or more of Compressive Stress Limit

>50% Compression - 50% or more of Compressive Stress Limit

Compressive Stress Limit = -0.55 Min(Sy, kEt/R), Section

slenderness ratio (elastic buckling) not considered.

>5%    >50%

Min        Pts      Compression

#  Load Type  Case    Stress    in Region   and Bending    Region

--  ---------  ----    ------   ----------  ------------  ----------------

1  SUSTAINED    2        -49.      3024      59%     5%  Circ Plate for Plate # 1

2  OPERATING    3        -49.      3024      59%     5%  Circ Plate for Plate # 1

3  EXPANSION    4        -33.      3024      61%     5%  Circ Plate for Plate # 1

4  SUSTAINED    2       -121.     13176      91%    29%  Long Plate for Plate # 1

5  OPERATING    3       -121.     13176      91%    29%  Long Plate for Plate # 1

6  EXPANSION    4        -71.     13176      89%    24%  Long Plate for Plate # 1

7  SUSTAINED    2        -28.      2112      59%     4%  Circ Plate for Plate # 2

8  OPERATING    3        -28.      2112      59%     4%  Circ Plate for Plate # 2

9  EXPANSION    4        -31.      2112      57%     4%  Circ Plate for Plate # 2

10  SUSTAINED    2       -192.      1600      77%    47%  Circ Plate for Plate # 3

11  OPERATING    3       -192.      1600      77%    47%  Circ Plate for Plate # 3

12  EXPANSION    4       -119.      1600      99%    68%  Circ Plate for Plate # 3

13  SUSTAINED    2       -175.      1440      68%    22%  Circ Plate for Plate # 4

14  OPERATING    3       -175.      1440      68%    22%  Circ Plate for Plate # 4

15  EXPANSION    4        -73.      1440      78%    22%  Circ Plate for Plate # 4

16  SUSTAINED    2        -17.       640      75%     4%  Long Plate for Plate # 4

17  OPERATING    3        -17.       640      75%     4%  Long Plate for Plate # 4

18  EXPANSION    4        -14.       640      88%     5%  Long Plate for Plate # 4

19  SUSTAINED    2         -7.     13376      10%     3%  Circ Plate for Plate # 5

20  OPERATING    3         -7.     13376      10%     3%  Circ Plate for Plate # 5

21  EXPANSION    4         -5.     13376       8%     1%  Circ Plate for Plate # 5

Table of Contents

03

03

03

03

03

03

03

03

03

03

03

03

03

03